THE CP/M VERSION OF THE CORVUS DISC

DIAGNOSTIC ( CDIAGNOS.COM ) PROVIDES

A SOME RELATIVELY "SAFE" DISC TESTS

ALONG WITH THE ABILITY TO LIST AND/OR

CHANGE SOME PARAMETERS WITHIN THE

CONTROLLER CODE.  THE FUNCTIONS

AVAILABLE IN THIS VERSION ARE:

1.  DISC FORMAT CHECK

THE CONTROLLER TRYS TO READ EACH

512 BYTE SECTOR TO VERIFY THAT IT

IS "GOOD" (HAS A CORRECT CRC).  IF

IT GETS A BAD CRC AFTER TWENTY READ

ATTEMPTS, IT WILL RE-WRITE THE

SECTOR TO RESET THE CRC.  THIS

USUALLY TAKES  1-2  MINUTES.  NOTE:

ON  REV A  CONTROLLERS, THIS FUNCTION

IS NOT AVAILABLE WITH   VERSION 0

CONTROLLER CODE.

2.  READ THE CONTROLLER CODE VERSION #

THIS OPTION WILL RETURN SOME

INFORMATION ABOUT THE CONTROLLER

CODE ON YOUR DRIVE.  REV B & H DRIVES

CAN RETURN MORE DETAILED INFORMATION

THAN  REV A  DRIVES.

3.  HEAD SERVO TEST

THIS TEST ALTERNATELY READS  128  BYTE

SECTORS AT  DISC ADDRESS  0  AND

1000 SECTORS IN FROM THE MAXIMUM.  THIS

TEST IS MAINLY USEFUL TO VERIFY THAT

THE DRIVE WILL ACTUALLY READ DATA AND

MOVE ITS HEAD.

4.  MANUAL CONTROLLER DIAGNOSTIC

THIS OPTION ALLOWS ONE TO READ OR WRITE

DIRECTLY TO THE CORVUS CONTROLLER I/O

PORTS AS WELL AS READ THE STATUS LINES.

IT IS MAINLY USEFUL FOR CORVUS SERVICE

PERSONEL WHEN ATTEMPTING TO DIAGNOS

A DRIVE PROBLEM OVER THE PHONE.

-------------- WARNING ---------------

THIS TEST CAN CAUSE THE USER DATA ON

THE DISC TO BE OVERWRITTEN.  IF USED ON

A  REV A  DRIVE WITH THE FORMAT JUMPER

IN PLACE, IT CAN EVEN CAUSE THE DISC TO

BE REFORMATED.

--------------------------------------

5.  UPDATE CONTROLLER CODE

THIS OPTION ALLOWS ONE TO UPDATE OR

INITIALLY INSTALL CONTROLLER CODE ON THE

CORVUS DRIVE.  THIS CODE RESIDES ON

PROTECTED TRACKS OF THE HARD DISC.

NORMALLY THIS CODE CANNOT BE WRITTEN TO

OR READ BY THE USER (EVEN ACCIDENTALLY).

HOWEVER, IT CAN BE MADE ACCESSABLE (TO

WRITING) BY TURNING ON THE FORMAT SWITCH

(OR BY ADDING A JUMPER TO THE DRIVE

BACKPLANE).  THIS OPTION REPLACES THE

SEPARATE PROGRAM: CCODE.COM  THAT HAD

SIMILAR CAPABILITIES.

-------------- WARNING ----------------

ON  REV A  DRIVES, DO NOT ATTEMPT TO

WRITE THE CODE OUT TO THE DRIVE WITHOUT

TURNING ON THE FORMAT SWITCH (OR INSTALLING

THE FORMAT JUMPER).  IF YOU DO, IT WILL BE

WRITTEN OUT TO THE USER AREA OF THE DISC

- OVERLAYING POSSIBLY VALUABLE USER PROGRAMS

OR DATA!!

---------------------------------------

TO USE THIS OPTION:

A.  POWER THE DRIVE DOWN.  IF YOUR DRIVE

HAS A  FORMAT SWITCH (UNDER THE FRONT

BEZEL OR ON THE BACK OF THE DRIVE),

TURN IT ON AND PROCEED TO STEP D.

B.  ON DRIVES WITHOUT A FORMAT SWITCH,

REMOVE THE PLASTIC COVER OVER THE

BACKPLANE PINS (ON THE BACK OF THE

DRIVE WHERE THE COMPUTER CABLE IS

ATTACHED.

C.  CONNECT A JUMPER BETWEEN PINS:

D37 & D38  (FOR REV A DRIVES)

C2  & C12  (FOR REV B DRIVES 10MB & 20MB)

AS ILLUSTRATED BELOW:

!* * * * * * * * * * * * * * * *

C  !2      10        20        30

!X * * * * X * * * * * * * * * *

XXXXXXXXXXX  <--- C2-C12 JUMPERED

!* * * * * ~ ~ ~ ~ * * * * X * *

D  !2      10        30       X40

!* * * * * ~ ~ ~ ~ * * * * X * *

^

^

D37-D38 JUMPERED

D.  POWER THE DRIVE BACK UP.

E.  RUN THIS PROGRAM FROM YOUR FLOPPY BASED

CP/M AND SELECT THE UPDATE OPTION.

F.  AFTER THE CODE IS WRITTEN OUT, TURN

OFF THE FORMAT SWITCH (OR REMOVE THE

JUMPER) AND RESET THE DRIVE BY EITHER

USING THE RESET SWITCH OR BY POWERING

THE DRIVE DOWN AND UP AGAIN.

6.  LIST/MODIFY DRIVE PARAMETERS

THIS OPTION ALLOW YOU TO EXAMINE

AND/OR CHANGE THE:

A.  THE DRIVE SPARE TRACK TABLE

THIS TABLE SPECIFIES PHYSICAL

TRACK NUMBERS WHICH ARE TO BE

SKIPPED OVER WHEN THE CONTROLLER

ACCESSES DATA ON THE DRIVE.

THIS ALLOWS ONE TO HIDE THE

EXISTANCE OF MEDIA DEFECTS

FROM THE EXTERNAL USER (THE

DATA SURFACE WILL LOOK PERFECT

TO HIM).

B.  THE SECTOR INTERLEAVING SPEC

THE DEFAULTS USED HERE HAVE BEEN

OPTIMIZED FOR SMALL COMPUTERS

LIKE  APPLE-II'S.  TYPICALLY,WE

HAVE SELECTED A VALUE OF:

9 - FOR 10MB AND 20 MB REV B DRIVES

OR FOR ANY REV H DRIVE.

12 - FOR 5MB REV B DRIVES

C.  THE VIRTUAL DRIVE OFFSET TABLE

THIS TABLE ALLOW ONE DRIVE TO BEHAVE

AS IF IT WERE SEVERAL SMALLER DRIVES

DAISEY-CHAINED TOGATHER.  THE TABLE

ENTRYS SPECIFY THE TRACK NUMBER AT

WHICH THESE VIRTUAL DRIVES START.

NORMALLY, ONE WOULD NOT HAVE OCCASION

TO DISTURB THESE PARAMETERS UNLESS

THE DRIVE HAS BEEN RE-FORMATTED OR A

BAD TRACK DEVELOPES.

7.  LIST/MODIFY CONSTELLATION PARAMETERS

THIS OPTION ALLOW ONE TO EXAMINE AND

/OR MODIFY THE:

A.  MASTER MULTIPLEXER CONNECTION TABLE

(I.E.- SPECIFY WHAT IS CONNECTED TO IT)

THIS TABLE IS USUALLY SET TO ASSUME

ALL THE SLOTS HAVE A MUX ON THEM.

B.  CONSTELLATION POLLING CONSTANTS

THESE NUMBERS DETERMINE CERTAIN

ASPECTS OF THE CONSTELLATION

POLLING ENVIRONMENT.

8.  PARK HEADS OF REV H DRIVE FOR SHIPMENT. THIS

OPTION MOVES THE DRIVE HEAD OUT TO A SPECIAL

"LANDING ZONE" NEAR THE CENTER OF THE DISK

HUB AND DISABLES THE DRIVE (IT MUST BE

"RESET" TO RE-ENABLE IT).  IT IS RECOMMENDED

HTAT YOU USE THIS OPTION BEFORE TRANSPORTING

THE DRIVE UNIT - TO AVOID "POSSIBLE" DAMAGE

TO DATA ON THE DISK.

###############################################################################################################