/----------------------------------------------------------------------------\

| 									     |

|		      ---------------------------- 			     |

|		      IRWIN MAGNETIC SYSTEMS, INC.			     |

|		      ----------------------------			     |

|									     |

|   File  : TFORMAT.DOC					                     |

|									     |

|   Title : Notes on the Tape Servo/Format Program (1.48)                    |

|									     |

|   By    : Applications Engineering Dept.                                   |

|									     |

|   Date  : March 11, 1985           				             |

|									     |

|									     |

\----------------------------------------------------------------------------/

----------------------------

|         Purpose          |--------------------------------------------------

----------------------------

This program servo writes and/or formats tape cartridges for use in the Irwin

Magnetics tape drive. This program can also display a cartridge's format

parameters, verify a formatted cartridge, or label a formatted cartridge.

----------------------------

| Operational Instructions |--------------------------------------------------

----------------------------

TFORMAT is a menu-driven program with the following menu options:

(1)	START SERVO WRITE AND EXIT.

The tape drive can servo write independent of the computer. Servo

writing takes approximately 18 minutes.

(2)	SERVO WRITE AND WAIT FOR COMPLETION.

The computer monitors drive status and will note any abnormal

completion.

(3)	SERVO WRITE AND THEN FORMAT CARTRIDGE.

Starting with a blank tape, the entire procedure takes approximately

34 minutes.

(4)	FORMAT CARTRIDGE ONLY.

Starting with a servo written cartridge, formatting takes

approximately 16 minutes (8 for formatting and 8 for verifying). Tape

parameters and a bad block map are written on the tape's leading

block.

(5)	DISPLAY CARTRIDGE FORMAT PARAMETERS.

The tape parameters and bad block data contained in the leading block

of any formatted cartridge are read into memory and displayed.

(6)	REVERIFY A FORMATTED CARTRIDGE.

Information about new badblocks will be displayed.

If the cartridge status is UNUSED, the bad block map will be rewritten

to reflect any new bad blocks.

(7)	ENTER VOLUME NAME.

A volume name can be written on any formatted cartridge.  If a name

already exists, a choice is given to preserve the old name or write a

new name.  The name can have any combination of 12 characters or

digits (the leading character cannot be blank!).

(8)	EXIT PROGRAM.

Exit to DOS.

----------------------------

| Error and Info. Messages |--------------------------------------------------

----------------------------

message:		 Servo write terminated abnormally

probable cause:	 	 at beginning of servo write this indicates an

edge-of-tape or write mechanism problem -- during

servo write this indicates a tape problem

action:			 return to menu

driver function:	 tp0comm(ISNORMAL)

error code:		 non-zero

TFORMAT function: 	 case: SERV_WAIT, SERV_FORM

------------------------------------------------------------------------------

message:		 FORMAT BUFFER ERROR

probable cause:		 problem with data transfer, floppy controller, or DMA

during format buffer fill routine

action:			 return to menu

driver function:	 tp0frmfl()

error code:		 non-zero

TFORMAT function:	 format()

------------------------------------------------------------------------------

message:		 SEEK TRACK ERROR

probable cause:		 problem with floppy controller during seek

action:			 return to menu

driver function:	 tp0comm(seek)

error code:		 non-zero

TFORMAT function:	 format()

------------------------------------------------------------------------------

message:		 FORMAT ERROR - one retry

probable cause:		 problem with data transfer, floppy controller, or DMA

during format routine

action:			 go to start of track

driver function:	 tp0frmt()

error code:		 non-zero

TFORMAT function:	 format()

------------------------------------------------------------------------------

message:		 (RE)VERIFY ERROR - retrying

probable cause:		 DMA problem or ID read problems in read during verify

action:			 call retry(), retry block up to 4 times.

driver function:	 tp0red()

error code:		 any non-zero but 6

TFORMAT function:	 reverify(), verify()

------------------------------------------------------------------------------

message:		 (RE)VERIFY FAILURE

REFORMAT CARTRIDGE

probable cause:		 controller problems (timeouts) in read during verify

action:			 return to menu

driver function:	 tp0red()

error code:		 any non-zero but 6

TFORMAT function:	 verify(), reverify()

------------------------------------------------------------------------------

message:		 FAILURE - CANNOT LOCATE BLOCK

probable cause:		 controller problems (id or timeouts) in read during

retry

action:			 return to menu

driver function:	 tp0id()

error code:		 any non-zero but 6

TFORMAT function:	 locate()

------------------------------------------------------------------------------

message:		 BAD FIRST BLOCK.  REFORMAT CARTRIDGE.

probable cause: 	 tape problem

action:			 return to menu

driver function:	 tp0red()

error code:		 6 (crc error for block 0)

TFORMAT function:	 verify(), reverify()

------------------------------------------------------------------------------

message:		 WRITE HEADER BLOCK ERROR

probable cause:		 problem with data transfer, floppy controller, or DMA

during write routine

action:			 return to menu

driver function:	 tp0wrt()

error code:		 non-zero

TFORMAT function:	 plot(), outmap()

------------------------------------------------------------------------------

message:		 READ HEADER BLOCK ERROR

probable cause:		 problem with data transfer, floppy controller, or DMA

during read routine

action:			 return to menu

driver function:	 tp0red()

error code:		 non-zero

TFORMAT function:	 readmap(), inmap(), plot()

------------------------------------------------------------------------------

TP0COMM return codes

0:     O.K.

1:     Command not accepted

2:	Receive timeout

3:	Send timeout

4:	Bad NEC chip

5:	Record not found

6:	CRC error

7:	DMA error

8:	Write protect

9:	ID mark error

10:	Interrupt timeout

11:	DMA boundary error

------------------------------------------------------------------------------

Call:	TFORMAT			Program executes with minimal error messages.

Call:	TFORMAT info		Command line parameter causes program

execution with full error messages.

------------------------------------------------------------------------------

##########################################################################