Getting CP/M Performance That You Want!!              By Michael J. Karas

-------------------------------------------------------------------------

The CP/M operating system, as a development environment, has got

to be one of the best things to happen to microcomputers. But once in a

while one finds a feature of CP/M or one of its utilities that you wished

would work "just a little different". I have been using CP/M for quite a

long time now and have just found a way to correct one of the annoying

little features of PIP that is sometimes kind of a pain. When using PIP

to transmit a file to the printer (LST: or LPT:) it is usually nice to

utilize the [Pnn] option to set the page size to some number of lines so

that ones listing does not go over the perforation line. When the [Pnn]

option is selected with PIP the utility does the infamous "initial page

eject". Under most situations of single file printouts, when the printer

device is already at TOF (Top of Form), the initial page eject wastes

a whole sheet of paper. Now many of you may say, what is one sheet of

paper? Well, at todays paper costs it is worth about 1.2 cents. In another

light, if you do a lot of small printouts, one at a time, while making

a small program that fits on less than three sheets of paper, then the

eject option wastes about 30% of the paper that you use. I think that is

a terrible thing and went about doing something about it.

The answer is the small patch program for PIP that enables PIP

to not do the first page eject if the [Pnn] option has been selected

in the command. This patch was made specifically for PIP Version 1.4,

(the version delivered with most later copies of CP/M 1.4), by a lengthy

tracing operation with DDT and use of a find byte routine in my monitor.

If you want to stop waste of paper and valuable trees, do so by typing

in the following program, assembling it, and making a waste proof copy

PIP using DDT. Full, annotated directions are given as part of the

comments section of the listing.

Use of this patch has not seemed to cause any kind of PIP

performance degradation when I tested it. Note also that the patch

is structured so that normal formfeeds in the file proper are still

put out to the destination device, unless of course you select the

filter formfeeds option. In addition, the first form feed is all

that is suppressed. Subsequent formfeeds that occur after "nn" lines

of the [Pnn] PIP command are still allowed to be outputted to the

destination device.

Implementation of this patch for the older PIP version 1.3

that some CP/M 1.4 users have or the new PIP delivered with CP/M 2.x

may be possible, but beware! This implementation is based upon direct

intrepretation of the PIP Version 1.4 object code, as sold by Digital

Research Inc., and other versons of PIP are probably not quite the

same as version 1.4. The concept of this patch will in all likelihood

work with other PIP versions if you can locate the part of the program

that:

a) outputs the formfeed characters upon max line count, and

b) find the location of the page option specified flag.

One other implementation note is that if your PIP supports the internal

drivers for the INP: and OUT: devices, then you will have to inspect

the PIP object code, with DDT, to see if there is room for the patch

at an address other than 0150H within the user patch area. Note that

PIP has about 256 bytes free for the INP: and OUT: routines and most

likely you can find room for both the user drivers and this patch

program.

;

;

;ROUTINE TO PATCH PIP.COM VERSION 1.4 WITHIN THE USER INPUT/OUTPUT

;AREA TO DELETE THE INITIAL PAGE EJECT IF THE [Pnn] OPTION IS

;SELECTED IN THE PIP COMMAND OPTIONS FIELD.

;

; 	THIS PATCH PATCHES THE PIP PROGRAM WHERE THE OUTPUT

;	OF A FORMFEED CHARACTER WAS FOUND. IN ORDER TO VERIFY

;	THAT THE "P" OPTION WAS SELECTED, A PIP PARAMETER FIELD

;	WITHIN THE PROGRAM OPERAND DATA AREA IS SET TO "00H" IF

;	THE OPTION WAS NOT SELECTED AND TO THE SELECTED PAGE SIZE

;	IF THE OPTION WAS SELECTED. THIS PROGRAM PATCH LOOKS FOR

;	A NON ZERO VALUE HERE TO DETERMINE IF THE OPTION HAS BEEN

;	SELECTED. AN INTERNAL FLAG WITHIN THE PATCH AREA IS

;	INITIALLY SET TO "0FFH". IF THE "P" OPTION WAS NOT SELECTED

;	OR THE INITIAL FORMFEED EJECT HAS BEEN DONE, THEN THIS

;	FLAG IS RESET TO "00H" TO DISABLE FURTHER ACTION OF THE

;	PATCH.

;

;	INSTALL THIS ROUTINE AS FOLLOWS:

;

;	1) EDIT IN THE FOLLOWING PROGRAM EXACTLY AS GIVEN BELOW.

;	   GIVE THE SOURCE FILE THE NAME "PIPPATCH.ASM".

;

;	2) FROM NOW ON THE DIRECTIONS WILL ASSUME THAT THE SYSTEM

;	   IS LOGGED ONTO DRIVE A: AND ALL FILES ARE PRESENT ON A:.

;	   IF YOU HAVE ANY OF THE FILES ON OTHER DRIVES, THEN CHANGE

;	   THE FOLLOWING COMMANDS TO INDICATE THE DRIVES WHERE YOU

;	   HAVE THE APPROPIATE FILES.

;

;	3) ASSEMBLE USING THE CP/M 1.4 ASSEMBLER WITH THE FOLLOWING

;	   COMMAND:

;		A>ASM PIPPATCH<cr>

;

;	4) LOAD THE PRESENT COPY OF PIP.COM WITH DDT AS FOLLOWS:

;	   (NOTE THAT I HAVE SHOWN ALL CP/M SYSTEM OUTPUT AS WELL

;	   AS THE STUFF YOUR GOING TO TYPE.)

;		A>DDT PIP.COM<cr>

;		DDT VERS 1.4

;		NEXT  PC

;		1C80 0100

;		-IPIPPATCH.HEX<cr>

;		-R<cr>

;		NEXT  PC

;		1C80 0100

;		-^C

;		A>

;

;	5) SAVE THE PATCHED VERSION OF PIP AS "PIPP.COM" WITH THE

;	   FOLLOWING COMMAND:

;		A>SAVE 28 PIPP.COM<cr>

;

;	6) YOU NOW HAVE A PIP THAT WILL NOT DO THE INITIAL PAGE EJECT

;	   THAT WASTES PAPER IF IF YOU HAVE SELECTED THE "P" OPTION.

;

;

;----------------------------------------------------------------------

;

;

;SETUP THE NECESSARY PATCH PARAMETERS

;

PATCH	EQU	0150H		;START OF PATCH AREA

CALLFF	EQU	0DFDH		;PIP VER 1.4 FORM FEED OUTPUT CALL

;..LOCATION. PATCHED TO JMP TO PATCH

PAGOPT	EQU	1D8FH		;LOCATION OF PIP PAGE OPTION SELECT

OUTCHAR EQU	0C74H		;ADDRESS OF CALL IN PIP THAT IS

;..PATCHED OVER

;

;

;SET START OF PATCH AREA WITHIN PIP USER I/O SPACE FOR THIS PATCH

;TO DELETE INITIAL FORMFEED

;

ORG	PATCH		;SET START OF PATCH

;

PUSH	PSW		;SAVE SO WE DONT BOMB PIP

;

LDA	PAGOPT		;SEE IF PAGE OPTION WAS SELECTED

ORA	A		;ZERO IF NOT SELECTED

JZ	NORMAL		;IF NO PAGING SKIP EJECT SUPPRESSION

;

LDA	SKIPFLG		;GET OUR SKIP FLAG AND TEST FOR

;..IF THIS IS FIRST FORM FEED

ORA	A		;IF ZERO THEN THIS WAS NOT FIRST PASS

;..THROUGH THIS ROUTINE

JZ	NORMAL

;

XRA	A		;CLEAR SKIP FLAG AS THIS IS FIRST

;..TIME THROUGH HERE WITH "P" OPTION

STA	SKIPFLG

POP	PSW

JMP	CALLFF+3	;EXIT WITHOUT NORMAL PIP FORMFEED CALL

;

NORMAL:

POP	PSW

CALL	OUTCHAR		;DO OUTPUT LIKE PIP USUALLY DOES IT

JMP	CALLFF+3	;RETURN TO NORMAL PIP INSTRUCTIONS

;

;

;SKIP FLAG SET TO "FFH" WITH LOAD OF PIP. USED TO SEE WHEN WE HAVE

;BYPASSED FIRST PAGE EJECT WITH "P" OPTION ENABLED

;

SKIPFLG:

DB	0FFH

;

;

;SET INSTRUCTION WITH WHICH TO PATCH PIP

;

ORG	CALLFF		;LOCATION FOR INTERNAL PATCH

JMP	PATCH		;INSTALL JUMP TO OUR PATCH

;

;

END

;

;*********************************************************************

###########################################################################################