CORVUS DISC ERROR CODES

THE CORVUS CONTROLLER HAS A NUMBER OF ERROR CODES

THAT MAY BE ISSUED IF EITHER  AN ILLEGAL COMMAND

IS GIVEN, OR THE CONTROLLER IS OUT OF SYNCHRONIZATION,

OR THERE IS A HARDWARE MALFUNCTION.  A NUMBER OF THE

UTILITIES AND DISC INTERFACE PROGRAMS CAN LIST THESE

ERROR CODES (IN HEX) IF SUCH AN ERROR OCCURES.  FOR

EXAMPLE,  PUTGET.COM  LIST THE CODE AS:

** DISC R/W ERROR # XXH **

WHERE XX IS THE ERROR CODE.  YOU CAN DEMONSTRATE THIS

BY TRYING TO READ A SECTOR (WITH PUTGET) FROM DRIVE 4

(UNLESS YOU HAVE FOUR DRIVES).  THIS WILL GIVE ERROR

CODE: A7H.  THE UPPER 3 BITS OF THE ERROR CODE HAVE

THE FOLLOWING SIGNIFICANCE:

BIT 5 :  SET IF THERE WAS A RECOVERABLE ERROR

(AS IN A RE-TRY ON READ OR WRITE).

BIT 6 :  SET IF AN ERROR OCCURED ON A RE-READ

(VERIFICATION) FOLLOWING A DISC WRITE.

BIT 7 :  SET IF ANY FATAL ERROR HAS OCCURED.

NOTE: MOST OF THE PROGRAMS WILL NOT

LIST THE ERROR UNLESS BIT 7 IS SET.

THE LOWER 5 BITS HAVE THE FOLLOWING SIGNIFICANCE:

BITS 4-0		MEANING

--------	-----------------------------

0		DISC HEADER FAULT

1		SEEK TIMEOUT

2		SEEK FAULT

3		SEEK ERROR

4		HEADER CRC ERROR

5		RE-ZERO (HEAD) FAULT

6		RE-ZERO TIMEOUT

7		DRIVE NOT ON LINE

8		WRITE FAULT

9		- - - - - -

A		READ DATA FAULT

B		DATA CRC ERROR

C		SECTOR LOCATE ERROR

D		WRITE PROTECTED

E		ILLEGAL SECTOR ADDRESS

F		ILLEGAL COMMAND

10		DRIVE NOT ACKNOWLEDGED

11		ACKNOWLEDGE STUCK ACTIVE

12		TIMEOUT

13		FAULT

14		CRC

15		SEEK

16		VERIFICATION

17		DRIVE SPEED ERROR

18		DRIVE ILLEGAL ADDRESS ERROR

19		DRIVE R/W FAULT ERROR

1A		DRIVE SERVO ERROR

1B		DRIVE GUARD BAND

1C		DRIVE PLO (PHASE LOCK) ERROR

1D		DRIVE R/W UNSAFE

##################################################################################################################