NS486SXF Demo Board

Theory of Operation

Prepared by

BCT Systems

Release Notes

03-JUL-95, Initial Release

- Open Items:

3-NOV-95, Rev. 1.1

- Added EPROM, FLASH and SRAM timing information.

- Corrected LCD power on/off sequence

- Corrected I/O map.

- Changed LPI left margin counter to 4 pixels per count

- Corrected Figure 2. Address and Data bus diagram. to show correct XSA buffer location.

Table of Contents

Release Notes

Table of Contents

List of Figures

List of Tables

1. Introduction

1.1 Purpose of Document

1.2 References

1.3 Overview of rest of document

2. Mother Board

2.1 Block Diagram

2.2 Bus Structure and Buffering

2.3 Functional Blocks

2.3.1 CPU

2.3.2 System ROM (EPROM and FLASH)

2.3.2.1 EPROM and FLASH Timing Requirements

2.3.3 Non-volatile SRAM

2.3.3.1 SRAM Timing Requirements

2.3.4 DRAM

2.3.5 ISA

2.3.5.1 Control Signals and Signal Termination

2.3.5.2 Standard Speed Wait State Generation

2.3.5.3 ISA Interrupt Capability

2.3.5.4 ISA DMA Capability

2.3.6 PCMCIA

2.3.7 Analog Reference Voltage

2.3.8 Digitizer Interface

2.3.9 LCD Panel Interface

2.3.10 LCD Bias voltage generation

2.3.11 Other LCD logic functions

2.3.12 Audio DAC and speaker interface

2.3.12.1 DAC Interface and DMA programming

2.3.12.2 Audio filter and amplifier

2.3.13 EEPROM

2.3.14 ECP Interface

2.3.15 Elentari Serial Port

2.3.15.1 Serial Infrared (SIR) Interface

2.3.15.2 EIA-232 Interface

2.3.16 General Purpose Jumpers and Indicators

2.3.17 Keypad Scanner Decoder

2.3.17.1 Keypad data circuitry

2.3.17.2 Keypad control state machine

2.3.17.3 Keypad layout and key codes

2.3.18 Super I/O

2.4 Elentari Chip Select Usage

2.4.1 CS0/ - EPROM & Flash Select

2.4.2 CS1/ & CS2/ - PCMCIA Selects

2.4.3 CS3/ - Non-volatile SRAM Select

2.4.4 CS4/ - Standard Speed ISA Select

2.4.5 CS5/ - High Speed ISA Select

2.5 Programmer’s Interface

2.5.1 Memory Map

2.5.2 I/O Map

2.5.3 Board Hardware Register Definitions

2.5.3.1 Audio DAC

2.5.3.2 Board Control Register

2.5.3.3 General Purpose Jumpers

2.5.3.4 General Purpose LED Displays

2.5.3.5 LCD Bias Voltage DAC

2.6 Configuration Jumper Summary

2.6.1 W1, W2 - Serial Port Type Selection

2.6.2 W3 - Super I/O™ Configuration

2.6.3 W4 - Unused

2.6.4 W5 - EPROM / Flash / Boot jumpers

2.6.5 W6 - Keypad interrupt enable

2.6.6 W7 - FDC MSEN

2.6.7 W8 - Audio DMA Request

2.6.8 W9 - Hard drive LED connection

2.6.9 W10 - General Purpose Configuration

2.6.10 W11 - Flash Interrupt Request Enable

2.6.11 W12 - LCD VEE Enable

3. Laser Printer Interface (LPI)

3.1 Overview

3.1.1 Video Interface

3.1.2 Engine Command and Status Interface

3.2 LPI Specifications

3.3 Block Diagram

3.4 LPI FPGA Functional Description

3.4.1 Engine command and status interface

3.4.2 Video interface

3.4.2.1 Beam Detect and video clock generation

3.4.2.2 Video data shift register and counter

3.4.2.3 Programmable counters

3.4.3 Address decode

3.5 Programmer’s Interface

4. Scanner Interface

4.1 Overview

4.2 Schematic

4.3 Programmer’s Interface

5. Appendix A

List of Figures

Figure 1. Elentari Demo Board Functional Block Diagram.

Figure 2. Address and Data bus diagram.

Figure 3. Digitizer Circuit Model.

Figure 4. Keypad control state diagram.

Figure 5. Keypad row and column definitions.

Figure 6. Board Control Register

Figure 7. Laser Printer Raster Scan.

Figure 8. Laser Printer Page Timing Diagram

Figure 9. Printer Engine Command Status Timing Diagram.

Figure 10. Simplified Block Diagram.

Figure 11. Laser Printer Interface Functional Block Diagram.

Figure 12. LPI Command Register

Figure 13. LPI Status Register

List of Tables

Table 1. EPROM jumper and address ranges.

Table 2. Flash  jumper and address ranges.

Table 3. EPROM and FLASH access times, ns.

Table 4. System RAM Configuration.

Table 5. ISA to Elentari Interrupt Mappings.

Table 6. DMA Channel Assignments.

Table 7. Digitizer Control Value Descriptions.

Table 8. Digitizer A/D Measurement Summary.

Table 9. Suggested Digitizer Operating Sequence.

Table 10. Elentari Serial Port Jumper Definitions

Table 11. Keypad key codes.

Table 12. Demo Board System Memory Map.

Table 13. Demo Board I/O Address and Port Map.

Table 14. MICROWIRE Device Selection.

Table 15. General Purpose Jumper Bit Definitions.

Table 16. Laser Printer Interface Specifications.

Table 17. LPI W4 Jumper Base I/O Address.

Table 18. LPI Port Definitions

Table 19. LPI W3 Interrupt Request Selection.

Table 20. LPI W1 and W2 Jumper DMA Settings.

Table 21. ScanFX W3 Jumper Base I/O Address.

Table 22. Scanner Interface Port Definitions

Table 23. ScanFX W4 Interrupt Request Selection.

Table 24. ScanFX W1 and W2 Jumper DMA Settings.

NS486SXF Demo Board

Theory of Operation

# Introduction

## Purpose of Document

This document provides a theory of operation for the Elentari demonstration system. The demo system consists of a motherboard, a laser printer interface board and a ScanFX interface board. This documents the details of the design of the demo system for use by hardware engineers, software engineers and technical documenters.

## References

The following documents are required references for understanding this document (items marked with * are supplied with this document, items with ** are supplied only for purchase of laser printer engine and scanner interface):

- Elentari Demo Board, Functional Specification, for National Semiconductor, BCT Systems.*

- Elentari Technical Documentation, March 1995, v0.1, National Semiconductor.

- Elentari MB Schematic, Rev. B.*

- Elentari MB Bill of Materials, Rev. B.*

- Elentari MB Cross Reference, Rev. B.*

- ELEN_GA Altera design files, Rev. A3.*

- FPGA2 Altera design files, Rev. A2.*

- KEYPAD Altera design files, Rev. B.*

- Laser Printer Interface Schematic, Rev. A.**

- Laser Printer Interface Bill of Materials, Rev. A.**

- Laser Printer Interface Cross Reference, Rev. A.**

- LPI Altera design files. Rev. A4.**

- ScanFX Interface Schematic, Rev. A.**

- ScanFX Interface Bill of Materials, Rev. A.**

- ScanFX Cross Reference, Rev. A.**

- ScanFX Altera design files, Rev. A.**

- PC87303VUL Super I/O™ Data Sheet, National Semiconductor, February 1995.

## Overview of rest of document

This document contains three major sections, one for each board.

Section 2 describes the mother board. This includes the block diagram and bussing diagram followed by an explanation of each functional block. Next, the chip select usage is described and the programmer’s interface is presented. The programmer’s interface includes memory and I/O maps and descriptions of programmable hardware registers on the board. This section concludes with a summary of the configuration jumpers.

Section 3 details the laser printer interface board. This includes the block diagram, description of the schematic and a theory of operation for the FPGA that implements the heart of the interface. Finally, a programmer’s interface for the LPI is presented.

Section 5 describes the scanner interface, including the schematic and programmer’s interface.

# Mother Board

## Block Diagram

Figure 1 is a functional block diagram of the Elentari demonstration mother board.

Figure 1. Elentari Demo Board Functional Block Diagram.

DRAM SIMM Bank 1

0-8 Megabytes

Battery Backed SRAM

LED Indicators (8)

20 Key Keypad

DRAM SIMM Bank 0

0.5-8 Megabytes

EPROM

0-8 Megabytes

FLASH

0-4 Megabytes

POWER

CLOCK

RESET

BATTERY

MICRO-WIRE™

A/D for Digitizer

PCMCIA

Type II Socket

ISA Bus Slots

Standard

&

High Speed

1     2     3     4

Super I/O

PC87303

Keyboard Interface

Floppy Disk Interface

IDE Interface

Debug Serial I/O

IEEE 1284 / ECP

Parallel Port

+32V Supply

LCD Bias Gen.

LCD panel I/F

Serial Infrared

or

EIA-232D I/F

DAC

&

Audio Amplifier

MICRO-WIRE™

EEPROM

Cascaded

Interrupt

Controller

## Bus Structure and Buffering

Figure 2 shows the major address and data bus buffering for the board. The unbuffered address (SA25..0) and data (SD15..0) busses from the Elentari CPU (U44) drive the DRAM directly. An individual set of buffers is provided for the PCMCIA socket so that PCMCIA can be powered up and down independent of the demo board. The PCMCIA busses are the PSA and PSD busses. A second set of buffers is provided for the rest of the memory and peripherals on the board. The are driven by the XSA and XSD address and data busses, respectively. The data bus buffers for the XSD include latches to improve the hold data on data write to ISA bus peripherals.

Functional Blocks

Figure 2. Address and Data bus diagram.

### CPU

The CPU along with the buffers described in section 2.2 are shown on sheet 2 of the schematic. The 32.768 Khz. crystal for the Elentari real time clock is Y2. The master Elentari time base is a crystal oscillator that is socketed for easy replacement. The frequency is nominally 50 MHz. J3 and J4 are the board power connectors, designed to plug into a standard AT compatible power supply. The J3 pin 1 connection provides the power good signal (PWRGD). This signal is combined with the “RESET” switch and the R-C network for power on initialization.

The IOCHK/ signal is combined with the “NMI” push-button to generate the non-maskable interrupt (NMI) to the CPU. All signals on the CPU are paralleled onto J1 (upper right corner of sheet 2). This connector is a collection of header pins that can serve two purposes. One is to allow easy connection to the Elentari emulator pod without requiring an expensive clip-on connector to the 160 pin PQFP. When the emulator is not connected, these pins are convenient for connector test equipment, such as logic analyzers or oscilloscopes to the signals.

### System ROM (EPROM and FLASH)

The System ROM is implemented as two arrays of devices, one for FLASH and the other for EPROM. The board contains a total of eight sockets to support a maximum of eight megabytes of EPROM and two locations for word-wide surface mount devices for a maximum of four megabytes of FLASH. The EPROM sockets are organized as four banks of two devices each to provide a 16-bit wide data path for the Elentari system bus. The EPROM and FLASH devices are all on sheet 6 of the schematic. The decode and chip select generation for the EPROM and FLASH is done in the Altera FPGA, labeled ELEN_GA in the lower left corner of sheet 7 of the schematic. The jumper decode and chip select generation is all in this FPGA. The jumper block that controls EPROM and FLASH size and location is W5. The FPGA is documented in the Altera design files for this device. ELEN_GA.GDF is the schematic of the top level of the FPGA. The decode logic for the EPROM devices is in the ROMBSU and ROMBSL sub-blocks. Decode logic for the flash is in the FLASHBS sub-block.

The EPROM devices are of two different package types. Two 32-pin DIP sockets accept standard, JEDEC EPROM DIP devices. Six 32-pin PLCC sockets accept standard, JEDEC EPROM PLCC devices. All EPROM devices must be the same size. The EPROM devices are organized as four banks, bank 0 through bank 3. The DIP devices are in bank 3, the bank at the highest address. Thus, the DIP devices can be the boot EPROM devices. The address range for each bank of EPROM devices is shown in Table 1 for all possible EPROM jumper settings.

Table 1. EPROM jumper and address ranges.

| W5 Jumper Pins |  |  |  |  |  | Address Range |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5-6 EJ2 | 7-8 EJ1 | 9-10 BJ | Device |  | Bank 0 U40, U41 |  | Bank 1 U26, U27 | Bank 2 U31, U32 | Bank 3 U50, U51 |
| X | X | X | 27C010 |  | FFF00000- FFF3FFFF |  | FFF4000- FFF7FFFF | FFF80000- FFFBFFFF | FFFC0000- FFFFFFFF |
| X | X | - | 27C010 |  | FF700000- FF73FFFF |  | FF74000- FF77FFFF | FF780000- FF7BFFFF | FF7C0000- FF7FFFFF |
| X | - | X | 27C020 |  | FFE00000- FFE7FFFF |  | FFE80000- FFEFFFFF | FFF00000- FFF7FFFF | FFF80000- FFFFFFFF |
| X | - | - | 27C020 |  | FF600000- FF67FFFF |  | FF680000- FF6FFFFF | FF700000- FF77FFFF | FF780000- FF7FFFFF |
| - | X | X | 27C040 |  | FFC00000- FFCFFFFF |  | FFD00000- FFDFFFFF | FFE00000- FFEFFFFF | FFF00000- FFFFFFFF |
| - | X | - | 27C040 |  | FF400000- FF4FFFFF |  | FF500000- FF5FFFFF | FF600000- FF6FFFFF | FF700000- FF7FFFFF |
| - | - | X | 27C080 |  | FF800000- FF9FFFFF |  | FFA00000- FFBFFFFF | FFC00000- FFDFFFFF | FFE00000- FFFFFFFF |
| - | - | - | 27C080 |  | FF000000- FF1FFFFF |  | FF200000- FF3FFFFF | FF400000- FF5FFFFF | FF600000- FF7FFFFF |

The FLASH devices are 48 pin thin small outline (TSOP) surface mount devices that are soldered on the board. The devices, available from AMD or Intel, are programmed using only the 5V supply. The board can be populated with zero, one or two FLASH devices. If there are two FLASH devices, they must be of the same size.

Table 2. Flash  jumper and address ranges.

| W5 Jumper Pins |  |  |  | Address Range |  |
| --- | --- | --- | --- | --- | --- |
| 1-2 FJ2 | 3-4 FJ1 | 9-10 BJ | Device | Bank 0 U45 | Bank 1 U33 |
| X | X | X | 29F100 | FF7C0000-FF7DFFFF | FF7E0000-FF7FFFFF |
| X | X | - | 29F100 | FFFC0000-FFFDFFFF | FFFE0000-FFFFFFFF |
| X | - | X | 29F200 | FF780000-FF7BFFFF | FF7C0000-FF7FFFFF |
| X | - | - | 29F200 | FFF80000-FFFBFFFF | FFFC0000-FFFFFFFF |
| - | X | X | 29F400 | FF700000-FF77FFFF | FF780000-FF7FFFFF |
| - | X | - | 29F400 | FFF00000-FFF7FFFF | FFF80000-FFFFFFFF |
| - | - | X | 29F800 | FF600000-FF6FFFFF | FF700000-FF7FFFFF |
| - | - | - | 29F800 | FFE00000-FFEFFFFF | FFF00000-FFFFFFFF |

Either the EPROM or FLASH devices can be the boot ROM. The boot ROM is the ROM that resides at the highest address in the ELENTARI memory space. The boot ROM occupies contiguous address space at the top of system memory (ending at 0ffffffffh). This boot ROM can be either EPROM or FLASH. The devices that are not boot ROM occupy contiguous address space ending at address FF7FFFFF. The boot selection is done by jumper W5 9-10 (BJ) as shown in Table 1 and Table 2, above. The CS0/ output is used to enable all system ROM (boot and non-boot) and must therefore be programmed to cover the entire system ROM address space, i.e. FF000000-FFFFFFFF.

#### EPROM and FLASH Timing Requirements

Timing requirements for the EPROM and FLASH are determined by the NS486 timing with the addition of the delays due to control and address/data buffering on the board. The control and buffering adds a delay of 37 ns. worst case to the access time. Access time requirements for the EPROM and FLASH depend on the frequency of the NS486 and the chip select wait programming. The formula for access time (in nanoseconds) is therefore:

taccess=WAIT+1T−67, with no command delay and

taccess=WAIT+2T−67, with command delay

Where WAIT is the number of programmend wait states and T is the period of the clock.

The FLASH must have the command delay of 1 state to function properly due to the requirement for valid address before the leading edge of the write strobe. Table 3 shows the EPROM and FLASH speeds supported for various numbers of wait states at both 20 and 25 MHz. operation.

Non-volatile SRAM

Table 3. EPROM and FLASH access times, ns.

| Number of wait states | 20 MHz. access time | 25 MHz. access time |
| --- | --- | --- |
| 1 | 83 | 53 |
| 2 | 133 | 93 |
| 3 | 183 | 133 |
| 4 | 233 | 173 |
| 5 | 283 | 213 |
| 6 | 333 | 253 |
| 7 | 383 | 293 |

The system NV-RAM consists of 256 kilobytes of battery-backed, static RAM. The static RAM is organized as two, 128K by 8 static RAM devices to form a 128K by 16 bit memory interfaced to the Elentari system bus.

The RAM circuitry is on schematic sheet 6. Included is a Nickel-Cadmium battery (BT1) that supplies voltage to the SRAM when VCC is removed. When VCC is applied, the battery is trickle charged via the transistor-resistor network. The 470 ohm resistor, R64, sets the charging current to nominal value of 3 ma. No diode switching is necessary since the 470 ohm resistor contributes no significant voltage drop for the SRAM leakage current when VCC is off.

#### SRAM Timing Requirements

Timing requirements for the SRAM are determined by the NS486 timing with the addition of the delays due to control and address/data buffering on the board. The control and buffering adds a delay of 37 ns. worst case to the access time. Access time requirements for the SRAM depend on the frequency of the NS486 and the chip select wait programming. The formula for access time (in nanoseconds) is therefore:

taccess=WAIT+1T−67, with no command delay and

taccess=WAIT+2T−67, with command delay

Where WAIT is the number of programmend wait states and T is the period of the clock.

An example programming of the chip select would be 2 wait states and command delay enabled yielding an access time requirement of 93 nanoseconds, which would be the fastest permissable access of 70 nanosecond SRAM.

### DRAM

The Demo Board is equipped with discrete SMT, DRAM devices soldered on the board for bank 0 and SIMM sockets for bank 1. The DRAM circuitry is on the right side of schematic sheet 5. The discrete DRAM and the two SIMM sockets can provide the full capability of 16Mbyte of memory. The discrete DRAM can consist of 1 Meg by 4 devices to provide a base RAM size of 2 megabytes or 4 Meg by 4 devices for 8 megabytes. The default configuration will be 8 megabytes. Industry standard, low-cost DRAM SIMM modules are used. The SIMM modules must be three-chip devices (to comply with Elentari DRAM control drive capability). Parity is supported so users should normally use nine-bit wide SIMMs.

The SIMM sockets support 256K by 9, 1M by 9, or 4M by 9 SIMMs. The two sockets must have the same sized SIMMs and must be less than or equal in memory size to bank 0. The diagnostic firmware sizes the system RAM and programs the Elentari to map the system RAM into contiguous address space beginning at 0h. Table 4 summarizes all possible system RAM configurations supported by the demo board. Since the DRAM controller is completely integrated within the Elentari, the reader is referred to the Elentari documentation for operation and programming of the DRAM controller.

Table 4. System RAM Configuration.

| Size | Bank 0 Devices | Bank 1 SIMMs | Address Range |
| --- | --- | --- | --- |
| 2 Meg | 1M by 4 | None | 0h-001FFFFFh |
| 2.5 Meg | 1M by 4 | 256K by 9 | 0h-0027FFFFh |
| 4 Meg | 1M by 4 | 1M by 9 | 0h-003FFFFFh |
| 8 Meg | 4M by 4 | None | 0h-007FFFFFh |
| 8.5 Meg | 4M by 4 | 256K by 9 | 0h-0087FFFFh |
| 10 Meg | 4M by 4 | 1M by 9 | 0h-009FFFFFh |
| 16 Meg | 4M by 4 | 4M by 9 | 0h-00FFFFFFh |

### ISA

Four 16-bit Industry Standard Architecture (ISA) bus slots are included on the demo board. The ISA connectors and support circuitry is shown on sheet 3 of the schematic. Although only four DMA channels are available, the slots support most standard 8-bit and 16-bit memory and I/O cards. By doubling the address space allocated to the ISA bus interface, each slot is able to support either standard speed ISA operation or high-speed Elentari native mode operation. The slots thus support a 16-bit local bus that provides higher than the normal ISA bus throughput but uses the same form factor, connectors and signals so that prototyping hardware is readily available. Additional circuitry on the demo board controls the Elentari RDY signal to better support the ISA ENDXFR/ and extend the timing to safely support 8-bit cards. This circuitry is described in Section 2.3.5.2, Standard Speed Wait State Generation.

The high-speed ISA is used to implement the high performance DMA required for laser printer engine raster scan drive as well as for high speed scanner input. The standard speed ISA can be used for FAX/modem cards, display cards, LAN cards or many other standard ISA cards that might be useful for Microsoft At Work product development.

The demo board ISA slots do not support the following standard ISA capabilities:

- MEMORY/ signal is not supported. The signal is driven unasserted (high).

- Temporary masters are not supported. The MASTER/ signal is ignored.

- The BALE (address latch enabled) is not supported. The signal is always asserted.

#### Control Signals and Signal Termination

The ISA interface on sheet 3 of the schematic includes control signal buffering for all control signals. Additionally, all signals on the bus contain AC termination. The RC termination, RC3 through RC12 provide a 100 ohm impedance coupled through a 47 pF capacitor. The design includes connection of VCC to the terminators, so they can optionally be replaced with standard resistor divider termination, if desired.

Many of the buffered control signals are generated directly by the Elentari. Some signals required logic to develop and this logic is in ELEN_GA on sheet 7 of the schematic. Within the ELEN_GA design, the ISA control signals are generated in the ISACTL module. This is a schematic in the file ISACTL.GDF. The four ISA memory control strobes, SMR/, SMW/, MWISA/ and MRISA/ must be qualified from the basic MEMR/ and MEMW/ generated by the Elentari. The strobes must be qualified by the CS4/ and CS5/ signals so the ISA cards do not respond to strobes for the EPROM, Flash and non-volatile SRAM on the board. This is necessary since the ISA bus only contains 24 address lines, not the full 32 bits required to distinguish the various sections of the demo board memory. The read strobes are generated directly in the FPGA and then simply buffered on the board. Because of the requirement to maximize data write hold time, enables are generated for the write strobes and the buffers on the board logically AND the enable signal with the appropriate strobe. The write strobes are ANDed in U34B and C on sheet 3 of the schematic. The SMWEN/ and SMRISA/ are the strobes for 8-bit ISA memory and must be active only when access is to the lowest one megabyte of ISA memory space. This is accomplished with the A20..A23 qualification in the FPGA.

The AEN signal for the ISA bus is set low to enable programmed I/O to ISA cards when either CS4/ or CS5/ is asserted and there is no DMA cycle in progress.

The XSD_WR/, XSD_RD/ and XSD_LEN control the data buffers from the Elentari to the ISA and other peripherals (see section 2.2, Bus Structure and Buffering). XSD_LEN controls the transparent latches (U56 and U59, sheet 2) so that they are fall through during the write strobe but latched after the write strobe terminates. The XSD_WR/ signal is active as long as the chip selects are active and a read cycle is not in progress. The XSD_RD/ enables the XSD bus read buffers (U57 and U58) during any bus read activity.

#### Standard Speed Wait State Generation

The wait state generation logic is in FPGA2 on sheet 5 of the schematic. The logic is contained in the RDY.GDF design file for this FPGA. Although the circuit looks complex on the drawing, the principal of operation is actually quite simple. The WAIT flip-flop is set whenever a command (IOR/, IOW/, MEMR/ or MEMW/) is asserted and the access is to the standard speed ISA (CS4/ asserted). The WAIT signal is synchronized to the 50mhz. clock and the WAITS signal enables the two 74163 counters to begin counting up. The ENDWAIT/ signal will be asserted whenever a terminating condition occurs. This will clear the counters and clear the WAIT flip-flop allowing RDY to be asserted to the ELENTARI. The two signals B and W are for Byte access and Word access on the bus, respectively. They are generated from the IOCS16/ and MEMCS16/ signals from the bus with the appropriate type cycle in progress. The B or W signal is combined with count values to generate most of the terminating conditions. The terminating conditions are:

- ENDXFR/. This is an end of transfer initiated by a 16-bit ISA memory card that can handle 0 wait states (and is asserted in response to 0WS/ ). There is no need to qualify this value by the counter, since the Elentari CS4/ must have a logical chip select bus cycle that meets the minimum ISA, zero wait state cycle. Note the ENDXFR/ is synchronized to the 50 MHz. clock.

- W200NS/ & W240NS/. These combine to provide a minimum of 200 ns. for 16-bit word cycles.

- B520NS/ & B560NS/. These combine to provide a minimum of 520 ns. for 8-bit byte cycles.

The RDY signal can also be driven by IOCRDY and PCMRDY signals. RDY will be asserted only when all conditions have been met, so the slower of the WAIT signal and the IOCRDY will determine the bus cycle time for ISA accesses.

The E_CS16/ signal is routed out of the device to drive the Elentari CS16/ signal. This is because ISA requires the bus master to properly qualify the IOCS16/ and MEMCS16/ signals with the appropriate strobes. (I.e., MEMCS16/ can, and usually will be, asserted by 16-bit ISA memory cards when an I/O access occurs. IOCS16/ will be asserted by 16-bit I/O cards when the low order address bits of a memory address match the I/O address).

ISA Interrupt Capability

Table 5. ISA to Elentari Interrupt Mappings.

| ISA bus Interrupt | Elentari Interrupt | Cascade PIC Channel |
| --- | --- | --- |
| IRQ3 | E_IRQ1 | N/A |
| IRQ4 | E_IRQ2 | N/A |
| IRQ5 | E_IRQ3 | N/A |
| IRQ6 | E_IRQ4 | N/A |
| IRQ7 | E_IRQ0 | 0 |
| IRQ9 | E_IRQ0 | 1 |
| IRQ10 | E_IRQ0 | 2 |
| IRQ11 | E_IRQ0 | 3 |
| IRQ12 | E_IRQ0 | 4 |
| IRQ14 | E_IRQ0 | 5 |
| IRQ15 | E_IRQ0 | 6 |

An AT compatible ISA bus supports thirteen interrupts. The Elentari only supports 6 external interrupts. In order to support the widest possible combination of on board and add-in card usage, the demo boards supports full interrupt capability on the ISA bus. This is accomplished by using a programmable interrupt controller (PIC) external to the Elentari. The PIC uses Elentari E_IRQ0 to add eight additional interrupt request lines for the ISA bus. Table 5 shows the interrupt mapping from ISA bus to the Elentari interrupt inputs.

ISA DMA Capability

Table 6. DMA Channel Assignments.

| ISA DMA Channel | Elentari DMA channel | Isolation Jumpers |
| --- | --- | --- |
| 0 | 0 | No |
| 1 | 2 | No |
| 2 | 4 | Yes |
| 3 | 6 | No |
| 5 | 0 | No |
| 6 | 2 | No |
| 7 | 6 | No |

The ISA bus supports a total of 8 DMA channels. ISA channel 4, which is the DMA cascade channel is not supported. Therefore, only seven DMA channels connections are supported by the demo board. Since there are only four Elentari DMA channels available external to the chip, the DMA channels are shared on the ISA bus as shown in Table 6. Additionally, Elentari DMA channel 4 (ISA channel 2) is used by the Floppy Disk controller, so an isolation jumper is provided to permit use of this channel on the ISA bus when the floppy controller is not used

### PCMCIA

The PCMCIA interface circuitry is on sheet 2 of the schematic with the CPU. The PCMCIA host interface requires minimal logic on the board as the Elentari contains the host adapter circuitry. The board circuitry consists of buffering and power control. By buffering all address, data and control signals to the PCMCIA socket, it is possible to turn power off to a device in the socket while the demo board is powered and fully operational.

### Analog Reference Voltage

The analog reference voltage is in the lower left corner of sheet 4 of the schematic. This uses an LM385 to produce a nominal 4.3V reference. The signal name is 4.3V. This reference is used for the digitizer interface, the two DACs and the A/D converter, all of which is on this same sheet 4 of the schematic.

### Digitizer Interface

The digitizer interface is designed to connect to an Alps resistive touch screen. The touch screen consists of a layer of transparent resistive film separated by a spacer from a transparent resistive layer on glass. This touch screen is typically placed on top of and LCD screen. Pressing on the transparent film will cause electrical contact between the two resistive layers at the point of the applied pressure. One resistive film is the Y plane with electrical connections at the top and bottom, referred to as Y1 and Y2, respectively. The other sheet is the X plane with left and right connections referred to as X1 and X2.

Figure 3. Digitizer Circuit Model.

Figure 3 shows a circuit model for the tablet when point pressure is applied. Since the sum of Rx1 and Rx2 is a fixed constant, the distance from the left side of tablet that the pressure is applied will be proportional to the value of Rx1. The vertical direction will have a similar proportion. By determining the resistor value ratios, circuitry can determine the precise location of the pressure. The circuitry to do this is shown on page 4 of the schematic. Application of a reference voltage (approximately 4.3V) or ground to the terminals of the tablet are controlled by the X1ON, X2ON, Y1ON and Y2ON signals that are generated in the Altera device FPGA2, U28 on sheet 5 of the schematic. The FPGA2 circuitry on sheet REG2 controls the drive values for these signals. There are four possible states, including the off state. The four possible states and the value required in the control register (see section 2.5.3.2) are shown in Table 7.

Table 7. Digitizer Control Value Descriptions.

| Control Value | X1 | X2 | Y1 | Y2 | Operation | Description |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | off | off | off | off | Idle |  |
| 1 | ON | ON | off | off | Measure X | Apply Vref to X1, ground to X2, measure Y2-X2 |
| 2 | off | off | ON | ON | Measure Y | Apply Vref to Y1, ground to Y2, measure X1-Y2 |
| 3 | off | ON | ON | off | Check closure | Places opposite voltage on the two planes to determine if a pen or finger is down |

The 12-bit analog to digital (A/D) converter, U70, can measure the differential voltage between selected points at the X1, X2, Y1 and Y2 terminals. The A/D is used as a four channel differential measurement system. Table 8 shows the measurements and describes the purposes for the four channels.

Table 8. Digitizer A/D Measurement Summary.

| Channel | Measurement | Control Value | Description |
| --- | --- | --- | --- |
| 0-1 | X1-X2 | 1 | Determine Vref-ground for X measurement |
| 2-3 | Y1-Y2 | 2 | Determine Vref-ground for Y measurement |
| 4-5 | Y2-X2 | 1 | Measure the X voltage divider |
| 6-7 | X1-Y2 | 2 | Measure the Y voltage divider |
|  |  | 3 | Read 0 volts, pen down, -Vref, pen up |

The suggested three step operating sequence for digitizer is shown in Table 9.

Table 9. Suggested Digitizer Operating Sequence.

| Step | Operation | Details |
| --- | --- | --- |
| 1 | Determine if Pen Up or Down | - Set control value to 3, read X1-Y2 (channel 6-7). - If value greater than -2 Volts, pen down, else if less than -2 Volts, pen up. - Continue only if pen is down. |
| 2 | Determine X position | - Set control value to 1 and measure Vxt = Channel 0-1 - Measure Y2-X2 (Channel 4-5) - Compute X position from X2 as (Y2-X2)/Vxt |
| 3 | Determine Y position | - Set control value to 2 and measure Vyt = Channel 2-3 - Measure X1-Y2 (Channel 6-7) - Compute Y position from Y2 as (X1-Y2)/Vyt |

### LCD Panel Interface

The Elentari LCD control circuitry supports either the ALPS 324 by 240 or 480 by 320 LCD display. The circuitry, at the bottom of sheet 4 of the schematic, includes programmable LCD bias supply and voltage divider. A flex circuit connector is used to connect to the LCD display.

### LCD Bias voltage generation

The LCD bias voltage is generated by a DC to DC converter (U71) from the +12V supply. This converter outputs approximately 30 Volts for driving the LCD display. An 8-bit digital to analog converter (DAC) provides a set point voltage for the display voltage level. The LCDSET signal (the output of I to V converter U21D) is an output proportional to the digital code written to the DAC and is in the range of 0 and -4.3V. Op amp U76C has a gain of -8.4 to scale the output from the DAC to the range of approximately 0 to +40 Volts. Note that the actual output cannot exceed the power supply to U76 of +30V. An emitter follower stage, Q1, that is driven from U76C provides buffered VEE for the LCD display and the voltage divider for the other required bias voltages. The most significant bit of the DAC input code is also used as a control to enable/disable the VEE output. Whenever bit 7 into the DAC is low, inverter U12E disables U76C output by driving a positive voltage into the summing junction through diode CR3. When bit 7 is high, the inverter output is low turning off the diode, allowing U76C to scale the DAC output and turning on LED DS2 as indication that the LCD display has power.

Jumper W12 provides a physical way to disable the power to the LCD panel for systems that do not yet have properly debugged supply hardware or software. This insures the LCD panel will not be inadvertently powered by improperly operating hardware or software. Because of the signal interface and possible difficulties with transients and parasitic SCR devices on the LCD display, W12 must never be removed or installed while power is applied to the board.

### Other LCD logic functions

Most of the control of the LCD display is via logic built into the Elentari. There are, however, two simple logic functions provided in FPGA2 (U28) on sheet 5 of the schematic. One function is control of the display on and off via the signal D_OFF/. This signal is reset low and is set by a write to the control register (see section 2.5.3.2). The other function is the division of the frame signal to provide an alternating logic level for each display frame to maintain an average DC bias of 0V across the LCD panel, as required by multiplexed LCD panels. This signal is DF and is generated by CL1 and CLF in FPGA2.

### Audio DAC and speaker interface

The audio circuitry includes an 8-bit DAC, audio filter and power amplifier, all on sheet 4 of the schematic.

#### DAC Interface and DMA programming

The DAC can be written via an OUT instruction or through DMA. The control strobe and DMA management for the DAC are in FPGA2 (U28) on sheet 5. The top level FPGA2 schematic contains the audio DMA request circuitry. The request (AUD_DRQ) is initiated by timer T1 on the Elentari. The FPGA2 logic latches the request and holds it until the audio DMA acknowledge signal (AUD_DACK/) is asserted and a DMA cycle places new data in the 74ALS374, U14. The rising edge of T1 sets the DMA request while the falling edge of T1 transfers data from U14 to the DAC register.

The assertion time programmed for T1 must therefore be greater than the maximum DMA latency in the system for each data item to be present for exactly the period of T1. Because the DAC requires a 900 nanosecond minimum write pulse, the low time of T1 must be at least 900 ns. This could be achieved by programming the T1 timer for rate generator operation and the clock divider register for T1 to divide the 25 MHz. clock by 32. Since the rate generator mode has a one clock period low per cycle, this would yield a 1280 ns. DAC load pulse and permit the duration of the timer cycle (the high time) to be available for DMA latency. If it is not possible to program the clock divider to divide by 32 or more (e.g., due to the use of the divider for another purpose on T0), then the timer would have to be programmed in the square wave mode, though at the loss of some efficiency.

#### Audio filter and amplifier

The first stage of the audio filter, U21B is a second-order high-pass Butterworth filter with an fcp of approximately 120 Hz. The second stage is a second-order low-pass Butterworth filter with an fcp of approximately 20 Khz. The final stage is a power amplifier AC coupled to an 8 ohm speaker.

### EEPROM

Non-volatile configuration can be saved in the serial EEPROM, U66 in the upper right of schematic sheet 4. The device is a MICROWIRE™ device that directly interfaces to the three-wire Elentari interface. Since there are a total of three slave devices on the bus, individual enables are provided for each serial peripheral. The CSEE signal is the enable for the EEPROM. It is generated in the control register (see section 2.5.3.2) in FPGA2 on sheet 5 of the schematic.

### ECP Interface

The Extended Capabilities Port (ECP) is implemented entirely within the Elentari. Signal filtering and connector are provided on the demo as shown in the upper left part of sheet 5 of the schematic. J16 is the ECP interface connector.

Elentari Serial Port

Table 10. Elentari Serial Port Jumper Definitions

| W1 | W2 | Port Enabled |
| --- | --- | --- |
| 1-2 | 1-2 | EIA-232 |
| 3-4 | 3-4 | Serial Infrared Interface |

The Elentari on-chip NS16550 UART supports either EIA-232 interface levels or Serial Infrared (SIR) mode. The demo board has both types of interface circuitry. Configuration jumpers W1 and W2 select which interface is used. Table 10 shows the jumper settings for the Elentari Serial Port.

#### Serial Infrared (SIR) Interface

An infrared LED and receiver are included on the board for infrared remote control application. The circuitry is shown at the top of sheet 7 on the schematic.

The transmitter uses a TSIP5201 Infrared emitting diode (IED) driven by the Elentari serial port. LED DS3 tracks the transmit signal to provide visual indication of serial data transmission activity. The LED will normally be off when the transmit signal is in the marking condition and thus turn on during data transmission.

#### EIA-232 Interface

Because the Elentari MICROWIREÔ is used on the demo board, the CTS/, DCD/ and RI signals are not available for the EIA-232 interface. Only the RTS/, DTR/ and DSR/ control signals plus the data are available for the interface. The interface is a data terminal equipment (DTE) interface using a 9 pin, male, D type connector. This is J22 on sheet 8 of the schematic. Level translation from the Elentari CMOS levels to EIA-232D levels is accomplished through U4 and U5.

### General Purpose Jumpers and Indicators

A general purpose 8-bit jumper field is provided for hardware and software configuration. The jumper is W10 and is located on sheet 5 of the schematic. The jumper field is read as 8-bit input data by the CPU. The bit definitions of the general purpose input jumper will be dependent upon the firmware operating on the board.

Eight general purpose LED displays are available for displaying hardware and software status on the board. These LED’s are on sheet 7 of the schematic. They are writable by the processor and with an 8-bit OUTPUT instruction. The register that drives the LEDs can also be read by software for programmer convenience.

### Keypad Scanner Decoder

A 20-key keypad is mounted on the board. The keyboard is a Grayhill series 88, 5 by 4 key, snap dome contact, keypad. The keypad has no legends. An assortment of printed legend labels are supplied for customization by the user. The keypad is mounted directly to the demo board with four studs. The keyboard is approximately 3.2” wide by 3.0” high.

The keypad interface circuitry is on sheet 7 of the schematic. U8 is an Altera EPM5032 programmable device that implements keypad scanning, debouncing and validation. Upon a valid key closure, an interrupt request is generated to notify the CPU. The key data can then be shifted out via the serial MICROWIRE™ interface to the CPU. The details of the keypad FPGA, U8, are documented in the keypad design files. KEYPAD.GDF is the top level of the keypad scanner/decoder. The design consists of two parts, the data and control. The data portion is in KEYDATA.GDF and the control is a state machine in KEYCNTL.TDF.

#### Keypad data circuitry

The keypad data circuitry continuously scans the rows of the keypad. The 74161 is a free-running counter that is the encoded row count. The row count is decoded by the 74138 to provide the individual row drive signals. The 74148 is an encoder that encodes the individual column inputs into a 3-bit binary code. The binary code present on the column along with a signal asserted whenever any column is asserted (low) are latched in the 74174 on the alternate edge of the key_clock that drives the row counter. This clocking scheme allows one-half period of the clock as the metastable settling time for the column data while managing to sample the column data while the row data is still valid. The 8DFFE latch can latch both the row count and encoded column data upon command from the control via the signal load_keycode. The latched row and column data is compared with the current row and column data to accomplish key debounce function. I.e., the code must get latched and then be identical during the next time the row matches in order to be accepted as a valid key closure. The exclusive-or based comparators separately report to the control circuitry the row and column comparisons via row_match and column_match. Once the control circuitry has validated a key, it asserts the load_keycode signal to cause the latched contents of the 8DFFE to be transferred to the 74165 output shift register. Since the H input to the shift register is loaded with a one, the shift register output provides indication that a key has been detected and is used to generate an interrupt signal. The interrupt is cleared by reading the key code via the serial MICROWIRE.™

Keypad control state machine

Figure 4. Keypad control state diagram.

The control circuitry in KEYCNTL.TDF implements a simple state machine with four states using the Altera AHDL language. Figure 4 is a state diagram for the keypad control circuitry. The state machine is clocked by the same clock as the 8DFFE latch that latches the row and column codes. This clock is set to a period of one fourth the total key bounce time desired. Once initialized and in the wait_for_keypress state, the state machine monitors the key_press signal. When asserted, the current row and column codes are latched in the 8DFFE as the state machine advances to the validate_key_press state. This state can not be exited until row_match occurs, indicating that the row counter has cycled around and is driving the same row that was latched in the advance from wait_for_key_press state. Once row_match occurs, then if column_match and key_press occur, a valid key has been detected, the key is output (output_key asserted) and the state machine advances to the key release side of the state machine. If a valid key was not detected in validate_key_press, then the state machine goes back to the wait_for_key_press state.

The release routine only advances when there is a row match, so that the key release time matches the key validation time of one full period of the row counter. This routine makes certain that the key is released and there is no key release bounce before advance to the wait_for_key_press state and waiting for the next valid key depression.

#### Keypad layout and key codes

Figure 5 shows the keypad layout with the row and column definitions for the scanning. Table 11 documents the key codes that are output for each key, referenced by the row and column of the key. Note that the most significant bit is set for each valid code. This is from the setting of this bit in the data shift register any time the register is loaded. Once the data is shifted out, the register is cleared, so polled reading of the register can use this bit to determine if the rest of the data represents a valid key depression.

Figure 5. Keypad row and column definitions.

Table 11. Keypad key codes.

| Keypad Label | Row | Col | Key Code (hex) |
| --- | --- | --- | --- |
| 1 | 1 | 1 | 87 |
| 2 | 1 | 2 | 86 |
| 3 | 1 | 3 | 85 |
|  | 1 | 4 | 84 |
|  | 1 | 5 | 83 |
| 4 | 2 | 1 | 97 |
| 5 | 2 | 2 | 96 |
| 6 | 2 | 3 | 95 |
|  | 2 | 4 | 94 |
|  | 2 | 5 | 93 |
| 7 | 3 | 1 | A7 |
| 8 | 3 | 2 | A6 |
| 9 | 3 | 3 | A5 |
|  | 3 | 4 | A4 |
|  | 3 | 5 | A3 |
| * | 4 | 1 | B7 |
| 0 | 4 | 2 | B6 |
| # | 4 | 3 | B5 |
|  | 4 | 4 | B4 |
|  | 4 | 5 | B3 |

### Super I/O

A PC87303VUL Super I/OÔ device is included to interface to several peripherals. This includes a debug UART, a standard PS/2 keyboard, a floppy disk and an IDE hard drive. The Super I/OÔ is on sheet 8 of the schematic. The Super I/O is interfaced to the XSA and XSD busses as ISA peripherals. The connection and usage is all per the PC87303VUL data sheet. The reader is referred to the data sheet for explanation of the circuitry.

## Elentari Chip Select Usage

All six external, physical chip select signals generated by the Elentari, CS0..CS5, are utilized by the demo board. The following sections describe the usage of these signals on the board.

### CS0/ - EPROM & Flash Select

CS0/ is used to select all of the EPROM and Flash on the demo board. The boot code must program a logical chip to include the entire EPROM Flash address range for CS0/ prior to jumping outside the upper 64 Kilobytes of memory. To include all EPROM and Flash, the chip select should be programmed to respond to access from 0xFF000000 to 0xFFFFFFFF. Since EPROM and Flash can have different speed requirements, two logical chip selects could be programmed for CS0/.

### CS1/ & CS2/ - PCMCIA Selects

The CS1/ and CS2/ are used for the PCMCIA interface.

### CS3/ - Non-volatile SRAM Select

The CS3/ is programmed for the non-volatile SRAM. It must have a logical memory chip select with an address range of 0x06000000 to 0x0603FFFF.

### CS4/ - Standard Speed ISA Select

The standard speed ISA select is programmed to be active for the standard speed ISA operations to either memory or I/O. The standard speed ISA is designed to match the speeds of a standard 8 MHz. AT and adhere to the IEEE P996 draft standard specification. Normally, this means access to ISA memory in the Elentari range of  0x04000000 to 0x4FFFFFF and I/O addresses in the range of 0x0000 to 0x7FFF.

### CS5/ - High Speed ISA Select

The high speed ISA select is programmed to be active for the special high-speed ISA operations to either memory or I/O. Normally, this means access to ISA memory in the Elentari range of  0x05000000 to 0x5FFFFFF and I/O addresses in the range of 0x8000 to 0xFFFF. Any CS5/ enabled access will operate at the speed set by the logical chip selects for CS5/ and no wait states will be added by demo board logic. Note that the ISA IOCRDY signal can still extend the length of ISA memory or I/O cycles.

## Programmer’s Interface

### Memory Map

The allocation of the external system memory space is shown in the address map in Table 12.

Table 12. Demo Board System Memory Map.

| Starting Address (hex) | Ending Address (hex) | Size | Description |
| --- | --- | --- | --- |
| 00000000 | 00FFFFFF | 16M | System RAM |
| 01000000 | 03FFFFFF | 48M | Unused |
| 04000000 | 04FFFFFF | 16M | Standard ISA |
| 05000000 | 05FFFFFF | 16M | High speed ISA |
| 06000000 | 0603FFFF | 256K | System NV-RAM |
| 06040000 | 06FFFFFF |  | Unused |
| 07000000 | 07FFFFFF | 16M | PCMCIA Interface |
| 08000000 | FEFFFFFF |  | Unused |
| FF000000 | FF7FFFFF | 8M | System secondary ROM (EPROM or FLASH) |
| FF800000 | FFFFFFFF | 8M | System boot ROM (EPROM or FLASH) |

### I/O Map

The allocation of the demo board I/O space, excluding internal Elentari ports, is shown in the I/O address map in Table 13. (Note that the high speed vs. standard speed ISA has no relevance for internal Elentari I/O ports, as access to the internal ports will not activate the external I/O strobes and thus not be effected by the board ready control logic.)

Table 13. Demo Board I/O Address and Port Map.

| Starting Address (hex) | Ending Address (hex) | Read - Write | Description | Section(s) |
| --- | --- | --- | --- | --- |
| 0x0000 | 0x7FFF | - | Standard speed ISA I/O |  |
| 0x8000 | 0xFFFF | - | High speed ISA I/O |  |
| 0x0060 | 0x006F | - | Super I/O 8042 keyboard port |  |
| 0x0A21 | 0x0A21 | W | Audio DAC | 2.3.12 & 2.5.3.1 |
| 0x0A22 | 0x0A22 | R/W | Board Control Register |  |
| 0x0A23 | 0x0A23 | R | General Purpose Jumpers | 2.3.16 & |
| 0x0A24 | 0x0A24 | R/W | General Purpose LEDs | 2.3.16 & 2.5.3.4 |
| 0x0A28 | 0x0A29 | R/W | 8259A Programmable Interrupt Controller |  |
| 0x0A26 | 0x0A26 | W | LCD Bias Voltage DAC | 2.3.10 & 2.5.3.5 |

### Board Hardware Register Definitions

#### Audio DAC

The Audio DAC is an NS DAC0830, 8-bit digital to analog converter. The DAC is written using an 8-bit OUT instruction. The Audio DAC output voltage will be approximately in the range of 0V and -4.3V (See section 2.3.7) according to the formula

Vout = −4.3DigitalInput256 Volts,

where Digital_Input is the 8-bit value written to the DAC, treated as an unsigned binary value. Note that the output voltage is negative, but the voltage is inverted by the audio power amplifier.

#### Board Control Register

The board control register is a 7-bit I/O read / write port for setting some control functions for the board. The board control register is cleared to all zeros by a power on reset. The board control register has three functions: Digitizer FET switch control, MICROWIRE™ device selection and LCD display on / off control.

The digitizer interface, described in section 2.3.8, includes FET switches to control the voltages applied to the digitizer. The digitizer control values, described in Table 7, are set by the two least significant bits (bits 0 and 1) of the control register.

The MICROWIRE™ device selection is included in the control register bits 2 and 3. The values for the device selection are in Table 14.

Table 14. MICROWIRE Device Selection.

| Control Register |  |  |  |
| --- | --- | --- | --- |
| Bit 3 | Bit 2 | Decimal | MICROWIRE™ Device Selected |
| 0 | 0 | 0 | None |
| 0 | 1 | 1 | Digitizer A/D Converter |
| 1 | 0 | 2 | 20-key Keypad |
| 1 | 1 | 3 | Serial EEPROM |

The LCD display is enabled via bit 5 in the board control register. The display is off when the bit is zero (the power on reset state) and on when the bit is a one. This bit should be turned on after the LCD Bias voltage is applied and turned off before the LCD bias voltage is removed. (See also sections 2.3.9 and 2.5.3.5).

The board control register is summarized in Figure 6. (Note: RFU is reserved for future use.)

| General Purpose Jumpers RFU | RFU | LCD | MICROWIRE™ Select |  | Digitizer Control Value |  |
| --- | --- | --- | --- | --- | --- | --- |
| 6 | 5 | 4 | 3 | 2 | 1 | 0 |

Figure 6. Board Control Register

Table 15. General Purpose Jumper Bit Definitions.

| W10 Pins | Bit Position |
| --- | --- |
| 1-2 | 0 |
| 3-4 | 1 |
| 5-6 | 2 |
| 7-8 | 3 |
| 9-10 | 4 |
| 11-12 | 5 |
| 13-14 | 6 |
| 15-16 | 7 |

The general purpose jumpers are read as an 8-bit INPUT port. The bit assignments for the jumpers are described in Table 15.

#### General Purpose LED Displays

The General Purpose LED Displays are located just above the keypad on the board. An 8-bit OUTPUT instruction is used to write the LED display register. The register is not cleared by a reset, so after board power on the LEDs will be in a random state. When viewing the board so the keypad is properly facing the user, the least significant bit of the 8-bit data value addresses the left most LED. A value of zero will turn the LED on and a value of one turns the LED off.

#### LCD Bias Voltage DAC

The LCD Bias Voltage DAC is an NS DAC0830, 8-bit digital to analog converter. The DAC is written using an 8-bit OUT instruction. The LCD bias voltage will be approximately in the range of 0V and 36V (See section 2.3.7) according to the formula:

VEE=36DigitalInput256Volts,

where Digital_Input is the 8-bit value written to the DAC, treated as an unsigned binary value. This formula is valid for Digital_Input in the range of 128 to 256. Any values below 128 produce an output of zero volts. The hardware only supports a maximum VEE of 30 Volts, so codes that would give greater than 30 Volts will be limited to 30 Volts.

## Configuration Jumper Summary

### W1, W2 - Serial Port Type Selection

W1 and W2 select the Elentari serial port interface to be EIA-232D or HP Serial Infrared. See section 2.3.15 and Table 10 on page 5.

### W3 - Super I/O™ Configuration

W3 is a nine position jumper that configures the Super I/O™ chip. Refer to the Super I/O™ data sheet for details

### W4 - Unused

### W5 - EPROM / Flash / Boot jumpers

W5 is a five position jumper that configures the EPROM size, Flash size and boot device. The jumper definitions are described in section 2.3.2. The EPROM jumpers are described in Table 1, page 5 and the Flash jumpers are described in Table 2, page 6.

### W6 - Keypad interrupt enable

W6 enables or disables the keypad interrupt to drive ISA interrupt request 12. This jumper should be removed if an ISA board requires the use of IRQ12.

### W7 - FDC MSEN

See Super I/O™ data sheet.

### W8 - Audio DMA Request

W8 enables the Audio DAC DMA request to drive Elentari DMA request 6 (E_DRQ6), when installed.

### W9 - Hard drive LED connection

W9 is a jumper header that can drive an LED to show Hard Drive activity.

### W10 - General Purpose Configuration

W10 provides general purpose configuration. The details are described in section 2.5.3.3 and Table 15, page 20.

### W11 - Flash Interrupt Request Enable

W11 enables or disables routing the Flash Ready signal to IRQ15. The interrupt is enabled when the jumper is installed.

### W12 - LCD VEE Enable

W12 connects the VEE to the LCD display interface connector. The jumper must be installed to apply power to the LCD Panel. W12 must never be installed or removed with power on to the board (also see section 2.3.10.)

# Laser Printer Interface (LPI)

## Overview

The laser printer interface (LPI) is a command/control and programmable video interface to a laser printer engine. It is an ISA form-factor compatible board that uses the demo board high-speed ISA interface, though the board is capable of operating in a standard speed ISA slot.

### Video Interface

The laser printer produces an image on the paper by raster scan scanning the image onto a drum by a laser modulated by the video signal from the laser printer controller. A simplified drawing of a single page and the raster scan path followed is shown in Figure 7. The imaging process begins in the upper left corner of the page. Each solid horizontal line represents a single horizontal scan line on the page. The horizontal broken lines represent the horizontal retrace.

Figure 7. Laser Printer Raster Scan.

The video interface to the laser printer contains the image data to be printed on the paper. The video interface sends raster scan data on the VDO/ signal. Additionally, the video interface includes two synchronization signals generated by the printer. TOP/ is the vertical sync, sent once per printed page. This corresponds to the Start position on the page. BD/ (Beam Detect) is the horizontal sync, sent once per horizontal scan line. The BD/ occurs at the left side of the page for each line. The relationship of these signals is shown in the timing diagram in Figure 8.

Figure 8. Laser Printer Page Timing Diagram

The top four signals in Figure 8 show the full page operation. The PRNT/ signal from the controller requests a page to be printed. The printer responds with TOP/ when the printer is at the top of the page. The TOP time is the time from the TOP/ signal until data can be imaged on the page. The value for this time is specified for a given laser printer and is programmable in this interface by the top_counter. The top_counter counts lines (occurrences of BD/) and is therefore programmed in units of lines. The top_counter has a count range of from 1 to 255 lines. Once the TOP time has completed, the controller begins sending video data (VDO/) in response to beam detects (BD/). The single scan line detail shows the imaging of a single horizontal scan line. The LEFT time controls the left margin from the left edge of the page to the beginning of the imaged data. This time is also specified for a given laser printer. The time is programmable in this controller via the left_counter. The left_counter makes one count for every two pixels with a range of 2 to 510 pixels. The WIDTH time specifies the time the video data is to be imaged. Actually, this is a count of dots or pixels to be imaged per line. The WIDTH count is programmable by setting the width_counter in this controller interface. The width_counter counts 16-bit words to be imaged. This provides a maximum line width of 8,160 pixels.

### Engine Command and Status Interface

Control and status of the laser printer engine is accomplished via a synchronous serial interface to the engine. This interface provides commands for setting paper size, resolution, and other parameters. Additionally, this interface can obtain status of the engine, such as paper out, toner out, trouble, etc. This slow speed serial interface always sends a command and then receives a status as a result of the command. The controller is thus the master and the printer the slave in the protocol. Figure 9 is a timing diagram showing the command/status interface operation.

Figure 9. Printer Engine Command Status Timing Diagram.

The CBSY/ signal is Controller BuSY, generated by the controller to indicate it has control and is shifting out a command. The command is shifted on the CMD/ signal via clock, CCLK/. When the printer responds with status, it asserts SBSY/ (Status BuSY) and sends the status data out over STS/, clocked by PCLK/.

## LPI Specifications

A summary of the important specifications for the Laser Printer Interface is contained in Table 16.

Table 16. Laser Printer Interface Specifications.

| Description | Specification |
| --- | --- |
| Maximum paper width | 11 inches |
| Maximum engine speed | 8 ppm |
| Maximum resolution | 600 dpi |
| Maximum top margin | 255 lines |
| Maximum left margin | 1020 pixels |
| Maximum line scan width | 8,160 pixels |
| Maximum video data rate | 8 Megahertz |
| Maximum left margin variation | 1/8 pixel |
| Maximum demand DMA data rate to FIFO | 25 Megabytes per second |

## Block Diagram

Figure 10 is a simplified block diagram of the laser printer interface. This diagram shows the blocks contained on the board. Sheet  2 of the LPI schematic shows the details of the ISA interface and data bus buffering. The data FIFO takes take from the ISA interface at 16-bits but data is removed from the FIFO by the FPGA 8 bits at a time. The FIFO and Altera LPI FPGA are shown on schematic sheet 3. The interface buffering and signal conditioning for driving the video interface over a cable is on sheet 4 of the schematic.

Figure 10. Simplified Block Diagram.

Figure 11 is a more detailed block diagram of the LPI. This diagram mixes the board sections with the various blocks within the FPGA. It provides a map for all of the blocks within the FPGA. Each FPGA block has a name:SYMBOL format where name is the name of the particular instantiation of a symbol and SYMBOL is the symbol name. The symbol name is identical to the file name of the symbol (without extension.) The top level symbol is LPI, which contains all of the blocks shown in the diagram.

Figure 11. Laser Printer Interface Functional Block Diagram.

## LPI FPGA Functional Description

The LPI FPGA is implemented using an Altera EPM7160. The design was done entirely in Altera Hardware Description Language (AHDL). The top level design file is LPI.TDF. This module provides interface to the pins and interconnection between the various blocks within the design. The lpi symbol includes four blocks, the lpi_decode, lpi_ecmd, lpi_estat and lpi_video. The lpi_video block itself is made up of five blocks as can be seen in the detailed block diagram, Figure 11. The following sections describe the functions of each of the blocks.

### Engine command and status interface

The command/control interface is a low-speed, synchronous serial interface to the print engine, as described in section 3.1.2. The LPI has parallel to serial and serial to parallel conversion registers for the command / control interface. The Elentari, therefore, writes commands to an eight-bit port, waits for the command to be sent to the engine and then reads the resulting status response returned from the engine by reading an eight-bit status port. An interrupt request can be asserted by the LPI when the status response is available so that the Elentari need not do a polled or programmatic wait for the status response.

The engine command output is implemented in the ENG_CMD.TDF file. The engine status input is implemented in the file ENG_STAT.TDF.

### Video interface

The video interface is a 16-bit DMA FIFO interface. Although the average video data transfer rate should not be a problem for the Elentari DMA bus bandwidth, the peak rates on high-speed, high-density laser printer engines will put too stringent a requirement on DMA latency to be practical. (The speed of the Elentari DMA has sufficient speed margin to support fast video rates, but other devices in the system could cause the latency to be the speed limiting factor). Fortunately, a substantial portion of the horizontal trace time is required for non-video transfer (beam re-trace, overhead, etc.) Therefore, a single scan line buffer reduces the peak DMA data rate requirement to the average video data rate. A 512 by 16-bit FIFO on the LPI provides adequate buffering for up to eleven inch wide paper at 600 dots per inch (dpi). The LPI is intended to use Elentari single or demand transfer with buffer chaining. The LPI, along with the FIFO status signals, manages the DMA requests so that the Elentari Byte Count Register can be programmed for up to 64 Kilobytes of DMA transfers between CPU interventions. DMA terminal count (TC) present on a LPI DMA cycle will generate an LPI interrupt request.

The video interface, LPI.TDF, includes three programmable one-shot counters (top_counter, left_counter and width_counter), the command and status registers, video data shift register (vdo_shifter) and beam detection (beam_detect).

#### Beam Detect and video clock generation

The beam_detect instantiation of BEAMDTCT.TDF does the clock generation and synchronization to the BD/ signal from the printer. The source file contains a simple timing diagram showing the operation of this circuitry. The primary purpose is to divide the master time base (clock_x8) by eight to produce the clock for shifting the video data to the printer (vid_clk). The video clock must be synchronized to the asynchronous BD/ signal and start data shifting with a repeatable resolution no worse than the master time base. This is required to provide proper alignment of the printed pixels on the page per the specifications of the printer. This circuitry also produces the signal vid_bol as a synchronized BD/ to communicate video beginning of line to other interface modules.

#### Video data shift register and counter

The vdo_shifter instantiation of VDO_SHF8 loads data from the FIFO, eight bits at a time and shifts the data out to the printer. The operation of this circuit is documented in the timing diagram in the design file. This circuitry shifts out the data and alternately loads the shift register from the low and high byte FIFOs.

#### Programmable counters

The position and width of the laser printer raster scan is controlled by three counters on the LPI. A programmable register is provided for each counter so that the registers only need to be set once and the counters will be automatically reloaded as required. The counters are implemented as programmable digital one-shots. There are two sizes, eight bits and ten bits, but both sizes are identical in operation.   The top_counter and left_counter are both eight bit counters while the width_counter is ten bits. The design can be studied by looking at either design file. They each include simple timing diagrams to show how they operate. The register/counters perform the following functions:

- Top Margin (top_counter:PRGCNT8) - This counter controls the number of Beam Detect (BD/) occurrences after the top of the page (VSREQ/ or TOP/) until video data will begin being shifted out. This is an eight bit counter that can provide a top margin of from 1 to 255 beam detect signals.

- Left Margin (left_counter:PRGCNT10) - This counter controls the time from the left margin (occurrence of BD/) until the start of video data output for each scan line. This is an eight bit counter that sets the left edge delay in increments of four pixels. Thus the left margin delay can be from 4 to 1020 pixels in width.

- Scan width (width_counter:PRGCNT10) - This counter controls the width of the video data output for each scan line in increments of 16 pixels, i.e. in the number of 16-bit words to be shifted from the FIFO to the video data stream. This is a ten bit counter, so that the number of words per line can be set from 1 to 1024.

### Address decode

The decode block, lpi_decode:DECODE decodes the ISA address bus for selection of interface registers. The base address is determined by the adr_select code from the board jumpers as can be seen in the DECODE.TDF file.

## Programmer’s Interface

| Control of the printer control signals, printer status signals and interrupt configuration are managed by the printer command and status registers (note these are printer command and status registers in contrast to the engine command and status registers described above). The command register in this interface controls the printer control signals PRNT/ (PRiNT) and CPRDY/ (Controller Power ReaDY). The status register reports the printer status signals PPRDY/ (Printer Power ReaDY), RDY/ (ReaDY) and either VSREQ/ (Video Sync REQuest) or TOP/ (Top Of Page), depending on which print engine is used. The printer command / status interface also controls the enabling of interrupt requests, interrupt acknowledgment and interrupt identification. In addition to the interrupt request from the engine status response and the DMA terminal count, assertion of VSREQ/ (or TOP/) can cause an interrupt request to be asserted. CNTL | Interrupt Enables |  |  | RFU | CPRDY | PRINT | PRNT |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ENGINE | TC | TOP |  |  | ENABLE |  |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

Figure 12. LPI Command Register

Figure 12 shows the command register bit definitions. PRNT and CPRDY directly drive the printer interface signals. PRINT ENABLE enables the FPGA circuitry to print. The interrupt enables are for top of page (TOP/ leading edge occurred), terminal count for a DMA block (TC) and engine status available (ENGINE). The interrupt requests are acknowledged (cleared) by clearing the respective interrupt enable. The CNTL bit controls the method of writing bits to the command register. The following describes the effect of bit 7, CNTL, on the command word:

- CNTL = 0, bits 6..0 of the command word will write bits 6..0 of the command register.

- CNTL = 1, a single bit is set or cleared. Data bits 3..0 are used as a bit address and must be in the range of 0..6. The data value in bit 4 will be the data written to the addressed bit. E.g., a command value of 0x91 would set the PRINT ENABLE bit in the command register without effecting any other bits.

Figure 13 shows the bits of the LPI Status register. Bits 0, 1 and 2, RDY, PPRDY and TOP reflect the current values of the respective printer interface signals. STATUS READY is high to indicate that there is an engine status value in the engine control status register. Bits 4-6 represent the interrupt request of the respective interrupt sources, qualified by the proper interrupt enable.

| The programmed I/O for the LPI takes eight contiguous I/O port addresses. The base address of the I/O port address is set by three jumpers to provide eight possible locations. The three pairs of jumper locations for W4 control the base address and the settings are summarized in Table 17. RFU | Interrupt Request |  |  | STATUS | TOP | PPRDY | RDY |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ENGINE | TC | TOP | READY |  |  |  |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

Figure 13. LPI Status Register

Table 17. LPI W4 Jumper Base I/O Address.

| 1-2 | 3-4 | 5-6 | BASE | 1-2 | 3-4 | 5-6 | BASE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| X | X | X | 220 | - | X | X | 320 |
| X | X | - | 230 | - | X | - | 330 |
| X | - | X | 260 | - | - | X | 360 |
| X | - | - | 270 | - | - | - | 370 |

The interface ports accessed by the software are shown in Table 18. The printer command and status register at base+0 are the registers described above. The Engine Control Command and Status registers at base+1 are the interface to the engine serial command and status. The counters for top, left and width are at base+2, base+3 and base+6, respectively.

Normally, data write to the FIFO is by DMA. This interface provides program I/O writes to the FIFO for testing and debug purposes.

Table 18. LPI Port Definitions

| Port Address | Size | Operation | Description |
| --- | --- | --- | --- |
| base + 0 | 8-bits | Write | Printer Command Register |
|  |  | Read | Printer Status Register |
| base + 1 | 8-bits | Write | Engine Control Command |
|  |  | Read | Engine Control Status |
| base + 2 | 8-bits | Write | Video Top Margin Register |
| base + 3 | 8-bits | Write | Video Left Margin Register |
| base + 4 | 16-bits | Write | Video Data FIFO input (also target for 16-bit DMA) |
| base + 6 | 16-bits | Write | Scan Width Register |

An interrupt request signal is jumper selectable from the LPI. Table 19 shows the LPI interrupt request jumper selection, W3.

Jumpers are provided to select which DMA channel to use. Consistent with standard ISA conventions, only the upper DMA channels are used for the sixteen-bit DMA operations. The possible DMA channels that can be used are thus channel 5, 6 or 7. These are controlled by W1 and W2, both of which must be set the same. Table 20 shows the W1 and W2 settings.

Table 19. LPI W3 Interrupt Request Selection.

| 1-2 | 3-4 | 5-6 | IRQ |
| --- | --- | --- | --- |
| X | - | - | 12 |
| - | X | - | 11 |
| - | - | X | 10 |

Table 20. LPI W1 and W2 Jumper DMA Settings.

| 1-2 | 3-4 | 5-6 | DMA |
| --- | --- | --- | --- |
| X | - | - | 7 |
| - | X | - | 6 |
| - | - | X | 5 |

# Scanner Interface

## Overview

The scanner interface is a custom ISA adapter board that interfaces the demo board to a ScanFX scanner. The ScanFX is a simple multi-page, color scanner with keyboard and LCD display supplied by Plustek, Inc.. Plustek also supplies an interface board with the scanner. The custom interface board is a superset of the Plustek supplied interface board. Both the custom interface board and standard ScanFX interface board provide programmed I/O eight bit command and data interface to the scanner. The custom interface board additionally supports eight-bit DMA of scan data from the scanner to the demo board DRAM. This section provides the details of the custom interface board.

## Schematic

The ScanFX interface is a simple 8-bit data path to the scanner. The one page schematic shows the ISA interface (P1 & P2), configuration jumpers (W1-W4), buffers (U2 & U3), the control FPGA (U1) and the interface connector (J1). The SCANFX FPGA, documented in the SCANFX.TDF and SCANFX.PIN files, provides I/O address decode and timing control for access to the scanner. The FPGA contains a timing shift register, shift_reg[5..1], that holds the bus using IOCHRDY to create 500 ns. bus cycles. Additionally, the strobes to the interface are conditioned to minimize bus drive overlap and provide additional data hold time.

A DMA interface has been added from the original ScanFX board. Until operation in a system has been examined, the DMA strategy can not be determined, thus the current FPGA design never asserts DMA request.

## Programmer’s Interface

The scanner interface consists of two basic interfaces: the programmed I/O and the DMA. The programmed I/O takes eight contiguous I/O port addresses. The base address of the I/O port address is set by three jumpers to provide eight possible locations. The three pairs of jumper locations for W3 control the base address and the settings are summarized in Table 21.

Table 21. ScanFX W3 Jumper Base I/O Address.

| 1-2 | 3-4 | 5-6 | BASE | 1-2 | 3-4 | 5-6 | BASE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| X | X | X | 220 | - | X | X | 320 |
| X | X | - | 230 | - | X | - | 330 |
| X | - | X | 260 | - | - | X | 360 |
| X | - | - | 270 | - | - | - | 370 |

The interface ports accessed by the software are shown in  Table 22. Details of the use of the ports and programming the ScanFX are contained in Appendix ?.

Table 22. Scanner Interface Port Definitions

| Port Address | Description |
| --- | --- |
| base + 0 | Data port |
| base + 1 | Command port for write Status port for read |
| base + 3 | Clear Interrupt Request |

An interrupt request signal is jumper selectable from the ScanFX. Table 24 shows the ScanFX interrupt request jumper selection, W3.

Jumpers are provided to select which (if any) DMA channel to use. Consistent with standard ISA conventions, only the lower four DMA channels are used for the eight-bit DMA operations. The possible DMA channels that can be used are thus channel 0, 1, 2 or 3. These are controlled by W1 and W2, both of which must be set the same. Table 23 shows the W1 and W2 settings.When enabled, the scanner SCANACK  initiates DMA reads from the scanner interface Data port.

Table 24. ScanFX W4 Interrupt Request Selection.

| 1-2 | 3-4 | 5-6 | IRQ |
| --- | --- | --- | --- |
| X | - | - | 12 |
| - | X | - | 11 |
| - | - | X | 10 |

Table 23. ScanFX W1 and W2 Jumper DMA Settings.

| 1-2 | 3-4 | 5-6 | 7-8 | DMA |
| --- | --- | --- | --- | --- |
| X | - | - | - | 0 |
| - | X | - | - | 1 |
| - | - | X | - | 2 |
| - | - | - | X | 3 |

# Appendix A

ScanFX Programming Documentation

Scanfx is a scanner machine but add with a fax keyborad and LCD, but Scanfx

didnt combine with a Data/Fax Modem, Scanfx didnt come with a printer.

At the this time scanfx will be more cheaper, and for the scanfx feature

Scanfx is a passive component, that mean Scanfx do not done job by themself

, they only pass the keyboard data/command to computer or receive data/command

form computer and show the data to the LCD screen.

The scanplus series communication will no longer be use in here. The new

bidirection method will be develop ,we develop a ESC sequences command call

The command call is divide in three part. one part is character transmit

second part is data/command from pc, the third part is data/command from scanfx

PART 1 ------ The Character pass between PC and Scanfx

a) The character pass from PC to Scanfx, before write character into Scanfx,

PC must be read status to see the busy flag and status indicator

if Scanfx is ready for receive the character or not.

when Scanfx is ready for receive data, you can pass the character into and

wait 100 us, (because Scanfx need time to wake up interrupt and set

busy flag to status) then PC can read status again to see if PC can write

data again.

THe status of Scanfx will be like this:

Bit 0	Busy flag; 1 is busy and 0 is ready

Bit 1	Fast flag; 1 is Fast move ready

Bit 2	Shading ready =1

Bit 3	Color ready =1

Bit 4	1 Data port data ready . 0 Data port not ready

Bit 5	Paper Indicater, 1 is paper in; 0 is paper out

Bit 6	0 ; Scanfx power ON and status indicator

Bit 7	1 ; Scanfx power ON and status indicator

Bit 4 use in no IRQ card

When PC receive an interrupt, and find the status port data is status

then it is read image data request.

b) The character pass from Scanfx to PC, at the scanplus we do not have

character pass from Scanfx to PC, so it need redesign. In the Scanfx side

we have status port but we dont have ststus port in PC side, and they

do not have channel to pass data from Scanfx to PC, So when Scanfx need

pass character to PC it will be like this.

Scanfx send a interrupt to PC and put the data to status port, when PC

wakeup it will read the status data and send back a 0 (binary) to Scanfx

To finish this process, in the Scanfx side when receive 0 will return status

data to status port, and wait 100 us to retrigger again (if have any data

need send again).

This procedure have a lot trouble, first status port have 4 usage not one,

So it have to distinguish is data or status, data can be

sure that bit 7 is 0 and for status is 1, bit 6 is 1 and status is 0

Bit 0	Data bit 0

Bit 1	Data bit 1

Bit 2	Data bit 2

Bit 3	Data bit 3

Bit 4	Data bit 4

Bit 5	Data bit 5

Bit 6	1 ; Scanfx power on and data indicator

Bit 7	0 ; Scanfx power on and data indicator

NOTE: When PC receive interrupt and find the status port is status, then it

mean scanner is ask for receive scanner image.

PART 2 ------ The data/command string from PC to Scanfx

They have some command that ask trans data/command to Scanfx

a) ESC C Command

Format:

ESC C command_code [parameter] 0

Specification:

ESC C A 0 Zero		;Line-art Mode Setting

ESC C A 1 Zero		;Grayscale Mode Setting

ESC C A 2 Zero		;Color Mode Setting

ESC C B xxx Zero	;Thresholded Data Type Setting (0 - 255)

ESC C X 0 Zero		;X Resolution 75 DPI Setting

ESC C X 1 Zero		;X Resolution 100 DPI Setting

ESC C X 2 Zero		;X Resolution 150 DPI Setting

ESC C X 3 Zero		;X Resolution 200 DPI Setting

ESC C X 4 Zero		;X Resolution 300 DPI Setting

ESC C Y xxx Zero	;Y Resolution 1 - 600 DPI Setting

ESC C O xxxx Zero	;Window X Position Setting

ESC C F xxxx Zero	;Window Y Position Setting

ESC C E xxxx Zero	;Window X Extend Setting

ESC C V 0 Zero		;Non-inverse Image Setting

ESC C V 1 Zero		;Inverse Image Setting

ESC C I Zero		;Reset Command Setting (Not Eject Paper)

ESC C R Zero		;Stop Command Setting (Eject Paper)

ESC C Z Zero		;Software Reset Scanner

ESC C D Zero		;Show ScanFx Title

ESC C G x Zero		;Step Control For Stop Line (0 - 9)

ESC C H Zero		;Do Shading

ESC C J xxx Zero	;ADF Step Extend Setting (-63 - +63)

ESC C K xxx Zero	;Paper Step Extend Setting (-60 - +60)

ESC C L 0 Zero		;Turn Lamp Off

ESC C L 1 Zero		;Turn Lamp On

ESC C N C Zero		;Close Data Bank

ESC C N x Zero		;Open Data Bank (0 - 7)

;	ESC C N O xxx Zero	;Open Data Bank (0 - 255)

ESC C U Zero		;Beep Sound Setting

ESC C T 5 Zero		;Test Mode For ADF Function Setting

ESC C W xxx Zero	;Dark Level Setting (0 - 255)

ESC C C Zero		;Reserved

ESC C M Zero		;Reserved

ESC C P Zero		;Reserved

ESC C Q Zero		;Reserved

ESC C S Zero		;Reserved

b) Scanning Command

Format :

Command_code

Specification:

*			;Request data & move one line

,			;Request data

+			;Move default steps then request data

c) Inquire Command

Format:

ESC I 0

Specification:

ESC I Zero		;inquire general purpose status

Different to Scanplus status port, the inquire is go to as command set,

The return of inquire will be show on the status port of Scanfx.

Bit 0

Bit 1

Bit 2

Bit 3	1 - ADF ready

Bit 4

Bit 5	0 ; inquire indicator

Bit 6	0 ; Inquire indicator

Bit 7	0 ; Inquire indicator

and a 0 command will set status port to return status

d) ESC V Command

Format:

ESC V 0

Specification:

ESC V Zero

Inquire model no and version

The return of version will be like this

Bit 0	Bit 3:0 is version no.

Bit 1

Bit 2

Bit 3

Bit 4	Model no; Bit 5:4 is 00 Gray; 01 is Color

Bit 5	Model no

Bit 6	1; Version indicator

Bit 7	1; Version indicator

e) Set Time & Date

Format:

ESC T parameter1 [parameter2] 0

Specification:

ESC T 0 YYYY Zero	;Set Year to LCD

ESC T 1 MM/DD Zero	;Set month & date to LCD

ESC T 2 HH:MM Zero	;Set hour & minute to LCD

ESC T 3 SS Zero 	;Set second to LCD

ESC T 4 Zero		;Show time to LCD

ESC T 5 Zero		;Not to show time

f) Show Time & Date

Format:

ESC S parameter 0

Specification:

ESC S 0 Zero		;Not to show time

ESC S 1 Zero		;Show time to LCD

g) Show message on LCD panel

Format:

ESC M parameter [message] 0

Specification:

ESC M 0 xxxxxxxx Zero

ESC M 1 xxxxxxxx Zero

ESC M 2 xxxxxxxx Zero

ESC M 3 xxxxxxxx Zero

0,1 indicate LCD line 1 first 8 char,and second 8 char

2,3 indicate LCD line 2 first 8 char,and second 8 char

The message length can not over 8 character, otherwise Scanfx will

see it as error command.

PART 3 ---- The Data/Command, scanner data from Scanfx to PC

There have two part of data from Scanfx to PC, one is scanner data, one is

keyboard data or command from Scanfx

1) The scanner data: when inquire step line, PC will wait scanner return

scanner data, so when scanner line complete, The scanner will trigger

PC to ask PC to read data, at this time the status port bit 4 will set

to high.

2) The Data/Command: The major Data/command is from panel keyborad.

so we will descript keyboard first, and all the data bit(7:6)=0:1

Group 1      1	  2    3

4	  5    6

7	  8    9

*	  0    #   wait key (for internation dial use)

Group 2      Start	Copy

Redial	Short dial

fine/normal/laser mode

Stop	Eject

Group 3      Fun 1

Fun 2

Fun 3

Fun 4

Fun 5

FAX		  EQU	  41H

COPY		  EQU	  42H

REDIAL	  EQU	  43H

PATTERN	  EQU	  44H

MODE		  EQU	  45H

STOP		  EQU	  46H

FAX_REC_EN	  EQU	  47H

MANU_REC	  EQU	  48H

F1		  EQU	  50H

F2		  EQU	  51H

F3		  EQU	  52H

F4		  EQU	  53H

F5		  EQU	  54H

F6		  EQU	  55H

F7		  EQU	  56H

F8		  EQU	  57H

F9		  EQU	  58H

F10		  EQU	  59H

F11		  EQU	  5AH

F12		  EQU	  5BH

F13		  EQU	  5CH

F14		  EQU	  5DH

3) There have some inquire data

TIME_INQUIRE	EQU	10H

when PC receive this command will return a new time/date to scanner

All of those command must be send back to PC, For communication easy;

every keyboard press will not process and direct send back to PC

So the procedure will be like this, When keyboard press except group one

and short dial there will no response on LCD,(in short dial the LCD will

response S and group 1 will response the DATA.) and the keypad data will

let Scanfx trigger PC and put data on status port the data will be 0- 9,

A- Z. The status bit (6:7)= 1:0 default.

Chapter 2    API ommunication Device driver

The major application use with API is COPY, and FAX function.

COPY function is more easy than FAX, that because COPY is not a real time

function but FAX is.

The API application have to resident, otherwise they can go direct through

Device driver. When the application resident first they must be LINK with

Device driver to get the API link entry and put set API entry for Device

driver. they have two entry set, one is COPY and one is FAX.

For COPY function when resident they have to set a mark in device driver

and put application entry in device driver COPY entry. so when Scanfx

send a copy command to device driver, device driver will send this information

through entry to COPY application.

The initialize of COPY function will be like this:

open HPSCAN device driver

send a COPY entry to device driver (ESC*c1Axxxx)

inquire a COPY entry (ESC*s1Z) (ESC*s1Zvxxxx)

close HPSCAN device driver

When application got a COPY command, the COPY ap will respond a

1) Install mark, install send entry, install receive entry

2) Receive COPY command

3) Set receive data to COPY entry

4) Show '1 Fast copy, 2 Time share copy' to know which kind copy

5) Receive 1 or 2 mode

6) Check printer port ready or not

7) Check InDOS and wait InDOS

8) Do the copy function

a) Open Device driver

b) Read a line and send to printer

c) Check if STOP command come or not if yes to (f)

d) Check scanner paper out or not if not to (b)

e) Check if next page waiting if yes to (b)

f) Close device driver

9) Reset receive data to copy entry, and set to FAX entry

1) Install mark, install send entry, install receive entry

2) Receive telephone no, put in queue, when 'start' search data base

if this telephone no is in data base, if yes then show company

name

2')Receive short dial and two digit, search data base and show

telephone no and company name, and set a 'BEEP' sound

3) Receive 'Start' command, if start come with telepone no

then will be autosend, if start not come with telephone no it

will be manual send, in the manual send the LCD will show

'Press START key again to make sure manual send'

4) Check modem is ready for send, if 'not' show error message in

LCD, change interrupt vector to application

5) After handshake is finish, set scanner receive entry to fax

function (for real time recive data)

a) Set command to device driver to change interrupt to fax entry

b) Open 2 block memory to device driver, the block size min is 1k

c) The block memory structure will be like this

Byte 0 - 0ffh indicate that this block finish fill with data

Byte 1 - how many line fill in this data

Byte (3:2) - Bytes per line

Byte 4 - 0ffh is paper out indicator (to next page)

Byte 4 to end is data area

d) When handshading finish send title data to fax (tel no, company name

and CSI), and open first block to fill data

e) Send finish with title, and open second bolck, and compress first

block

f) If fax card buffer is busy, then back to forground

g) Wait until fax card ready again (by interrupt), if one block send

finish then turn this block to be ready to receive data, set byte 0

to be 0.

h) Each block finish must be check if 'paper out' show on byte 4,

if yes then need to check with device driver about 'if have any

next page on scanner', and do somthing on fax card.

i) When a block is release must enable scanner interrupt again, to make

sure scanner can scan in this block