prmbrn

NAME

prmbrn	- loads object file to tty01 in dataio ascii format

SYNOPSIS

prmbrn objfile [ > port ]

DESCRIPTION

sends file in m.out or w.out format to /dev/tty01 or

specified format in dataio promburner format. Sizing

for 2708, 2716, 2732 and 2764. Unused bytes sent as

0xff. If object file too large for one prom file is

sent in promsize pieces.

EXAMPLE

make 2732 proms from m.out

prmbrn m.out

prom type? 2732

set prom burner to load, hit <cr> when ready

( set up dataio )

end of prom; program part; hit <cr> when done

( program prom )

insert new prom; set burner for load

( set up dataio )

done