********************************************************************

*                                                                  *

*          INSTALLATION  INSTRUCTIONS FOR                          *

*          THE XEBEC CORP. HARD DISK DRIVER                        *

*          CP/M-80 (R)       S100 BUS HOST COMPUTER                *

*                                                                  *

********************************************************************

REVISION 2.0   FEBRUARY 28, 1983

THE XEBEC HARD DISK DRIVER SOFTWARE PACKAGE CONSISTS OF FOUR

FILES SUPPLIED ON THE DISTRIBUTION DISKETTE.  THESE ARE

"HDINSTL.DOC", "HDFMT.ASM", "HDLINK.ASM", AND "HDDVR.ASM".

HDINSTL.DOC (THIS FILE) IS ALL THE DOCUMENTATION NECESSARY

FOR THE USER TO INSTALL THE DRIVER ON HIS S100 BUS SYSTEM.

HDFMT.ASM IS A UTILITY PROGRAM WHICH ENABLES THE USER TO

FORMAT THE HARD DISK AND INITIALIZE THE DIRECTORY AREA.

THE FILES HDLINK.ASM AND HDDVR.ASM ARE THE FILES WHICH

ENABLE THE USER TO GENERATE THE HARD DISK DRIVER PROGRAM.

THE DRIVER OPERATES BY LINKING ITSELF TO THE EXISTING FLOPPY

DISK BIOS.  THIS IS ACCOMPLISHED BY FINDING THE FLOPPY BIOS

JUMP TABLE AND PATCHING THE ENTRY POINTS FOR ALL DISK OPERATIONS.

THE USER SHOULD ONLY RUN "HDBOOT" ONCE PER COLD BOOT OF

THE SYSTEM, DUE TO THE NATURE OF THE PATCHING PROCESS.

THE ADVANTAGE OF THIS DRIVER OVER THE CONVENTIONAL

TYPE OF DRIVER IS THAT THE USER IS NOT REQUIRED TO CREATE

A CUSTOM BIOS TO GET HIS HARD DISK SUBSYSTEM RUNNING.

BY SIMPLY EDITING A FEW VARIABLES INTO THE SOURCE

FILES AND ASSEMBLING THEM, THE DRIVER IS READY TO OPERATE

ON THE USER'S SPECIFIC SYSTEM.

FOLLOWING ARE THE STEPS NECESSARY TO CREATE A DRIVER

TAILORED TO THE USER'S SYSTEM.

1.  FOR A SINGLE HARD DISK SUBSYSTEM, A 1K BYTE BLOCK OF

MEMORY MUST BE CREATED ABOVE CP/M FOR THE DRIVER PROGRAM.

FOR A TWO DRIVE SYSTEM ( C & D ), A 2K BYTE BLOCK OF

MEMORY MUST BE CREATED ABOVE CP/M FOR THE DRIVER PROGRAM.

THIS IS NORMALLY DONE BY A CP/M SYSGEN.

2.  EDIT THE DESIRED STARTING ADDRESS FO THE DRIVER

INTO THE FILE "HDLINK.ASM".  THIS WILL BE THE

BEGINNING ADDRESS OF THE BLOCK CREATED IN STEP 1.

THE VARIABLE NAME IS "START".

A.  IF A TWO DRIVE SYSTEM ( C & D ) IS DESIRED,

REMOVE THE COMMENTS IN "HDLINK.ASM" LOCATED

BETWEEN THE DOUBLE ROWS OF '$' NEAR THE END.

3.  EDIT THE FOLLOWING VARIABLES INTO THE FILE "HDDVR.ASM":

A. DESIRED STARTING ADDRESS OF THE DRIVER (THIS MUST

BE THE SAME AS THE ADDRESS IN "HDLINK.ASM").THE

VARIABLE NAME IS "START".

B. THE HOST ADAPTOR BASE ADDRESS.  THE VARIABLE NAME

IN THE FILE IS "BASE".  THE FACTORY STANDARD

IS 80H...IF A DIFFERENT BASE ADDRESS IS DESIRED

THE ADDRESS SWITCHES ON THE HOST ADAPTOR AND THIS

EQUATE WILL NEED TO BE CHANGED.

C. IF YOUR SYSTEM HAS A Z80 CPU, SETTING THE EQUATE

Z80 TO TRUE AND WILL ALLOW THE DRIVER TO USE THE

Z80 BLOCK MOVE INSTRUCTIONS.

D. IF A TWO DRIVE SYSTEM ( C & D ) IS DESIRED,

REMOVE THE COMMENTS IN "HDDVR.ASM" IN FIVE PLACES

BETWEEN THE DOUBLE ROWS OF '$'.

4.  EDIT THE HOST ADAPTOR BASE ADDRESS INTO THE FILE

"HDFMT.ASM".  AGAIN, THE FACTORY STANDARD IS 80H.

THE VARIABLE NAME IN THE FILE IS "BASE".

5.  ASSEMBLE HDFMT, HDLINK, AND HDDVR USING THE CP/M "ASM" PROGRAM.

6.  CREATE HDFMT.COM USING THE CP/M "LOAD" PROGRAM.

7.  TO CREATE THE "HDBOOT.COM", DO THE FOLLOWING:

A. INSURE THAT DDT.COM IS ON THE SAME DISKETTE

AS THE FILES "HDLINK.HEX" AND "HDDVR.HEX".

B. TYPE:

DDT HDLINK.HEX

C. WHEN DDT RETURNS WITH THE - PROMPT, TYPE:

IHDDVR.HEX

RXXXX

WHERE XXXX IS DETERMINED BY THE FORMULA:

XXXX=FC00H - DESIRED STARTING ADDRESS (H) + 800H

EXAMPLE:

STARTING ADDRESS F800

XXXX=FC00H - F800H + 800H

XXXX=0C00H

D. TYPE CTRL-C TO RETURN TO THE CP/M CONSOLE

COMMAND PROCESSOR.

E. TYPE:

SAVE 8 HDBOOT.COM

YOU NOW HAVE THE DRIVER PROGRAM UNDER THE NAME "HDBOOT.COM"

8.  FORMAT THE HARD DISK USING THE UTILITY PROGRAM "HDFMT".

9.  WHEN THE HARD DISK HAS BEEN SUCESSFULLY FORMATTED, YOU

ARE READY TO INSTALL THE DRIVER.

10. TYPE:

HDBOOT

THIS WILL INSTALL THE DRIVER FOR THIS SESSION.

EACH TIME YOU COLD BOOT THE SYSTEM, YOU WILL

NEED TO RUN THE "HDBOOT" PROGRAM.  REMEMBER,

ONLY DO THIS ONCE PER COLD BOOT.

(R)   CP/M IS A REGISTERED TRADEMARK OF DIGITAL RESEARCH.