Adding New I/O Device Drivers to CDOS

This  disk  now contains the source file to the CDOS I/O

drivers (called  Drivers.Z80).   This  may  be  used  in

conjunction  with the Batch file, Drivers.CMD, to easily

modify drivers for devices connected to CDOS.   This  is

done in several easy steps.

First,  edit  the  original  source file into a new file

called Mydriver.Z80.  (Insert a copy of Assembler Disk B

in drive A and a blank, initialized and labeled disk  in

drive  B,  and  type "Screen Drivers.Z80 Mydriver.Z80".)

Make  the  changes  you  desire  to   this   new   file,

Mydriver.Z80.   Many  changes may be performed simply by

changing the EQU's at the beginning of the source.

For example, if the console to which CDOS  is  connected

is  a  Model  3101  rather  than  a  Model 3102, the I/O

drivers can be changed to reflect this by  changing  the

definition  of  "C3102"  in  the  source  to "FALSE" and

"C3101" to "TRUE".  Model 3100 terminals may be selected

by changing both "C3102" and "C3101" as above,  as  well

as changing "FUN.KEYS" to "FALSE".

Once Mydriver.Z80 has been changed as  you  desire,  you

may exit from SCREEN and start the Batch process.  To do

this   the   Batch  file,  Drivers.CMD,  must  first  be

transferred (using Xfer) to the disk in drive B.   Next,

insert  a  copy  of Assembler Disk A in drive A and type

"@/B B:Drivers" to execute the Batch file on the disk in

drive B.  The source file, Mydriver.Z80,  will  then  be

assembled  twice  and  relocated when executing CDOSGEN.

Following the completion of this process, you will  have

new  I/O  drivers in your CDOS.  To add these drivers to

any copies of CDOS you make from now on, simply type the

last line of the Batch file:

CDOSGEN Myd0.hex Myd100.hex

The  ability  to change the CDOS I/O drivers has several

uses.  First, it is a convenient way to remove  portions

of  CDOS in order to make it occupy less machine memory.

Second, it  allows  you  to  write  custom  drivers  for

non-standard  I/O  devices  and  be able to access these

through CDOS.  Third, it is possible  to  have  the  I/O

drivers  make  a decision on which of several devices to

access according to the condition of the CDOS I/O Byte.

An example of this is contained in the file Drivers.Z80.

Two printers,  one  serial  and  one  parallel,  may  be

connected  to CDOS by specifying both the labels "C3703"

and "S.PRINTER" as "TRUE", and  the  label  "NO.LST"  as

"2";  then  reassembling  and  relocating the drivers as

already described.

The program STAT (version 02.09 or higher) may  then  be

used  to  select one of these two printers by one of the

following commands:

STAT PRT:=0	(or STAT PRT:=PAR:)

STAT PRT:=1	(or STAT PRT:=SER:)

Other multiple devices may be accessed through  CDOS  by

first  changing the I/O drivers as above, and then using

STAT to change the I/O Byte.  The configurations allowed

by STAT are as follows:

STAT dev:=n:

where dev: = CON:, RDR:, PUN:, or PRT: and n = 0-7, 0-3,

0-1, or 0-3, respectively.  The actual bit format of the

CDOS I/O Byte is:

Bits 0,1,2 are assigned to CONsoles 0 through 7

Bits 3,4 are assigned to ReaDeRs 0 through 3

Bit 5 is assigned to PUNches 0 and 1

Bits 6,7 are assigned to PRinTers 0 through 3