.5ML v`

.c 0f

\[Directory`Formats\]

\[Standard`Floppy`Disk`(CP/M`1.4`style)\] (32 bytes)

.0c

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | A | B | C | D | E | F |

|---------------------------------------------------------------|

FCB |DSK| m*| - | - | - | - | p*| u*| r*| w*| s*| a*| - | - | - | - |

DISK|ATT|<- - - - -FILE NAME- - - - - ->|<- -TYPE ->|EX | 0 | 0 |RC |

|---------------------------------------------------------------|

|                                                               |

|<- - - - - - - - - -ALLOCATION DISK MAP- - - - - - - - - - - ->|

|---------------------------------------------------------------|

|10 |11 |12 |13 |14 |15 |16 |17 |18 |19 |1A |1B |1C |1D |1E |1F |

\[ATT\]		- File attributes (on disk)

Bits:   \[|`7```6```5```4`|`3```2```1```0`|\]

\[|`p```w```r```u`|`s```a```0```F'|\]

Meanings if set (to 1)

p: permanent		s: system file

w: write protected	a: file has been archived

r: read protected	0: clear bit, not used

u: user file		F': not a file

m: fcb modified

*	    - Bits set in FCB by 17/18 calls or open.  These

attributes have the same meaning as the bits of the

same name above, and are the MSB (80H) of the bytes shown.

\[DSK\]		- 1 Byte (in FCB)

0	- current drive

1..15	- drive A: through drive O:

\[FILE`NAME\]	- 1-8, 8 characters

\[FILE`TYPE\]	- 9-B, 3 characters

7 bit ASCII file name right filled with spaces.

\[EX\]		- Extent number

Each extent uses one directory entry and can have

16Kbytes allocated to it.

\[RC\]		- Record Count

Records (128 bytes each) in this extent containing

written data. This number should be < 80H only in the

last extent, and equal to 80H in all but the last extent.

\[Allocation`Disk`Map\]

Up to 16 cluster pointers, declaring up to 16K bytes

of file, in cluster sized increments.  They are each

one byte (1..254)

0	- Not allocated

255	- cluster 0 (directory)

1..254	- cluster 1 through 254

The first cluster is 0, the directory.

#########################################################################################