.c

\[Standard`Floppy`Label\]

(If present, must be first entry in directory)

.0c

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | A | B | C | D | E | F |

|---------------------------------------------------------------|

|81H|<- - - - -DISK LABEL - - - - ->|<- -RSVD ->|CS |DF | 0 |RD |

|---------------------------------------------------------------|

|<- - - - - - - - - -DIRECTORY DISK MAP - - - - - - - - - - - ->|

|---------------------------------------------------------------|

|10 |11 |12 |13 |14 |15 |16 |17 |18 |19 |1A |1B |1C |1D |1E |1F |

\[81(hex)\]	- Disk label indicator

\[Disk`Label\]	- Up to 8 characters, right filled

with spaces.  Characters are 7 bit ASCII.

\[RSVD\]	- Reserved

\[CS\]		- Cluster size (records per cluster)

\[Density	CS	CS(HEX)	Kbytes`per`cluster\]

Single Density	8	08H		1K

Double Density	16	10H		2K

Double Density	32	20H		4K

Double Density	64	40H		8K

Double Density	128	80H		16K

\[RD\]		- Records in directory (8-63 allowed)

Single Density	16	64 entries

Double Density	32	128 entries

\[DF\]		- Directory type flags

Bits:	\[|`7```6```5```4`|`3```2```1```0`|\]

\[|`-```d```-```-`|`-```-```-```-`|\]

d: double sided disk

################