\[File_Allocation_Schema_-_Big_Directory\]

\[Disk Structure\]

Same as floppy, except:

3 bytes for size, max.

\[Disk Cluster\]

Same as floppy except:

- Up to 65534 clusters

- Numbered 1 to 65534

\[Cluster Pointer\]

Same as floppy,

(Note 24 bit precsion requires for calculations)

\[Directory\]

- "n" disk blocks (LABEL.RD) each containing

2 entries of 64 bytes each,

0<n<255 (in cluster increments)

- First entry is a label, with first byte of 81(hex)

- Second entry is the file "MAP.DIR"

- Each subsequent entry is either free (first byte

0E5(hex) or describes a file.

- A \[small file\] (ATT.B=0) has cluster pointer to

data clusters

- A \[big file\] (ATT.B=1) has cluster pointers to

index cluster

\[Data Cluster\]

A cluster on the disk, containing data

\[Index Cluster\]

A cluster on the disk, containing cluster pointers.

########################################################################################################