.c

\[Details`of`BIG`DIR`bitmap\]

.0c

\[BIG`DIR`BITMAP`DEFINITION`FILE:\]

ORG	0

BM$FG:	DS	1	; FLAG BYTES

;	80	LAST RECORD IN BIT MAP FILE

;	40	NO FREE BLOCKS IN THIS RECORD

BM$ST:	DS	2	; STARTING CLUSTER FOR THIS RECORD

BM$EN:	DS	2	; ENDING CLUSTER FOR THIS RECORD

BM$CT:	DS	1	; NUMBER OF BYTES OF BITS

BM$BT:	DS	96	; 96 (FIRST REC) OR 120 (ALL OTHER)

BM$TC:	DS	2	; TOTAL NUMBER OF CLUSTERS (REC 0 ONLY)

BM$CS:	DS	1	; CLUSTER SIZE (RECORDS)   (REC 0 ONLY)

BM$RS:	DS	1	; RESERVED BITS AT START   (REC 0 ONLY)

BM$DT:	DS	3	; M-D-Y CREATED/FIXED      (REC 0 ONLY)

DS	18	; TO FILL RECORD TO 128

BM$CK:	DS	1	; CHECKSUM (XOR OF ALL FROM 0 TO 127)=00

BM$BT*/0006  	BM$CK*/007F  	BM$CS*/0068  	BM$CT*/0005

BM$DT*/006A  	BM$EN*/0003  	BM$FG*/0000  	BM$RS*/0069

BM$ST*/0001  	BM$TC*/0066

\[Pictorial`Description`of`Bit`Map\]

+-----------------------------------------------+

| flag(1) start(2) end(2) count(1)		|

| allocation bit map:				|

|    1st block	- 96 bytes			|

|    others blocks	- 120 bytes		|

| control fields	- first block only	|

|    total(2)	csize(1)	reserved(1)	|

|    date(3)			filler(17)	|

| free(1)					|

| checksum(1)					|

+-----------------------------------------------+

\[Sample`Output`of`BMPDUMP\]

InfoSoft BIG Disk Bitmap Dump (BMPDUMP 00.01)

Bitmap for unit E:

Total clusters: 7FF8H

Cluster Size:   08H

Reserved Bits:  15H

Creation Date:  00/00/00

Map Block: 01H Flag byte: 40H

Start Cluster: 0001H End Cluster: 0300H

0000 FF FF FF FF FF FF FF-FF FF FF FF FF FF FF FF FF

0010 FF FF FF FF FF FF FF-FF FF FF FF FF FF FF FF FF

0020 FF FF FF FF FF FF FF-FF FF FF FF FF FF FF FF FF

0030 FF FF FF FF FF FF FF-FF FF FF FF FF FF FF FF FF

0040 FF FF FF FF FF FF FF-FF FF FF FF FF FF FF FF FF

0050 FF FF FF FF FF FF FF-FF FF FF FF FF FF FF FF FF

Checksum: 00H

###############################################################