M680X MACRO CROSS-ASSEMBLERS

USER GUIDE

Alan R. Baldwin

Physics Department

Kent State University

Kent, Ohio  44242

PAGE 2

MACRO Package for MACRO-11 to Assemble MOTOROLA 680X Code,

Version:  September 1983.

Author:  Alan R.  Baldwin

Kent State University Department of Physics

Operating System:  RT-11 V4.0

Source Language:  MACRO-11

Abstract:  This package contains four(4) sets of MACRO-11 macros

to interpret MOTOROLA 6800(6802 and 6808), 6801(6803), 6805, and

6809 mnemonics.  As is normal in MACRO-11, the output  may  con-

tain  two  files;   one  file  contains  the binary image of the

object in .LDA format, the other contains the  assembly  listing

of  the  assembled  program.  A program called LISTER reads .LDA

format files and prints the binary data in  Octal,  Decimal,  or

Hex  formats.   A second program, HEXDCM, can be used to process

the .LST file to  produce  Decimal  or  Hex  listings  from  the

MACRO-11 generated Octal listing.

Several 6800 and 6809 programs are included as coding exam-

ples,  these  include:  (1) LOADER, a 'DEC' .LDA format absolute

binary loader for the 6800, (2) 6821A, a general purpose MOTORO-

LA  6821  PIA  I/O  handler  (6800  code),  (3) 8250A, a general

purpose NATIONAL  INS8250  ACE  serial-port  I/O  handler  (6800

code), (4) MC6845, a general purpose MOTOROLA 6845 CRT controll-

er handler (6809  code),  and  (5)  NS8250,  a  general  purpose

NATIONAL INS8250 ACE serial-port handler (6809 code).

Documentation on magnetic media.

M680X MAGNETIC MEDIA DIRECTORY                                    PAGE 3

M68   .MAC     1  09-Aug-80     Conditional file for assembly of

M680X.MAC files

END   .MAC     1  09-Aug-80     File containing .END directive

M6800 .MAC    23  08-Aug-83     6800 (6802  and  6808)  MACRO-11

CROSS-ASSEMBLER

M6801 .MAC    24  08-Aug-83     6801       (6803)       MACRO-11

CROSS-ASSEMBLER

M6805 .MAC    19  08-Aug-83     6805 MACRO-11 CROSS-ASSEMBLER

M6805T.MAC     1  06-Aug-83     6805 CROSS-ASSEMBLER test file

M6809 .MAC    39  08-Aug-83     6809 MACRO-11 CROSS-ASSEMBLER

M6809T.MAC     9  08-Feb-83     6809 CROSS-ASSEMBLER test file

HEXDCM.MAC    18  07-Sep-79     .LST  file  processor   converts

Octal listings to Decimal or Hex

listings

LISTER.MAC    20  07-Sep-79     .LDA  format  file  listing   in

Octal, Decimal, or Hex

LOADER.MAC     6  12-Apr-83     Absolute  binary  loader   (6800

code)

6821A .MAC    29  16-Apr-82     MOTOROLA 6821 PIA handler  (6800

code)

8250A .MAC    34  31-Mar-83     NATIONAL 8250 ACE handler  (6800

code)

MC6845.TBL     2  03-Sep-83     Setup table  for  MOTOROLA  6845

CRTC

MC6845.SML     3  16-Feb-83     System  macro  calls   for   the

M6845.MAC file

MC6845.MAC    11  16-Feb-83     MOTOROLA 6845 CRTC hanlder (6809

code)

NS8250.TBL     2  03-Sep-83     Setup table  for  NATIONAL  8250

ACE

NS8250.SML     5  16-Feb-83     System  macro  calls   for   the

NS8250.MAC file

NS8250.MAC    35  16-Apr-83     NATIONAL 8250 ACE handler  (6809

code)

M680X .RNO    29  05-Sep-83     RUNOFF version of this document

M680X .DOC    32  05-Sep-83     This listing

21 Files, 343 Blocks

Using the M680X MACRO-11 CROSS-ASSEMBLERS                         PAGE 4

Each of  the  M680X  cross-assemblers  contains  a  set  of

MACRO-11  macros  which  define the mnemonics of that particular

microprocessor.   The  general  format  for  using   the   M680X

cross-assemblers is:

.R MACRO

*FILE(.OBJ),FILE(.LST)=M680X(.MAC),FILE(.MAC)

where the default file types are indicated in paranthesis.   The

absolute  binary  output file is in the .LDA format (even though

the extension is .OBJ).  As an example, the file  LOADER.MAC  is

assembled and listed by the following input sequence:

*LOADER,TT:=M6800,LOADER,END

The file  M68.MAC  contains  the  definition  .M68=1  which

allows  the  listing of the cross-assembler macros during assem-

bly, if .M68 is not defined  then  the  cross-assembler  is  not

listed  or  subtitled  by  MACRO-11 during assembly (this is the

default mode).

NOTE

The M6800 and M6801 cross-assemblers always default to

the  direct  addressing  mode  if a label has not been

defined before it is used by an instruction.  i.e.:

LDA A   VRBL

.

.   other coding

.

.=1000

VRBL:   .BYTE   0

Because the varaible VRBL does  not  have  an  address

between  0  and  377  (octal), phase errors will occur

during pass 2 of MACRO-11.  (Caused by a 2-byte  allo-

cation  for  LDA A   VRBL  during  pass 1 and a 3-byte

allocation during pass 2.) The solution is  to  always

define  direct  variables  before  they  are  used  in

instructions.

For the 6805 and 6809 cross-assemblers the direct mode

is  used  only  if the (*) argument is specified.  The

variables may be defined at any  position  within  the

source program.

M6800 MACRO-11 CROSS-ASSEMBLER                                    PAGE 5

;**************************************************************

;*							      *

;*	MACRO PACKAGE FOR THE MOTOROLA 6800 MICROPROCESSOR    *

;*		TO RUN UNDER MACRO 11.			      *

;*							      *

;*		6800, 6802, AND 6808 PROCESSORS SUPPORTED     *

;*							      *

;*							      *

;*	BY ALAN R. BALDWIN				      *

;*		PHYSICS DEPARTMENT			      *

;*		KENT STATE UNIVERSITY			      *

;*		KENT, OHIO  44242			      *

;*							      *

;*		V03 - OCTOBER 1980			      *

;*							      *

;**************************************************************

;

;

;THE FOLLOWING DIFFERENCES EXIST BETWEEN THIS CROSS-ASSEMBLER

;AND MOTOROLA'S M6800 ASSEMBLER

;	LABELS MUST TERMINATE WITH A :

;	COMMENTS START WITH A ;

;	IMMEDIATE MODE IS DENOTED BY A SEPERATE ARGUMENT - #

;

;

;DEFINITION OF ASSEMBLER DIRECTIVES WITH DIFFERENCES

;	END - USE	.END	END OF PROGRAM

;	EQU - USE  	=	EQUATE SYMBOL

;	FCB			FORM SINGLE-BYTE CONSTANT

;				NO MORE THAN 10 ARGUMENTS

;	FCC   <ASCII CHRS>	FORM CONSTANT CHARACTERS

;	FDB			FORM DOUBLE-BYTE CONSTANT

;				NO MORE THAN 10 ARGUMENTS

;	MON - NOT IMPLEMENTED	RETURN TO MONITOR CONSOLE

;	NAM - USE	.SBTTL	PROGRAM NAME

;	OPT - NOT IMPLEMENTED	OPTION

;	ORG			ORIGIN

;	PAGE - USE	.PAGE	ADVANCE LISTING TO TOP OF PAGE

;	RMB			RESERVE MEMORY BYTES

;	SPC - NOT IMPLEMENTED	SPACE N LINES

;

;

;PROCESSOR CONDITION CODE REVIEW

;	0 - CARRY BIT (C)

;	1 - OVERFLOW BIT (V)

;	2 - ZERO BIT (Z)

;	3 - NEGATIVE BIT (N)

;	4 - INTERRUPT MASK BIT (I)

;	5 - HALF CARRY BIT (H)

;	6 - ALWAYS 1

;	7 - ALWAYS 1

;

M6801 MACRO-11 CROSS-ASSEMBLER                                    PAGE 6

;**************************************************************

;*							      *

;*	MACRO PACKAGE FOR THE MOTOROLA 6801 MICROPROCESSOR    *

;*		TO RUN UNDER MACRO 11.			      *

;*							      *

;*							      *

;*		6801 AND 6803 PROCESSORS SUPPORTED	      *

;*							      *

;*							      *

;*	BY ALAN R. BALDWIN				      *

;*		PHYSICS DEPARTMENT			      *

;*		KENT STATE UNIVERSITY			      *

;*		KENT, OHIO  44242			      *

;*							      *

;*		V01 - OCTOBER 1980			      *

;*							      *

;**************************************************************

;

;

;THE FOLLOWING DIFFERENCES EXIST BETWEEN THIS CROSS-ASSEMBLER

;AND MOTOROLA'S M6801 ASSEMBLER

;	LABELS MUST TERMINATE WITH A :

;	COMMENTS START WITH A ;

;	IMMEDIATE MODE IS DENOTED BY A SEPERATE ARGUMENT - #

;

;

;DEFINITION OF ASSEMBLER DIRECTIVES WITH DIFFERENCES

;	END - USE	.END	END OF PROGRAM

;	EQU - USE  	=	EQUATE SYMBOL

;	FCB			FORM SINGLE-BYTE CONSTANT

;				NO MORE THAN 10 ARGUMENTS

;	FCC   <ASCII CHRS>	FORM CONSTANT CHARACTERS

;	FDB			FORM DOUBLE-BYTE CONSTANT

;				NO MORE THAN 10 ARGUMENTS

;	MON - NOT IMPLEMENTED	RETURN TO MONITOR CONSOLE

;	NAM - USE	.SBTTL	PROGRAM NAME

;	OPT - NOT IMPLEMENTED	OPTION

;	ORG			ORIGIN

;	PAGE - USE	.PAGE	ADVANCE LISTING TO TOP OF PAGE

;	RMB			RESERVE MEMORY BYTES

;	SPC - NOT IMPLEMENTED	SPACE N LINES

;

;

;PROCESSOR CONDITION CODE REVIEW

;	0 - CARRY BIT (C)

;	1 - OVERFLOW BIT (V)

;	2 - ZERO BIT (Z)

;	3 - NEGATIVE BIT (N)

;	4 - INTERRUPT MASK BIT (I)

;	5 - HALF CARRY BIT (H)

;	6 - ALWAYS 1

;	7 - ALWAYS 1

;

M6805 MACRO-11 CROSS-ASSEMBLER                                    PAGE 7

;**************************************************************

;*							      *

;*	MACRO PACKAGE FOR THE MOTOROLA 6805 MICROPROCESSOR    *

;*		TO RUN UNDER MACRO 11.			      *

;*							      *

;*							      *

;*	BY ALAN R. BALDWIN				      *

;*		PHYSICS DEPARTMENT			      *

;*		KENT STATE UNIVERSITY			      *

;*		KENT, OHIO  44242			      *

;*							      *

;*		V01 - AUGUST 1983			      *

;*							      *

;**************************************************************

;

;

;THE FOLLOWING DIFFERENCES EXIST BETWEEN THIS CROSS-ASSEMBLER

;AND MOTOROLA'S M6805 ASSEMBLER

;	LABELS MUST TERMINATE WITH A :

;	COMMENTS START WITH A ;

;	IMMEDIATE MODE IS DENOTED BY A SEPERATE ARGUMENT - #

;	DIRECT MODE IS DENOTED BY A SEPERATE ARGUMENT - *

;

;

;DEFINITION OF ASSEMBLER DIRECTIVES WITH DIFFERENCES

;	END - USE	.END	END OF PROGRAM

;	EQU - USE  	=	EQUATE SYMBOL

;	FCB			FORM SINGLE-BYTE CONSTANT

;				NO MORE THAN 10 ARGUMENTS

;	FCC   <ASCII CHRS>	FORM CONSTANT CHARACTERS

;	FDB			FORM DOUBLE-BYTE CONSTANT

;				NO MORE THAN 10 ARGUMENTS

;	MON - NOT IMPLEMENTED	RETURN TO MONITOR CONSOLE

;	NAM - USE	.SBTTL	PROGRAM NAME

;	OPT - NOT IMPLEMENTED	OPTION

;	ORG			ORIGIN

;	PAGE - USE	.PAGE	ADVANCE LISTING TO TOP OF PAGE

;	RMB			RESERVE MEMORY BYTES

;	SPC - NOT IMPLEMENTED	SPACE N LINES

;

;

;PROCESSOR CONDITION CODE REVIEW

;	0 - CARRY BIT (C)

;	1 - ZERO BIT (Z)

;	2 - NEGATIVE BIT (N)

;	3 - INTERRUPT MASK BIT (I)

;	4 - HALF CARRY BIT (H)

;	5 - ALWAYS 1

;	6 - ALWAYS 1

;	7 - ALWAYS 1

;

M6809 MACRO-11 CROSS-ASSEMBLER                                    PAGE 8

;**************************************************************

;*							      *

;*	MACRO PACKAGE FOR THE MOTOROLA 6809 MICROPROCESSOR    *

;*		TO RUN UNDER MACRO 11.			      *

;*							      *

;*							      *

;*	BY ALAN R. BALDWIN				      *

;*		PHYSICS DEPARTMENT			      *

;*		KENT STATE UNIVERSITY			      *

;*		KENT, OHIO  44242			      *

;*							      *

;*		V01 - JUNE 1980				      *

;*							      *

;**************************************************************

;

;

;  SPECIAL NOTES ABOUT THE CROSS ASSEMBLER

;

;	LABELS MUST TERMINATE WITH A COLON (:)

;	COMMENTS START WITH A SEMI-COLON (;)

;	IMMEDIATE MODE IS DENOTED BY A SEPERATE ARGUMENT - #

;	DIRECT MODE IS DENOTED BY A SEPERATE ARGUMENT - *

;	DEFFERRED MODE IS DENOTED BY A SEPERATE ARGUMENT - @

;

;

;DEFINITION OF ASSEMBLER DIRECTIVES WITH DIFFERENCES

;	END - USE	.END	END OF PROGRAM

;	EQU - USE  	=	EQUATE SYMBOL

;	FCB			FORM SINGLE-BYTE CONSTANT

;				10 ARGUMENTS ALLOWED

;	FCC   <ASCII CHRS>	FORM CONSTANT CHARACTERS

;	FDB			FORM DOUBLE-BYTE CONSTANT

;				10 ARGUMENTS ALLOWED

;	MON - NOT IMPLEMENTED	RETURN TO MONITOR CONSOLE

;	NAM - USE	.SBTTL	PROGRAM NAME

;	OPT - NOT IMPLEMENTED	OPTION

;	ORG			ORIGIN

;	PAGE - USE	.PAGE	ADVANCE LISTING TO TOP OF PAGE

;	RMB			RESERVE MEMORY BYTES

;	SPC - NOT IMPLEMENTED	SPACE N LINES

;	SETDP	ADDRESS		SETS ASSEMBLER DIRECT PAGE VARIABLE

;

;

;PROCESSOR CONDITION CODE REVIEW

;	0 - CARRY BIT (C)

;	1 - OVERFLOW BIT (V)

;	2 - ZERO BIT (Z)

;	3 - NEGATIVE BIT (N)

;	4 - INTERRUPT MASK BIT (I)

;	5 - HALF CARRY BIT (H)

;	6 - FAST INTERRUPT REQUEST MASK BIT (F)

;	7 - ENTIRE STATE ON STACK BIT (E)

;

M6809 MACRO-11 CROSS-ASSEMBLER                                    PAGE 9

;

;	REGISTERS

;

;	A -	8 BIT ACCUMULATOR A

;	B -	8 BIT ACCUMULATOR B

;	D -	16 BIT ACCUMULATOR D (A:B)

;	CC -	8 BIT CONDITION CODE REGISTER CC

;	DP -	8 BIT DIRECT PAGE REGISTER DP

;	X -	16 BIT INDEX REGISTER X

;	Y -	16 BIT INDEX REGISTER Y

;	U -	16 BIT USER STACK POINTER U

;	S -	16 BIT HARDWARE STACK POINTER S

;	PC -	16 BIT PROGRAM COUNTER PC

;

;	ADDRESSING MODES

;

;	LABEL IS AN ASSEMBLER LABEL OR

;	 A NUMBER

;

;	SYNC			INHERENT

;	CLRA			ACCUMULATOR

;	CLRA			ACCUMULATOR

;	TFR	D,Y		REGISTER

;	LDA	LABEL		ABSOLUTE (EXTENDED)

;	LDA	*,LABEL		DIRECT PAGE

;	LDA	@,LABEL		ABSOLUTE INDIRECT (EXTENDED INDIRECT)

;	LDA	5,X 		CONSTANT-OFFSET INDEXED

;	LDA	LABEL,PCR	ASSEMBLER PC RELATIVE

;	LDA	@,5,X 		CONSTANT-OFFSET INDEXED INDIRECT

;	LDA	@,LABEL,PCR	ASSEMBLER PC RELATIVE INDIRECT

;	LDA	A,X		ACCUMULATOR INDEXED

;	LDA	@,A,X		ACCUMULATOR INDEXED INDIRECT

;	LDA	X+		AUTO-INCREMENT (1)

;	LDA	X++		AUTO-INCREMENT (2)

;	LDA	@,X++		AUTO-INCREMENT (2) INDIRECT

;	LDA	-X		AUTO-DECREMENT (1)

;	LDA	--X		AUTO-DECREMENT (2)

;	LDA	@,--X		AUTO-DECREMENT (2) INDIRECT

;	BRA	LABEL		RELATIVE

;	LBRA	LABEL		RELATIVE

Using the LISTER and HEXDCM programs                             PAGE 10

LISTER

;****************************************************************

;*								*

;*		  A PROGRAM TO LIST .LDA FORMAT BINARY		*

;*		FILES IN OCTAL, DECIMAL, OR HEXIDECIMAL		*

;*		FORMATS.					*

;*								*

;*		FORMAT:						*

;*		 DEV:FILE.EXT	/O	OCTAL(DEFAULT)		*

;*				/D	DECIMAL			*

;*				/H	HEXIDECIMAL		*

;*								*

;*				/B	BYTE FORMAT (DEFAULT)	*

;*				/W	WORD FORMAT		*

;*								*

;*								*

;*		WRITTEN BY - ALAN R. BALDWIN			*

;*				SEPT. 1979			*

;*								*

;****************************************************************

HEXDCM

;****************************************************************

;*								*

;*		  A HEX / DECIMAL CONVERSION PROGRAM		*

;*		TO TRANSLATE OCTAL LISTINGS FROM		*

;*		CROSS ASSEMBLERS WRITTEN IN MACRO-11.		*

;*		  THE LISTING FILE MUST HAVE BEEN LISTED	*

;*		IN THE FORM .NLIST  TTM (IE - LINEPRINTER)	*

;*		ONLY THE MACRO GENERATED BINARY 		*

;*		IS TRANSLATED TO HEX / DECIMAL.			*

;*								*

;*		FORMAT:						*

;*		 DEV:FILE.EXT = DEV:FILE.EXT/H/D		*

;*		 (DEFAULT=.HEX) = (DEFAULT=.LST)		*

;*				  (DEFAULT=/H)			*

;*								*

;*								*

;*		WRITTEN BY - ALAN R. BALDWIN			*

;*				SEPT. 1979			*

;*								*

;****************************************************************

The programs LISTER and HEXDCM are supplied in source  form

and must be assembled and linked before they can be used.

#######