DYC - A Mapped RX03 Handler for TSX-plus

V2c 9-Sep-86

Author:  C G Wilson, 71 Galatea St, Charleville, Australia 4470.

DYC provides an RX03 (double-sided RX02) handler capable of  reading  or

writing  RX01  and  RX02  discs.   It  may  be used under RT-11, but was

primarily designed to run in high memory ("mapped") under TSX-plus, thus

saving low memory space.

This handler is designed for Sigma/Dilog RX03 controllers, with optional

assembly for MTI boards.  It may need minor modification (notably to the

extended addressing code) for other manufacturers' boards.

Full 22-bit addressing is used for either of these boards, removing  the

need for intermediate address buffers.

An SPFUN to allow formatting during  normal  system  operation  (without

requiring  access  to the I/O page) is included.  The handler may be run

as a  programme,  providing  the  formatting  options  with  no  further

programme being required.

DYC.TSX is designed for use as SY:DY.TSX, to replace the  DEC  or  S & H

DY.TSX on the system device.

DYC - Mapped RX03 Handler for TSX-plus                            Page 2

1.0  Assembly Options - DY$22B

If DY$22B is set non-zero  (default),  code  for  22-bit  addressing  is

included.  This unfortunately is controller dependent:

Sigma/Dilog DQ419:  (default)

22 bit addressing is enabled by setting the <cs22bit>  bit  in  the

command.   An  extra  <TR> phase is added to all cycles requiring a

memory access.  In this cycle the high-order address is  passed  to

the controller, as six bits right justified.  The bits <csma16> and

<csma17> are not used.

MTI:  (select if MTI$$ is set non-zero)

22 bit addressing is enabled by setting the <cs22bit>  bit  in  the

command.   An  extra  <TR> phase is added to all cycles requiring a

memory access.  In this cycle the  high  order  four  bits  of  the

address are passed to the controller, right justified.  Bits 16 and

17 still need to be set in the command word as  per  a  DEC  18-bit

controller.

DYC - Mapped RX03 Handler for TSX-plus                            Page 3

2.0  Formatting

An SPFUN (code 200) is provided to allow formatting, with the  following

parameters:

In Buffer:

First byte:

If zero, format side zero

If non-zero, format size one

Second byte:

If zero, use single density

If non-zero, use double density

Block Number:      If zero, use soft formatting (header rewrite)

If non-zero, use hard formatting

The Sigma/Dilog protocol is assumed.  I hope this goes for all

controllers:

Command:

10 + <20 if unit 1> + <400 if double dens> + <1000 for side 1>

Followed by (in data register)

111 for soft format, or 222 for hard format.

DYC - Mapped RX03 Handler for TSX-plus                            Page 4

3.0  Running DYC as a Programme

If DYC is run as a programme, it provides  formatting  capabilities  for

relatively  easy use, requiring no particular privileges under TSX-plus.

[If however you have users who may not access ".TSX" files on the system

device, make a copy of DYC.TSX called DY.SAV on SY:  and run that.]

This facility is best demonstrated by showing how to get the  HELP  text

for this programme:

.r dy.tsx

*/h

DY Commands:

/H (without any file) - Type this text

The device specified should be a DY floppy,

write-enabled.

/H - hard format;

/F - soft format;

/D - double density;

/S - both sides.

/S:0 - side 0 only (default);

/S:1 - side 1 only.

/I - initialize directory when formatted

(default 4 segments), or

/I:n for n segments (1 to 31.)

/T - tell size of disc (in blocks)

/Z - equivalent to /F/D/S/I

(/D and /S imply /F unless /H is specified.

*

Examples:

.r dy.tsx

*dy:/z/h

Hard formats an RX03 floppy and initializes it.

.r dy.tsx

*dy1:/t

Tells the size of the floppy in dy1:.

.r dy.tsx

*dy1:/d/i

Soft formats an RX02 floppy in dy1: and initializes it.

DYC - Mapped RX03 Handler for TSX-plus                            Page 5

4.0  TSMUNG - for modifying TSX plus

With the advent of TSX-plus version 6, no matter what  is  specified  in

the TSGEN file (eg DEVDEF <DY>,MAPH,HANBUF,DMA) TSX refuses to allow the

DY or the DU handler to be mapped into high memory.   This  is  fine  to

protect  people  trying  to do this with DEC or S & H handlers, but is a

nuisance when we have a legitimate handler capable of this performance.

A programme TSMUNG is provided.  This modifies the default values set up

for  the DU and/or DY devices.  It is run, using the name of the TSX.SAV

file to be modified appropriately.  To modify the DY value, append /Y to

the  filename.   (To modify the DU, append /U).  [No switches implies /U

only.]

.Run TSMUNG

*sy:tsx.sav/y

Successful TSX modification

.

or

.Run TSMUNG sy:tsx.sav/y

Successful TSX modification

.

If it should be desired to run the DU handler mapped in addition to  the

DY  handler (WARNING - again, make sure your handler is capable of being

run mapped and is not the standard DEC or S & H handler), append a /U to

the filename in addition to the /Y.  For example:

.Run TSMUNG du:tsx.sav/y/u

Successful TSX modification

.

If TSMUNG cannot find the relevant string of device  names  in  TSX,  it

will  tell  you  so.   If  this  is the case, you are probably trying to

operate upon a version of TSX different from that for which  TSMUNG  was

written (version 6.01).

#####################