######                                          MXV22

DISK CONTROLLER

### ###                                         MANUAL

PREFACE

The purpose of this manual is to provide the user adequate information

to configure and operate the MXV22 floppy disk controller.  The

information provided should clarify the controller connection to any

Shugart compatible drive and assist in the selection of associated

interface options.  Both register definition and command protocol are

provided for reference and as an aid in development of user software.

Operational procedures outline the use of the controller features as

well as explaining operation in an RT-11 or RSX11M software

environment.

SECTION 1

GENERAL

INFORMATION

1. INTRODUCTION         ____________

The MXV22 is a dual density controller compatible with the DEC* RX02

floppy disk system.  The MXV22 emulates two separate RXV21 controllers,

each of which controls two 8-inch floppy disk drives.  When configured

with any Shugart compatible drive, it is a direct replacement for two

complete RX02 subsystems.  The controller provides either single

density encoding compatible with IBM 3740 equipment or double density

encoding providing up to 1 Mbyte of storage on a single diskette.

All electronics are contained on one dual-height board which plugs

directly into any standard LSI-11 backplane and interfaces through a 50

conductor ribbon cable with up to four (4) Shugart compatible drives.

All controllers are 100% tested and ready for operation.  The primary

controller registers are configured for the standard device address

177170#U8#D and interrupt vector 264#U8#D.  The secondary controller registers

are configured with a device address of 177174#U8#D and interrupt level is

factory set to level four.  Features include:

.22 bit addressing

.Transparent firmware bootstrap automatically loads either single

or double density diskettes.

.Formatting capabilities permit writing sector headers, checking

the written headers, and writing the data fields in the user

selected density.

.Jumper selection of both alternate address and vector.

.Jumper selectable four-level device interrupt priority compatible

with the LSI-11/23.

.Provides power fail protection for data integrity.

.Write current control signal for tracks greater than forty-three.

.Write precompensation for reduced error rates.

*DEC, PDP-11 LSI-11, RSX, Unibus & Q bus are registered trademarks of

Digital Equipment Corporation.

1

1.1. COMPONENTS         __________

The controller is provided with the following components:

P/N 81010-07-2   Floppy disk controller

M 81010-02-2     MXV22 disk controller manual

1.2. COMPATIBILITY         _____________

This section discusses the aspects of hardware, software and media

compatibility with Digital Equipment's RX02 system.  The information

will aid the user in data interchanging with foreign systems.

### ###                                        Hardware### ###                                        ________

The controller is compatible with the LSI-11, LSI-11/2 and LSI-11/23

processors. All circuitry is contained on one dual-wide board that

plugs directly into any standard LSI-11 backplane.  Alternate address

selection and a four-level device interrupt priority scheme provide the

user added flexibility for expanded system configurations.  Shugart

compatible drive logic is interfaced through a 50-pin ribbon cable.

The connector pins are compatible with both the 800 and 850 series

drives.

### ###                                        Software### ###                                        ________

The MXV22 is completely compatible with RXV21 register definition and

command protocol.  All DEC-supplied software designed to operate with

the RX02 system will operate with the controller without modification.

Media                                          _____

The media used with the MXV22 are  compatible with the IBM 3740 family

of equipment.  Either preformatted or blank soft sectored diskettes may

be used with the controller.  The following list summarizes the

suggested media.

IBM                Single or Double Density

DEC                RX01/RX02

2

1.2.1. Logical Track Format         ____________________

The diskette surface is divided into 77 concentric tracks numbered

0-76.  Each track consists of 26 sectors numbered 1-26.  The track

begins and ends at the index address mark.  The track is formatted in

such a way that this "soft" index is preceded by the leading edge of

the physical index hole in the diskette.  Following the physical index

are 40 bytes of "FF" data, 6 bytes of "0" data, and the index address

mark indicating the beginning of the track.  Following the index

address mark is the post index gap consisting of 26 bytes of "FF" data

and 6 bytes of "0" data.  The next field is the sector header for

sector 1.  Following the data field is the data gap consisting of 28

bytes of "FF" data and 6 bytes of "0" data.  This field leads to the

next sector header.  Following the 26th data record is the pre-index

gap consisting of approximately 274 bytes of "FF" data.

Each track is formatted in the above manner.  Refer to Figure 1-1.  The

sector header field of each sector contains information describing the

sector number, track number, and diskette side.  All the above fields

are recorded in FM except as noted in the following sections.

Figure 1-1:  Logical Track Format

3

1.2.2. Sector Header Field         ___________________

The header field consists of 7 bytes of information.  Preceding the

header is a field of 6 bytes of "zero" data for synchronization.  The

header and this preamble are always recorded in FM.

1.   Byte 1.  ID Address Mark - A unique mark

consisting of 1 byte of FE (hex) data with

three missing clock-transitions using a C7

(hex) clock pattern.  This mark is decoded by

the controller and indicates the start of the

sector header.

2.   Byte 2.  Track Address - This byte indicates

the absolute (0-114#U8#D) track address.  Each

sector contains this track information to

locate its position on one of the 77 tracks.

3.   Byte 3.  Side - This byte indicates the

diskette side (0 or 1).

4.   Byte 4.  Sector Address - This byte indicates

the absolute (1-32#U8#D) sector address.  Each

sector contains this information to identify

its position on the track.

5.   Byte 5.  "Zero"

6.   Byte 6, 7.  CRC - This is the 16 bit cyclic

redundancy character and is calculated for

each header from the first 5 bytes of

information, using the IBM 3740 polynomial.

(Refer to Cyclic Redundancy Check, Section

1.2.7.).

1.2.3. Data Field         __________

The data field consists of either 131#U10#D or 259#U10#D bytes of information

depending upon the recording method.  Preceding the data field is a

field of 6 bytes of "zero" data for synchronization.

The preamble and data address mark are always written in FM.  The user

data and CRC character are either written in FM or MFM modified

depending upon the formatted diskette density.

4

1.   Byte 1.  Data Address Mark - A unique mark consisting of a

data byte (see Table 1-1) with three missing clock

transitions using a C7 (hex) clock pattern.  This byte is

always written in FM and is decoded by the controller to

indicate the start of the data field, its recording method

(FM vs MFM), and if the field is a deleted data field.

------------------------------------------------

|  ADDRESS    |  INDICATED  |  DATA   |  CLOCK   |

|   MARK      |   DENSITY   |         |          |

|------------------------------------------------|

|------------------------------------------------|

|  INDEX      |      NA     |   FC    |    D7    |

|------------------------------------------------|

|    ID       |      NA     |   FE    |    C7    |

|------------------------------------------------|

|             |      FM     |   FB    |    C7    |

|   DATA      |----------------------------------|

|             |   Modified  |   FD    |    C7    |

|------------------------------------------------|

|  DELETED    |      FM     |   F8    |    C7    |

|   DATA      |----------------------------------|

|             |   Modified  |   F9    |    C7    |

------------------------------------------------

Table 1-1:  Address Marks

2.   Bytes 2-129 (FM) or Bytes 2-257 (MFM Modified).  User Data.

This field is recorded in either FM or MFM modified.

Depending upon the encoding scheme, either 128 or 256 bytes

of information can be stored.

3.   Bytes 130-131 or 258-259.  CRC - This is the 16 bit cyclic

redundancy character and is calculated for each data field

from the first 129 or 257 bytes of information using the IBM

3740 polynomial.  (Refer to Cyclic Redundancy Check, Section

1.2.7.).  These bytes are recorded with the same encoding

scheme as the data field.

1.2.4. Recording Scheme         ________________

Two recording schemes are used by the MXV22:  double frequency (FM) and

DEC modified Miller code (MFM).  FM is used for single density

recording and is compatible with IBM 3740 or DEC RX01 media.  DEC

modified MFM is used for recording double density and is only

compatible with the DEC RX02 system.

5

1.2.5. Double Frequency (FM)         _____________________

FM recording is characterized by a flux transition at the beginning of

each bit cell which is commonly termed a clock pulse or transition as

shown in Figure 1-2.  A logic "one" is represented by a flux transition

withing the bit cell; a logic "zero" is represented by the lack of any

flux transition within a bit cell.  In FM the bit cell time is 4us.

"0"       "0"       "1"       "0"       "1"       "0"

_         _         _    _    _         _    _    _         _

| |       | |       | |  | |  | |       | |  | |  | |       | |

--   -------   -------   --   --   -------   --   --   -------   --

C         C         C    D    C         C    D    C         C

|   bit   |

-->|   cell  |<--

|   4us   |

Figure 1-2:  FM Recording Characteristics

1.2.6. DEC Modified MFM         ________________

MFM recording consists of flux transitions for a logic "one" and no

flux transitions for a logic "zero".  A clock transition only occurs

between two consecutive logic "zeros" as shown in Figure 1-3 below.

The MFM bit cell time is 2us.

"1"   "1"  "0"     "0"  "1"   "0"   "1"   "1"

_     _        _        _           _     _

| |   | |      | |      | |         | |   | |

---   ---   ------   ------   ---------   ---   ---

D     D        C        D           D     D

| bit |

-->| cell|<--

| 2us |

### ###                       Figure 1-3:  MFM Recording Characteristics

6

Table 1-2 summarizes the standard MFM encoding algorithm.

------------------------------------------------

|        DATA       |         ENCODED DATA       |

|------------------------------------------------|

|  DN-1        DN   |    DN-1     CN      DN     |

|------------------------------------------------|

|    0         0    |     0        1       0     |

|    1         0    |     1        0       0     |

|    0         1    |     0        0       1     |

|    1         1    |     1        0       1     |

------------------------------------------------

Table 1-2:  Standard MFM Encoding

Because single density headers are used for both FM and MFM

recording formats, and since certain MFM patterns map into single

density address marks, a modified algorithm is used.  The mapping

occurs when a data pattern of exactly four consecutive "ones" is

encoded.  Whenever this pattern is encoded a special algorithm is

applied.  Table 1-3 defines the encoding algorithm for this

special case.

-----------------------------------------------------------

|                           DATA                            |

|-----------------------------------------------------------|

|  DN-5   | DN-4    |  DN-3   |  DN-2   |  DN-1   |   DN    |

|-----------------------------------------------------------|

|    0    |   1     |    1    |    1    |    1    |   0     |

|-----------------------------------------------------------|

|  X   0  | 1   0   |  0   0  |  1   0  |  0   0  |  1  0   |

|CN-5 DN-5|CN-4 DN-4|CN-3 DN-3|CN-2 DN-2|CN-1 DN-1| CN  DN  |

|-----------------------------------------------------------|

|                       ENCODED DATA                        |

-----------------------------------------------------------

Table 1-3:  Modifying Algorithm (Special Case)

When reading double density data fields the controller checks for

a missing clock bit between two zero data cells, and if found,

substitutes ones for the two zero data bits (generated by the

special encoding algorithm).

1.2.7. Cyclic Redundancy Check         _______________________

Each sector header field and data field has two byte CRC character

appended.  This 16 bit character is the remainder that results when

dividing the data bits [represented as a polynomial M(x)] by a

generator polynomial G (x).  The polynomial used for IBM 3740 is G(x) =

X #D16#U+ X #D12#U+ X #D5#U+ 1.  For the sector header the data bits include byte 1

thru 5.  For an FM data field the data bits include byte 1 thru byte

129.  For an MFM data field the data bits include byte 1 thru byte 257.

7

1.3. SPECIFICATIONS         ______________

RECORDING TECHNIQUE:

Single Density       IBM 3740 FM

Double Density       DEC Modified MFM

POWER REQUIREMENTS:

Voltage              Single 5V supply

(from LSI-11 backplane)

Current              2.5A typical

ENVIRONMENTAL

Temperature          0 degree - 45 degrees C

Humidity             10% - 95% non-condensing

8

SECTION 2

INSTALLATION

2. GENERAL         _______

The controller is shipped with standard options configured.  The

standard address 177170#U8#D and vector 264#U8#D are set.  The device interrupt

priority is set to level four.  The firmware bootstrap is disabled.

Most options are factory foil-etched to the most often used

configuration.  The foil jumpers must first be cut before the alternate

jumpers are inserted.  Refer to Tables 2-1, 2-2, and 2-3 for alternate

options and Figure 2-1 for jumper location.  Several of the options are

selectable by using AMP 530153-2 pin jumpers.  If these pin jumpers are

not available use #30 wire wrap.

2.1. CONFIGURATION         _____________

2.1.1. Address Vector Selection         ________________________

The controller is shipped with the DEC standard device address and

vector assignments preset to 177170#U8#D and 264#U8#D, respectively.  Any

change in these assignments would necessitate a change in system

software.  However, a dual address and vector option is selectable

enabling a second register assignment at 177174#U8#D and 270#U8#D,

respectively.

The dual address mode is used in applications requiring more than two

drives.  In this case the controller simulates the operation of two

controller as required by standard DEC software.  To select this option

remove the jumper between W16 and W17.  Jumper W15 to W16 and jumper W7

to W8 as shown in Table 2-1.

9

--------------------------------------------------

|                   |           JUMPERS            |

|   OPTION          |------------------------------|

|                   |  15-16  |  16-17  |   7-8    |

|--------------------------------------------------|

|  Standard         |         |         |          |

|  Address/Vector*  |   OUT   |    IN   |   OUT    |

|  177170/264       |         |         |          |

|--------------------------------------------------|

|  Dual             |         |         |          |

|  Address/Vector   |    IN   |   OUT   |    IN    |

|  177170/264       |         |         |          |

|  177174/270       |         |         |          |

|--------------------------------------------------|

|  *Factory Preset                                 |

--------------------------------------------------

Table 2-1:  Address/Vector Option Configuration

2.1.2. Device Interrupt Priority         _________________________

The MXV22 supports the four-level device interrupt priority scheme

compatible with the LSI-11/23.  The controller asserts interrupt

requests and monitors higher level request lines during interrupt

arbitration as described in Table 2-2.  The level four request is

always asserted by the controller, regardless of its priority, to

maintain compatibility with the LSI-11 and LSI-11/2 processors.

The interrupt priority level is configured to level four at the

factory.  If a different interrupt level is desired the following

foil-etched jumpers must be cut.  Refer to Table 2-2 for the proper

jumpers to insert for the desired priority level.

W19 - W20          W28 - W29

W22 - W23          W30 - W31

W24 - W25

--------------------------------------------------------------------------

|PTY|ASRT |MON |________________________JUMPERS____________________________|                                             _______

|LEV|     |    |18-19|19-20|21-22|22-23|24-25|25-26|27-28|28-29|30-31|31-32|

|--------------------------------------------------------------------------|

| 4*|  4  |5,6 | Out | In  | Out | In  | In  | Out | Out | In  | In  | Out |

|--------------------------------------------------------------------------|

| 5 | 4,5 | 6  | Out | In  | In  | Out | In  | Out | Out | In  | Out | In  |

|--------------------------------------------------------------------------|

| 6 | 4,6 | 7  | In  | Out | Out | In  | Out | In  | In  | Out | In  | Out |

|--------------------------------------------------------------------------|

| 7 |4,6,7|None| In  | Out | In  | Out | Out | In  | In  | Out | Out | In  |

|--------------------------------------------------------------------------|

|  *Factory Preset                                                         |

--------------------------------------------------------------------------

### ###                           Table 2-2:  Priority Level Configuration

10

Factory Test

---------

\        /

\      /

---------------------------++++++++++++++++++++++++---

|                           ||||||||||||||||||||||||   |

|                                               W35    |

|                           W1 W2 W3             *     |

|                           *  *  *     *  *  *  *     |

|                                      W44 45 33 34    |

|                                                      |

|                                                      |

|                                               W6     |

|                                               *      | Factory

|                                               W5     | Test

|                                               *      |

|                                               W4     |

|                                               *      |

|                                                      |

22 Bit  |                   W7                                 |

Address |                   *                                  |

|            W47 48 W8                                 |

|             *  *  *                                  |

|                   *                                  |

|                   W9                                 |  Address

|                                                      | Selection

Step    |            W36 39                                    |

Rate    |             *  *                                     |

|             *  *                                     |

|            W37 38                                    |

|                                                      |  Write

|                                                      | Precom-

Boot-   |            W41        W15                            | pensation

Strap   |             *          *                 *  *  *     |

|              *  *  *  *                 W14 13 12    |

|             W42 43 17 16                             |

|                                                      |

|                                   W20 19 18          |

|                 W10                *  *  *           | Interrupt

TBS7    |                  *                 *  *  *           | Priority

|                  *                W23 22 21          |

|                 W11                                  |

|                                 W24 30 29            |

|                                  *  *  *             |

|                            _    W25 31 28            |

|_                        __| |    *  *  *            _|

|                      |    |    *  *  *           |

|                      |    |   W26 32 27          |

----------------------      ---------------------

Figure 2-1:  Configuration Jumper Locations

11

2.1.3. Bootstrap         _________

The controller board incorporates a transparent firmware bootstrap.

The bootstrap is initiated whenever program execution is started at

location 173000#U8#D, homing all drives to track 0.  Next, track 1, sector

1, of unit 0 is read and diskette density is determined.  If the

diskette is single density, sectors 1, 3, 5, and 7 are loaded into

memory starting at location 0. If the diskette is double density,

sectors 1 and 3 are loaded.  Program execution is then transferred to

location 0.  Controllers are shipped with this feature disabled.  To

enable the bootstrap remove the jumper between W42 and W43 and insert

the jumper between W41 and W42 as shown in Table 2-3.

NOTE                                  ____

Only one bootstrap should be enabled in a system for

proper operation.  If another bootstrap exists in the

system, it must be disabled before enabling the

controller bootstrap.

--------------------------------------------------

|                   |           JUMPERS            |

|  BOOTSTRAP        |------------------------------|

|                   |     41-42     |     42-43    |

|--------------------------------------------------|

|  ENABLED          |       IN      |       OUT    |

|--------------------------------------------------|

|  DISABLED *       |       OUT     |       IN     |

|--------------------------------------------------|

|  *Factory Preset                                 |

--------------------------------------------------

### ###                                   Table 2-3:  Bootstrap Option

2.1.4. Write Precompensation         _____________________

The MXV22 controller provides hardware write precompensation to reduce

the bit shift exhibited by all drives as the recorded flux density

increases.  The controller recognizes the patterns which produce bit

shift and precompensates the written pattern.  This unique feature

allows the controller to perform reliably with any Shugart compatible

drive.

12

Controllers are shipped with this feature enabled and it is recommended

that for more reliable operation the feature not be disabled.  However,

if so desired, the feature can be defeated by cutting the foil-etched

jumper between W12 and W13 and inserting a jumper between W13 and W14

as shown in Table 2-4.

--------------------------------------------------

|                   |           JUMPERS            |

|      WRITE        |------------------------------|

|  PRECOMPENSATION  |     12-13     |     13-14    |

|--------------------------------------------------|

|  ENABLED*         |       IN      |       OUT    |

|--------------------------------------------------|

|  DISABLED         |       OUT     |       IN     |

|--------------------------------------------------|

|  *Factory Preset                                 |

--------------------------------------------------

Table 2-4:  Write Precompensation

2.1.5. Step Rate Control         _________________

Up to four drives can be interfaced with the MXV22 controller.  These

drives are organized into two pairs, each of which is associated with a

particular register set.  The first pair of drives (DS1/DS2) is

associated with the primary register set (177170#U8#D) and the remaining

pair (DS3/DS4) with the secondary register set (177174#U8#D).  The

controller is shipped configured with a 6ms step rate for both pair of

drives.  Alternate step rates can be selected for each pair of drives.

Refer to Table 2-5 for desired step rate option and associated jumpers.

--------------------------------------------------

|   STEP RATE       |           JUMPERS            |

|-------------------|------------------------------|

| DS1/DS2 | DS3/DS4 |     36-37     |     38-39    |

|--------------------------------------------------|

|     6*  |    6*   |       IN      |       IN     |

|-------------------|---------------|--------------|

|     3   |    6    |       OUT     |       IN     |

|--------------------------------------------------|

|     6   |    3    |       IN      |       OUT    |

|-------------------|---------------|--------------|

|     3   |    3    |       OUT     |       OUT    |

|--------------------------------------------------|

|  *Factory Preset                                 |

--------------------------------------------------

Table 2-5:  Step Rate

13

2.1.6. 22 Bit Addressing         _________________

The controller is shipped with 22 bit addressing disabled.  Enabling

this option provides extended address control during DMA transactions

allowing the controller to transfer information throughout 22 bit

address space.  The additional four bits of address (A#U18#D-A#U21#D) are

communicated to the controller as described in section 3.1.2.  Before

enabling this option it is necessary to modify the corresponding

software drivers in order to maintain proper register communication as

described in section 5.  To enable 22-bit addressing jumper W46 and W47

as shown in Table 2-6.

-----------------------------------

|                   |    JUMPERS    |

|      22 BIT       |---------------|

|    ADDRESSING     |     46-47     |

|-------------------|---------------|

|  ENABLED          |       IN      |

|-------------------|---------------|

|  DISABLED *       |       OUT     |

|-----------------------------------|

|  *Factory Preset                  |

-----------------------------------

Table 2-6:  22 Bit Addressing

2.1.7. Miscellaneous Options         _____________________

There are several options related to factory configuration of the

controller.  These options must be configured as shown for proper

operation of the controller.  Refer to Table 2-7 for these options.

During DMA operations if the bus address established extends into the

peripheral address page the controller asserts bank select 7 (BS7) as

required by normal bus protocol.  If the application requires extend

memory overlapping the peripheral address page this option can be

disabled as indicated in Table 2-7.

---------------------------------------------------------------------

|   OPTION      |                       JUMPERS                        |

|               |------------------------------------------------------|

|               | 1-2  |  2-3  |  4-5  |  5-6  | 33-34 | 34-35 | 10-11 |

|----------------------------------------------------------------------|

|  FACTORY*     | OUT  |  IN   |  OUT  |  IN   |  OUT  |  IN   |   -   |

|----------------------------------------------------------------------|

|  BS7 ENABLED  |  -   |   -   |   -   |   -   |   -   |   -   |  IN   |

|----------------------------------------------------------------------|

|  BS7 DISABLED |  -   |   -   |   -   |   -   |   -   |   -   |  OUT  |

|----------------------------------------------------------------------|

|    * Factory Preset                                                  |

----------------------------------------------------------------------

Table 2-7:  Miscellaneous Options

14

2.2. DRIVE CONFIGURATION         ___________________

The controller provides an industry standard floppy interface

compatible with most available drives.  However, for proper operation,

each drive must be configured with attention to several options.

Tables 2-8 thru 2-12 summarize these options for several of the more

popular drives.

--------------------------------------------------------------------

|          |                            |       DUAL         SINGLE  |

|  OPTION  |      DESCRIPTION           | DRIVE 0   DRIVE 1  DRIVE 0 |

|--------------------------------------------------------------------|

|   DS1    | Drive select 1             |   IN        OUT      IN    |

|   DS2    | Drive select 2             |   OUT       IN       OUT   |

|   DS3    | Drive select 3             |   OUT       OUT      OUT   |

|   DS4    | Drive select 4             |   OUT       OUT      OUT   |

|   A      | Radial head loading option |   IN        IN       IN    |

|   B      | Radial head loading option |   IN        IN       IN    |

|   C      | Head load option           |   IN        IN       IN    |

|   D      | In use option              |   OUT       OUT      OUT   |

|   X      | Radial head loading option |   OUT       OUT      OUT   |

|   WP     | Inhibit write when protect |   IN        IN       IN    |

|   NP     | Allow write when protect   |   OUT       OUT      OUT   |

|   DS     | Stepper power from drive   |   IN        IN       IN    |

|          | select                     |                            |

|   HL     | Stepper power from head    |   OUT       OUT      OUT   |

|          | load                       |                            |

|   Z      | In use from drive select   |   OUT       OUT      OUT   |

|   Y      | In use from head load      |   IN        IN       IN    |

|   R      | Ready output               |   IN        IN       IN    |

|   I      | Index output               |   IN        IN       IN    |

|   DC     | Disk change output         |   X         X        X     |

|   S      | Sector output              |   X         X        X     |

|   800    | Sector option disable      |   IN        IN       IN    |

|   801    | Sector option enable       |   OUT       OUT      OUT   |

|   L      | -5V DC bias                |   IN        IN       IN    |

|   T1     | Termination HL             |   OUT       IN       IN    |

|   T2     | Termination drive select   |   IN        IN       IN    |

|   T3     | Termination direction      |   OUT       IN       IN    |

|   T4     | Termination step           |   OUT       IN       IN    |

|   T5     | Termination write data     |   OUT       IN       IN    |

|   T6     | Termination write gate     |   OUT       IN       IN    |

--------------------------------------------------------------------

Table 2-8:  Shugart 801/851 Drive Configuration

15

---------------------------------------------------------------------

|           |                             |       DUAL        SINGLE  |

|  OPTION   |      DESCRIPTION            | DRIVE 0  DRIVE 1  DRIVE 0 |

|---------------------------------------------------------------------|

|   U9      |Terminations for multiplexed |  IN        OUT      IN    |

|           |inputs                       |                           |

|   SI      |Internal write current switch|  X         X        X     |

|   SE      |External write current switch|  X         X        X     |

|   TR      |True ready output            |  IN        IN       IN    |

|   RTR     |Radial true ready **         |  IN        IN       IN    |

|   2S      |Two-sided status output*     |  IN        IN       IN    |

|   DC      |Disk change option           |  OUT       OUT      OUT   |

|   S1      |Side select option using     |  OUT       OUT      OUT   |

|           |direction select *           |                           |

|   S2      |Side select input *          |  IN        IN       IN    |

|   S3      |Side select option using     |  OUT       OUT      OUT   |

|           |drive select                 |                           |

|1B,2B,3B,4B|Side select option using     |  OUT       OUT      OUT   |

|           |drive select                 |                           |

|   D       |Alternate input-in-use       |  OUT       OUT      OUT   |

|   MS      |Motor on from drive select   |  IN        IN       IN    |

|   MO      |Alternate input-motor-on     |  OUT       OUT      OUT   |

|   MMO     |Alternate input-multiplexed  |  OUT       OUT      OUT   |

|           |motor on  **                 |                           |

|   MD      |Motor off delay              |  IN        IN       IN    |

|   R       |Ready output                 |  IN        IN       IN    |

|   RR      |Radial ready                 |  IN        IN       IN    |

|   SR      |Standard ready **            |  IN        IN       IN    |

|   MT      |Modified true ready (outputs |  OUT       OUT      OUT   |

|           |true ready on pin 22)**      |                           |

|   DS1     |Drive select 1               |  IN        OUT      IN    |

|   DS2     |Drive select 2               |  OUT       IN       OUT   |

|   DS3     |Drive select 3               |  OUT       OUT      OUT   |

|   DS4     |Drive select 4               |  OUT       OUT      OUT   |

|   Y       |Door lock/activity light     |  IN        IN       IN    |

|           |activated from motor on **   |                           |

|   Z       |Door lock/activity light     |  OUT       OUT      OUT   |

|           |activated from drive select**|                           |

|   PD      |Stepper power down           |  OUT       OUT      OUT   |

|   WP      |Inhibit write when write     |  IN        IN       IN    |

|           |protected                    |                           |

|   NP      |Allow write when write       |  OUT       OUT      OUT   |

|           |protected                    |                           |

|   TS      |Data separation option select|  OUT        OUT     OUT   |

--------------------------------------------------------------------

| X  Don't Care  * 860 Drive Only  ** Available on PCB Pin 25249 Only |

|                                                                     |

---------------------------------------------------------------------

Table 2-9:  Shugart 810/860 Drive Configuration

16

----------------------------------------------------------------------

|            |                             |       DUAL        SINGLE  |

|   OPTION   |      DESCRIPTION            | DRIVE 0  DRIVE 1  DRIVE 0 |

|----------------------------------------------------------------------|

|   DS1      |Drive select 1               |  IN        OUT      IN    |

|   DS2      |Drive select 2               |  OUT       IN       OUT   |

|   DS3      |Drive select 3               |  OUT       OUT      OUT   |

|   DS4      |Drive select 4               |  OUT       OUT      OUT   |

|   A,B      |Radial head load             |  IN        IN       IN    |

|   X        |Radial head load             |  OUT       OUT      OUT   |

|   Z        |In use from drive select     |  OUT       OUT      OUT   |

|   HL       |Stepper power from head load |  OUT       OUT      OUT   |

|   R        |Alternate output ready pad   |  IN        IN       IN    |

|   I        |Alternate output index pad   |  IN        IN       IN    |

|   C        |Alternate input head load    |  IN        IN       IN    |

|   D        |Alternate input-in-use       |  OUT       OUT      OUT   |

|   DC       |Alternate output disk change |  OUT       OUT      OUT   |

|   2S       |Alternate output 2-sided disk|  IN        IN       IN    |

|   DS       |Stepper power from drive     |                           |

|            | select                      |                           |

|   Y        |In use from head load        |  IN        IN       IN    |

|   DL       |Door lock latch              |  OUT       OUT      OUT   |

|   RR       |Radial ready                 |  IN        IN       IN    |

|   RI       |Radial index                 |  IN        IN       IN    |

|   WP       |Inhibit write when write     |  IN        IN       IN    |

|            |protect                      |                           |

|   NP       |Allow write when write       |  OUT       OUT      OUT   |

|            |protect                      |                           |

|D1,D2,D4,DDS|Drive address select         |  OUT       OUT      OUT   |

|   B1-B4    |Two-sided drive select       |  OUT       OUT      OUT   |

|   S1,S3    |Head select option           |  OUT       OUT      OUT   |

|   S2       |Head select option           |  IN        IN       IN    |

|   T40      |Test track 40                |  OUT       OUT      OUT   |

|   HA       |Test actuate head load       |  OUT       OUT      OUT   |

|4,6,8,10,12,|Alternate I/0 pins           |  OUT       OUT      OUT   |

|16,18,24    |                             |                           |

---------------------------------------------------------------------

Table 2-10:  Qume 842 Drive Configuration

17

----------------------------------------------------------------------

|            |                             |       DUAL        SINGLE  |

|   OPTION   |      DESCRIPTION            | DRIVE 0  DRIVE 1  DRIVE 0 |

|----------------------------------------------------------------------|

|   DS1      |Drive select 1               |  IN        OUT      IN    |

|   DS2      |Drive select 2               |  OUT       IN       OUT   |

|   DS3      |Drive select 3               |  OUT       OUT      OUT   |

|   DS4      |Drive select 4               |  OUT       OUT      OUT   |

|   A,B      |Radial head load             |  IN        IN       IN    |

|   X        |Radial head load             |  OUT       OUT      OUT   |

|   Z        |In use from drive select     |  OUT       OUT      OUT   |

|   HL       |Stepper power from head load |  OUT       OUT      OUT   |

|   R        |Alternate output ready pad   |  IN        IN       IN    |

|   I        |Alternate output index pad   |  IN        IN       IN    |

|   C        |Alternate input head load    |  IN        IN       IN    |

|   D        |Alternate input-in-use       |  OUT       OUT      OUT   |

|   DC       |Alternate output disk change |  OUT       OUT      OUT   |

|   2S       |Alternate output 2-sided disk|  IN        IN       IN    |

|   Y        |In use from head load        |  IN        IN       IN    |

|   DL       |Door lock latch              |  OUT       OUT      OUT   |

|   RR       |Radial ready                 |  IN        IN       IN    |

|   RI       |Radial index                 |  IN        IN       IN    |

|   WP       |Inhibit write when write     |  IN        IN       IN    |

|            |protect                      |                           |

|   NP       |Allow write when write       |  OUT       OUT      OUT   |

|            |protect                      |                           |

|D1,D2,D4,DDS|Drive address select         |  OUT       OUT      OUT   |

|   B1-B4    |Two-sided drive select       |  OUT       OUT      OUT   |

|   S1,S3    |Head select option           |  OUT       OUT      OUT   |

|   S2       |Head select option           |  IN        IN       IN    |

|   T40      |Test track 40                |  OUT       OUT      OUT   |

|   HA       |Test actuate head load       |  OUT       OUT      OUT   |

|4,6,8,10,12,|Alternate I/0 pins           |  OUT       OUT      OUT   |

|16,18,24    |                             |                           |

|   SF       |Switch filter                |  IN        IN       IN    |

|   SP       |Stepper power (used with HL) |  OUT       OUT      OUT   |

---------------------------------------------------------------------

Table 2-11:  Qume 242 Drive Configuration

18

------------------------------------------------------------------------

|            |                               |       DUAL        SINGLE  |

|   OPTION   |      DESCRIPTION              | DRIVE 0  DRIVE 1  DRIVE 0 |

|------------------------------------------------------------------------|

|   DS1      |Drive select 1                 |  IN        OUT      IN    |

|   DS2      |Drive select 2                 |  OUT       IN       OUT   |

|   DS3      |Drive select 3                 |  OUT       OUT      OUT   |

|   DS4      |Drive select 4                 |  OUT       OUT      OUT   |

|   Z        |In use from drive select       |  OUT       OUT      OUT   |

|   R        |Alternate output ready pad     |  IN        IN       IN    |

|   I        |Alternate output index pad     |  IN        IN       IN    |

|   D        |Alternate input-in-use         |  OUT       OUT      OUT   |

|   DC       |Alternate output disk change   |  OUT       OUT      OUT   |

|   2S       |Alternate output 2-sided disk *|  IN        IN       IN    |

|   DS       |Stepper power from drive       |  OUT       OUT      OUT   |

|            |select                         |                           |

|   DL       |Diskette lever (optional)      |  OUT       OUT      OUT   |

|   Y        |In use from head load          |  IN        IN       IN    |

|   HL       |Stepper power from head load   |  IN        IN       IN    |

|   C        |Head load option               |  IN        IN       IN    |

|   RR       |Radial ready                   |  IN        IN       IN    |

|   RM       |Ready modified                 |  OUT       OUT      OUT   |

|   RI       |Radial index                   |  IN        IN       IN    |

|   WP       |Inhibit write when write       |  IN        IN       IN    |

|            |protected                      |                           |

|   NP       |Allow write when write         |  OUT       OUT      OUT   |

|            |protected                      |                           |

|   B1-B4    |Side select option using       |  OUT       OUT      OUT   |

|            |drive select*                  |                           |

|   S1       |Head select option             |  OUT       OUT      OUT   |

|   S2       |Head select option             |  IN        IN       IN    |

|   S3       |Head select option             |  OUT       OUT      OUT   |

|   M1       |Spindle motor option           |  IN        IN       IN    |

|   M2       |Spindle motor option           |  OUT       OUT      OUT   |

|   MC1-MC4  |Motor control select           |  OUT       OUT      OUT   |

|------------------------------------------------------------------------|

|     *  TM 848-2 Only                                                   |

|________________________________________________________________________|

Table 2-12 Tandon TM848-1/848-2 Drive Configuration

19

2.2.1. Drive Selection Signals                _______________________

The controller provides four radial drive select signals.  Each

drive must be configured to one of the appropriate drive select

signals.  Drive select 1 and drive select 2 (DS1/DS2) are

associated with the primary register set (177170#U8#D) while drive

select 3 and drive select 4 (DS3/DS4) are associated with the

secondary register set (177174#U8#D).

Upon initiation of a function the controller selects the

appropriate drive, performs the operation and after ten

revolutions of inactivity deselects the drive.

2.2.2. Head Load Signal                ________________

A separate head load signal is provided to prolong media life.

The read/write heads are only loaded on the media during

read/write operations.  All other operations are completed with

the heads in the unloaded position.

Because step positioning can take place with the heads unloaded,

the drive must be configured to provide stepper power

independent of head loading.

2.2.3. Motor Control                _____________

The controller has been configured for ease of use with the

newer DC motor drives.  A motor on signal is provided and is

activated by the controller upon the initiation of any function.

Before the drive is used for read/write operations, a motor

delay timer insures the drive is up to speed.  After ten

revolutions of inactivity, the signal is deselected, prolonging

the life of the drive.

The motor on delay timer has been configured to operate with

drives jumpered with motor on as a function of drive select.

When a drive is selected the motor on signal is set.  If the

motor on signal was previously set the drive ready status is

immediately interrogated and if valid, the function is

initiated.  If the drive is not ready a motor on delay is issued

after which the drive ready status is again interrogated.  A

valid ready status initiates the function while a not ready

status results in a error being reported and the operation

aborted.

20

2.3. CABLING         _______

A 50-conductor ribbon cable connects to any Shugart compatible drive(s).

If the optional cable is purchased with the controller, connect the socket

connector to the 50-pin header located at the edge of the controller

board.  Observe the alignment of pin 1 of the socket connector and header

as indicated by the arrows shown in Figure 2-2.  The two 50-pin connectors

should be connected to the corresponding drives, again observing the

location of pin 1.  If the optional cable is purchased from an independent

source, the following list of materials (or equivalent) will help in the

construction of the required cable.

QTY        DESCRIPTION                  MFG        NUMBER              _________________________________________________________

1 ea       50 pin socket connector      3M         3425-3000

2 ea       50 pin edge connector        3M         3415-0001

A/R        50 conductor ribbon cable    3M         3365/50

Pin 1 ___

\ ___      ___      ___      ___

------|   |----|   |----|   |----|   |

/      |   |    |   |    |   |    |   |

/       |   |    |   |    |   |    |   |

/--------|   |----|   |----|   |----|   |                                                    ___      ___      ___      ___

\        \           \

\        \           \

\        \            Card edge

\        \           connector

\        \          (50 pin)

Ribbon cable    \        \

(50 conductor)  \       |

\      |

/\      \     |

/  \      \    |

/    \    / \   |

/      \  /   \  |

/        \/     \ |

/ Pin 1___/       /

/           \     /

/             \   /    Socket

/               \/\_____connector

/                   /      (50 pin)

/  Component Side   /

/                   /

/        MXV22      /

\                  /

\ B              /

\              /

\/\          /

/         /

\ A      /

\      /

\    /

\  /

\/

Figure 2-2:  Drive/Controller Cabling

21

The connector pins illustrated in Figure 2-3 are compatible with both

the Shugart 800 series and 850 series drives.  Any drive that has both

a Shugart compatible interface and connector should function properly

with the controller.

-------       _______

----| 1 | 2 |----->TG 43___                                       ________

|----| 3 | 4 |----->MOTOR ON

|----| 5 | 6 |

|----| 7 | 8 |      _________

|----| 9 |10 |<-----TWO SIDED

|----|11 |12 |      ___________

|----|13 |14 |----->SIDE SELECT

|----|15 |16 |      _________

|----|17 |18 |----->HEAD LOAD                                       _____

|----|19 |20 |<-----INDEX                                       _____

|----|21 |22 |<-----READY

|----|23 |24 |      ____________

|----|25 |26 |----->DRIVE SELECT 1                                       ______________

|----|27 |28 |----->DRIVE SELECT 2                                       ______________

|----|29 |30 |----->DRIVE SELECT 3                                       ______________

|----|31 |32 |----->DRIVE SELECT 4                                       ______________

|----|33 |34 |----->DIRECTION                                       ____

|----|35 |36 |----->STEP______                                       ____

|----|37 |38 |----->WRITE DATA                                       __________

|----|39 |40 |----->WRITE GATE                                       ________

|----|41 |42 |<-----TRACK 00_____                                       ________

|----|43 |44 |<-----WRITE PROTECT                                       _________

|----|45 |46 |<-----READ DATA

|----|47 |48 |

|----|49 |50 |

|     -------                   _

-

### ###                              Figure 2-3:  Connector Pin Definitions

2.4. CONTROLLER INSTALLATION         _______________________

The controller  can be inserted and will function in any LSI-11 bus

slot provided that both interrupt and DMA continuity are maintained.

Since these signals are daisy chained through the bus slots, no unused

slots between LSI-11 processor and the floppy controller may exist.

Determine the order that the priority chain follows by consulting the

documentation supplied with the LSI-11 system.  Note that when two

interrupts of the same priority level are asserted, the closer a device

is located to the processor, the higher its priority.

22

2.5. INITIAL OPERATION AND CHECKOUT         ______________________________

Before the following procedures are done and for purposes of checkout,

verify that the controller has been configured with two drives as

described in Sections 2.1 - 2.4.

NOTE                                             ____

The bootstrap must be disabled for

the following procedures.

1.   Apply power to the drive(s).

2.   Place the Run/Halt switch on the processor to the Halt position and

turn on the processor.  An "@" character should be printed on the

terminal signifying that console ODT has been entered.  Both drives

(first drive 1, then drive 0) will step the heads inward 2-tracks, then

step the heads outward until the home signal is detected.  The heads

will not load.  If the above events do not occur, check the cabling and

drive power supplies.

3.  Place a preformatted scratch diskette in drive 0.                ____________

4.  If the standard address assignment is selected, open the CS register

using ODT by typing 177170/ on the terminal.  The processor will

display the contents of the CS register.  If the controller is

operating properly a 004040#U8#D should be printed.  Deposit a 40000#U8#D in

the CS register by typing 40000 <CR>.  This command will initialize the

controller.  Both drives should calibrate for home position.  First,

drive 1 steps inward 2 tracks then outward one track at a time until

the drive indicates track 0 has been reached.  The procedure is

repeated on drive 0.  After both drives are calibrated, the head on

drive 0 is loaded.  Sector 1 of track 1 on drive 0 is read into the

controller buffer.  This operation is indicated by the in-use LED on

the drive indicating the head load operation.  The LED will remain on

for a short time after the read operation is complete.

If, after initializing, the drives do not calibrate or the LED is not

activated, check the cabling and power supplies.

5.   Reopen the CS register (location 177170#U8#D) using ODT as described above.

The contents of this location should be 004040#U8#D.  Examining the next

location 177172#U8#D by using the line feed key or typing in 177172/ should

yield either a 204#U8#D, 244#U8#D, 206#U8#D or 246#U8#D.  For a detailed description of

the register protocol and bit definition, refer to Section 3.

6.   If the above procedures function as described, the controller is ready

for use.  Either diagnostics or an operating system can be booted.  For

details on bootstrapping refer to Section 4.

7.   If the above procedures cannot be validated, consult the factory or

your local representative for assistance.

23

24

SECTION 3

FUNCTIONAL

DESCRIPTION

3. GENERAL         _______

This section describes device registers and command protocol for the

MXV22.

All software control of the MXV22 is performed by means of two sets of

two device registers:  the command and status (MXVCS) register and a

multipurpose data buffer (MXVDB) register.  These registers are

assigned the bus address 177170#U8#D and 177172#U8#D, respectively, for the

primary register set and 177174#U8#D and 177176#U8#D, respectively, for the

secondary register set.  The registers can be read or loaded, with a

few exceptions, using any instruction referring to their addresses.

The MXVCS register passes control information from the CPU to the

controller and reports status and error information from the controller

to the CPU.  The MXVDB is provided for additional control and status

information between the CPU and the controller.  The information that

is present in the MXVDB at any given time is a function of the

controller operation in progress.

The controller contains a sector buffer capable of storing a complete

sector.  For read/write operations the buffer is either "filled" before

a write command or "emptied" after a read command under DMA control.

During a write command the controller locates the desired sector and

the buffer information is transferred to the diskette.  During a read

operation the desired sector is located and the sector data are

transferred to the buffer.

3.1. REGISTER DEFINITIONS         ____________________

3.1.1. MXVCS - Command and Status Register (177170#U8#D or 177174#U8#D)         ___________________________________________#U_#D__________#U_#D_

The format of the MXVCS register is shown below. Functions are

initiated by loading the command and status (CS) register, when not

busy (bit 5 = 1), with bit 0 = 1.  Command protocol is discussed in

detail in Section 3.2.

25

15  14  13  12  11  10   9   8   7   6   5   4   3   2   1   0

---------------------------------------------------------------

|ERR|INT|EXT ADD|RX |   |HD |DEN|TR |INT|DN |UNT|  FUNCTION |GO |

|   |   |       |02 |   |SEL|   |   |ENB|   |SEL|           |   |

---------------------------------------------------------------

BIT DESCRIPTION         _______________

15   ERROR:  This bit is set by the controller to indicate that an

error has occurred during an attempt to execute a command.

This bit is cleared by the initiation of a new command or by

setting the initialize bit.  When an error is detected the

MXVES is read into the MXVDB.  This bit is a read-only bit.

14   MXV22 INITIALIZE:  This bit is set by the program to

initialize the controller without initializing all the

devices on the LSI-11 bus.  This is a write-only bit.

CAUTION                                              _______

Loading the lower byte of the MXVCS will

also load the upper byte of the MXVCS.

When this bit is set, the controller will negate Done and

move the head position mechanism of drive 1 (if two drives

are available) to track 0.  When completed, the controller

will repeat the operation on drive 0.

The controller will then clear the error and status register,

set Initialize Done, and set Drive Ready if drive 0 is ready.

Finally, the controller will read sector 1, track 1, of drive

0.

13-12   EXTENDED ADDRESS BITS:  These bits are used to specify an

extended bus address.  Bit 12 = MA16.  Bit 13 = MA17.  These

are write-only bits.

11   RX02:  This bit is asserted by the controller to indicate

that this is an RX02 type system.  This is a read-only bit.

10   RESERVED:  Must be written as a zero.

09   HEAD SELECT:  This bit selects one of the two possible sides

of the disk for execution of the desired function.  When

cleared, side 0 is selected, when set, side 1 is selected.

This is a read/write bit.

08   DENSITY SELECT:  This bit selects either single or double

density operation.  When cleared, single density is selected;

when set, double density is selected.  This is a read/write

bit.

07   TRANSFER REQUEST:  This bit signifies that the controller

needs data or has data available. This is a read-only bit.

26

06   INTERRUPT ENABLE:  This bit is set by the program to enable

an interrupt when the controller has completed an operation

and asserted the Done bit.  The condition of this bit is

cleared by initialize.  This is a read/write bit.

05   DONE:  This bit indicates the completion of a function.  Done

will generate an interrupt when asserted if interrupt enable

(MXVCS bit 6) is set.  This is a read-only bit.

04   UNIT SELECT:  This bit selects one of the two possible disks

for execution of the desired function.  This is a read/write

bit.

03-01   FUNCTION SELECT:  These bits code one of the eight possible

functions described in detail within this section.  These are

write-only bits.

000   Fill Buffer

001   Empty Buffer

010   Write Sector

011   Read Sector

100   Set Media Density/Format

101   Read Status

110   Write Deleted Data Sector

111   Read Error Code

00   GO:  Initiates a command (write-only bit).

3.1.2. MXVDB - Data Buffer (177172#U8#D or 177176#U8#D)         ___________________________#U_#D__________#U_#D_

This register serves as a general purpose data path between the

controller and the LSI-11.  It will represent one of seven registers

according to the protocol of the function in process.  These registers

include the MXVDB, MXVTA, MXVSA, MXVWC, MXVBA, MXVBAE and MXVES.

This register is a read/write register if the controller is not in the

process of executing a command (i.e., it may be manipulated without

affecting the controller).  When the controller is executing a command,

the register can only be accessed when MXVCS bit 7 (TR) is set.

Data Buffer Register (MXVDB)         ____________________________

All information transferred to and from the floppy media passes through

the MXVDB register and is addressable only under the protocol of the

function in progress.

15  14  13  12  11  10   9   8   7   6   5   4   3   2   1   0

---------------------------------------------------------------

|   |   |   |   |   |   |   |   |   |   |   |   |   |   |   |   |

|   |   |   |   |   |   |   |   |   |   |   |   |   |   |   |   |

---------------------------------------------------------------

\---------------------------\/----------------------------------/

READ/WRITE DATA

### ###                                      MXVDB FORMAT

27

Track Address Register (MXVTA)         ______________________________

This register is loaded to indicate on which of the 115#U8#D (77 decimal)

tracks a given function is to operate.  It can be addressed only under

the protocol of the function in progress.  Bits 8 through 15 are not

used and are ignored.

15  14  13  12  11  10   9   8   7   6   5   4   3   2   1   0

---------------------------------------------------------------

|///|///|///|///|///|///|///|///| 0 |   |   |   |   |   |   |   |

|///|///|///|///|///|///|///|///|   |   |   |   |   |   |   |   |

---------------------------------------------------------------

\---------------\/--------------/\--------------\/--------------/

NOT USED                         0-114#U8#D

### ###                                      MXVTA FORMAT

Sector Address Register (MXVSA)         _______________________________

This register is loaded to indicate on which of the 32#U8#D (26 decimal)

sectors a given function is to operate.  It can be addressed only under

the protocol of the function in progress.  Bits 8 through 15 are not

used and are ignored.

15  14  13  12  11  10   9   8   7   6   5   4   3   2   1   0

---------------------------------------------------------------

|///|///|///|///|///|///|///|///| 0 | 0 | 0 |   |   |   |   |   |

|///|///|///|///|///|///|///|///|   |   |   |   |   |   |   |   |

---------------------------------------------------------------

\--------------\/---------------/\-------------\/---------------/

NOT USED                         1-32#U8#D

### ###                                      MXVSA FORMAT

Word Count Register (MXVWC)         ___________________________

This register is loaded with the number of words (maximum of 128

decimal) to be transferred.  At the end of each transfer the word count

register is decremented.  When the contents of the register are

decremented to zero transfers are terminated; Done is set (MXVES bit

5);and, if enabled, an interrupt is requested.  If the word count is

greater than the limit for the density specified, the controller

asserts a Word Count Overflow (bit 10 of the MXVCS).  This register can

be addressed only under the protocol of the function in progress.  Bits

8 through 15 are not used and are ignored.

15  14  13  12  11  10   9   8   7   6   5   4   3   2   1   0

---------------------------------------------------------------

|///|///|///|///|///|///|///|///|   |   |   |   |   |   |   |   |

|///|///|///|///|///|///|///|///|   |   |   |   |   |   |   |   |

---------------------------------------------------------------

\--------------\/---------------/\------------\/----------------/

NOT USED                        0-200#U8#D

### ###                                      MXVWC FORMAT

28

Bus Address Register (MXVBA)         ____________________________

This register is used to generate the bus address which specifies the

location to and from which data are to be transferred.  The register is

incremented after each transfer.  It will increment across 32K boundary

lines via the extended address bits in the control and status register

and the bus address extension register.  Systems with only 16 address

bits will "wrap around" to location zero when the extended  address

bits are incremented.  This register can be addressed only under the

protocol of the function in progress.  Bit 0 is not used and is

ignored.

15  14  13  12  11  10   9   8   7   6   5   4   3   2   1   0

---------------------------------------------------------------

|   |   |   |   |   |   |   |   |   |   |   |   |   |   |   |///|

|   |   |   |   |   |   |   |   |   |   |   |   |   |   |   |///|

---------------------------------------------------------------

### ###                                      MXVBA FORMAT

Bus Address Extension Register (MXVBAE)         _______________________________________

This register contains the extended address bits (A18-A21) when 22 bit

addressing is enabled.  Bits 4 thru 15 are not used and are ignored.

15  14  13  12  11  10   9   8   7   6   5   4   3   2   1   0

---------------------------------------------------------------

|///|///|///|///|///|///|///|///|///|///|///|///|A21|A20|A19|A18|

|///|///|///|///|///|///|///|///|///|///|///|///|   |   |   |   |

---------------------------------------------------------------

MXVBAE FORMAT

Error and Status Register (MXVES)         _________________________________

This register contains the current error and status conditions of the

drive selected by bit 4 (Unit Select) of the MXVCS.  This read-only

register can be addressed only under the protocol of the function in

progress.  The MXVES is loaded in the MXVDB upon completion of a

function.

15  14  13  12  11  10   9   8   7   6   5   4   3   2   1   0

---------------------------------------------------------------

|///|///|///|///|NXM|WC |HD |UNT|DRV|DD |DRV|DEN|AC |ID |SID|CRC|

|///|///|///|///|   |OVF|SEL|SEL|RDY|   |DEN|ERR|LO |   |RDY|   |

---------------------------------------------------------------

\------\/-------/

NOT USED

### ###                                      MXVES FORMAT

29

BIT DESCRIPTION         ___ ___________

15-12   Not Used.

11   NONEXISTENT MEMORY ERROR:  This bit is asserted by the

controller when the memory address specified for a DMA

operation is nonexistent.

10   WORD COUNT OVERFLOW:  This bit indicates that the word count

specified is greater then the limit for the density selected.

Upon detecting this error the controller terminates the fill or

empty buffer operation and asserts the Error and Done bits.

09   HEAD SELECT:  This bit indicates the side currently selected.

If cleared, it indicates drive 0; if set, it indicates side 1.

08   UNIT SELECT:  This bit indicates the drive currently selected.

If cleared, it indicates drive 0; if set, it indicates drive 1.

07   DRIVE READY:  This bit is asserted if the unit currently

selected exists, is properly supplied with power, has a

diskette installed correctly, has its door closed, and has a

diskette up to speed.  This bit is only valid when retrieved

via a read status function or at the completion of initialize

when it indicates the status of drive 0.

06   DELETED DATA:  During data recovery, the identification mark

preceding the data field was decoded as a deleted data mark.

05   DRIVE DENSITY:  The bit indicates the density of the diskette

in the selected drive.  When zero, it indicates single density;

when set to one, it indicates double density.

04   DENSITY ERROR:  A density error was detected as the information

was retrieved from the data field of the diskette (a density

error occurs when the density selected differs from that of the

data field).  Upon detecting this error the controller loads

the MXVES into the MXVDB and asserts the Error and Done bits.

03   ACLO: Set by the controller to indicate a power failure.

02   INITIALIZE DONE:  This bit is asserted to indicate completion

of the initialize routine, which can be caused by system power

failure, or programmable LSI-11 bus initialize.

01   SIDE READY:  This bit is asserted by the controller when a

double-sided drive is selected, is ready, and has double-sided

media inserted.  The assertion of this bit indicates that side

1 of the selected drive is available for read and write

operations.

00   CRC ERROR:  A cyclic redundancy check error was detected as

information was retrieved from a data field of the diskette.

The information stored in the buffer should be considered

invalid.  Upon detection of this error the controller loads the

MXVES into the MXVDB and asserts the Error and Done bits.

30

3.1.3. Extended Status Registers         _________________________

The controller has four internal status registers.  These registers

provide specific error information in the form of error codes as well

as drive status information depending upon the general error type.  The

registers can be retrieved by a read error code function as described

in Section 3.2.8.

Word 1 <7:0> - Definitive Error Code         ____________________________________

Octal Code Error Code Meaning         _____________________________

040   Tried to access a track greater than 76.

050   Home was found before desired track was reached.

070   Desired sector could not be found after looking at 52

headers (2 revolutions).

120   A preamble could not be found.

150   The header track address of a good header does not compare

with the desired track.

160   Too many tries for an IDAM (identifies header).

170   Data AM not found in allotted time.

200   CRC error on reading the sector from the disk.

240   Density Error

250   Wrong Key word for Set Media Density Command

260   Illegal Data AM

270   Invalid POK during write sequence

300   Drive not ready.

310   Drive write protected.

Word 1 <15:8> - Not Used         ________________________

This register is always cleared by the controller.

Word 2 <7:0> - Current Track Address of Drive 0         _______________________________________________

This register is cleared during the initialize command in order to

synchronize with actual track position.  The register is updated with

each seek on drive 0 and maintains current track position.

31

Word 2 <15:8> - Current Track Address of Drive 1         ________________________________________________

This register is cleared during the initialize command in order to

synchronize with actual track position.  The register is updated with

each seek on drive 1 and maintains current track position.

Word 3 <7:0> - Target Track of Current Disk Access         __________________________________________________

If legal, the track specified for the last read/write command is saved

in this register.

Word 3 <15:8> - Target Sector of Current Disk Access         ____________________________________________________

The sector specified for the last read/write command is saved in this

register.

Word 4 <15:8> - Track Address of Selected Drive         _______________________________________________

This register contains the track address read from the sector header of

the desired sector during the last read/write command.

3.2. COMMAND PROTOCOL         ________________

Data storage and recovery using the MXV22 controller is accomplished by

careful manipulation of the MXVCS and MXVDB registers according to the

strict protocol of the individual functions.  The penalty for violation

of protocol can be permanent loss of data.  Each of the functions are

encoded and written into the command and status register bits 1-3 as

described in Section 3.1.1.  The detailed protocol for each function is

described below.

3.2.1. Fill Buffer (000)         _________________

This function is used to fill the controller buffer with data from the

host processor.  The number of words to transfer is specified by the

host.  The command density bit determines the buffer size (64 or 128

words).  The controller zero-fills the remaining buffer space.  If the

word count is too large for the density selected, the function is

aborted, Error and Done are asserted and the Word Count Overflow bit is

set in the MXVES.

The contents of the buffer may be written on the diskette with a

subsequent write sector command or returned to the host processor using

an empty buffer command.

When the command is loaded, MXVCS bit 5 (Done) is negated.  MXVCS bit 8

(density) must be set to define the buffer size.  MXVCS bits 12 and 13

(extended address bits A16 and A17) must also be asserted to define the

extended memory segment used with the buffer address, yet to be

specified, to form the absolute memory address of the data to be

transferred.  MXVCS bit 4 (unit select) and bit 9 (head select) are

ignored since no drive operation is required.  When MXVCS bit 7 (TR) is

first asserted, the program must move the word count into the MXVDB

which will negate TR.

32

When the controller again asserts TR, the program must move the buffer

address into the MXVDB.  If 22 bit addressing is enabled, the

controller again asserts TR and the program must move the extended

address bits (A18-A21) into the MXVDB.  The controller then negates TR,

initiates a DMA cycle, and transfers the first word from the host

processor to the controller buffer.  At the end of the transfer the

word count register is decremented and the buffer address is

incremented by two.  This cycle is repeated until the word count

register becomes zero.  The controller zero-fills the remaining buffer

space, sets the Done bit, and if enabled, causes an interrupt request.

After Done is asserted the MXVES is moved into the MXVDB.

During the Data Transaction, if any non-existent memory is addressed,

the controller will time out and abort the function.  The Error and

Done bits will be asserted.  MXVES bit 11 (NXM) will be set and the

MXVES will be moved into the MXVDB; if enabled, an interrupt request

will be generated.

3.2.2. Empty Buffer (001)         __________________

This function is used to transfer the contents of the controller to the

host processor.  The number of words to transfer is specified by the

host.  The command density bit determines the maximum legal word count.

If the word count specified is too large for the density selected the

function is aborted, Error and Done are asserted and the Word Count

Overflow bit is set in the MXVES.

The contents of the buffer may be transfered to the host as many times

as desired or may be written on the diskette with a subsequent write

sector command.  Unless a fill buffer or read sector command is issued,

the controller buffer is not destroyed.

When the command is loaded, MXVCS bit 5 (Done) is negated, MXVCS bit 8

(density) must be set to allow the proper word count limit.  MXVCS bits

12 and 13 (extended address bits A16 and A17) must also be asserted to

define the extended memory segment used with the buffer address, yet to

be specified, to form the absolute memory destination address.  MXVCS

bit 4 (unit select) and bit 9 (head select) are ignored since no drive

operation is required.  When MXVCS bit 7 (TR) is first asserted the

program must move the word count into the MXVDB which will negate TR.

When the controller again asserts TR the program must move the buffer

address into the MXVDB.  If 22 bit addressing is enabled, the

controller again asserts TR and the program must move the extended

address bits (A18-A21) into the MXVDB.  The controller then negates TR,

initiates a DMA, and transfers the first word of the buffer to the host

processor.  At the end of the transfer, the word count register is

decremented and the buffer address register is incremented by two.

This cycle is repeated until the word count register becomes zero.  The

controller then sets the Done bit and if enabled causes an interrupt

request.  After Done is asserted the MXVES is moved into the MXVDB.

33

During the DMA transaction, if any non-existent memory is addressed,

the controller will time out and abort the function.  The Error and

Done bits will be asserted.  MXVES bit 11 (NXM) will be set and the

MXVES will be moved into the MXVDB.  If enabled, an interrupt request

will be generated.

3.2.3. Write Sector (010)         __________________

This function is used to locate a desired track and sector and write

the sector with the contents of the internal sector buffer.  When the

MXVCS is loaded with this command, the MXVES is cleared and both the TR

and Done bits are negated.  When TR is first asserted the program must

load the desired sector address into the MXVDB which will negate TR.

When TR is again asserted the program must load the desired track

address into the MXVDB which will negate TR.  The controller then seeks

the desired track and attempts to locate the desired sector.  The

desired track is compared with the track field of the sector header.

If they do not match the operation is aborted, the Error and Done bits

are asserted, the MXVES is moved into the MXVDB, and if enabled the

controller will assert an interrupt request.

If the densities agree but the controller is unable to locate the

desired sector within two diskette revolutions, the controller will

abort the operation, move the contents of MXVES into MXVDB, assert the

Error and Done bits, and if enabled, assert an interrupt request.

If the desired track and sector located and the densities agree, the

controller will write the contents of the internal sector buffer

followed by a CRC character, all in the function selected density.  The

controller completes the operation by moving the MXVES to the MXVDB,

asserts Done, and if enabled, asserts an interrupt request.

CAUTION                                         _______

The contents of the internal sector buffer are lost

during a power failure.  However, after power is brought

back to normal, a write sector command will cause the

random contents of the buffer to be written on the

diskette with a valid CRC character.

NOTE                                        ____

The contents of the sector buffer are not destroyed by a

write sector operation.

34

3.2.4. Read Sector (011)         _________________

This function is used to locate the desired track and sector and

transfer the contents of the data field into the controller's internal

sector buffer.  When the MXVCS is loaded with this command, the MXVES

is cleared and both the TR and Done bits are negated.  When TR is first

asserted the program must load the desired sector address into the

MXVDB which will negate TR.  When TR is again asserted the program must

load the desired track address into the MXVDB which will negate TR.

Both the TR and Done bits remain negated while the controller attempts

to locate the desired sector.  If after two revolutions the controller

is unable to locate the desired sector, the operation is aborted.  The

controller will move the MXVES into the MXVDB, assert the Error and

Done bits, and if enabled, assert an interrupt request.

When the desired sector is located, the controller will then compare

the desired track with the track field of the sector header.  If they

do not match, the operation is aborted.  The Error and Done bits are

asserted, the MXVES is moved into the MXVDB, and if enabled, the

controller asserts an interrupt request.

If a legal data address mark is located and the densities of the

diskette and function agree, the controller will read the data from the

sector into the internal buffer.  If the data address mark indicated a

deleted data field, MXVES bit 6 (DD) is set.  As data are stored in the

internal buffer, a CRC is computed on the data and the CRC bytes

recorded.  A non-zero result indicates a read error.  When a CRC error

is encountered, the controller sets MXVES bit 0 (CRC), moves the MXVES

into the MXVDB, asserts the Error and Done bits, and if enabled,

asserts an interrupt request.

If the desired sector is located, the density of the diskette and

function agree, and the data are transferred with no CRC error, the

controller will assert Done, and if enabled, will assert an interrupt

request.

3.2.5. Set Media Density (100)         _______________________

This function is dual purpose.  The controller can set the media

density by rewriting all the data address marks (single or double

density) and writing zero data fields in the selected density.  The

controller can also "reformat" the entire diskette by rewriting both

the sector headers and the data fields.  The data fields are written in

the selected density preceded by the corresponding data address mark.

Both commands are initiated by the set media function but differ in the

keyword required by the controller to execute the command.

35

When the MXVCS is loaded with the command, the MXVES is cleared and the

Done bit is negated.  When TR is set, the program must respond with a

keyword.  This keyword must be deposited in the MXVDB to complete the

protocol.  When the controller recognizes this character, it begins

executing the command.  If an illegal keyword is used, the operation is

aborted.  The MXVES is moved into the MXVDB, the Error and Done bits

are set, and if enabled, the controller asserts an interrupt request.

If the keyword used is a 111#U8#D, the controller initiates  a set media

density operation.  This operation starts at track 0, sector 1.  Each

sector header is located and a write operation is initiated.  A data

field is written with zero data in the density selected.  If an error

occurs reading any header, the operation is aborted.  The MXVES is

moved into the MXVDB, the Error and Done bits are set, and if enabled,

the controller asserts and interrupt request.  If the operation is

successfully completed, Done is set and if enabled, the controller

asserts an interrupt request.

If the keyboard used is a 222#U8#D, the controller initiates a format

operation.  This function starts at the physical index of track 0.

Each track is written first with an index address mark, then 26 sector

headers are written sequentially about the track.  When each track has

been written, the controller initiates a set media density function as

described above.

The following input string will format the selected unit, in the

desired density.

777170/        4040        XXXX   <LF>                   _______                    ___________

177172/        000000      222    <CR>                                              ___________

### ###                                    CAUTION### ###                                    _______

The set media density function takes about 15 seconds

and the format function takes about 45 seconds.  Neither

should be interrupted.  If either operation is

interrupted, an illegal diskette has been generated, and

the operation should be repeated.  If an error occurs

during a set media density function or a format

function, an illegal diskette has been generated.  The

operation should be repeated.

36

3.2.6. Read Status (101)         _________________

This function is used to update the drive status information and is

initiated by loading the command into the MXVCS.  The Done bit is

negated.  MXVES bit 7 (Drive Ready) is updated by sampling the drive

ready status line.  Drive density is updated by loading the head of the

selected drive and reading the first data address mark.  The controller

then moves the MXVES into the MXVDB, asserts Done, and if enabled,

asserts an interrupt request.  This operation requires about 250ms to

complete.

### ###                                          NOTE

If double-sided media are mounted in a double-sided

drive, MXVES bit 1 (side ready is set).

3.2.7. Write Deleted Data Sector (110)         _______________________________

This operation is identical to Write Sector (010) with one exception.

The data address mark preceding the data is not the standard data

address mark.  A single or double density deleted data address mark is

written according to the density of the function.

3.2.8. Read Error Code (111)         _____________________

This function is used to retrieve the extended status registers and is

initiated by loading the MXVCS with the command.  The Done bit is

negated.  When TR is asserted, the program must load the Bus Address

into the MXVDB.  If 22 bit addressing is enabled, the controller again

asserts TR and the program must move the extended address bits

(A18-A21) into the MXVDB.  The controller then negates TR and assembles

one word at a time and, under DMA control, transfers them to memory

starting at the address specified.

If non-existent memory is encountered during the transfer, the

operation is aborted.  The Error and Done bits will be asserted, MXVES

bit 11 (NXM) will be set, and the MXVES will be moved into the MXVDB.

If enabled, an interrupt request will be generated.

When all four words have been transferred the Done bit is set and if

enabled, an interrupt request is generated.

37

38

SECTION 4

CONTROLLER

OPERATIONS

4. GENERAL         _______

This section provides the user pertinent information concerning the

description and use of the controller functions.  The functions covered

include:  bootstrapping, formatting, fill/write operations, read/empty

operations, write current control, write precompensation, and power

fail protection.

4.1. BOOTSTRAPPING THE CONTROLLER         ____________________________

If the bootstrap is enabled as described in Section 2.1.3, the

controller will respond to the standard bootstrap address 173000#U8#D.  The

controller is bootstrapped by typing 773000G while in console ODT. This

causes a bus INIT and transfers program execution to location 173000#U8#D.

An alternate method is to strap the LSI-11 processor to power up Mode

2.  In this mode, when a power up occurs, the processor automatically

starts execution at 173000#U8#D.  Power-up strapping procedures for the

LSI-11 processor can be found in the Microcomputer Processors

Handbook.*

To boot either a single or double density diskette use the following

procedure:

1.  Place the diskette in drive 0.

2.  If the processor is strapped for power-up Mode 2,

operate the INIT (boot) switch or cycle DC power OFF

and ON.

3.  If the processor is not strapped for power up Mode 2                                      ___

while in console ODT, type 773000G.

*Published by Digital Equipment Corporation.  Maynard, Mass., 1979.

39

4.1.1. Bootstrap Operation         ___________________

The bootstrap is not a standard ROM program.  It uses the controller's

microprocessor to capture the bus; to read block 0 of the diskette into

memory starting at location 0; and finally to transfer program

execution to memory location 0.

Any attempt to read location 173000#U8#D will result in a non-existent

memory trap.  The controller only responds to this address immediately

after a bus INIT.  For this reason the bootstrap is called

"transparent".  When the processor attempts to fetch location 173000#U8#D

following a bus INIT, the controller responds by passing the processor

a "CLEAR RO" instruction.  The processor clears RO and then attempts to

fetch location 173002#U8#D.  The controller passes the processor a "LOAD

IMMEDIATE" instruction with R1 as the destination.  The processor then

attempts to fetch the source operand from location 173004#U8#D. The

controller passes the the device address 177170#U8#D if the standard

address is selected, or 177174#U8#D if the alternate address is selected.

The processor moves the address into R1 and then attempts to fetch

location 173006#U8#D.  The controller first asserts a Direct Memory Access

Request (DMR) then passes the processor a "CLEAR PC" instruction.

Before the processor executes the instruction it passes bus mastership

to the controller.  The controller moves a "BRANCH TO CURRENT LOCATION"

instruction (777#U8#D) into memory location 0 under DMA control.  When the

controller releases bus mastership the processor executes the "CLEAR

PC" instruction and, in so doing, transfers program execution to

location 0.  The processor is thus forced to loop at location 0.  The

controller initiates a Read Status function on drive 0 to determine

diskette density.  If the diskette is single density the controller

reads sectors 1,3,5, and 7 of track 1 of drive 0 into locations 2

through 176, 200 through 376, 400 through 576, and 600 through 776

respectively.  If the diskette is double density the controller reads

sectors 1 and 3 of track 1 of drive 0 into locations 2 through 376, and

400 through 776 respectively.  Finally, the controller DMA's location 0

with a NOP instruction (240#U8#D) allowing the processor to execute the

system bootstrap.  If there is no diskette in drive 0 nothing will be

transferred to memory and the processor will continue to loop at

location 0 until halted.

4.2. FORMAT OPERATIONS         _________________

The controller has the capability of formatting diskettes in a

specified density.  The formatting is accomplished in two passes.

During pass 1, an index address mark is written on track 0 following

the index hole.  Twenty-six sector headers, appropriately spaced, are

written following the index address.  Each of the remaining 76 tracks

is written in the same manner.  When track 76 is completed, pass 2 is

initiated.  The controller seeks track 0 and write a zero data field in

sector 1 using the selected density.  The remaining sectors are written

in the same manner.

40

The format command selects diskette density, unit and side (for dual

headed drives).  Table 4-1 lists the various command word formats.

-------------------------------------------------------

|                          |   Unit 0     |   Unit 1    |

|-------------------------------------------------------|

| Single Density Side 0    |     11#U8#D      |     31#U8#D     |

| Single Density Side 1    |   1011#U8#D      |   1031#U8#D     |

| Double Density Side 0    |    411#U8#D      |    431#U8#D     |

| Double Density Side 1    |   1411#U8#D      |   1431#U8#D     |

-------------------------------------------------------

### ###                            Table 4-1:  Command Word Formats

Figure 4-1 illustrates a format subroutine.  The format command is

loaded into MXVCS.  When TR is set, the keyword 222#U8#D is loaded into

MXVDB.  When the diskette has been formatted a return is made.

FORMAT:

MOV  #11, CMD          ;FORMAT

BIS  DENS, CMD         ;DENSITY

BIS  UNIT, CMD         ;UNIT

BIS  SIDE, CMD         ;SIDE

MOV  CMD, @#MXVCS      ;SELECT FUNCTION

JSR  PC, TRWAIT        ;WAIT FOR TR

MOV  #222, @#MXVDB     ;KEYWORD

JSR  PC, DNWAIT        ;WAIT FOR DONE

TST  @#MXVCS           ;ERROR

BMI  FRMERR            ;BR IF SO

RTS  PC

FRMERR:

### ###                             Figure 4-1:  Format Subroutine

Alternatively a diskette can be formatted using console ODT.  Open the

CS register and deposit the appropriate command.  Then deposit the

format key word 222#U8#D in the DB register.  The following is an example

of formatting unit 0 side 0 in double density.

177170/       004040          411 <LF>         _______                       ________

177172/       000000          222 <CR>                                       ________

41

4.3. FILL/WRITE OPERATIONS         _____________________

Figure 4-2 illustrates subroutines to write data on a diskette which is

done by performing a fill buffer operation followed by a write sector.

The Fill Buffer command, specifying single or double density is loaded

into the MXVCS.  When TR is set, the word count is loaded into the

MXVDB.  When TR is again set, the bus address of the data is loaded

into the MXVDB.  If 22 bit addressing is enabled TR is again set and

the extended address bits are moved into the MXVDB.  A return is made

when the controller's sector buffer is filled.  The Write Sector

command (specifying density, unit and side) is loaded into the MXVCS.

When TR is set the sector address is loaded into the MXVDB.  When TR is

again set, the track address is loaded into the MXVDB.  When the

contents of the controller's sector buffer are written at the selected

sector, a return is made.

FILLBF:

MOV     #1, CMD               ;FILL BUFFER

BIS     DENS, CMD             ;DENSITY

MOV     CMD, @#MXVCS          ;SELECT FUNCTION

JSR     PC, TRWAIT            ;WAIT FOR TR

MOV     COUNT, @#MXVDB        ;WORD COUNT

JSR     PC,TRWAIT

MOV     #BUFOUT, @#MXVDB      ;BUS ADDRESS OF DATA

JSR     PC, DNWAIT            ;WAIT FOR DONE

MOV     #EXTAD,@#MXVDB        ;EXTEND ADDR BITS*

JSR     PC, DNWAIT            ;WAIT FOR DONE*

TST     @#MXVCS               ;ERROR

BMI     ERFIL                 ;BR IF SO

RTS     PC

ERFIL:

WSECT:

MOV     #5, CMD               ;WRITE, SECTOR

BIS     DENS, CMD             ;DENSITY

BIS     UNIT, CMD             ;UNIT

BIS     SIDE, CMD             ;SIDE

MOV     CMD, @#MXVCS          ;SELECT FUNCTION

JSR     PC, TRWAIT            ;WAIT FOR TR

MOV     SECTOR, @#MXVDB       ;SECTOR

JSR     PC TRWAIT

MOV     TRACK @#MXVDB         ;TRACK

JSR     PC, DNWAIT            ;WAIT FOR DONE

TST     @#MXVCS               ;ERROR

BMI     WSERR                 ;BR IF SO

RTS     PC

WSERR:

*only required if 22 bit addressing is enabled!

Figure 4-2:  Write Data Subroutines

42

4.4. READ/EMPTY OPERATIONS         _____________________

Figure 4-3 illustrates subroutines to read data from a diskette which

is done by performing a Read Sector operation followed by an Empty

Buffer operation.

The Read Sector command (specifying density, unit and side) is loaded

into the MXVCS.  When TR is set the sector address is loaded into the

MXVDB.  When TR is again set, the track address is loaded into the

MXVDB.  When the contents of the selected sector are read into the

controller's sector buffer, a return is made.

The Empty Buffer command, specifying density, is loaded into the MXVCS.

When TR is set, the word count is loaded into the MXVDB.  When TR is

again set, the bus address of storage buffer is loaded into the MXVDB.

If 22 bit addressing is enabled TR is again asserted and the extension

address bits are loaded into the MXVDB.  A return is made after the

contents of the controller's buffer are transferred to the memory

storage buffer.

RSECT:  MOV     #7, CMD               ;READ SECTOR

BIS     DENS, CMD             ;DENSITY

BIS     UNIT, CMD             ;UNIT

BIS     SIDE, CMD             ;SIDE

MOV     CMD, @#MDVCS          ;SELECT FUNCTION

JSR     PC, TRWAIT            ;WAIT FOR TR

MOV     SECTOR, @#MDVDB       ;SECTOR

JSR     PC, TRWAIT

MOV     #EXTAD, @#MXVDB       ;EXTENDED ADDRESS BITS*

JSR     PC, DNWAIT            ;WAIT FOR DONE*

MOV     TRACK, @#MXVDB        ;TRACK

JSR     PC, DNWAIT            ;WAIT FOR DONE

TST     @#MXVCS               ;ERROR

BMI     RSERR                 ;BR IF SO

RTS     PC

RSERR:

EMPBF:  MOV     #3, CMD               ;EMPTY BUFFER

BIS     DENS, CMD             ;DENSITY

MOV     CMD, @#MXVCS          ;SELECT FUNCTION

JSR     PC, TRWAIT            ;WAIT FOR TR

MOV     COUNT, @#MXVDB        ;WORD COUNT

JSR     PC, TRWAIT

MOV     #BUFFIN, @#MXVDB      ;BUS ADDRESS FOR DATA

JSR     PC, DNWAIT            ;WAIT FOR DONE

TST     @#MXVCS               ;ERROR

BMI     EREMP                 ;BR IF SO

RTS     PC

EREMP:

*only required if 22 bit addressing is enabled!

### ###                               Figure 4-3:  Read Data Subroutines

43

4.5. WRITE CURRENT CONTROL         _____________________

The controller provides a Write Current Control signal (TG43) which is

asserted whenever a track address greater than 43 is accessed.  This

signal is required by some drives to reduce the effects of write

saturation on the inner tracks.  Since the Shugart 800 series drives do

not require this signal, the controller is shipped with this feature

disabled.  However, Shugart 850 series double sided drives require this

signal (refer to the Shugart double sided diskette storage drive manual

section 7.13).  This signal is provided on pin 2 of the 50 pin ribbon

connector and is enabled according to section 2.1.4.

4.6. WRITE PRECOMPENSATION         _____________________

Bit shift occurs on both single and double density diskettes.  This

shift is more noticeable with double density due to the smaller bit

cell size and corresponding data and clock windows.  Some aspects of

bit shift are predictable and are dealt with the precompensation scheme

implemented in this controller; unpredictable effects are reduced by

using PLL techniques.

Predictable bit-shift effects result from normal read/write operation.

Data are recorded by flux changes in the gap of the read/write head.

These flux changes produce changes in magnetization induces current in

the read/write head.  Since this change is current is not

instantaneous, it takes a finite time to build up to a peak and return

to zero.  When the magnetic flux changes are close together the

previous current transition may not reach zero before a second

transition occurs.  The summation of current pulses produces shifted

peaks.  Because the flux changes are closer together on the inner

tracks (43 through 76) the bit shift is greater in this area.  Values

of up to +350ns are typical.                  _

Other causes of bit shift are variation disk drive rotational speed.

The specified +2% variation will produce bit shifts of +40ns.                        _                                        _

Incomplete erasure of previously recorded data can produce bit shifts

of up to 50ns.  Other miscellaneous components of bit shift include

instantaneous speed variation, electrical noise, radial track alignment

and nonsymmetry of the read/write head and associated electronics.

These effects can produce up to +10ns of bit shift, bringing the total                                         _

effect to +450ns.                   _

Since the data/clock window for double density is only 1000ns, a +450ns                                                                          _

bit shift leaves only a 50ns margin before soft errors begin to occur.

To improve this margin the controller incorporates a scheme to

recognize the data patterns which produce excessive bit shift and

introduces a compensating bit shift.  For tracks greater than

forty-three the recorded bits are shifted 165ns early, or late, as

determined by the two previously recorded bits and the subsequent two

bits to be recorded.  The controller also incorporates a phase-locked

data recovery scheme which dynamically adjusts the recovery clock

frequency to the data, reducing bit shifts due to rotational speed

errors.  These two features improve data recovery margins by 175ns or

more, providing approximately 225ns of margin.

44

4.7. POWER FAIL PROTECTION         _____________________

The controller continuously monitors both the BPOK and BDCOK bus

signals.  Refer to the Microcomputer Processors Handbook for detailed

descriptions of these signals.  When asserted, BPOK signals an

impending DC power failure and guarantees 4ms of operation before BDCOK

is asserted and DC power fails.  Assertion of BDCOK indicates invalid

DC power.  This signal is hardwired in the controller as an interlock

on the Write Gate signal.  When BDCOK is asserted the Write Gate signal

is blocked and write operations are prevented.

Before initiating a write sequence, the controller interrogates the

BPOK line.  If an impending DC failure is indicated the operation is

aborted.

45

46

SECTION 5

SOFTWARE

CONSIDERATIONS

5. GENERAL         _______

The MXV22 controller configured for 18-bit addressing can be used with

all software designed to communicate with DEC's RXV21 controller.  This

is of particular importance when using software not supported by the

driver changes presented in this section.  This section describes the

changes required to RT-11 and RSX-11M in order to take advantage of the

22-bit DMA support provided by the MXV22 controller.  Once the changes

described in this section have been incorporated into the applicable

drivers the system software can be used with the MXV22 controller

configured in either 18-bit or 22-bit modes.

CAUTION

Operation via the MXV22 controller using unmodified

driver will yield questionable results if the MXV22

controller is configured for 22-bit addressing.

5.1. OPERATION USING RT-11 V4.0         __________________________

Operations involving the MXV22 controller are logically equivalent to

those of the RXV21 except a modified "DY" driver is required when

configured in 22-bit mode.  Several techniques can be used to

incorporate the changes described in section 5.1.1; however, the

changes can not be performed on the "DY" via the MXV22 controller

without attention to the caution noted in section 5.

The MXV22 (and RX02) controller requires a different handler than the

single density controllers.  This new handler is configured to utilize

the DMA transfer scheme of the controller.  In addition, diskette

density is determined by the handler without system intervention,

allowing the use of either single or double density diskettes

interchangeably.

This handler, designated "DY", is available in RT11-V03B and later

revisions.

47

Although earlier versions of RT-11 can be used, only the changes to

V4.0 are provided in this document.  Changes to earlier versions can be

accomplished using the methodology described here and the judicial

placement of similar code.  Earlier version will also require

modification to the Bootstrap program BSTRAP

5.1.1. Modifying RX02 Driver for RT11         ______________________________

Changes listed in Appendix A are those required to modify DEC's V4.0

RX02 driver for operation with the MXV22 controller.  Changes listed in

Appendix A.1 are in a format expected by the Source Language Patch

program (SLP).  Generate a file "DYMXV.DIF" using an editor of your

choice containing changes listed in Appendix A.1.  Use the following

steps to include the changes:

.RUN SLP

*DYMXV=DY,DYMXV

### ###                                          NOTE

Changes in Appendix A are to edit level 2 of the

DY.MAC driver program.

The file DYMXV.MAC will contain the new MXV22 compatible driver.

Assemble, link and install the new driver using the method described in

your RT-11 System Generation Manual.

5.1.2. Creating a DY-Compatible System Disk from a DX-based System         ___________________________________________________________

The MXV22 controller requires the DY-based RT-11 monitor rather than

the DX-based RT-11 monitor.  The following procedures explain how to

create a DY-based system.

Using an RX01 or equivalent system, or system which has an RX01 or

equivalent peripheral device, the monitor file and other associated

system files should be copied onto a single density diskette.  These

files can be obtained from the binary distribution media or by

performing a SYSGEN and specifying DY as the system device (refer to

the RT11 System Generation Manual).  The following commands will

initialize the diskette and copy the necessary files to Drive 1:

RT11V04

.INIT/NOQUERY DX1:

.COPY/SYS DEV:SWAP.SYS DX1:

.COPY/SYS DEV:RT11XX.SYS DX1:

.COPY/SYS DEV:TT.SYS DX1:

.COPY DEV:DIR.SAV DX1:

.COPY DEV:PIP.SAV DX1:

.COPY DEV:DUP.SAV DX1:

48

The bootstrap must then be copied from the monitor file to block 0 of

the diskette.  The following command will accomplish this on the

diskette in drive 1.

RT11V04

.COPY/BOOT DX1:RT11XX DX1:

This diskette can be used with the MXV22 controller but it is single

density.  To build a double density diskette the user must first format

a diskette to double density as explained in Section 4.2  Boot the

single density system diskette in drive 0.  Use the following commands

to initialize the formatted diskette in drive 1 and copy the system

software from drive 0 to drive 1.

RT11V04

.INIT/NOQUERY DY1:

.COPY/SYS DY:*.* DY1:

Finally, copy the bootstrap to block 0 of the diskette in drive 1.

RT11V04

.COPY/BOOT DY1:RT11XX DY1:

The diskette in drive 1 can now be booted as a double density system

diskette.

5.2. OPERATION WITH RT-11 V5.0         _________________________

Currently undefined.

5.3. OPERATION WITH LAYERED PRODUCTS         _______________________________

Using the driver modifications described for RT-11, layered products

such as TSX-Plus V3.01, and SHARE 11 can be used to provide 22-bit

system support with the MXV22 controller.

5.4. OPERATION WITH RSX11M         _____________________

Extended address support provided by the MXV22 controller is

incorporated by the DYDRV changes listed in Appendix B.  An additional

change is required to the device data I/O structure.  The file

SYSTB.MAC created by Phase I of SYSGEN must be edited prior to assembly

and task building process.  Characteristic word one of the Unit Control

Block (UCB) must be edited to reflect 22-bit direct addressing support

for the DYDRV device.  Refer to section 4 of the Guide to Writing and

I/O Device for details.

49

5.4.1. Modifying RX02 Driver RSX11M         ____________________________

Changes to the RX02 driver program DYDRV.MAC are provided in Appendix

B.  The change are referenced to the distributed Version 2.13.  Changes

listed in Appendix B are in the format expected by the Source Language

Input Program (SLP).  These change can be incorporated into the

standard driver using methods described in Section 17 of the RSX-11

Utilities Manual.

5.5. DOUBLE SIDED OPERATION RT-11/RSX-11M         ____________________________________

Change for RT-11 double sided support is listed in Appendix A.2.  These

changes can be used with the 22-bit changes.

Appendix B.2 contain the changes required for RSX-11M double sided

support.

5.6. OPERATION WITH OTHER DEC SOFTWARE         _________________________________

The MXV22 Controller used in 18-bit mode emulates the operation of the

RXV21.  This is particularly important when using programs or systems

that might require access to the RX02 type device.

### ###                                          NOTE### ###                                          ____

Operation of the XXDP Diagnostic program require

that the MXV22 be configured to emulate the RXV21

in 18-bit mode.

50

APPENDIX A.1

The following changes are for incorporating 22-bit support via the

Source Language Patch program (SLP):

-/.IIF NDF DY$DD/,,/;MXV22/

$22BIT = 1     ;Remove this line to disable 22-bit addressing

.IIF NDF $22BIT, $22BIT = 0

.IF NE $22BIT

DYTYP=2000

.ENDC

-/@$MPPTR/+1,,/;MXV22/

.IF EQ $22BIT

-/BIS/,,/;MXV22/

.IFF

MOV     @SP,-(SP)

ASL     @SP

ASL     @SP

MOV     (SP)+,EXMBIT

SWAB    @SP

BIC     #^C<30000>,@SP

BIS     (SP)+,R4

-/.ENDC/,,/;MXV22/

.ENDC

-/BUFRAD:/+3,,/;MXV22/

.IF NE $22BIT

BIT     #40000,R0       ;Boundary crossed?

BEQ     8$              ;Branch if not

INCB    EXMBIT

BIC     #40000,R0       ;Remove bit

-/.ENDC/,,/;MXV22/

.ENDC

-/DYDOFN:/+6,,/;MXV22/

.IF NE $22BIT

BIT     #DYTYP,@R4      ;Protocol complete

BNE     3$

2$:

.ENDC

-/RTS/,,/;MXV22/

51

.IF NE $22BIT

3$:    MOV     R2,@R5

MOVB    EXMBIT,R2

4$:    BITB    #CSTR!CSDONE,@R4

BEQ     4$

BMI     2$

JMP     DYERR2

.BYTE   0

EXMBIT:        .BYTE   0               ;Extended memory addr bits

.ENDC

-/8$:/+2,,/;MXV22/

.IF NE $22BIT

BIT     #DYTYP,@R4      ;22-bit controller?

BEQ     22$             ;If equal no!

CLRB    23$

22$:

.ENDC

-/MOV/,,/;MXV22/

.IF NE $22BIT

23$:   BR      9$

JSR     PC,WAIT         ;One more transfer for 22-bit

CLR     @R5

.ENDC

/

52

APPENDIX A.2

The following changes are for incorporating double sided support for

RT-11 via the Source Language Patch (SLP) program:

-/.IIF NDF DY$DD/,,/;2SIDED/

DY$DS=1

MAXLSN=DYDSIZ*4

DBSID2=2

-/DOXFER:/,,/;2SIDED/

MOV     DYLSN,R3

-/DYLSN/,.,/;2SIDED/

/

53

APPENDIX B.1

The following changes are for incorporating 22-bit support for RSX-11M

via Source Language Input Program (SLP):

-/SDEN/,,/;MXV22/

ADREXT =       2000            ; 22 BIT CONTROLLER BIT

-/WD. 20/,,/;MXV22/

;      WD. 21 -- NOT USED

-/(WD. 20)/,,/;MXV22/

;      I.PRM+16 (WD. 21) - EXTENDED ADDRESS BITS (BA18-BA21)

-/SWAB/,.,/;MXV22/

MOVB    UBUF(R5),R0     ; EXTENDED MEMORY BITS

ROR     R0

ROR     R0

MOVB    R0,I.PRM+16(R1) ; SAVE BA18-BA21

.REPT   3

ROR     R0

.ENDM

BIC     #30000,R0       ; ISOLATE BA16 & BA17

MOV     R0,UBUF(R5)     ; INITIALIZE CSR WORD

-/280$:/+1,.,/;MXV22/

CMPB    R0,#'I          ; END REGISTER PROTOCOL ?

BEQ     284$            ; IF EQ EXIT

BIT     #ADREXT,(R2)    ; 22-BIT CONTROLLER ?

BEQ     284$            ; IF EQ YES

MOV     R0,RXDB(R2)     ; LOAD TRACK, BUFFER ADDRESS, OR ASCII I

281$:  BITB    #TR!DONE,(R2)   ; READY FOR LAST WORD ?

BMI     283$            ; IF MI YES

BEQ     281$

BR      160$            ; ERROR, NO TRANSFER REQUEST

283$:  MOV     I.PRM+16(R3),R0 ; EXTENDED ADDRESS BITS

284$:  MOV     R0,RXDB(R2)     ; LOAD TRACK, BUFFER ADDRESS, OR ASCII I

/

54

APPENDIX B.2

The following changes are for incorporating double sided support for

RSX-11M  via Source Language Input Program (SLP):

-/RSAE/,,/;2SIDED/

SSIDED =       20000           ; SECOND SIDE INDICATOR BIT (U.CW2)

-/CRCERR/,,/;2SIDED/

SIDES  =       2               ; DOUBLE SIDED MEDIA

-/220$:/,.,/;2SIDED/

220$:  BIT     #SSIDED,U.CW2(R5) ; IS IT SECOND SIDE OPERATION?

BEQ     225$            ; IF EQ NO

BIS     #DSIDED,U.BUF(R5) ; USE SECOND SIDE

225$:  MOVB    S.CON(R4),R1    ; RETRIEVE CONTROLLER INDEX

-/440$:/,.,/;2SIDED/

440$:  BIC     #SILO!SCHAR!SSIDED!ERR1,U.CW2(R5) ; CLEAR BITS

-/470$:/,.,/;2SIDED/

470$:  BIC     #SCHAR!DEN!DSIDED,U.CW2(R5) ; CLEAR FLAGS

-/480$:/,.,/;2SIDED/

480$:  BIT     #SIDES,I.PRM+6(R1) ; IS IT DOUBLE SIDED?

BEQ     485$            ; IF EQ NO

ASL     U.CW3(R5)       ; DOUBLE THE MAXIMUM LBN'S

BIS     #DSIDED,U.CW2(R5) ; SET THE DOUBLE SIDED BIT

485$:  MOV     #IS.SUC&377,R0  ; SET SUCCESS

-/600$:/,.,/;2SIDED/

600$:  BIT     #SIDES,I.PRM+6(R1) ; IS IT DOUBLE SIDED?

BEQ     605$            ; IF EQ NO

BIS     #DSIDED,U.CW2(R5) ; SET DOUBLE SIDED BIT

ASL     U.CW3(R5)       ; DOUBLE MAX LBN'S

605$:  MOV     R0,I.PRM+10(R1) ; STORE LOGICAL SECTOR NUMBER

-/CMP  R0,#77./,/BEQ   60$/,/;2SIDED/

CMP     R0,#76.         ; IS IT SECOND SIDE?

BLT     30$             ; IF LT NO

BITB    #IO.RPB&377,I.FCN(R3) ; PHYSICAL BLOCK FUNCTION?

BEQ     23$             ; IF EQ NO, IT'S A LOGICAL BLOCK

CMP     #76.,R0         ; YES

BEQ     30$             ; IF EQ ALLOW ACCESS TO #76.

SUB     #77.,R0         ; CHANGE SIDES - PHYBLK ACCESS

BR      25$             ; CHANGE READ HEADS

23$:   SUB     #76.,R0         ; ADJUST FOR SECOND SIDE

25$:   BIT     #DSIDED,U.CW2(R5) ; TWO SIDE MEDIA?

BEQ     60$             ; IF EQ NO, BAD BLOCK ERROR

BIS     #SSIDED,U.CW2(R5) ; SET HEAD 1 SELECT BIT

/

55

CONTENTS                                     ________

PAGE         ____

SECTION 1   GENERAL INFORMATION . . . . . . . . . . . . . . . . . . . 1

COMPONENTS  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 2

COMPATIBILITY . . . . . . . . . . . . . . . . . . . . . . . . . . . . 2

Logical Track Format . . . . . . . . . . . . . . . . . . . . . . 3

Sector Header Field  . . . . . . . . . . . . . . . . . . . . . . 4

Data Field . . . . . . . . . . . . . . . . . . . . . . . . . . . 4

Recording Scheme . . . . . . . . . . . . . . . . . . . . . . . . 5

Double Frequency (FM)  . . . . . . . . . . . . . . . . . . . . . 6

DEC Modified MFM . . . . . . . . . . . . . . . . . . . . . . . . 6

Cyclic Redundancy Check  . . . . . . . . . . . . . . . . . . . . 7

SPECIFICATIONS  . . . . . . . . . . . . . . . . . . . . . . . . . . . 7

SECTION 2   INSTALLATION  . . . . . . . . . . . . . . . . . . . . . . 9

GENERAL . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 9

CONFIGURATION . . . . . . . . . . . . . . . . . . . . . . . . . . . . 9

Address Vector Selection  . . . . . . . . . . . . . . . . . . . . . . 9

Device Interrupt Priority  . . . . . . . . . . . . . . . . . .  10

Bootstrap  . . . . . . . . . . . . . . . . . . . . . . . . . .  12

Write Precompensation  . . . . . . . . . . . . . . . . . . . .  12

Step Rate Control  . . . . . . . . . . . . . . . . . . . . . .  13

22 Bit Addressing  . . . . . . . . . . . . . . . . . . . . . .  14

Miscellaneous Options  . . . . . . . . . . . . . . . . . . . .  14

DRIVE CONFIGURATION . . . . . . . . . . . . . . . . . . . . . . . .  15

Drive Selection Signals  . . . . . . . . . . . . . . . . . . .  20

Head Load Signal . . . . . . . . . . . . . . . . . . . . . . .  20

Motor Control  . . . . . . . . . . . . . . . . . . . . . . . .  20

CABLING . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  21

CONTROLLER INSTALLATION . . . . . . . . . . . . . . . . . . . . . .  22

INITIAL OPERATION AND CHECKOUT  . . . . . . . . . . . . . . . . . .  23

SECTION 3   FUNCTIONAL DESCRIPTION  . . . . . . . . . . . . . . . .  25

REGISTER DEFINITIONS  . . . . . . . . . . . . . . . . . . . . . . .  25

MXVCS - Command and Status Register (177170#U8#D)  . . . . . . . .  25

MXVDB - Data Buffer (177172#U8#D)  . . . . . . . . . . . . . . . .  27

Extended Status Register . . . . . . . . . . . . . . . . . . .  31

COMMAND PROTOCOL  . . . . . . . . . . . . . . . . . . . . . . . . .  32

Fill Buffer (000)  . . . . . . . . . . . . . . . . . . . . . .  32

Empty Buffer (001) . . . . . . . . . . . . . . . . . . . . . .  33

Write Sector (010) . . . . . . . . . . . . . . . . . . . . . .  34

Read Sector (011)  . . . . . . . . . . . . . . . . . . . . . .  35

Set Media Density (100)  . . . . . . . . . . . . . . . . . . .  35

Read Status (101)  . . . . . . . . . . . . . . . . . . . . . .  37

Write Deleted Data Sector (110)  . . . . . . . . . . . . . . .  37

Read Error Code (111)  . . . . . . . . . . . . . . . . . . . .  37

SECTION 4   CONTROLLER OPERATIONS . . . . . . . . . . . . . . . . .  39

GENERAL . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  39

BOOTSTRAPPING THE CONTROLLER  . . . . . . . . . . . . . . . . . . .  39

Bootstrap Operation  . . . . . . . . . . . . . . . . . . . . .  40

i

FORMAT OPERATIONS . . . . . . . . . . . . . . . . . . . . . . . . .  40

FILL/WRITE OPERATIONS . . . . . . . . . . . . . . . . . . . . . . .  42

READ/EMPTY OPERATIONS . . . . . . . . . . . . . . . . . . . . . . .  43

WRITE CURRENT CONTROL . . . . . . . . . . . . . . . . . . . . . . .  44

WRITE PRECOMPENSATION . . . . . . . . . . . . . . . . . . . . . . .  44

POWER FAIL PROTECTION . . . . . . . . . . . . . . . . . . . . . . .  45

SECTION 5   SOFTWARE CONSIDERATIONS . . . . . . . . . . . . . . . .  47

GENERAL . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  47

OPERATION USING RT-11 V4.0  . . . . . . . . . . . . . . . . . . . .  47

Modifying RX02 Driver for RT11 . . . . . . . . . . . . . . . .  48

Creating a DY-Compatible System Disk from a DX-based System  .  48

OPERATION WITH RT-11 V5.0 . . . . . . . . . . . . . . . . . . . . .  49

OPERATION WITH LAYERED PRODUCTS . . . . . . . . . . . . . . . . . .  49

OPERATION WITH RSX11M . . . . . . . . . . . . . . . . . . . . . . .  49

Modifying RX02 Driver RSX11M . . . . . . . . . . . . . . . . .  50

APPENDIX A  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  51

APPENDIX B  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  54

ii

###############################################################################################################################################################################################################################################################################################################################################################