DRQIO NOTES.

The contents of this file are notes taken on the module

DRQIO  (RSX-11M  V3.0)  by Ralph Stamerjohn.  This module is

used to processed the QIO and QIOW executive request.

The notes  assume  the  following  features  have  been

selected/not selected/optional:

A$$CHK  Address checking                selected

A$$CPS  ACP support                     selected

D$$IAG  On-line diagnostics             not selected

L$$DRV  Loadable drivers                optional

M$$MGE  Memory management               selected

M$$MUP  Multi-user protection           optional

M$$NET  Network support                 selected

1.0  CHECK QIO LEGAL AT THIS TIME (265 - 292)

This section of code checks that a QIO is legal at  for

the current task and device state.  The following checks are

made.  If all checks prove false, control is continues  with

the next section.

1.  If      the      task       is       checkpointable

(bt[T2.CHK]wd[T.ST2]=0),  the  current I/O count is

nonzero (by[T.IOC]#0), and the I/O function  not  a

kill  request  (by[Q.IOFN+1]#0),  the  task  PC  is

backed to reissue the QIO and the task is placed in

WAIT FOR SIGNIFICANT EVENT state.

2.  If  the  lun  is  illegal  (by[Q.IOLN]>H.NLUN),   a

directive error of -96.  is returned.

3.  If the lun is unassigned ([first  lun  word]=0),  a

directive error of -5.  is returned.

4.  If the lun is interlocked for  use  (bt[0]wd[second

lun  word]=1),  the task PC is backed up to reissue

the  QIO  and  the  task  is  placed  in  WAIT  FOR

SIGNIFICANT EVENT state.

NOTE

This  feature  is  used  by   certain   ACP

functions  to  block  further  I/O  request

until the function is complete.

DRQIO NOTES - SEP, 77                                 PAGE 2

5.  If the driver  is  not  resident  (wd[D.DSP]=0),  a

directive error of -6.  is returned.

2.0  INITIALIZE I/O ENVIRONMENT (293 - 341)

This section gets various parameters from the QIO  DPB,

checks for legal ranges, and initaiize various environmental

elements for the  I/O  request.   The  following  steps  are

performed.

1.  If an  event  flag  was  specified,  clear  it.   A

directive  error  of -97.  is returned if the event

flag is illegal.

2.  If an I/O status block was  specified,  clear  both

words.

3.  Allocate an I/O PACKET.  If this fails a  directive

error of -1.  is returned.

NOTE

This is the last possible directive  error.

Both  the  event  flag  and  I/O status are

cleared before this error can be detected.

4.  Increment     the     outstanding     I/O     count

(by[T.IOC]_T.IOC+1).

5.  If the function was QIOW  and  an  event  flag  was

specified,  place  the  task in WAIT FOR EVENT FLAG

state for the specified event flag.

3.0  FILL IN I/O PACKET (341 - 361)

This section of code  fills  in  I/O  PACKET  locations

I.PRI to I.AST.

DRQIO NOTES - SEP, 77                                 PAGE 3

4.0  DECODE I/O FUNCTION (362 - 469)

This section of code determines what to do with the I/O

function  depending  on  the  device  type  and I/O function

tables.  The following steps are performed.

1.  If   the   I/O   function   code   was   kill   I/O

(by[Q.IOFN+1]=0), control is passed to the kill I/O

code.  This code calls $IOKIL and  returns  success

for the kill I/O function.

2.  If multiuser support was selected (M$$MUP), the I/O

request access to the device is checked.  Access is

denied and an I/O status code of IE.PRI is returned

if the all of the following are true.

1.  The device is owned (wd[U.OWN]#0).

2.  Public     access      is      not      allowed

(bt[US.PUB]by[U.ST2]=0).

3.  The current owner is not the task  that  issued

the  I/O request (wd[U.OWN]#wd[T.UCB of current

task]).

4.  The   current   task   is    not    priviledged

(bt[T3.PRV]wd[T.ST3]=0).

3.  The I/O function code is checked to make sure it is

in range (1-31.).  If it is not, an I/O status code

of IE.IFC is returned.

4.  The I/O function code is changed into a bit mask.

5.  The I/O function code is checked against the  legal

function  table  in  the DCB.  If it is not, an I/O

status code of IE.IFC is returned.

6.  The device is checked  to  see  if  it  is  offline

(bt[US.OFL]by[U.ST2]=1).   If  it is, an I/O status

code of IE.OFL is returned.

7.  The  I/O  function  code  is  checked  against  the

control  function  table  in  the  DCB.   If it is,

control is passed to control function service.  See

section 5.2.

8.  The I/O function code is checked against the  no-op

function table in the DCB.  If it is, an I/O status

code of IS.SUC is returned.

DRQIO NOTES - SEP, 77                                 PAGE 4

9.  The device is checked to see if it is not mountable

(wd[U.CW1]>0).   If  so  the  following  checks are

made.

1.  The I/O function code is  checked  against  the

ACP function table in the DCB.  If it is not an

ACP function, control is passed to the transfer

function service code.  See section 5.1.

2.  If function is an ACP  function,  the  function

code  is  changed  to  IO.WLB  unless  the  I/O

function code was IO.RVB which is converted  to

IO.RLB.  Control is then passed to the transfer

function service code.  See section 5.1.

NOTE

If you get here, the I/O function  code

must  be  either IO.RVB or IO.WVB.  All

others will be handled incorrectly.

10. If the device is not mounted or mounted as  foreign

(bt[US.MNT!US.FOR]by[U.STS]=0),    the    following

occurs.

1.  If the function is  an  ACP  function,  an  I/O

status return of IE.PRI is made.

2.  Otherwise the control is passed to the transfer

function service code.  See section 5.1.

11. If the device is mounted and the I/O function  code

is not an ACP function, the following occurs.

1.  If  the  I/O  function  code  is  load  overlay

(IO.LOV)  control  is  passed  to  the transfer

function service code.  See section 5.1.

2.  If   the   issuing    task    is    priveledged

(bt[T3.PRV]wd[T.ST3]=1),  control  is passed to

the  transfer  function  service   code.    See

section 5.1.

3.  Otherwise an  I/O  status  code  of  IE.PRI  is

returned.

12. If the I/O function code is an ACP function and the

DRQIO NOTES - SEP, 77                                 PAGE 5

device   is  mounted,  control  is  passed  to  the

appropriate polish code based on the function code.

See section 6.

5.0  COMPLETE NON-ACP I/O REQUEST

This  section  of  code  completes  the  processing  of

non-ACP  I/O  request.   It  is  broken  up  into two parts:

transfer request service and control request service.

5.1  Transfer Request Service (470#-#509)

This section  of  code  checks  the  users  buffer  for

correct  attributes  and  sets  up the mapping into the user

buffer  for  the  I/O  driver.   The  following  steps   are

performed:

1.  Get address (Q.IOPM) and length (Q.IOPM+2) of  user

buffer.

2.  If word alignment is required (by[U.CTL]<0),  check

that  buffer  starts  on word boundary.  If it does

not, return an I/O status code of IE.BYT.

3.  Check  that  the  buffer  length  has  the  correct

modulo.   If it does not, return an I/O status code

of IE.BYT.

4.  Address check the buffer to make sure it is  inside

the  users task space.  If it is not, return an I/O

status code of IE.SPC.

5.  Convert user buffer address  into  relocation  bias

and APR6 offset.

6.  If    the    device    is     an     NPR     device

(bt[UC.NPR]by[U.CTL]=1), convert to proper form for

device setup.

NOTE

The above step is  done  only  if  the  I/O

function code on the stack is nonzero.  The

read/write virtual functions for a mounted,

ACP devices zero this word.

DRQIO NOTES - SEP, 77                                 PAGE 6

7.  Store relocation bias  and  displacement  into  I/O

PACKET.

5.2  Control Request Service (542#-#594)

This  section  of  code  completes  the  processing  of

non-ACP I/O functions.  Transfer request fall into this code

(see above section).  The following steps are performed:

1.  Complete copying the QIO parameters  into  the  I/O

PACKET.

NOTE

Control  functions  copy  all  6  parameter

locations.  Transfer function copy only the

last five.

NOTE

Control functions set I/O PACKET  locations

I.PRM  to  I.PRM+14  to  location Q.IOPM to

Q.IOPM+14.   Transfer  functions  set   I/O

PACKET  location  I.PRM  to  the relocation

bias, I.PRM+2 to the displacement  address,

and   I.PRM+4   to  I.PRM+16  to  locations

Q.IOPM+2 to Q.IOPM+14.

2.  If the function is load overlay  (IO.LOV)  and  the

device  is  mountable  (wd[U.CW1<0),  calculate the

correct disk block address for the function.

3.  Control is passed to the  queue  I/O  PACKET  code.

See section 7.

DRQIO NOTES - SEP, 77                                 PAGE 7

6.0  COMPLETE ACP I/O REQUEST

This section of code completes the  processing  of  ACP

functions.   It  is  broken  into  two  parts:   the  polish

interpreter data base and the various polish routines:

6.1  Polish Interpreter Data Base (94#-#221)

These  tables  are  used  to  correctly  dispatch   ACP

functions  to  the  appropriate  setup routines.  The tables

have two parts.  Table FCDSP contains the  starting  address

of  the appropriate polish table for each function.  This is

followed by the polish tables.

NOTE

An   ACP   function   with   a

function  code  of  0 - 7 will

cause the system to crash.

The following code is a copy of the polish  interpreter

data base take directly from the module DRQIO.  See the next

section for a description  of  each  routine  referenced  by

these tables.

; POLISH INTERPRETER DATA BASE

;

; FUNCTION CODE DISPATCH VECTOR

;

.IF DF	A$$CPS

FCDSP:	.WORD	FCIFC	;10-ILLEGAL FUNCTION

.WORD	FCPKT	;11-FIND FILE NAME IN DIRECTORY

.WORD	FCIFC	;12-ILLEGAL FUNCTION

.WORD	FCPKT	;13-REMOVE FILE NAME FROM DIRECTORY

.WORD	FCPKT	;14-ENTER FILE NAME IN DIRECTORY

.WORD	FCACC	;15-ACCESS FILE FOR READ

.WORD	FCACC	;16-ACCESS FILE FOR READ AND WRITE

.WORD	FCACC	;17-ACCESS FILE FOR RWE

.WORD	FCDAC	;20-DEACCESS FILE

.WORD	FCRVB	;21-READ VIRTUAL BLOCK

.WORD	FCWVB	;22-WRITE VIRTUAL BLOCK

.WORD	FCPKT	;23-EXTEND FILE

.WORD	FCCRE	;24-CREATE FILE

DRQIO NOTES - SEP, 77                                 PAGE 8

.WORD	FCPKT	;25-MARK FILE FOR DELETE

.WORD	FCPKT	;26-READ FILE ATTRIBUTES

.WORD	FCPKT	;27-WRITE FILE ATTRIBUTES

.WORD	FCPKT	;30-USER MAGTAPE CONTROL FUNCTION

.WORD	FCWVB	;31-TRANSMIT PROCESS MESSAGE

.WORD	FCRVB	;32-RECEIVE PROCESS MESSAGE

.WORD	FCCON	;33-CONNECT TO PROCESS

.WORD	FCDIS	;34-DISCONNECT FROM PROCESS

.WORD	FCNCT	;35-NETWORK CONTROL FUNCTION

.WORD	FCIFC	;36-ILLEGAL FUNCTION

.WORD	FCIFC	;37-ILLEGAL FUNCTION

.IF NDF	M$$NET

FCCON:			;REF LABEL IF NO NETWORK SUPPORT

FCDIS:			;REF LABEL IF NO NETWORK SUPPORT

FCNCT:			;REF LABEL IF NO NETWORK SUPPORT

.IFTF

;

; ILLEGAL FUNCTION

;

FCIFC:	.WORD	IEIFC	;SET ILLEGAL FUNCTION STATUS

;

; ACCESS FILE FOR READ, READ/WRITE, OR READ/WRITE/EXTEND

;

FCACC:	.WORD	CKDMO	;CHECK IF VOLUME MARKED FOR DISMOUNT

FCCAW:	.WORD	CKALN	;CHECK IF FILE ACCESSED ON LUN

.WORD	BDPKT	;BUILD AN I/O PACKET

.WORD	CKRLK	;SYNCHRONIZE ACCESS AND EXIT

;

; DEACCESS FILE

;

FCDAC:	.WORD	CKNLN	;CHECK IF FILE ACCESSED ON LUN

.WORD	BDPKT	;BUILD AN I/O PACKET

.WORD	CKRLK	;EXIT

;

; READ VIRTUAL BLOCK

;

FCRVB:	.WORD	CKNLN	;CHECK IF FILE ACCESSED ON LUN

.WORD	CKRAC	;CHECK READ ACCESS AND EXIT

;

; WRITE VIRTUAL BLOCK

DRQIO NOTES - SEP, 77                                 PAGE 9

;

FCWVB:	.WORD	CKNLN	;CHECK IF FILE ACCESSED ON LUN

.WORD	CKWAC	;CHECK WRITE ACCESS AND EXIT

;

; CREATE FILE

;

FCCRE:	.WORD	CKDMO	;CHECK IF VOLUME MARKED FOR DISMOUNT

.WORD	CKACC	;CHECK IF ACCESS ALSO REQUESTED

;

; BUILD AN I/O PACKET FOR FIND, ENTER, REMOVE, EXTEND,

; DELETE, READ  ATTRIBUTES, AND WRITE ATTRIBUTES

;

FCPKT:	.WORD	BDPKT	;BUILD AN I/O PACKET

.WORD	CKXIT	;EXIT

.IFF

;

; CONNECT TO PROCESS

;

FCCON:	.WORD	CKDMO	;CHECK IF VOLUME MARKED FOR DISMOUNT

.WORD	CKALN	;CHECK IF PROCESS CONNECTED ON LUN

.WORD	CKCON	;ADDRESS CHECK CONNECT BUFFER

.WORD	CKRLK	;INTERLOCK LUN USAGE AND EXIT

;

; DISCONNECT FROM PROCESS

;

FCDIS:	.WORD	CKNLN	;CHECK IF PROCESS CONNECTED ON LUN

.WORD	CKDIS	;CHECK BUFFER AND COPY PARAMETERS

.WORD	CKRLK	;INTERLOCK LUN USAGE AND EXIT

;

; NETWORK CONTROL FUNCTION

;

FCNCT:	.WORD	CKDMO	;CHECK IF VOLUME MARKED FOR DISMOUNT

.WORD	CKCON	;ADDRESS CHECK BUFFER

.WORD	CKXIT	;EXIT

.ENDC

.ENDC

DRQIO NOTES - SEP, 77                                PAGE 10

The registers and stack are setup as shown  below  when

the dispatch into the polish code begins.

R0     = Address of UCB.

R1     = Address of second lun word.

R2     = Index into polish function dispatch table.

R3     = Address of Q.IOPM in QIO DPB.

R4     = Address of I.PRM  in I/O PACKET.

R5     = Address of next polish dispatch vector.

(SP)   = Address of second lun word.

(SP)+2 = Address of UCB.

(SP)+4 = I/O function code.

6.2  Polish Routines

This section describes the various polish routines used

to complete

the processing of ACP functions. To see which routines are

used

by a particular functions, see the above section. The

routines

are listed in alphabetical order.

6.2.1  BDPKT (724#-#867) -

This routine completes the I/O PACKET

for various ACP I/O request. It expects the following I/O

parameters

in the QIO request:

Q.IOPM+0  Address of file ID block.

Q.IOPM+2  Address of attribute descriptor block.

Q.IOPM+4  Extend control word 1.

Q.IOPM+6  Extend control word 2.

Q.IOPM+10 Access control word.

Q.IOPM+12 Address of filename block.

The file ID block and filename block are  relocated  if

present.   The  attribute  descriptor  blocks  addresses are

relocated and stored in the secondary control block.  If any

address  do  not  check out, an I/O status code of IE.SPC is

returned.  Otherwise it dispatches to the next entry.

DRQIO NOTES - SEP, 77                                PAGE 11

6.2.2  CKACC (953#-#961) - This routine checks if access  is

requested  on  a  create  (wd[Q.IOPM+10]<0)  and changes the

dispatch pointer  (R5)  to  FCCAW  if  true.   Otherwise  it

dispatches to the next entry.

6.2.3  CKALN (936#-#944) - This routine checks if a file  is

already  accessed  on  the  lun  (wd[second lun word]#0) and

returns an I/O status code of IE.ALN if  so.   Otherwise  it

dispatches to the next entry.

6.2.4  CKCON  (880#-#893) - This  routine  copies  the   QIO

parameters  into  the  I/O packet.  The routine requires the

first two parameters to be a buffer  address  and  size  and

relocates  the  buffer before storing.  If the buffer is not

specified or is illegal, an I/O status  code  of  IE.BAD  is

returned.  Otherwise it dispatches to the next entry.

6.2.5  CKDIS  (868#-#879) - This  routine  copies  the   QIO

parameters  into  the  I/O packet and dispatches to the next

entry.

6.2.6  CKDMO (898#-#905) - This routine checks if the volume

is  marked  for dismount (bt[US.MDM]by[U.STS]=1) and returns

an I/O status code of IE.PRI if so.  Otherwise it dispatches

to the next entry.

6.2.7  CKNLN (945#-#952) - This routine checks if a files is

accessed  on  the lun (wd[second lun word]#0) and returns an

I/O status code of IE.NLN if not.  Otherwise  it  dispatches

to the next entry.

6.2.8  CKRAC (906#-#913) - This routine sets R2 to check for

read access (bt[WI.RDV]) and falls into the CKWAC code.

6.2.9  CKRLK  (963#-#968) - This  routine  locks   the   lun

(wd[second  lun  word]_wd[second lun word]+1) and falls into

the CKXIT code.

DRQIO NOTES - SEP, 77                                PAGE 12

6.2.10  CKWAS  (914#-#934) - This   routine   performs   the

following steps.

1.  Set R2 to check for write access (bt[WI.WRV])

2.  Pop address of second lun word off stack.

NOTE

CKRAC continues at the above step.

3.  Mark  function  as  ACP  transfer   function   (see

Transfer Request Service, step 6).

4.  Check if desired access allowed to file.   If  not,

return I/O status code of IE.PRI.

5.  Pop UCB address of of stack.

6.  Continue in Transfer Request Service  (see  section

5.1).

6.2.11  CKXIT (969#-#978) - This routine cleans  the  stack,

increments       the      volume      transaction      count

(wd[V.TRCT]_wd[V.TRCT]+1), and exits queue I/O  PACKET  code

(see section 7).

7.0  QUEUE I/O REQUEST TO DRIVER (595#-#645)

This section of code  queues  the  I/O  packet  to  the

device  driver  and  calls the driver at the initiator.  The

following steps are performed.

NOTE

The entry $DRQRQ can  be  used

to  queue  an  I/O packet that

has been constructed by  other

means.    R1   should  be  the

packet address and R5 the  UCB

address.

DRQIO NOTES - SEP, 77                                PAGE 13

1.  Get the SCB address (wd[U.SCB]).

2.  If   the   packet   is    not    to    be    queued

(bt[UC.QUE]by[U.CTL]=1), goto step 4.

3.  Queue the I/O packet to the SCB I/O queue.

4.  If the driver is  loadable,  map  KISAR5  into  the

driver.

5.  Call the driver at the initaitor entry point.

6.  If the driver is loadable, restore  KISAR5  to  its

orginal contents.

7.  Exit from the DRQIO code.