PN-85.2

A CAMAC DRIVER FOR RSX-11M

Daniel B. Curtis

22-May-78

ABSTRACT                                   ________

This program note describes a driver for RSX-11M.  The driver  allows  a

task  to  interface  to a BD-011 or JY411 parallel CAMAC branch highway.

This  driver  is  not  suitable  for  single  purpose  very  fast   data

acquisition systems.

The program note is written with the same format as  a  chapter  in  the

RSX-11M I/O Drivers manual.

Only RSX-11M is supported.

RSX-11M CAMAC DRIVER                                              Page 2

A CAMAC DRIVER FOR RSX11-M

1.0  INTRODUCTION        ____________

The EG&G BD-011 and Jorway JY411 CAMAC branch interfaces  supply  RSX11M

with the ability to access CAMAC modules on a parallel branch highway.

This device driver  is  designed  for  use  with  many  tasks  accessing

different  CAMAC  modules.  Due to the I/O overhead necessary in RSX, it

may not be suitable for extremely fast data acquisition  from  a  single

purpose  CAMAC system.  For fast data acquisition, use a privileged task

|  or use COMMON and DEVICE partitions.

|

|  The device driver is able to support multiple  interfaces  of  the  same

|  type.   It  is  not possible to mix JY411 and BD-011 interfaces with the

|  same device driver.

2.0  QIO MACRO        ___ _____

This section summarizes the valid functions of the  QIO  macro  for  the

CAMAC driver.

2.1  Standard QIO Functions        ________ ___ _________

Table 1 lists the standard functions of the QIO macro that are valid for

the CAMAC driver.

Table 1

Standard QIO Functions for CAMAC

Format                            Function          ______                            ________

QIO$C  IO.ATT,...                       Attach device

QIO$C  IO.DET,...                       Detach device

QIO$C  IO.KIL,...                       Cancel I/O requests

RSX-11M CAMAC DRIVER                                              Page 3

2.2  Device Specific QIO Functions For CAMAC        ______ ________ ___ _________ ___ _____

Table 2 lists the device specific functions of the QIO  macro  that  are

|  supplied for the CAMAC driver.  These functions may be modified by ORing

|  a modifier, described in section 5, with the function.

|

|  There are two units  associated  with  each  branch  driver.   The  even

|  numbered  unit is used for non NPR type transfers, while the odd unit is

|  used  for  NPR  type  transfers.   Therefore,  all  IO.CDM  and   IO.LPR

|  operations  MUST  be  sent  to  odd  numbered  units, all others to even|              ____

|  numbered units.

Table 2

Device Specific QIO Functions for CAMAC

Format                            Function          ______                            ________

QIO$C  IO.CPW,...,<low,hiw,0,cs,fna>    Programmed data write

QIO$C  IO.NDT,...,<0,0,0,cs,fna>        Non-data transfer

QIO$C  IO.CPR,...,<stadd,size,cs,fna>   CAMAC programmed data read

QIO$C  IO.CRR,...,<stadd,14,0,0,0>      CAMAC register read

|  QIO$C  IO.CDM,...,<stadd,size,cs,fna,bm> CAMAC DMA transfer

|  QIO$C  IO.LPR,...,<stadd,size,,,cstadd,csize> CAMAC list processing

2.2.1  IO.CRR - The IO.CRR QIO  function  is  used  to  read  the  CAMAC

registers.   The  registers are read out and put in the buffer in order.

|  If a BD-011 is used, the last word will be zero.  The second  word  will

|  contain   the   "Selected  Option"  word  (see  section  5).   For  more

information refer to the branch  driver  description  (HN-15  or  HN-2).

Note that the status of the CAMAC registers will depend on the time that

this function is dequeued by the driver.  So, for some applications,  it

may be best to attach the driver to your task before using IO.CRR.

The 'stadd' parameter is the starting  address  of  a  14  (octal)  byte

buffer.  The format of this buffer is shown in table 3.

The 'size' parameter must be 14 (octal).

The next 3 arguments must be zero or blank.

RSX-11M CAMAC DRIVER                                              Page 4

Table 3

Register Read Input Buffer

from low to high address

*********************************

* Control status register       * N

*********************************

|                   * Selected Option Word          * N+2

*********************************

* Memory Address Register       * N+4

*********************************

* Crate select Register         * N+6

*********************************

* Instruction Register          * N+10

*********************************

* Extended Word Count (JY411)   * N+12

*********************************

2.2.2  IO.NDT - The IO.NDT QIO function  is  used  to  execute  a  CAMAC

non-data transfer operation.

The first three arguments must be blank or zero

'cs' is the crate select mask word and must be setup as in table 4

'fna' is the function, station, and address control  word  and  must  be

setup as in table 5

Table 4

Crate Select Mask Word

15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 00

*  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  *

*C7 C6 C5 C4 C3 C2 C1 00 00 00 00 00 00 00 00 00*

*  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  *

Table 5

FNA Control Word

15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 00

*  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  *

*BB SZ F5 F4 F3 F2 F1 N5 N4 N3 N2 N1 A4 A3 A2 A1*

*  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  *

RSX-11M CAMAC DRIVER                                              Page 5

2.2.3  IO.CPR - The IO.CPR QIO function is used to do a programmed  data

|  read.   Refer  to  section  5 to find the interaction of the 24 bit mode

|  request with the byte count.

'stadd' is the starting address to store the data.  See table 6 for  the

|  buffer format.

'Size' is the size in bytes and must be a '2' or '4'

'cs' is the crate select mask

'fna' is the function, station, address word

Table 6

Programmed Data Read Buffer Format

**************************

* Low Order 16 bits      * N

**************************

* zero      - High byte  * N+2

**************************

2.2.4  IO.CPW - The IO.CPW QIO function is used to do a programmed  data

write.

'low' is the low order 16 bits of the 24 bit transfer

'hiw' is the high order 8 bits of the 24 bit transfer

The next parameter must be zero

'cs' is the crate select mask word

'fna' is the function, station, and address control word

If a 16 bit transfer is to be done, the 'hiw' must still be specified.

NOTE

If a 16 bit write is  done  to  a  24  bit

module,  the  upper  8  bits of the module

will be set to a random value.

RSX-11M CAMAC DRIVER                                              Page 6

2.2.5  IO.CDM - The IO.CDM QIO function is used to cause a NPR  transfer

|  to  and from CAMAC.  This function must be sent to an odd numbered unit;

|  refer to the programming notes.

'stadd' is the starting address of the buffer

'size' is the size in bytes of the buffer to be used

'cs' is the crate select word

'fna' is the function, station, address control word

|

|  'bm' is the control word for the setting of the BM bit (bits 14&15 only)

|

|

|

|  2.2.6  IO.LPR - The IO.LPR QIO function is used to cause a list  of  NPR

|  type functions to be sent to the driver.  As such, the operation must be

|  sent to an odd numbered unit.  The QIO must indicate  two  buffers,  one

|  for  data  and  another  for  the  operation list.  This QIO function is

|  described in section 3.

|

|  'stadd' is the starting address of the data buffer

|

|  'size' is the size in bytes of the data buffer

|

|  'cstadd' is the starting address of the list buffer

|

|  'csize' is the size of the list buffer

|

|

|                                    NOTE

|

|                 The list buffer must be terminated with  a

|                 zero  word.   Currently  there is no check

|                 for overflowing the control  list.   There

|                 is  also no check for correct (odd number)

|                 unit.  These  will  be  considered  for  a

|                 future release.

|

|

RSX-11M CAMAC DRIVER                                              Page 7

|  3.0  LIST PROCESSING OPTION|       ____ __________ ______

|

|  As mentioned in section 2.2.6, the user may request the execution  of  a

|  list  of  CAMAC operations by using a single QIO$ request.  This greatly

|  reduces the system overhead.  The procedure to perform a  list  directed

|  CAMAC I/O request is:

|

|       1.  Set up a list describing the CAMAC operations to be performed

|

|       2.  Reserve a buffer area for the storage of any data  to  be  read

|           from CAMAC modules

|

|       3.  Issue an I/O request  using  the  QIO$  MACRO  and  the  IO.LPR

|           function   code   (All   requests  for  list  processing  CAMAC

|           operations must be made to odd numbered units as in the case of

|           normal CAMAC DMA transfers.)

|

|  The exact format of the QIO$ MACRO parameters is  described  in  section

|  2.2.6.  The format of the COP list is described in section 3.1.  Section

|  5 contains a discussion of driver error returns that  may  occur  during

|  list processing operations.

|

|

|

|  3.1  CAMAC Operations Packet (COP) List Format|       _____ __________ ______ _____ ____ ______

|

|  The CAMAC operations packet (COP) list is a list of CAMAC operations  to

|  be  performed  by the CAMAC driver.  Each packet in the COP list has the

|  following format:

|

|          ---------------------------------

|          | BYTE COUNT    | OPTION FLAGS  |

|          ---------------------------------

|          | CRATE SELECT                  |

|          ---------------------------------

|          | WORD COUNT                    |

|          ---------------------------------

|          | FNA                           |

|          ---------------------------------

|          | OPTIONAL DATA  (0 TO 3 WORDS) |

|          ---------------------------------

|          | FIRST WORD OF THE NEXT PACKET |

|          ---------------------------------

|

|  Where:

|

|    BYTE COUNT    specifies the number of bytes in the  COP  list  packet.

|                  This  value will be between eight and fourteen depending

|                  on the number of optional data words.

|

|    OPTION FLAGS  is a bit mask used to request special operations  to  be

|                  performed.

|

|    CRATE SELECT  specifies the word to be loaded into the  branch  driver

|                  crate  select/word  count  register  when  the operation

RSX-11M CAMAC DRIVER                                              Page 8

|                  specified by this packet is performed.

|

|    WORD COUNT    specifies the value to be loaded into the branch  driver

|                  extended  word  count  register  when  the  operation is

|                  performed.

|

|    FNA           specifies the value to be loaded into the branch  driver

|                  instruction register when the operation is performed.

|

|    OPTIONAL DATA specifies the values to be used by any  of  the  options

|                  selected  by the OPTION  FLAGS byte in the first word of

|                  the  packet.   The  contents  of  this  area  is   fully

|                  described below in section 3.1.1.

|

|  The COP list is terminated by a packet whose first word is zero.

|

|

|

|  3.1.1  Description Of OPTION FLAGS -

|

|  This section describes the function of the  OPTION FLAGS  field  of  the

|  COP.   This byte is used as a bit mask to select special operations.  In

|  the following discussion, bit 0 refers to the least significant  bit  of

|  the byte.

|

|  The option byte is interpreted as follows.  The bits within the byte are

|  examined one at a time starting with bit zero.  A set bit indicates that

|  the option associated with that bit has been selected.   If  the  option

|  selected  requires a data word then the value stored in the next word of

|  the OPTIONAL DATA area is used.  The bit assignments and associated data

|  word descriptions are listed below.

|

|      BIT 0  If this bit is set, the next word in the  OPTIONAL DATA  area

|             specifies  the buffer offset at which the first word of CAMAC

|             data read by this command is to  be  placed.   If  no  buffer

|             offset  is specified, any data that is read will be placed in

|             the next available buffer word (starting with the first  word

|             of  the  buffer if no offset is specified in the first packet

|             in the COP list).

|

|      BIT 1  If this bit is set, the next word in the  OPTIONAL DATA  area

|             specifies   the   expected  end  CAMAC  crate,  station,  and

|             subaddress of this operation.  The format of this  data  word

|             is:

|

|                  bit 0 -> bit 3 = subaddress

|                  bit 4 -> bit 8 = station

|                  bit 9 -> bit 15 = crate mask

|

|      BIT 2  If this bit is set the next word in  the  OPTIONAL DATA  area

|             specifies  a  repeat count for the operation requested in the

|             packet.

|

|      BIT 3  Reserved

RSX-11M CAMAC DRIVER                                              Page 9

|      BIT 4  Reserved

|

|      BIT 5  This bit is set to  allow  unknown  length  transfers  to  be

|             specified.   It  will cause the word count part of the COP to

|             be overwritten if the specified word count  would  cause  the

|             operation  to  overflow  the  end  of  the data buffer.  This

|             modified COP will execute just  as  normal  but  the  maximum

|             number  of  operations  that  it  is  allowed  to  execute is

|             adjusted to fit into the buffer.  You must reset the COP word

|             count on subsequent operations as it may have been modified.

|

|      BIT 6  If this bit is set the driver will ignore any CAMAC time  out

|             errors  (T02  bit in the interface CSR) that may occur during

|             the execution of  the  command.   If  not  set,  a  time  out

|             condition will produce an error.

|

|      BIT 7  If this bit is set the driver will  ignore  the  CAMAC  crate

|             overflow  error  condition.   If  not,  crate  overflow  will

|             produce an error.

|

4.0  STATUS RETURNS        ______ _______

The error and status conditions listed in table 7 are  returned  by  the

|  CAMAC  driver  in the first byte of the I/O status block.  The next byte

|  will contain a packed representation of the control status word  of  the

|  branch  driver  (see  table 8) for all functions except for IO.LPR.  For

|  the IO.LPR function, the byte will contain the number of  the  operation

|  in the list ( the first operation is operation '0').

|

|  The second word of the I/O status block will contain the value  selected

|  from  the function modifiers described in section 5.  The default is the

|  control  status  word  of  the  branch  interface  after  the  operation

|  completed.

RSX-11M CAMAC DRIVER                                             Page 10

Table 7

Status returns

IS.SUC    Successful Completion- The  operation  specified  in  the  QIO

directive was completed successfully.

IE.VER    Unrecoverable Error- The operation  caused  a  CAMAC  timeout.

Typically this is due to an offline crate, the selection of an

unoccupied station, or the lack of an  'X'  response  for  the

|            JY411.   If the 'CM.NMS' modifier is not set, a 'SELECT ERROR'

|            message will be printed on CO:.

|

|  IE.BAD    Bad Parameter- This occurs if an attempt was made to do a  NPR

|            transfer  to an even numbered unit or a non-NPR transfer to an

|            odd numbered unit.

IE.ONP    Illegal Subfunction- This is due only to programmer error.  It

|            is caused by attempting to do an illegal transfer function.

|

|

|                     Status Returns for List Operations

|

|

|  IE.FHE    Fatal Hardware Error- The expected 18 bit address of the  last

|            word of the list operation was not correct.  This implies that

|            the NPR function of the branch driver  is  malfunctioning  and

|            the  system  may  be  corrupted.  A 'DEVICE NOT READY' message

|            will be output at CO: and the NPR unit will  be  set  offline.

|            This  will  cause any other attempt to use that unit to return

|            with an IE.OFL (device offline) error.

|

|  IE.RCN    Record Number Error-This error is returned  if  an  unexpected

|            CAMAC timeout (T02) occurs.

|

|  IE.VER    Should never happen when list processing.  If it does, contact

|            the RSX-11M maintanence person in the Computing Department.

|

|  IE.RBG    Illegal  Record  Size-  An  unexpected  crate   overflow   was

|            encountered.

|

|  IE.SPC    Illegal User Buffer- This is set if an  attempt  was  made  to

|            write  data outside of the data buffer.  It will be set if the

|            buffer offset is outside of the buffer or if a a COP  attempts

|            to  do  a  transfer that could extend past the end of the data

|            buffer (see section 3.1.1 for the description of BIT 5 of  the

|            option flag byte).

|

|  IE.DAO    Data Overrun- This is set if an end CNA check was requested in

|            the list and it failed.

|

|

RSX-11M CAMAC DRIVER                                             Page 11

|                                  Table 8

|                             Packed Status Byte

|

|

|

|  Bit 0     Branch Q of operation

|

|  Bit 1     Branch X of operation

|

|  Bit 2     Abort bit

|

|  Bit 3     Crate Overflow bit

|

|  Bit 4     CAMAC Timeout

|

|  Bit 5     PDP Timeout

|

|  Bit 6-7   Zero

|

|

The driver may produce a 'Device Not  Ready'  message  at  the  console.

|  This  is  due  to  programmer error or a bad interface.  If this message

|  occurs, a device display should be done.  If only the odd unit  but  not

|  the  even unit is marked offline the branch driver is faulty and must be

|  replaced<1>.

|  <1> The hardware addressing of the Jorway is faulty and it  is  possible

|  that the system has been corrupted.

RSX-11M CAMAC DRIVER                                             Page 12

5.0  PROGRAMMING HINTS        ___________ _____

This section contains information on programming considerations that are

|  relevant to the users of the CAMAC driver.

|

|

|

|  5.1  Function Modifier Actions|       ________ ________ _______

|

|  The function modifier is used to control the information returned to the

|  I/O status block and to suppress SELECT ERROR messages.

|

|  The possible modifiers follow:

|

|       1.  Only one of this type of modifier may be used.   Any  of  these

|           modifiers may be used with the next set of modifiers.

|

|           1.  CM.ASM- This will  cause  the  Option  Select  word  to  be

|               returned to the second word of the I/O status block.

|

|           2.  CM.CS- This will cause the Crate  Select  register  of  the

|               interface  to  be  returned  to  the second word of the I/O

|               status block.

|

|           3.  CM.CSR- This (the default  case)  will  cause  the  control

|               status  register  of  the  interface  to be returned to the

|               second word of the I/O status block.

|

|           4.  CM.EWC- This will cause the extended word count register of

|               the  interface to be returned to the second word of the I/O

|               status block.

|

|           5.  CM.FNA- This will cause the FNA register of  the  interface

|               to be returned to the second word of the I/O status block.

|

|           6.  CM.TOP-  This  may  only  be  selected  when   doing   list

|               processing.    It  returns  the  index  of  the  last  byte

|               transferred from the buffer.  This index when added to  the

|               starting address of the buffer generates the address of the

|               last word transferred.

|

|

|       2.  CM.NMS- This modifier may be added with any one  of  the  above|                                                       ___

|           modifiers.   It  will suppress the generation of a SELECT ERROR

|           message when a module does not respond or if the transfer falls

|           off of the branch.

|

RSX-11M CAMAC DRIVER                                             Page 13

|  5.2  Selected Option Word|       ________ ______ ____

|

|  This word indicates which assembly options were selected for the driver.

|  The  word  is  bit encoded for the operations and the currently assigned

|  bits are:

|

|       1.  Bit 0- Indicates  driver  is  able  to  generate  SELECT  ERROR

|           messages.

|

|       2.  Bit 1- Indicates able to do list processing.

|

|       3.  Bit 2- Driver expects to use a Jorway Branch interface.

|

|

|

|

|  5.3  IO.CPR|       ______

|

|  If the byte count is '2' and a 24 bit transfer is  requested,  only  the

|  lower  16  bits  are transferred.  If the byte count is '4' and a 16 bit

|  transfer is requested, the second word will be zero.

5.4  IE.VER        ______

|  When a non list processing operation detects an IE.VER error return  and

|  CM.NMS  was  not  selected,  you  should wait at least 15 seconds before

|  attempting to do another CAMAC operation.  This is  due  to  the  select

error  message  printed  out at the console.  This message is printed by

TKTN.  If TKTN and the MCR reside in the same partition, MCR will  never

be  able  to  get  in to abort the task causing the select errors.  This

condition does not occur if TKTN and MCR reside in different partitions.

5.5  Unit Selection        ____ _________

|  All non-NPR (non block transfer) operations to CAMAC must be sent to  an

|  even  numbered unit.  All NPR operations must be sent to an odd numbered

|  unit.  If this rule  is  violated  an  IE.BAD  status  will  usually  be

returned.   If  the  program  attempted  to  do  a  NPR operation with a

function that does not require a transfer, the driver will time out with

a  IE.ONP status return and a 'Device not ready' message will be printed

|  at the console.  The NPR functions are IO.CDM and IO.LPR.

5.6  FORTRAN Use        _______ ___

When using the driver from a FORTRAN program some of the QIO  directives

require an address in the parameter array.  These QIO functions are:

RSX-11M CAMAC DRIVER                                             Page 14

1.  IO.CPR

2.  IO.CRR

|       3.  IO.CDM

|

|       4.  IO.LPR

5.7  Function Code Definitions        ________ ____ ___________

The following function codes may be defined in the user program,  placed

in  the  system  macro  library,  or  placed  in  the system default I/O

|  definition module in the system  object  library.   All  values  are  in

|  octal.

1.  IO.CPW=400

2.  IO.NDT=402

3.  IO.CPR=1000

4.  IO.CRR=1001

|       5.  IO.CDM=3000

|

|       6.  IO.LPR=3001

|

|       7.  CM.CSR=00

|

|       8.  CM.FNA=10

|

|       9.  CM.CS= 20

|

|      10.  CM.EWC=30

|

|      11.  CM.ASM=40

|

|      12.  CM.TOP=50

|

|      13.  CM.NMS=200

|

|

|

|  6.0  INSTALLING THE DRIVER|       __________ ___ ______

|

|  The driver is usually installed during the sysgen  procedure.   If  this

|  was  not  done,  obtain  a  copy  of  the current installation tape (DOS

|  format),  copy  it  to  'LB:[200,200]'  and  type  'HELP  %'   or   'PIP

|  TI:=LB:[200,200]HELP.TXT'.

RSX-11M CAMAC DRIVER                                             Page 15

7.0  FOR MORE INFORMATION        ___ ____ ___________

|  For more information obtain a copy of the CAMAC branch  driver  hardware

|  specification   (HN-2  or  HN-15)  or  contact  the  FERMILAB  Computing

|  Department.

Written by Daniel B.  Curtis