PN-85.3

A CAMAC DRIVER FOR RSX-11M

Daniel B. Curtis

30-MAR-79

ABSTRACT                                ________

This program note describes a driver for RSX-11M.  The driver  allows  a

task  to  interface  to a BD-011 or JY411 parallel CAMAC branch highway.

This  driver  is  not  suitable  for  single  purpose  very  fast   data

acquisition systems.

The program note is written with the same format as  a  chapter  in  the

RSX-11M I/O Drivers manual.

Only RSX-11M is supported.

RSX-11M CAMAC DRIVER                                              Page 2

A CAMAC DRIVER FOR RSX11-M

1.0  INTRODUCTION     ____________

The EG&G BD-011 and Jorway JY411 CAMAC branch interfaces  supply  RSX11M

with the ability to access CAMAC modules on a parallel branch highway.

This device driver  is  designed  for  use  with  many  tasks  accessing

different  CAMAC  modules.  Due to the I/O overhead necessary in RSX, it

may not be suitable for extremely fast data acquisition  from  a  single

purpose  CAMAC system.  For fast data acquisition, use a privileged task

or use COMMON and DEVICE partitions.

The device driver is able to support multiple  interfaces  of  the  same

type.   It  is  not possible to mix JY411 and BD-011 interfaces with the

same device driver.

2.0  QIO MACRO     ___ _____

This section summarizes the valid functions of the  QIO  macro  for  the

CAMAC driver.

2.1  Standard QIO Functions     ________ ___ _________

Table 1 lists the standard functions of the QIO macro that are valid for

the CAMAC driver.

Table 1

Standard QIO Functions for CAMAC

Format                            Function       ______                            ________

QIO$C  IO.ATT,...                       Attach device

QIO$C  IO.DET,...                       Detach device

QIO$C  IO.KIL,...                       Cancel I/O requests

RSX-11M CAMAC DRIVER                                              Page 3

2.2  Device Specific QIO Functions For CAMAC     ______ ________ ___ _________ ___ _____

Table 2 lists the device specific functions of the QIO  macro  that  are

supplied for the CAMAC driver.  These functions may be modified by ORing

a modifier, described in section 5, with the function.

There are two units  associated  with  each  branch  driver.   The  even

numbered  unit is used for non NPR type transfers, while the odd unit is

used  for  NPR  type  transfers.   Therefore,  all  IO.CDM  and   IO.LPR

operations  MUST  be  sent  to  odd  numbered  units, all others to even            ____

numbered units.

Table 2

Device Specific QIO Functions for CAMAC

Format                            Function       ______                            ________

QIO$C  IO.CPW,...,<low,hiw,0,cs,fna>    Programmed data write

QIO$C  IO.NDT,...,<0,0,0,cs,fna>        Non-data transfer

QIO$C  IO.CPR,...,<stadd,size,cs,fna>   CAMAC programmed data read

QIO$C  IO.CRR,...,<stadd,14,0,0,0>      CAMAC register read

QIO$C  IO.CDM,...,<stadd,size,cs,fna,bm> CAMAC DMA transfer

QIO$C  IO.LPR,...,<stadd,size,,,cstadd,csize> CAMAC list processing

2.2.1  IO.CRR - The IO.CRR QIO  function  is  used  to  read  the  CAMAC

registers.   The  registers are read out and put in the buffer in order.

If a BD-011 is used, the last word will be zero.  The second  word  will

contain   the   "Selected  Option"  word  (see  section  5).   For  more

information refer to the branch  driver  description  (HN-15  or  HN-2).

Note that the status of the CAMAC registers will depend on the time that

this function is dequeued by the driver.  So, for some applications,  it

may be best to attach the driver to your task before using IO.CRR.

The 'stadd' parameter is the starting  address  of  a  14  (octal)  byte

buffer.  The format of this buffer is shown in table 3.

The 'size' parameter must be 14 (octal).

The next 3 arguments must be zero or blank.

RSX-11M CAMAC DRIVER                                              Page 4

Table 3

Register Read Input Buffer

from low to high address

*********************************

* Control status register       * N

*********************************

* Selected Option Word          * N+2

*********************************

* Memory Address Register       * N+4

*********************************

* Crate select Register         * N+6

*********************************

* Instruction Register          * N+10

*********************************

* Extended Word Count (JY411)   * N+12

*********************************

2.2.2  IO.NDT - The IO.NDT QIO function  is  used  to  execute  a  CAMAC

non-data transfer operation.

The first three arguments must be blank or zero

'cs' is the crate select mask word and must be setup as in table 4

'fna' is the function, station, and address control  word  and  must  be

setup as in table 5

Table 4

Crate Select Mask Word

15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 00

*  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  *

*C7 C6 C5 C4 C3 C2 C1 00 00 00 00 00 00 00 00 00*

*  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  *

Table 5

FNA Control Word

15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 00

*  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  *

*BB SZ F5 F4 F3 F2 F1 N5 N4 N3 N2 N1 A4 A3 A2 A1*

*  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  *

RSX-11M CAMAC DRIVER                                              Page 5

2.2.3  IO.CPR - The IO.CPR QIO function is used to do a programmed  data

read.   Refer  to  section  5 to find the interaction of the 24 bit mode

request with the byte count.

'stadd' is the starting address to store the data.  See table 6 for  the

buffer format.

'Size' is the size in bytes and must be a '2' or '4'

'cs' is the crate select mask

'fna' is the function, station, address word

Table 6

Programmed Data Read Buffer Format

**************************

* Low Order 16 bits      * N

**************************

* zero      - High byte  * N+2

**************************

2.2.4  IO.CPW - The IO.CPW QIO function is used to do a programmed  data

write.

'low' is the low order 16 bits of the 24 bit transfer

'hiw' is the high order 8 bits of the 24 bit transfer

The next parameter must be zero

'cs' is the crate select mask word

'fna' is the function, station, and address control word

If a 16 bit transfer is to be done, the 'hiw' must still be specified.

NOTE

If a 16 bit write is  done  to  a  24  bit

module,  the  upper  8  bits of the module

will be set to a random value.

RSX-11M CAMAC DRIVER                                              Page 6

2.2.5  IO.CDM - The IO.CDM QIO function is used to cause a NPR  transfer

to  and from CAMAC.  This function must be sent to an odd numbered unit;

refer to the programming notes.

'stadd' is the starting address of the buffer

'size' is the size in bytes of the buffer to be used

'cs' is the crate select word

'fna' is the function, station, address control word

'bm' is the control word for the setting of the BM bit (bits 14&15 only)

2.2.6  IO.LPR - The IO.LPR QIO function is used to cause a list  of  NPR

type functions to be sent to the driver.  As such, the operation must be

sent to an odd numbered unit.  The QIO must indicate  two  buffers,  one

for  data  and  another  for  the  operation list.  This QIO function is

described in section 3.

'stadd' is the starting address of the data buffer

'size' is the size in bytes of the data buffer

'cstadd' is the starting address of the list buffer

'csize' is the size of the list buffer

NOTE

The list buffer must be terminated with  a

zero  word.   Currently  there is no check

for overflowing the control  list.   There

is  also no check for correct (odd number)

unit.  These  will  be  considered  for  a

future release.

RSX-11M CAMAC DRIVER                                              Page 7

3.0  LIST PROCESSING OPTION     ____ __________ ______

As mentioned in section 2.2.6, the user may request the execution  of  a

list  of  CAMAC operations by using a single QIO$ request.  This greatly

reduces the system overhead.  The procedure to perform a  list  directed

CAMAC I/O request is:

1.  Set up a list describing the CAMAC operations to be performed

2.  Reserve a buffer area for the storage of any data  to  be  read

from CAMAC modules

3.  Issue an I/O request  using  the  QIO$  MACRO  and  the  IO.LPR

function   code   (All   requests  for  list  processing  CAMAC

operations must be made to odd numbered units as in the case of

normal CAMAC DMA transfers.)

The exact format of the QIO$ MACRO parameters is  described  in  section

2.2.6.  The format of the COP list is described in section 3.1.  Section

5 contains a discussion of driver error returns that  may  occur  during

list processing operations.

3.1  CAMAC Operations Packet (COP) List Format     _____ __________ ______ _____ ____ ______

The CAMAC operations packet (COP) list is a list of CAMAC operations  to

be  performed  by the CAMAC driver.  Each packet in the COP list has the

following format:

---------------------------------

| BYTE COUNT    | OPTION FLAGS  |

---------------------------------

| CRATE SELECT                  |

---------------------------------

| WORD COUNT                    |

---------------------------------

| FNA                           |

---------------------------------

| OPTIONAL DATA  (0 TO 3 WORDS) |

---------------------------------

| FIRST WORD OF THE NEXT PACKET |

---------------------------------

Where:

BYTE COUNT    specifies the number of bytes in the  COP  list  packet.

This  value will be between eight and fourteen depending

on the number of optional data words.

OPTION FLAGS  is a bit mask used to request special operations  to  be

performed.

CRATE SELECT  specifies the word to be loaded into the  branch  driver

crate  select/word  count  register  when  the operation

RSX-11M CAMAC DRIVER                                              Page 8

specified by this packet is performed.

WORD COUNT    specifies the value to be loaded into the branch  driver

extended  word  count  register  when  the  operation is

performed.  Only bits 15,14, and 11-0 may be set.   Bits

13 and 12 are set by the driver.  To set the BM bit, you

set bit 15 in this register.

FNA           specifies the value to be loaded into the branch  driver

instruction register when the operation is performed.

OPTIONAL DATA specifies the values to be used by any  of  the  options

selected  by the OPTION  FLAGS byte in the first word of

the  packet.   The  contents  of  this  area  is   fully

described below in section 3.1.1.

The COP list is terminated by a packet whose first word is zero.

3.1.1  Description Of OPTION FLAGS -

This section describes the function of the  OPTION FLAGS  field  of  the

COP.   This byte is used as a bit mask to select special operations.  In

the following discussion, bit 0 refers to the least significant  bit  of

the byte.

The option byte is interpreted as follows.  The bits within the byte are

examined one at a time starting with bit zero.  A set bit indicates that

the option associated with that bit has been selected.   If  the  option

selected  requires a data word then the value stored in the next word of

the OPTIONAL DATA area is used.  The bit assignments and associated data

word descriptions are listed below.

BIT 0  If this bit is set, the next word in the  OPTIONAL DATA  area

specifies  the buffer offset at which the first word of CAMAC

data read by this command is to  be  placed.   If  no  buffer

offset  is specified, any data that is read will be placed in

the next available buffer word (starting with the first  word

of  the  buffer if no offset is specified in the first packet

in the COP list).

BIT 1  If this bit is set, the next word in the  OPTIONAL DATA  area

specifies   the   expected  end  CAMAC  crate,  station,  and

subaddress of this operation.  The format of this  data  word

is:

bit 0 -> bit 3 = subaddress

bit 4 -> bit 8 = station

bit 9 -> bit 15 = crate mask

BIT 2  If this bit is set the next word in  the  OPTIONAL DATA  area

specifies  a  repeat count for the operation requested in the

packet.

RSX-11M CAMAC DRIVER                                              Page 9

BIT 3  Reserved

BIT 4  Reserved

BIT 5  This bit is set to  allow  unknown  length  transfers  to  be

specified.  It will cause the word count to be overwritten if

the  specified  word  count  would  cause  the  operation  to

overflow  the  end  of the data buffer.  The COP will execute

just as normal but the maximum number of operations  that  it

is allowed to execute is adjusted to fit into the buffer.

BIT 6  If this bit is set the driver will ignore any CAMAC time  out

errors  (T02  bit in the interface CSR) that may occur during

the execution of  the  command.   If  not  set,  a  time  out

condition will produce an error.

BIT 7  If this bit is set the driver will  ignore  the  CAMAC  crate

overflow  error  condition.   If  not,  crate  overflow  will

produce an error.

4.0  STATUS RETURNS     ______ _______

The error and status conditions listed in table 7 are  returned  by  the

CAMAC  driver  in the first byte of the I/O status block.  The next byte

will contain a packed representation of the control status word  of  the

branch  driver  (see  table 8) for all functions except for IO.LPR.  For

the IO.LPR function, the byte will contain the number of  the  operation

in the list ( the first operation is operation '0').

The second word of the I/O status block will contain the value  selected

from  the function modifiers described in section 5.  The default is the

control  status  word  of  the  branch  interface  after  the  operation

completed.

RSX-11M CAMAC DRIVER                                             Page 10

Table 7

Status returns

IS.SUC    Successful Completion- The  operation  specified  in  the  QIO

directive was completed successfully.

IE.VER    Unrecoverable Error- The operation  caused  a  CAMAC  timeout.

Typically this is due to an offline crate, the selection of an

unoccupied station, or the lack of an  'X'  response  for  the

JY411.   If the 'CM.NMS' modifier is not set, a 'SELECT ERROR'

message will be printed on CO:.

IE.BAD    Bad Parameter- This occurs if an attempt was made to do a  NPR

transfer  to an even numbered unit or a non-NPR transfer to an

odd numbered unit.

IE.ONP    Illegal Subfunction- This is due only to programmer error.  It

is caused by attempting to do an illegal transfer function.

Status Returns for List Operations

IE.FHE    Fatal Hardware Error- The expected 18 bit address of the  last

word of the list operation was not correct.  This implies that

the NPR function of the branch driver  is  malfunctioning  and

the  system  may  be  corrupted.  A 'DEVICE NOT READY' message

will be output at CO: and the NPR unit will  be  set  offline.

This  will  cause any other attempt to use that unit to return

with an IE.OFL (device offline) error.

IE.RCN    Record Number Error-This error is returned  if  an  unexpected

CAMAC timeout (T02) occurs.

IE.VER    Should never happen when list processing.  If it does, contact

the RSX-11M maintanence person in the Computing Department.

IE.RBG    Illegal  Record  Size-  An  unexpected  crate   overflow   was

encountered.

IE.SPC    Illegal User Buffer- This is set if an  attempt  was  made  to

write  data outside of the data buffer.  It will be set if the

buffer offset is outside of the buffer or if a a COP  attempts

to  do  a  transfer that could extend past the end of the data

buffer (see section 3.1.1 for the description of BIT 5 of  the

option flag byte).

IE.DAO    Data Overrun- This is set if an end CNA check was requested in

the list and it failed.

RSX-11M CAMAC DRIVER                                             Page 11

Table 8

Packed Status Byte

Bit 0     Branch Q of operation

Bit 1     Branch X of operation

Bit 2     Abort bit

Bit 3     Crate Overflow bit

Bit 4     CAMAC Timeout

Bit 5     PDP Timeout

Bit 6-7   Zero

The driver may produce a 'Device Not  Ready'  message  at  the  console.

This  is  due  to  programmer error or a bad interface.  If this message

occurs, a device display should be done.  If only the odd unit  but  not

the  even unit is marked offline the branch driver is faulty and must be

replaced<1>.

<1> The hardware addressing of the Jorway is faulty and it  is  possible

that the system has been corrupted.

RSX-11M CAMAC DRIVER                                             Page 12

5.0  PROGRAMMING HINTS     ___________ _____

This section contains information on programming considerations that are

relevant to the users of the CAMAC driver.

5.1  Function Modifier Actions     ________ ________ _______

The function modifier is used to control the information returned to the

I/O status block and to suppress SELECT ERROR messages.

The possible modifiers follow:

1.  Only one of this type of modifier may be used.   Any  of  these

modifiers may be used with the next set of modifiers.

1.  CM.ASM- This will  cause  the  Option  Select  word  to  be

returned to the second word of the I/O status block.

2.  CM.CS- This will cause the Crate  Select  register  of  the

interface  to  be  returned  to  the second word of the I/O

status block.

3.  CM.CSR- This (the default  case)  will  cause  the  control

status  register  of  the  interface  to be returned to the

second word of the I/O status block.

4.  CM.EWC- This will cause the extended word count register of

the  interface to be returned to the second word of the I/O

status block.

5.  CM.FNA- This will cause the FNA register of  the  interface

to be returned to the second word of the I/O status block.

6.  CM.TOP-  This  may  only  be  selected  when   doing   list

processing.    It  returns  the  index  of  the  last  byte

transferred from the buffer.  This index when added to  the

starting address of the buffer generates the address of the

last word transferred.

2.  CM.NMS- This modifier may be added with any one  of  the  above                                                     ___

modifiers.   It  will suppress the generation of a SELECT ERROR

message when a module does not respond or if the transfer falls

off of the branch.

RSX-11M CAMAC DRIVER                                             Page 13

5.2  Selected Option Word     ________ ______ ____

This word indicates which assembly options were selected for the driver.

The  word  is  bit encoded for the operations and the currently assigned

bits are:

1.  Bit 0- Indicates  driver  is  able  to  generate  SELECT  ERROR

messages.

2.  Bit 1- Indicates able to do list processing.

3.  Bit 2- Driver expects to use a Jorway Branch interface.

5.3  IO.CPR     ______

If the byte count is '2' and a 24 bit transfer is  requested,  only  the

lower  16  bits  are transferred.  If the byte count is '4' and a 16 bit

transfer is requested, the second word will be zero.

5.4  IE.VER     ______

When a non list processing operation detects an IE.VER error return  and

CM.NMS  was  not  selected,  you  should wait at least 15 seconds before

attempting to do another CAMAC operation.  This is  due  to  the  select

error  message  printed  out at the console.  This message is printed by

TKTN.  If TKTN and the MCR reside in the same partition, MCR will  never

be  able  to  get  in to abort the task causing the select errors.  This

condition does not occur if TKTN and MCR reside in different partitions.

5.5  Unit Selection     ____ _________

All non-NPR (non block transfer) operations to CAMAC must be sent to  an

even  numbered unit.  All NPR operations must be sent to an odd numbered

unit.  If this rule  is  violated  an  IE.BAD  status  will  usually  be

returned.   If  the  program  attempted  to  do  a  NPR operation with a

function that does not require a transfer, the driver will time out with

a  IE.ONP status return and a 'Device not ready' message will be printed

at the console.  The NPR functions are IO.CDM and IO.LPR.

5.6  FORTRAN Use     _______ ___

When using the driver from a FORTRAN program some of the QIO  directives

require an address in the parameter array.  These QIO functions are:

RSX-11M CAMAC DRIVER                                             Page 14

1.  IO.CPR

2.  IO.CRR

3.  IO.CDM

4.  IO.LPR

5.7  Function Code Definitions     ________ ____ ___________

The following function codes may be defined in the user program,  placed

in  the  system  macro  library,  or  placed  in  the system default I/O

definition module in the system  object  library.   All  values  are  in

octal.

1.  IO.CPW=400

2.  IO.NDT=402

3.  IO.CPR=1000

4.  IO.CRR=1001

5.  IO.CDM=3000

6.  IO.LPR=3001

7.  CM.CSR=00

8.  CM.FNA=10

9.  CM.CS= 20

10.  CM.EWC=30

11.  CM.ASM=40

12.  CM.TOP=50

13.  CM.NMS=200

6.0  INSTALLING THE DRIVER     __________ ___ ______

The driver is usually installed during the sysgen  procedure.   If  this

was  not  done,  obtain  a  copy  of  the current installation tape (DOS

format),  copy  it  to  'LB:[200,200]'  and  type  'HELP  %'   or   'PIP

TI:=LB:[200,200]HELP.TXT'.

RSX-11M CAMAC DRIVER                                             Page 15

7.0  FOR MORE INFORMATION     ___ ____ ___________

For more information obtain a copy of the CAMAC branch  driver  hardware

specification   (HN-2  or  HN-15)  or  contact  the  FERMILAB  Computing

Department.

Written by Daniel B.  Curtis