TPARS Users Manual

**

**

************************

*                      *

*  TPARS Users Manual  *

*                      *

*     28-May-1976      *

*                      *

*   A. C. Goldstein    *

*                      *

*    130-958-021-02    *

*                      *

************************

TPARS Users Manual                                    PAGE 2

1.0  	INTRODUCTION

Fair warning:  The reader  is  assumed  to  have  a  cursory

knowledge  of  table  driven  parsing  and/or  finite  state

automata.

TPARS is a general  purpose  table  driven  parser.   It  is

fundamentally  capable  of  handling anything that is finite

state parsable.  Due to its structure and  choice  of  basic

lexical  types,  however,  it  works  most  efficiently  for

keyword grammars such as  operating  system  command  lines,

etc.

A string is parsed by  interpreting  the  transitions  in  a

state  table  supplied  by the caller.  The state table is a

list  of  states.   Each  state  consists  of  a   list   of

transitions to other states.  Each transition is composed of

the following elements:

Type      The  type  of  syntactic   element   which,   when

recognized  in  the  input string, will cause this

transition to be taken.

Target    The label of the state to transfer to.

Action    Address of a user supplied action routine to  call

when taking this transition.

Mask      A bit mask to set when taking this transition.

Address   The address into which to set the mask when taking

this transition.

The action, mask, and address components of a transition are

optional.   The  target  component  of  a  transition may be

optional under certain circumstances explained later.

When a transition is taken, the section of the input  string

that  matches  the  specified  type  is passed over, and the

characters following it become  available  for  matching  by

transitions in the next state.

The structure of the state table specifies  the  syntax  the

parser   will  accept;   the  action  routines  specify  the

semantics.

TPARS Users Manual                                    PAGE 3

2.0  	STATE TABLE SOURCE CODING

A set of macros is provided to  generate  the  state  table.

The  state  table  proper  is built in a Psect named $STATE.

All keyword strings are accumulated in  a  separate  keyword

table  in  Psect $KSTR.  A keyword pointer table is built in

Psect $KTAB.  If the user defines  the  symbol  $RONLY,  the

$STATE,  $KSTR,  and  $KTAB Psects will be generated as read

only.  The state table is coded at the source level with the

following macro calls:

2.1  	ISTAT$	Statetable,Keytable

ISTAT$ initializes table generation.  It equates  the  label

"Statetable"  to the start of the state table, and the label

"Keytable"  to  the  start  of  the  keyword  pointer  table

(explained below).

2.2  	STATE$	Label

STATE$ declares the beginning of  a  state.   The  label  is

optional.   If  present, it is equated to the address of the

state.  Because of the way the table generation macros work,

the  state  table  must  be terminated with one extra STATE$

call to force completion of the last transition entry.

2.3  	TRAN$	Type,Label,Action,Mask,Address

TRAN$ specifies a state transition.

2.3.1     Type -

"Type" specifies the syntactical type that will  cause  this

transition to be taken.  "Type" may be one of the following:

$ANY      Matches any single character.

$ALPHA    Matches any single alphabetic character.

$DIGIT    Matches any single digit (0-9).

$LAMDA    Matches the  empty  string.   This  transition  is

always  successful.  Lambda transitions are useful

for getting action routines called without passing

any of the input string.

TPARS Users Manual                                    PAGE 4

$NUMBR    Matches a number.  A number consists of  a  string

of  digits,  followed  optionally by a period.  If

the number is not followed  by  a  period,  it  is

taken as octal;  if it is, it is taken as decimal,

and the decimal point is included in the  matching

string.  A number is terminated by any non-numeric

character.  Values up through 2**32 -  1  will  be

converted   correctly   into  a  32  bit  unsigned

integer.

$DNUMB    Matches a decimal number.  The string of digits is

interpreted  as  decimal.  The matched string does

not include trailing decimal  points.   Otherwise,

it is treated the same as $NUMBR.

$STRNG    Matches any alphanumeric string.  The string  will

not be null.

$RAD50    Matches  any  legal  Radix-50  string,  i.e.,  any

non-null  string  containing  alphanumerics and/or

dot and dollar sign.  Conversion to Rad-50  is  up

to the user's action routine.

$BLANK    Matches any non-null string of blanks and tabs.

$EOS      Matches end of the input string.  Once  TPARS  has

reached  the  end  of  the input string, $EOS will

match as many times as it is  encountered  in  the

state table.

Char      Matches the  single  character  whose  ASCII  code

corresponds to the value of the expression "Char".

The value of the expression must be a 7 bit  ASCII

code;   i.e.,  must  lie  in  the  range  0 to 177

(octal).  Constructions such as 'X are  legal  and

in fact recommended.

"Keyword" Matches the specified keyword.   Keywords  may  be

any   length,   must   contain  only  alphanumeric

characters,  and   must   be   terminated   by   a

non-alphanumeric  character  in  the  string being

parsed.  A state table may contain a maximum of 64

keywords.

!Label    Matches  the  sub-expression  described   by   the

section  of  the  state  table  entered  at  state

"Label" and terminated by a $EXIT.  i.e., TPARS is

caused   to   call   itself   recursively.    This

construction  is  useful  for   handling   complex

syntactical  types  that  must  appear  in several

places in the state table.  It is also useful  for

recursive     or    indeterministic    syntactical

structures.  Note, however,  that  each  level  of

TPARS Users Manual                                    PAGE 5

recursion requires six words of stack space.

2.3.2     Label -

"Label" specifies the target state of  the  transition.   If

"Label"  is  null,  the  target  is  defaulted  to the state

immediately following.  A null label may be used only in the

last  transition  in  a  state.   If the label is $EXIT, the

parse is terminated and TPARS returns to the calling program

with carry clear to indicate a successful parse.

2.3.3     Action -

"Action" is the address of a caller supplied action  routine

which  is  called when the transition is taken.  If "Action"

is null, no call is made.  The following internal  variables

are available to action routines:

.PSTCN    Character count and

.PSTPT    address of the portion of the input string matched

by  this transition.  These two variables are good

for  all  types  recognized  by  TPARS,  including

subexpressions.

.PNUMH    High order and

.PNUMB    low order binary value  of  the  number  found  by

$NUMBR or $DNUMB.

.PCHAR    The character found by $ANY,  $ALPHA,  $DIGIT,  or

"Char".

.PFLAG    The flag word supplied by the call to .TPARS in R1

(see  calling sequence below).  The action routine

may modify this word to change the options on  the

fly.

R3        Byte count and

R4        address of the remainder of the input string.   At

the  time the action routine is called, the string

does not include the  characters  matched  by  the

transition being taken.

Action routines are called with JSR PC.  They may modify R0,

R1, and R2 but must preserve all other registers.

An action routine may reject the state transition from which

it  is  being  called  by returning at call+4 instead of the

normal call+2.   (i.e.,  the  action  routine  performs  the

equivalent of ADD  #2,(SP) before returning.) This mechanism

TPARS Users Manual                                    PAGE 6

permits   an   action   routine   to   perform    additional

discrimination  of  syntactic  types,  and  allows arbitrary

extensibility of the set of  "hard  wired"  syntactic  types

made  available  by TPARS.  When an action routine rejects a

transition, the overall effect is as if that transition  had

never  matched.  TPARS will continue to attempt to match the

remaining transitions in the state.

2.3.4     Mask -

"Mask" specifies a mask word that is to be stored  when  the

transition is taken.  If "Mask" is null, nothing is stored.

2.3.5     Address -

"Address" specifies the location into which this  mask  word

is   to   be   stored.    Functionally,   TPARS  executes  a

"BIS  #MASK,ADDRESS" when the  transition  is  taken.   This

construct  allows  the  user  to  avoid  having to supply an

action routine when all he wants to do is flag the fact that

a  particular  transition  has  been  taken.  The "Mask" and

"Address" arguments must be present or absent together.

2.4  	General Coding Considerations

Note that the sets of strings that can be represented by the

transitions   in  a  state  are  not  necessarily  disjoint.

Therefore the order in which  types  are  presented  in  the

transition  table  of  a state is critical.  Transitions are

always scanned in the order in which they are listed;  hence

the following ordering of mixed types is recommended:

All Char

All "Keyword"

$EOS

$ALPHA

$DIGIT

$BLANK

$NUMBR

$DNUMB

$STRNG

$RAD50

$ANY

$LAMDA

The placement of !Label transitions in a  state  depends  on

the basic types used within the subexpression.

TPARS Users Manual                                    PAGE 7

The handling of arguments by the assembler's macro processor

causes  some problems when character expressions are used in

the "Char" syntactical type.  While most characters  can  be

matched with a straightforward expression, such as

TRAN$ '=

a comma must be enclosed in angle brackets  to  avoid  being

taken as an argument separator

TRAN$ <',>

Angle brackets cannot be entered as  character  literals  at

all, but must be expressed symbolically, i.e.

LA = '<

TRAN$ LA

2.5  	Blank Suppress

TPARS may optionally be caused to flush blanks in the string

being  parsed.  If so requested, all blanks and tabs will be

ignored at the syntactic element level;   i.e.,  blanks  and

tabs may appear between any syntactic elements without being

called for in the state table.  The string being  parsed  is

not  modified.   Note that under this option the $BLANK type

will never match.  Note also that when a "!Label" transition

is taken, the matching string specified by .PSTCN and .PSTPT

may contain blanks or tabs, even though none were called for

in  the  subexpression.   The matching strings for the basic

types will never contain extraneous blanks, however.

Under the blank flush option, blanks and  tabs  function  as

invisible terminators.  Consider the string

ABC DEF

This string is matched by the following state table segments

with and without the blank flush option:

Without             With

STATE$

TRAN$     $STRNG    STATE$

STATE$              TRAN$     $STRNG

TRAN$     $BLANK    STATE$

STATE$              TRAN$     $STRNG

TRAN$     $STRNG

TPARS Users Manual                                    PAGE 8

3.0  	INTERNAL REPRESENTATION

The state table is coded as follows.  Each state consists of

its  transition  entries  concatenated in the order in which

they are specified.  The state label, if present, is equated

to  the  address of the first transition in the state.  Each

transition entry consists of 1 to 6 words as follows:

Type - 1 byte.      May assume the following values:

$LAMDA    = 300

$NUMBR    = 302

$STRNG    = 304

$BLANK    = 306

$SUBXP    = 310     Used in the !Label type.

$EOS      = 312

$DNUMB    = 314

$RAD50    = 316

$ANY      = 320

$ALPHA    = 322

$DIGIT    = 324

Char      = Ascii value of the character.

"Keyword" = 200+n   Where n is an  index  into  the  keyword

table.  The keyword table is an array of

pointers to the keyword  strings.   Each

string   is  stored  terminated  with  a

single "377".

Flags - 1 byte.     Bits are used as follows:

Bit 0     Type extension exists.

Bit 1     Action exists.

Bit 2     Mask word exists.

Bit 3     Address word exists.

Bit 4     Target exists.

Bit 7     This is last transition in this state.

The remaining five words are optional:

Type extension - 1 word.  Present only in the  !Label  type.

Contains starting state of the subexpression.

Action - 1 word.  Present if action routine  was  specified.

Contains address of the action routine.

Mask - 1 word.  Present if mask argument was supplied.  This

is the bit mask to set.

Address - 1 word.  Present if address argument was supplied.

Contains address into which to set the mask.

TPARS Users Manual                                    PAGE 9

Target - 1 word.   Present  if  explicit  target  state  was

specified.  Contains address of target state.  The target of

$EXIT is coded as a zero word.

The assembly listing produced by the state table  macros  is

not  straightforward.   The  object  code  was  designed for

compactness and convenience in processing,  and  the  source

macro  language  was designed for clarity and convenience in

writing state tables.   As  a  result,  the  mechanism  that

translates  between  the  two  (the  macros) is of necessity

somewhat obscure.

The binary output of the macros is delayed by one statement.

Thus,  if  the  listing  of  macro  generated binary code is

enabled, the binary code that  appears  after  a  particular

macro  call  is  in  fact the result of the preceding macro.

Error  messages  generated  by  the  macros  are   similarly

delayed.   This  is  the reason for the extra STATE$ call at

the end of a state table.

The macros generate code in three Psects ($STATE, $KTAB, and

$KSTR).   At  the  end of each macro expansion, the Psect is

reset to the blank Psect, to prevent user  code  from  being

loaded  into any of these Psects accidentally.  As a result,

the location counter displayed for each macro  call  is  not

meaningful,  since  it is simply the current position in the

blank Psect.

TPARS Users Manual                                   PAGE 10

4.0  	CALLING SEQUENCE

TPARS is called with the following arguments:

R1 = Options word

R2 = Pointer to the keyword table

R3 = Length of string to be parsed

R4 = Address of string to be parsed

R5 = Label of starting state in state table

CALL      .TPARS

On return,

R3 = Length and

R4 = address of the unscanned portion of the

string

Other registers are preserved.

The low byte of the options word contains  flag  bits.   The

only  flag  defined is bit 0;  it controls the processing of

blanks.  If the bit is set, blanks are processed normally by

transitions  in the state table;  if it is clear, blanks are

ignored as described above.

The high byte of the options word  is  the  match  count  on

keywords.   If it is zero, all keyword strings in the string

being parsed must match the keywords exactly  as  listed  in

the state table.  If this byte is set to the value "n", then

all keywords may be abbreviated  down  to  a  minimum  of  n

characters.   Keywords  given  with  more  than  the minimum

number  of  characters  must  still  be  spelled  correctly.

Keywords  that  are  shorter than n characters must still be

given   exactly.    Prevention   of   ambiguities   is   the

responsibility of the state table designer.

The C bit is cleared to indicate a successful  parse.   This

occurrs  when  a  transition  is  made to $EXIT which is not

within a subexpression.  If a syntax error  is  encountered,

the  parse  is  terminated  and TPARS returns with the C bit

set.  A syntax error occurrs when there are no types  listed

in  the  current  state which match the portion of the input

string currently being scanned.   Illegal  type  codes,  and

other  forms of bad data in the state table, will also cause

a syntax error return.  Note that TPARS trusts addresses  in

the  state  and  keyword  tables, and that bad addresses may

cause program termination.  Note that when the  end  of  the

input  string is reached, the only types that will match are

$LAMDA and $EOS.

TPARS Users Manual                                   PAGE 11

5.0  	SPACE REQUIREMENTS

The routine .TPARS  proper  is  253  words  in  length.   In

addition,  it  calls .OD2CT and .DD2CT in the system library

(an additional 68 words).

6.0  	AVAILABILITY

TPARS and the table generation macros are available  in  the

system  library of RSX-11M, RSX-11D, and IAS.  Copies of the

routines may also be obtained from the author.

7.0  	EXAMPLE 1

A sample state table and set of action  routines  appear  on

the following pages.  Their function is to parse the command

line to the UFD command in RSX-11M.  The general form of the

command line is as follows:

UFD DK0:LABEL[201,202]/ALLOC=100./PRO=[RWED,RWED,RWE,R]

The associated action routines return the following values:

$UDEV     Device name (2 characters ascii)

$UUNIT    Unit number (binary)

$UVNML    Byte count and

$UVNAM    address of volume label string

$UUIC     Binary UIC for which to create directory

$UALL     Number of directory entries to preallocate

$UPRO     Binary protection word for UFD

$FLAGS    Flags word containing the following bits:

UF.ALL    Set if allocation was specified.

UF.PRO    Set if protection was specified.

Observe  that  the  "LABEL"  and  the  "/ALLOC"  and  "/PRO"

switches are optional.

TPARS Users Manual                                   PAGE 12

.TITLE  STATE TABLE FOR UFD COMMAND LINE

.MCALL  ISTAT$,STATE$,TRAN$

; TO BE USED WITH BLANK SUPPRESS OPTION

ISTAT$  UFDSTB,UFDKTB

; READ OVER COMMAND NAME

.GLOBL  START

STATE$  START

TRAN$   "UFD"

; READ DEVICE AND UNIT NUMBER

STATE$

TRAN$   $ALPHA,,SETDV1

STATE$

TRAN$   $ALPHA,,SETDV2

STATE$

TRAN$   $NUMBR,DEV1,SETUNT

TRAN$   $LAMDA

STATE$  DEV1

TRAN$   ':

; READ VOLUME LABEL

STATE$

TRAN$   $STRNG,RUIC,SETLAB

TRAN$   $LAMDA

; READ UIC

STATE$  RUIC

TRAN$   !UIC

; SCAN FOR OPTIONS AND END OF LINE

STATE$  OPTS

TRAN$   $EOS,$EXIT

TRAN$   '/

STATE$

TRAN$   "ALLOC",ALC,,UF.ALL,$FLAGS

TRAN$   "PRO",PRO,,UF.PRO,$FLAGS

TPARS Users Manual                                   PAGE 13

; SET ALLOCATION

STATE$  ALC

TRAN$   '=

STATE$

TRAN$   $NUMBR,OPTS,SETALC

; PROTECTION

STATE$  PRO

TRAN$   '=

STATE$

TRAN$   '[,,IGROUP

STATE$  SPRO

TRAN$   '],OPTS,ENDGRP

TRAN$   <',>,SPRO,NXGRP

TRAN$   'R,SPRO,SETRP

TRAN$   'W,SPRO,SETWP

TRAN$   'E,SPRO,SETEP

TRAN$   'D,SPRO,SETDP

; SUBEXPRESSION TO READ AND STORE UIC

STATE$  UIC

TRAN$   '[

STATE$

TRAN$   $NUMBR,,SETGN

STATE$

TRAN$   <',>

STATE$

TRAN$   $NUMBR,,SETPN

STATE$

TRAN$   '],$EXIT

STATE$

State Table Size:            60 words

Keyword Table Size:          8 words

Keyword Pointer Space:       3 words

TPARS Users Manual                                   PAGE 14

.SBTTL ACTION ROUTINES FOR THE COMMAND LINE PARSER

; DEVICE NAME CHAR 1

SETDV1::MOVB    .PCHAR,$UDEV

RETURN

; DEVICE NAME CHAR 2

SETDV2::MOVB    .PCHAR,$UDEV+1

RETURN

; UNIT NUMBER

SETUNT::MOV     .PNUMB,$UUNIT

RETURN

; VOLUME LABEL

SETLAB::MOV     .PSTCN,$UVNML

MOV     .PSTPT,$UVNAM

RETURN

; PPN - GROUP NUMBER

SETGN:: MOVB    .PNUMB,$UUIC+1

BR      TSTPPN

; PPN - PROGRAMMER NUMBER

SETPN:: MOVB    .PNUMB,$UUIC

TSTPPN: TST     .PNUMH       ; CHECK FOR ZERO HIGH ORDER

BNE     10$

TSTB    .PNUMB+1     ; CHECK FOR BYTE VALUE

BEQ     20$

10$:    ADD     #2,(SP)      ; BAD VALUE - REJECT TRANSITION

20$:    RETURN

; NUMBER OF ENTRIES TO ALLOCATE

SETALC::MOV     .PNUMB,$UALL

RETURN

TPARS Users Manual                                   PAGE 15

; SET PERMISSIONS

; INITIALIZE

IGROUP::MOV     #4,GRCNT

; MOVE TO NEXT PERMISSIONS CATEGORY

NXGRP:: SEC                  ; FORCE ONES

ROR     $UPRO

ASR     $UPRO        ; SHIFT TO NEXT GROUP

ASR     $UPRO

ASR     $UPRO

DEC     GRCNT        ; COUNT GROUPS

BGE     30$          ; TOO MANY IS AN ERROR

BADGRP: ADD     #2,(SP)      ; IF SO, REJECT TRANSITION

30$:    RETURN

; SET READ PERMIT

SETRP:: BIC     #FP.RDV*10000,$UPRO

RETURN

; SET WRITE PERMIT

SETWP:: BIC     #FP.WRV*10000,$UPRO

RETURN

; SET EXTEND PERMIT

SETEP:: BIC     #FP.EXT*10000,$UPRO

RETURN

; SET DELETE PERMIT

SETDP:: BIC     #FP.DEL*10000,$UPRO

RETURN

; END OF PROTECTION SPEC

ENDGRP::TST     GRCNT        ; CHECK THE GROUP COUNT

BNE     BADGRP       ; MUST HAVE 4

RETURN

.END    UFD

TPARS Users Manual                                   PAGE 16

8.0  	EXAMPLE 2

As  an  illustration  of  the  use  of  subexpressions   and

transition  rejection,  consider  the following excerpt of a

state table which parses a string  quoted  by  an  arbitrary

character  (i.e.,  the  first  character  is taken to be the

quote character).  Such a construct occurrs in many  editors

and  some  programming  languages.   The  associated  action

routines leave the byte count and address of the  string  in

locations  "QSTC"  and "QSTP", respectively, and the quoting

character in location "QCHAR".

; MAIN LEVEL STATE TABLE

;

; PICK UP THE QUOTE CHARACTER

STATE$  STRING

TRAN$   $ANY,,SETQ

; ACCEPT THE QUOTED STRING

STATE$

TRAN$   !QSTRG,,SETST

; GOBBLE UP THE TRAILING QUOTE CHARACTER

STATE$

TRAN$   $ANY,NEXT,RESET

; SUBEXPRESSION TO SCAN THE QUOTED STRING

; THE FIRST TRANSITION WILL MATCH UNTIL IT IS REJECTED

; BY THE ACTION ROUTINE

STATE$  QSTRG

TRAN$   $ANY,QSTRG,TESTQ

TRAN$   $LAMDA,$EXIT

TPARS Users Manual                                   PAGE 17

; ACTION ROUTINES

;

; STORE THE QUOTING CHARACTER

SETQ:   MOVB    .PCHAR,QCHAR

INCB    .PFLAG       ; TURN OFF SPACE FLUSH

RETURN

; TEST FOR QUOTING CHARACTER IN THE STRING

TESTQ:  CMPB    .PCHAR,QCHAR

BNE     10$

ADD     #2,(SP)      ; REJECT TRANSITION ON MATCH

10$:    RETURN

; STORE THE STRING DESCRIPTOR

SETST:  MOV     .PSTPT,QSTP

MOV     .PSTCN,QSTC

RETURN

; RESET THE SPACE FLUSH FLAG

RESET:  DECB    .PFLAG

RETURN

TPARS Users Manual                                   PAGE 18

9.0  	EXAMPLE 3

The  following   state   table   excerpt   illustrates   how

subexpressions   may   be   used  to  parse  indeterministic

grammars.  The state table excerpt accepts a number followed

by  a  qualifier  keyword;   depending upon the keyword, the

number is interpreted either as octal or  as  decimal.   The

binary  value  of  the  number  is  left in the words tagged

NUMBER.  The following sort of strings will be accepted:

10/OCTAL

359/DECIMAL

77777/OCTAL

This syntax is  ambiguous  for  an  LR(1)  grammar;   it  is

unambiguous for an LR(2) grammar.

; MAIN STATE TABLE ENTRY - ACCEPT THE EXPRESSION AND

; STORE ITS VALUE

STATE$

TRAN$   !ONUMB,NEXT,SETNUM

TRAN$   !DNUMB,NEXT,SETNUM

; SUBEXPRESSION TO ACCEPT OCTAL NUMBER

STATE$  ONUMB

TRAN$   $NUMBR

STATE$

TRAN$   '/

STATE$

TRAN$   "OCTAL",$EXIT

; SUBEXPRESSION TO ACCEPT DECIMAL NUMBER

STATE$  DNUMB

TRAN$   $DNUMB

STATE$

TRAN$   '/

STATE$

TRAN$   "DECIMAL",$EXIT

; ACTION ROUTINE TO STORE THE NUMBER

SETNUM: MOV     .PNUMB,NUMBER

MOV     .PNUMH,NUMBER+2

TPARS Users Manual                                   PAGE 19

RETURN

Note that  the  contents  of  .PNUMB  and  .PNUMH  are  left

undisturbed  by  all state transitions except for the $NUMBR

and $DNUMB types.

Because of the way in which  subexpressions  are  processed,

calls  to action routines from within subexpressions must be

handled with some care.  When a subexpression is encountered

in  a  transition, TPARS saves its current context and calls

itself, using the label of  the  subexpression  as  the  new

starting  state.   If  the subexpression parses successfully

and returns with a $EXIT, all is well and that transition is

taken.   If  the  subexpression  fails  (encounters a syntax

error), then TPARS restores the saved context to back out of

the  subexpression  and  tries  the  next  transition in the

original state.  However, there is no facility  for  undoing

actions  performed  by  the user's action routines that were

called from within  the  subexpression.   Therefore,  action

routines  called  by transitions in a subexpression that may

fail should store their results in some  intermediate  area,

and  let  their  results  be  made  "permanent" by an action

routine which is called from the main level state table.