Send/Receive Driver for RSX-11M

Variable Length Send/Receive Driver for RSX-11M v.3.2

Joseph Sventek

Computer Science & Mathematics Department

Lawrence Berkeley Laboratory

Berkeley, CA  94720

-1-

Send/Receive Driver for RSX-11M

The   send/receive   driver  permits  a  variable-length  message

capability for RSX-11M v.3.2.  The receiver task does not have to

be installed for the message to be buffered.   The  messages  are

buffered  in  an internal pool within the driver, thus freeing up

the Dynamic Storage Region of  RSX  for  other  myriad  uses.   A

lifetime  is  associated  with  each  message,  after  which  the

internal buffer space is freed up for further  use.  The  buffers

are also freed when the message is read by the receiver task.

The flushing of messages is performed as follows: when the driver

is  loaded, it is called at its powerfail entry point.  There, it

attempts to allocate a clock queue control block  from  DSR.   If

this  is  successful, it fills in the appropriate information and

queues a timer request for itself TINTPT seconds in  the  future.

If  the  clock block could not be allocated, the device is marked

off-line and a "DEVICE  NOT  READY"  message  is  issued  at  the

console via TKTN. If this happens to you, your system is ready to

crash anyway, but the check is performed nonetheless.

When  the  timer  routine  is  entered,  the queue of messages is

searched for messages which have outlived their usefulness.   Any

found  are  removed  from the queue and their buffers returned to

the  driver's  internal  pool.   Just  before  returning  to  the

executive,  the  timer  routine  queues another timer request for

itself TINTPT seconds in the future.

Due to this perpetual timer, just unloading the driver  could  be

catastrophic,  since the timer request is for a fixed location in

physical memory.  In order to permit unloading of the  driver,  a

separate  task is provided (UNX, extended unload).  UNX issues an

IO.STP QIO to the driver.  If this QIO succeeds, UNX then  spawns

the following commands to MCR...:

DEVICE SR:

UNLOAD SR:

DEVICE SR:

UNX  is  distributed  with  the  Virtual  Aether  driver  on  UIC

[307,35].   The  QIO  will  fail  if  UNX  is  invoked   from   a

non-priveleged  terminal.   If  you use SR: for passing arguments

within the Software Tools system, it is suggested that you VMR it

into DRVPAR and leave it there.

To build the driver,  copy  the  contents  of  [307,34]  to  your

current UIC and invoke

>@SRGEN

Answer the questions and have fun.

After  loading  the  driver,  it  can  be tested by compiling and

-2-

Send/Receive Driver for RSX-11M

task-building SRTEST.FTN.  This program prompts the  user  for  a

message  to  send  to itself, and then reads the message from the

driver.  Unless you get an error message from SRTEST,  everything

is working OK.

In  case  your wondering at the distinction between SRDRV.MAC and

srdrv.m, the latter  is  an  archive  of  the  individual  source

modules  needed  to  generate the former.  To generate SRDRV.MAC,

type the following command to the shell:

ar p srdrv.m | tr "a-z_" "A-Z$" >srdrv.mac

All  driver  routines  subscribe  to   the   following   register

conventions:

r0	free for use

r1	free for use

r2	address of message header block

r3	address of i/o request packed

r4	address of TCB

r5	address of UCB

If any of r2-r5 are to be used, they must be saved and restored.

The  maximum  message  size  is  kept in U.CW4 of the UCB.  It is

assembled to be the value specified to SRGEN, but can be  changed

dynamically via the SET /BUF=SR:n MCR command.

-3-

Send/Receive Driver for RSX-11M

Vital Statistics

Device name - SR0:

Legal Function codes

IO.WVB,IO.WLB

p1	buffer address

p2	# of bytes to write

p3	high order half of receiver taskname (rad50)

p4	low order half of same

p5	message lifetime (in seconds)

p6	if non-zero, flush all currently buffered messages for

this sender-receiver pair before queueing this message.

IO.RVB,IO.RLB

p1	buffer address

p2	size of buffer in bytes

IO.STP

status returns

writes

IS.SUC - word 2 of iosb has number of bytes written == p2

IE.BAD - no task name specified in p3-p4

IE.ALC - could not allocate internal buffers for message

IE.NOD - could not allocate message header block

reads

IS.SUC - word 2 of iosb has number of bytes read

IE.DAO - message bigger than buffer, only p2 bytes copied

IE.DNA - data not available

stop

IS.SUC

IE.PRI - not invoked by a priveleged terminal

-4-