OPA - A PRIVILEGED TASK TO DISPLAY A SNAPSHOT MAP OF THE CURRENT CONTENTS

OF POOL!!!! (for RSX-11M 3.2)

NOTE WELL:  This program is sufficiently valuable to all system managers/

programmers who have pool space problems that it is (to my mind) well worth

the risk of using.  However, it is necessary while this program is scanning

through pool to be on the system stack.  It has been made somewhat more

rugged than the 3.1 version submitted for San Diego DECUS, as it now catches

its own odd address or memory protect traps, and reports them rather than

causing a system crash.  It is, however, conceivable that it might corrupt

something in the EXEC, although rather less likely.  The biggest problem

from overuse is that some device drivers may require frequent access to

system state (i.e. forking) in order not to lose interrupts.  While this

task is collecting data, the EXEC cannot handle fork requests.  It does,

however, seem to work well on our 11/34 system, and has given us a

good deal of insight into what fragments and/or gobbles up pool.

The program will identify about 95% of the data structures kept in

pool, in its current version.  Remaining known structures not identified are:

some AST control blocks (see version 3.2 CDA manual description of same to see

why I haven't yet tracked all of them down).

RECEIVE packets (certain tasks [notably TKTN, LOADER] have non-standard message

sizes, and without significant checking any attempt to map those packets along

with the standard ones would certainly cause difficulties).

FCB's for other than INDEXF.SYS (I have done the window blocks for files

which are open by tasks currently in memory, but didn't bother with the

associated FCB's since many are buffered in the ACP anyway.)

Buffers allocated by the half-duplex terminal driver for as-yet incomplete

unsolicited input, or completed messages for MCR from either terminal driver.

Offspring control blocks, which I have looked for but have not yet found

the linkages as suggested in the CDA manual.

OTHERS which I don't remember at the moment.

The program output will appear on the invoking terminal, unless the

task is installed and the LUN reassigned to something like the lineprinter.

Output format:

1st line: # of fragments of pool

2nd line: total words of pool

next n lines: addresses and sizes of all free segments in pool

next line: total # words free in pool

Then the good part:

In this display, each ASCII character represents a minimum allocatable piece

of pool, i.e. 4 bytes.  Each line is 64(10) characters, corresponding to

400(8) bytes of pool.

Free area is shown by --------

Used but not identified is shown by ????????

<A> represents an attachment discriptor block

<An> is a specifed-ast control block, where n is the ast type code.

<M is a mount control block

<CP> is a checkpoint file control block

<TCB  -  ...MCR> is the task control block for task ...MCR, e.g.

<Cn> is a clock queue control block, where n is the type (see system data

structure definitions for types)

<Lx is a logical assignment control block.  The most interesting from the

point of pool fragmentation is <L/> which is a login assignment, which of course

will not move while the terminal is logged in.

<VCB  - DBn:> the volume control block for device DBn:

<FCB @DBn:> a file control block found for a file on DBn:

<WB=========> a window block for a file

<I DV: > an interrupt control block for device DV:

<DCB+UCB+SCB = DV:##> The data structures for a loadable driver with a loadable

database.  This may not be quite right if you have a 22-bit system with a DMA

device that does not say it is but still allocates space in the SCB for saving

the UNIBUS mapping register assignments.  There is a real example of such a

device (the VERSATEC DMA interface)!  It also will not handle data structures

which have oddball SCB lengths for other reasons.

<HEADER POOLFL###########> is of course a task header for task POOLFL.

Note that this is one of the significant pool fragmenters, particularly if

you get either MCR or an ACP checkpointed, since the header then will wind

up in the middle of pool somewhere when the task comes back in, and since the

task never exits (just stops) the header never goes back out.

<IO DVn:> is an I/O packet either currently in use or queued to device DVn:

Note that this does not include 'buffered I/O packets' used by the full-duplex

terminal driver.  Those appear (apparently) as AST packets but I haven't

tracked them down yet.

<PKT OPT> is a saved I/O size packet for QIO optimization.  These devils will

really fragment your pool, as they get allocated mostly when your system is

busy, then hang around later so it's hard to move/get rid of them, except of

course by turning off the optimization (we've had to do that to avoid pool

problems occurring on a regular basis).

( FCSRES) is a PCB for a permanent partition,

while (/POOLFL) is a subpartition control block, typically for the partition

control block for a running task in a system controlled partition. A sub

partition control block for a driver is (/ ZB:  )

<MCR LINE FOR ...RMD> is an example of a task invoked as an external MCR

function (i.e. JNK<cr> for a task called ...JNK) which hasn't gotten around

to doing a GMCR$.  This structure may stay until the task exits, so you

should remember to to a GMCR$ for any task which will be invoked as an external

MCR function and which will be active for awhile, even if it doesn't need to

get any information from the command line.

A sample output might look like:

FREE POOL FRAGMENTS=17.

TOTAL WORDS OF POOL=8940.

FREE SEGMENT LIST:

055064-055067 2.

055234-055237 2.

055430-055437 4.

.

.

.

070634-071753 296.

072004-071753 4178.

TOTAL FREE WORDS=4652.

POOL MAP:

053400:                                                              <A>

054000: <A>-<A><TCB = ...LOA><C0><C0>-<VCB=DB0:><FCB @DB0:>(/ TM:  )<M(/

054400:  VT:  )<HEADER F11ACP############><TCB = ...SLG>(/...PSZ)<A>---<

055000: A><WB==================================><CP>---(/ PL:  )--------

055400: -<DCB+UCB+SCB = TM:%%><A4>--<L/<TCB = ...PSZ>-------------<HEADE

.

.

.

071000: #############<A><A>????????<C2><C2><HEADER POOL  ###############

071400: ##><HEADER ...PSZ###########><IO LP0:>------<C2>----------------

072000: <MCR LINE FOR ...JNK>-------------------------------------------

072400: ------------------------------------------------<L/<L/<PKT OPT><

073000: PKT OPT>----------------------------------<PKT OPT>-------------

.

.

.

117000: -----------------( SHFPAR)<HEADER TKTN  ###########><TCB  -  TKT

117400: N  >( TKNPAR)<HEADER ...LDR###########><TCB  -  ...LDR>( LDRPAR)

Note that in general the stuff at the top of pool is put there by VMR, which

incidentally from this program I learned does a terrible job by not filling

any holes created by removing tasks, for example.

Comments and suggestion are welcome, but please don't complain if your system

crashes.  Hopefully I or others will get around to doing a few more data

structures,and there will be fewer areas labelled ????????.  Keep in mind

that it will never be possible to do all areas, since privileged tasks and

drivers may take blocks out of pool which are not referred to by any standard

data structures.  Also some structures are pointed to only by things in

the task headers, and if the task is checkpointed there is no way to find

them.  A good example is window blocks for files opened by various active

tasks.

Jim Neeland

Hughes Research Labs

3011 Malibu Canyon Rd.

Malibu, California 90265

(213) 456-6411 ext. 333