[352,2]NOTES.DOC - NOTES ON THE TALK PROGRAM

This UIC contains version V05.11 of the TALK program.

The first step for building TALK is the modification of TALKSYS.MAC

for your configuration.  This procedure is described below.

The term "host" computer in the documentation and source files refers to the

computer with which TALK is communicating, not the RSX-11M system on which

TALK is running.  This designation was chosen since the computer on which TALK

is running appears as a terminal to the computer attached to the port.

The current version of TALK supports the following host computers:  PDP 11

RSX-11M, VAX/VMS, and the Sigma 9.  Contact me if you need a list of changes

required for other system hosts.

The documentation files have not been changed since the previous SIG tape

version.

I plan to make additional changes and intend to submit a new version on

the next SIG tape.

SUMMARY OF TALKSYS.MAC MODIFICATIONS TO SET UP FOR YOUR SITE:

Define the vector/CSR addresses, following the instructions above the

DEFPRT  300,176500  line.  These addresses should not be known to the terminal

driver.  However, if they are, testing can be done by either changing

the vector address on the interface board, or by moving the value

$NONSI or $NS0 found in RSX11M.MAP into the vector address and the vector

address+4.  In the latter case, be sure not to try to access that terminal

before restoring the original values.

Define the appropriate host system symbol:  SIGMA9, RSX11, or VAX.

Find the line containing the symbol CKRERR.  Define this symbol if you

want TALK to check for and report character receive errors.

[For SIGMA 9 users only:]

Find the line containing the symbol COPOVR.  Define this symbol if

you want command A to transmit a "COPY OVER filename" command.

@TALK will then build the TALK program.

This command file allows a two-character suffix to be appended

to assembler and task builder output files, so that separate

versions may be built for communicating with different host

computers.  If a suffix is specified, then TALKSYS.MAC should

be renamed with the suffix.  For example, if using suffix VX,

rename TALKSYS.MAC to TALKSYSVX.MAC.

INS TALK (or TALKss, where ss is the suffix mentioned above).

TLK (or Tss)     ! Run TALK, entering the port number.

! Then log onto the host computer.

To enter command mode, type Control-W.  Then type M for a menu of commands.

Before transferring files to an RSX-11M system, check to see that the host

computer terminal line has these characteristics set (if appropriate for

the files you will be transferring):  /HHT=TI:, /LOWER=TI:, /BUF=TI:132.

On a Sigma 9 system, set PLATEN 140 before transferring files with records

longer than the current PLATEN value.

***

While testing TALK, we found that we could transfer

files between a PDP-11/23 and a VAX-11/780 without a direct

link between these computers, by running TALK on the 11/23

to connect to a PDP-11/70, and then running TALK on the

11/70 to connect to the VAX.  (TALK on the 11/23 was built

as if it were communicating directly to a VAX, so that the

correct COPY commands would be generated.)  In order to

exit, TALK must be in command mode and this is entered by

typing a control-W on the user's terminal.  Typing a

control-W on the 11/23 would allow us to exit TALK on the

11/23, but the second TALK would be left active on the

11/70.  This demonstrates the need for command C, which

sends control-W to the host computer.  We originally

included command C simply for completeness, in case someone

connected to a computer on which control-W had some meaning.

Well, in this situation we needed to send a control-W to

TALK on the 11/70 in order to be able to exit the TALK

program.

***

The following changes have been made since the previous version (V05.09):

o  The interrupt service routine no longer disables interrupts.

Some users had reported occasional character loss at 2400 baud,

and much greater loss at higher speeds.  We have now been able

to test TALK at 9600 baud from an 11/70 to a VAX 11/780 without

character loss.

o  The TALK task is set non-privileged after the connection to

the port is established.  Previously, any TALK user could

have specified an input or output file in any UIC.  Also, if a

user logs off the terminal after "detaching" from TALK with

command D, instead of exiting with command Z, TALK will be

aborted by BYE.  Previously, since TALK was privileged, the task

simply remained in the stopped state.  This version corrects

a problem encountered with the version resulting from

performing the modifications published in the Multi-Tasker.

Note that the next user of the line may need to enter CONTROL/S

and CONTROL/Q, since TALK suspends the host output when it

stops itself.

o  The new version eliminates the extra null records at the

beginning and end of a file transferred from a VAX/VMS.  Other

null records are written to the PDP-11 output file with this

version (unlike the version created by applying the correction

file published in the Multi-Tasker).

o  VAX and Sigma 9 files containing the monitor prompt

character ($ or !) at the beginning of records, may now be

transferred to the PDP-11 using command B.

o  A few changes have been made to TALK.CMD.  TALK is now built to

run at priority 65.