Erlang Program

The ERLANG program provides solutions to various  trunking  problems  using

the Erlang blocking formulas.

To use the program, sign on to the system and type in "RUN  $ERLANG".   The

program  will  respond  with  "ENTER  WANTED  FACTOR"  followed by a list of the

unknown factors for which the program can find a value, as listed  below.   Type

in  the  name  of one of these factors as it appears in upper case.  The program

will then prompt for the values required to calculate the desired  answer.   The

data is entered as follows:

1.  Number of trunks:  this is an  integer  (whole  number),  greater  than

zero,  entered with NO decimal point.  This term is synonymous with the

general term "servers".

2.  Blocking:  this is a real number, expressed as a percentage between 0.0

and  100.0  percent, entered WITH a decimal point.  If no decimal point

is entered, two decimals are assumed.

3.  Traffic:  this is the traffic on the group of servers in  Erlangs.   It

is a real number, entered with a decimal point.

4.  Probabilities are printed out as real numbers  between  zero  and  one.

They may be converted to percentages of time occupied by multiplying by

100.

When more than one value is to be entered, seperate the values with commas.  The

program  will then type out the entered data and the calculated answer.  It will

then ask if you want to perform another calculation of this type:  entering  "Y"

will  repeat the prompt for input, otherwise the program will ask if you want to

perform another type of calculation.  Answering "Y" will cause the "ENTER WANTED

FACTOR"  prompt  to  be  repeated, otherwise the program exits.  Responding to a

prompt with a "Control-Z" will also terminate the program.

Types of problems.

1.  BLOCKING:  The number of trunks (servers) present  and  the  amount  of

traffic  in  Erlangs  presented  to  these  trunks  is entered, and the

resulting percentage of traffic blocked is calculated.

2.  TRUNKS:  the traffic  in  Erlangs  to  be  presented  and  the  desired

blocking  is  entered, and the result is the number of trunks (servers)

required for the required blocking to not be exceeded.

3.  INPUT:  The traffic received over a group of trunks (servers)  and  the

number  of  trunks  is  entered.   The program calculates the amount of

traffic which must have been presented at the  far  end  of  the  trunk

bundle,  and  the blocking observed at the far end for the given amount

of traffic to have been received.

4.  TRAFFIC:  The number of trunks (servers) and the percentage of blocking

which may be tolerated is entered.  The result is the amount of traffic

which may be presented to that number of  trunks  for  that  amount  of

blocking not to be exceeded.

Erlang Program                                               PAGE 2

5.  QUEUE:  The number of trunks and the traffic in  Erlangs  presented  to

those  trunks  is  entered.   The result is the probability of delay in

service (P>0) for a queueing system.

6.  PROB(ability):  only the first four letters are entered.   The  program

will prompt for one of three calculations:

1.  SINGLE:  The amount of traffic in Erlangs and the number of  trunks

(servers)  is  entered.   The  program will list the probability of

exactly that number of trunks being occupied:  this is identical to

the  amount  of time during the observation period that that number

of trunks will be occupied.

2.  AVERAGE:  The amount of traffic in  Erlangs  is  entered,  and  the

program  will  list  the  mean  number  of trunks occupied, and the

standard deviation to the mean, for that amount of  traffic  on  an

unlimited number of trunks (servers).

3.  TABLE:  The amount of  traffic  in  Erlangs  is  entered,  and  the

program  will  list  a  table showing the probabilities that at any

instant in time no trunks are occupied, one trunk is occupied,  two

trunks  are occupied, etc.  If a full table is requested, the table

is printed until the cumulative probability (the  probability  that

that  number  of trunks or fewer trunks are occupied) is effectivly

equal  to  one  (all  practical  cases   have   been   considered).

Otherwise,  the program prompts for a starting and ending number of

trunks, and the table is printed out between these limits.