.he ZAPLOAD- Binary to Intel Hex (c) T. Jennings 1980, 1981, 1982 Page #

ZAPLOAD- MSDOS version

ZAPLOAD  can  be  used  to convert any  file  (binary  is

assumed)  to  INTEL  hex  format,   and  send  to  one  or   more

destinations:  a  disk file,  the punch device,  or the  printer.

There  are  many options to select various things;  ones or  twos

complement checksums,  where to send the hex, specifying the load

and start addresses, etc. All of the available options are listed

below, followed by a more detailed description of each.

The  very  last section deals with connecting  a  Sunrise

Electronics ZAP-80 EPROM burner to a computer via ZAPLOAD,  since

that was it's original intent.

ZAPLOAD  gives  a  quick list  and  description  of  it's

commands when operated improperly. To get this list, type ZAPLOAD

without  any  command line options.  You will also get  the  list

whenever you make any command errors.

.pa

Zapload  is executed from an MSDOS system,  and  converts

MSDOS files from the disk,  to INTEL hex format.  All options are

specified  on the command line when ZAPLOAD is run.  ZAPLOAD  has

the following syntax:

ZAPLOAD <file to convert> <any legal options ...>

After  ZAPLOAD has sent the entire file,  it will display

on  the console a 16 bit two's complement checksum of every  data

byte sent.  This checksum is of the data bytes only.  This can be

used  in  conjunction  with the  destination  device  to  provide

another check of data integrity.

An  INTEL hex file consists of one or more data  records,

followed  by  an  end  of file record.  These  records  have  the

following format:

DATA RECORDS: : bb llll tt dd dd dd .... cc

: says this is the start of a record,

bb is the number of data bytes to follow,

llll is the address to load the data byte at,

tt is the record type, 00 for data,

dd dd ... are the data bytes,

cc is the checksum of bb llll tt and all dd data

bytes.

END OF FILE RECORDS: : bb ssss tt cc

: is the same as a data record,

bb is zero, (no data),

ssss is the program start address,

tt is 01 to indicate end of file,

cc is the checksum of bb ssss and tt.

There are no data bytes.

After  the cc bytes have been sent,  a CR LF sequence  is

sent  for readability.  Programs that read INTEL hex  ignore  any

characters before the :,  and after the cc checksum.  Or at least

are supposed to.

If  you specify no options at all,  ZAPLOAD processes the

file but does not send the output anywhere. The following are the

defaults for Intel Hex generation:

- 32 byte data records (20 hex),

- 2's complement checksums,

- Initial load address of 0,

- Start address of 0.

.pa

ZAPLOAD OPTIONS

There  are  many options;  these fall  roughly  into  two

categories;  those  that  control  the record  format,  for  non-

standard devices (damn them) and those that control where to send

it as it is generated.

Options are specified on the command line,  following the

filename.  Each option is a single letter,  and may appear in any

order. Some options take an additional argument; like setting the

load  address.  In  these cases the argument follows  the  option

letter, seperated by one or more spaces, like so:

ZAPLOAD file l 8000

Option  letters may be optionally preceded by  a  -,  for

UNIX freaks, like so:

ZAPLOAD file -l 8000

RECORD FORMAT OPTIONS

b <hex number>

Set  the number of bytes sent per record from the default

20  hex  to <hex number>.  <hex number> must be less  than  1024.

Please do not use 0. (BUG: ZAPLOAD will not catch a byte count of

zero.  It  might  try to produce 65535  byte  records,  and  bomb

everything.)

z

Generate  ones  complement  checksums instead  of  two's.

Obscure option for use with non-standard devices.

l <hex number>

Set  the load address to <hex number>.  The default is  0

hex if this option is not used.  The load address for each record

sent will increment by the byte count.

r <hex number>

Change the default run address of 0 to <hex number>. This

is used only in the end of file record,  to indicate the starting

address of the file just sent.  Things like EPROM burners do  not

use this; some computers with download facilities might.

.pa

FILE DESTINATION CONTROLS

The following options control where the file is sent, and

how it is sent. If none of these are specified, it is merely sent

to  the  console.  Each  record is always typed on  the  console,

character by character,  as it is sent,  unless the 'X' option is

given.  Running ZAPLOAD with only the 'X' option would cause  the

file  to be sent nowhere at all.  (Why?) A control-Z  terminating

character  is  sent to the destination device after the  file  is

transferred.

a

Send to the aux device.  This could also be done via  the

'f' option: f AUX. See below.

f <file name>

Create a file <file name>, and send each byte there. This

can be used to generate INTEL hex disk to disk, without using any

external devices.

w <decimal number>

Software  delay by <decimal number> after each  character

is sent.  This can be used for those poorly designed devices that

require  but  do not use character by  character  handshake.  The

default here is 0, obviously, and 10 is about 50 milliseconds for

a  4MHz  Z80.  This  was necessary for the ZAP-80,  as is  has  a

software UART, and takes too long to process a byte to keep up.

Pretty useless with disk files.

e <decimal number>

Software  delay after the CR LF sequence at the end of  a

record.  Some loaders require more time at the end of the record,

in order to generate and check the checksum. The delay w <decimal

number> is still done after sending the last cc byte,  but not on

the CR LF sequence.

.pa

General Controls

s <dddd>

Skip  <dddd> decimal bytes at the beginning of  the  file

before processing.  This is useful when generating HEX files from

CP/M .CMD files or the like that have a header at the beginning.

o

e

Process  only  even or odd numbered bytes.  This is  very

useful when generating EPROM code for 16 bit processors like  the

8086,  that  have  even bytes in one PROM,  and the odd  ones  in

another. By using the form:

zapload file.cmd s 128 e f low.hex

and

zapload file.cmd s 128 o f hi.hex

You  can  make the two EPROM files necessary for an  8086

bootstrap ROM, or whatever.

x

List output on the console. All hex records generated are

typed.

.pa

TYPICAL USAGE

Here  are  some examples for loading INTEL  hex  to  some

common devices:

zapload test.com x

This  loads  test.com to the console,  and sends 32  byte

records, load address of 0, start address of zero.

zapload test.com e 10 w 10

This  loads  a ZAP80 tied to the  punch  port,  with  the

necessary software delays to accomodate the ZAP80.

zapload test.com f test.hex

Convert  file  test.com  to  hex,  and make  a  hex  file

test.hex. This "undoes" Digital Research's LOAD utility.

zapload test.com l 100 r 100 p f test.hex x

Convert test.com to Intel hex,  send it to the CP/M  PUN:

device,  to the disk file TEST.HEX,  and listing it on

the console. The load address starts at 100h, and the run address

is 100h.

.pa

Connecting the ZAP-80 to FIDO or any other computer

You  will  not find this data in any Sunrise  Electronics

documentation.  The ZAP80 is filled with bugs,  and the manual is

terrible.

- The ZAP80 requires 300 baud,  1 1/2 stop bits.  Idiots.

Use two stop bits.

- The ZAP 80 cannot be run faster than 300 baud,  and can

not handle even this without software delays.

- Though  they  claim  that  it  uses  the  RTS  line  or

whatever, it really does not.

- Even  though each record may be loading  properly,  the

display  will  always say there is a load error after  the  first

record.  Ignore  it,  and once the entire file is  sent,  do  the

following:

Reset  the ZAP80,  and checksum the entire data ares just

sent.  (CKSM key,  then the lower and upper  addresses).  Compare

this  to  the data checksum typed by ZAPLOAD.  If the  last  four

digits are the same, all is well. (It usually is)

HARDWARE CONNECTIONS

-Use  a typical 3 wire connection,  swapping pins 2 and 3

until it works. (Necessary on FIDO, and done in it's cable).

-On the ZAP80, tie pins 4 and 5 together. (Fool RTS)

Don't forget to set FIDO's punch port,  TUART port B,  to

300 baud 2 stop bits.

END OF MANUAL