KERMIT PROTOCOL MANUAL

Fifth Edition

Frank da Cruz

Columbia University Center for Computing Activities

New York, New York 10027

3 April 1984

Copyright (C) 1981,1982,1983,1984

Trustees of Columbia University in the City of New York

Permission is granted to any individual or institution to copy or

use this document and the programs described in it, except for

explicitly commercial purposes.

Preface to the Fourth Edition                                            Page 1

Preface to the Fourth Edition

The  fourth  edition (November 1983) of the KERMIT Protocol Manual incorporates

some new ideas that grew from our experience in attempting to implement some of

the features described in earlier editions, particularly user/server functions.

These include a mechanism to allow batch transfers to be interrupted gracefully

for either the current file or the entire batch of files; a "capability  mask";

a protocol extension for passing file attributes.  In addition, numbers are now

written  in  decimal  notation  rather  than octal, which was confusing to many

readers.  Also, several incompatible changes were made in minor areas where  no

attempts at an implementation had yet been made; these include:

- The format and interpretation of the operands to the server commands.

- Usurpation  of the reserved fields 10-11 of the Send-Init packet, and

addition of new reserved fields.

Most of the remaining material has been rewritten and reorganized, and much new

material added, including a section on the recommended vocabulary for  documen-

tation and commands.

The  previous edition of the Protocol Manual attempted to define "protocol ver-

sion 3"; this edition abandons that concept.  Since KERMIT  development  is  an

unorganized,  disorderly, distributed enterprise, no requirement can be imposed

on KERMIT implementors to include a certain set of capabilities  in  their  im-

plementations.    Rather,  in  this  edition  we  attempt  to  define the basic

functionality of KERMIT, and then describe various optional functions.

The key principle is that any implementation of KERMIT  should  work  with  any

other, no matter how advanced the one or how primitive the other.  The capabily

mask and other Send-Init fields attempt to promote this principle.

FIFTH EDITION

The  fifth  edition  (March 1984) attempts to clarify some fine points that had

been left ambiguous in the 4th edition, particularly with respect to  when  and

how  prefix  encoding  is done, and when it is not, and about switching between

block check types.  A mechanism is suggested (in the  Attributes  section)  for

file archiving, and several attributes have been rearranged and some others ad-

ded  (this  should  do no harm, since no one to date has attempted to implement

the attributes packet).  A more complete protocol state table  is  provided,  a

few minor additions are made to the collection of packet types.

A FEW WORDS...

Before  deciding to write a new version of KERMIT, please bear in mind that the

philosophy of KERMIT has always been that is not, and never  should  become,  a

commercial  product, sold for profit.  Its goal is to promote communication and

sharing, and KERMIT itself should be freely shared, and not sold.    Media  and

reproduction costs may be recouped if desired, but profit should not be the mo-

tive.    Vendors of commercial software, however, may request permission to in-

clude KERMIT with, or in, their programs provided certain conditions  are  met,

including  that credit for the protocol be given to Columbia and that the price

of the product not be raised substantially beyond media and reproduction  costs

Preface to the Fourth Edition                                            Page 2

for  inclusion of KERMIT.  Contact the KERMIT group at Columbia if you have any

questions about this.  Prospective KERMIT implementors should check with us  in

any  case, to be sure that someone else has not already done, or started to do,

the same thing you propose to do.

KERMIT is distributed from Columbia University on magnetic tape.  Complete  or-

dering  instructions  can be found in the Kermit Users Guide.  Direct inquiries

about KERMIT to:

KERMIT Distribution

Columbia University Center for Computing Activities

7th Floor, Watson Laboratory

612 West 115th Street

New York, NY  10025

ACKNOWLEDGEMENTS

Bill Catchings and I designed the basic KERMIT protocol at Columbia  University

in  1981.   For ideas, we looked at some of the ANSI models (X3.57, X3.66), the

ISO OSI model, some real-world "asynchronous protocols" (including the Stanford

Dialnet project, the University of Utah TTYFTP project), as  well  as  at  file

transfer on full-blown networks like DECnet and ARPAnet.

Bill  wrote  the  first  two  programs  to  implement the protocol, one for the

DEC-20, one for a CP/M-80 microcomputer, and in the process worked out most  of

the details and heuristics required for basic file transfer.  Meanwhile, Daphne

Tzoar  and  Vace  Kundakci, also of Columbia, worked out the additional details

necessary for IBM mainframe communication.

Much credit should also go to Bernie Eiben of Digital Equipment Corporation for

promoting widespread use of KERMIT and for adding many  insights  into  how  it

should  operate, and to Nick Bush and Bob McQueen of Stevens Institute of Tech-

nology, for many contributions to the "advanced" parts of the protocol, and for

several major KERMIT implementations.

Thanks to the many people all over the world who have  contributed  new  KERMIT

implementations,  who have helped with KERMIT distribution through various user

groups, and who have contributed to the quality of the protocol  and  its  many

implementations  by  reporting  or  fixing problems, criticizing the design, or

suggesting new features.

DISCLAIMER

No warranty of the software nor of the accuracy of the documentation  surround-

ing it is expressed or implied, and neither the authors nor Columbia University

acknowledge any liability resulting from program or documentation errors.

Introduction                                                             Page 3

1. Introduction

This  manual  describes the KERMIT protocol.  It is assumed that you understand

the purpose and operation of the Kermit file transfer  facility,  described  in

the  Kermit  Users Guide, and basic terminology of data communications and com-

puter programming.

1.1. Background

The KERMIT file transfer protocol is intended for use in an  environment  where

there  may  be  a  diverse  mixture of computers -- micros, personal computers,

workstations, laboratory computers, timesharing systems -- from  a  variety  of

manufacturers.    All  these systems need have in common is the ability to com-

municate in ASCII over ordinary serial telecommunication lines.

KERMIT was originally designed at Columbia University to meet the need for file

transfer between our DECSYSTEM-20 and IBM  370-series  mainframes  and  various

microcomputers.   It turned out that the diverse characteristics of these three

kinds of systems resulted in a design that was general enough to fit almost any

system.  The IBM mainframe, in  particular,  strains  most  common  assumptions

about how computers communicate.

1.2. Overview

The  KERMIT  protocol is specifically designed for character-oriented transmis-

sion over serial telecommunication lines.  The design allows for  the  restric-

tions and peculiarities of the medium and the requirements of diverse operating

environments  --  buffering,  duplex, parity, character set, file organization,

etc.  The protocol is carried out by KERMIT programs on each end of the  serial

connection  sending "packets" back and forth; the sender sends file names, file

contents, and control information; the  receiver  acknowledges  (positively  or

negatively) each packet.

The  packets  have  a layered design, more or less in keeping with the ANSI and

ISO philosophies, with the outermost fields used by  the  data  link  layer  to

verify  data integrity, the next by the session layer to verify continuity, and

the data itself at the application level.

Connections between systems are established by the ordinary user.  In a typical

case, the user runs KERMIT on a microcomputer, enters terminal emulation,  con-

nects  to  a remote host computer (perhaps by dialing up), logs in, runs KERMIT

on the remote host, and then issues commands to that KERMIT  to  start  a  file

transfer,  "escapes"  back  to the micro, and issues commands to that KERMIT to

start its side of the file transfer.  Files may be  transferred  singly  or  in

groups.

Basic  KERMIT  provides only file transfer, and that is provided for sequential

files only, though the protocol attempts to allow for various types of  sequen-

tial  files.    Microcomputer  implementations  of  KERMIT are also expected to

provide terminal emulation, to facilitate the initial connection.

More advanced implementations simplify the "user interface" somewhat by  allow-

ing  the  KERMIT  on  the  remote host to run as a "server", which can transfer

files in either direction upon command from the local "user" Kermit.  The serv-

Introduction                                                             Page 4

er  can  also  provide  additional functionality, such as file management, mes-

sages, mail, and so forth.  Other optional features  also  exist,  including  a

variety  of  block  check  types,  a mechanism for passing 8-bit data through a

7-bit communication link, a way to compressing a repeated sequence  of  charac-

ters, and so forth.

As  local area networks become more popular, inexpensive, and standardized, the

demand for KERMIT and similar protocols may dwindle, but will never wither away

entirely.  Unlike hardwired networks, KERMIT gives the ordinary user the  power

to  establish  reliable  error-free connections between any two computers; this

may always be necessary for one-shot or long-haul connections.

Definitions                                                              Page 5

2. Definitions

2.1. General Terminology

TTY:  This  is the term commonly used for a device which is connected to a com-

puter over an EIA RS-232 serial telecommunication line.  This  device  is  most

commonly  an  ASCII  terminal,  but  it  may be a microcomputer or even a large

multi-user computer emulating  an  ASCII  terminal.    Most  computers  provide

hardware (RS-232 connectors and UARTs) and software (device drivers) to support

TTY  connections;  this is what makes TTY-oriented file transfer protocols like

KERMIT possible on almost any system at little or no cost.

LOCAL: When two machines are connected, the LOCAL machine is the one which  you

interact  with  directly,  and which is in control of the terminal.  The "local

Kermit" is the one that runs on the local machine.  A local Kermit always  com-

municates  over an external device (the micro's communication port, an assigned

TTY line, etc).

REMOTE: The REMOTE machine is the one on the far side of the connection,  which

you  must  interact with "through" the local machine.  The "remote Kermit" runs

on the remote machine.  A remote  Kermit  usually  communicates  over  its  own

"console", "controlling terminal", or "standard i/o" device.

HOST:  Another word for "computer", usually meaning a computer that can provide

a home for multiple users or applications.  This term should be avoided in KER-

MIT lore, unless preceded immediately by LOCAL or REMOTE, to denote which  host

is meant.

SERVER:  An  implementation of remote Kermit that can accept commands in packet

form from a local Kermit program, instead of directly from the user.

USER: In addition to its usual use to denote  the  person  using  a  system  or

program,  "user"  will also be used refer to the local Kermit program, when the

remote Kermit is a server.

2.2. Numbers

All numbers in the following text are expressed in decimal (base  10)  notation

unless otherwise specified.

Numbers  are  also  referred  to  in terms of their bit positions in a computer

word.  Since KERMIT may be implemented on computers with various word sizes, we

start numbering the bits from the "right" -- bit 0 is  the  least  significant.

Bits  0-5  are  the  6 least significant bits; if they were all set to one, the

value would be 63.

A special quirk in terminology, however, refers to the  high  order  bit  of  a

character  as  it  is  transmitted on the communication line, as the "8th bit".

More properly, it is bit 7, since we start counting from 0.  References to  the

"8th  bit"  generally are with regard to that bit which ASCII transmission sets

aside for use as a parity bit.  KERMIT concerns itself with  whether  this  bit

can  be  usurped  for  the  transmission  of data, and if not, it may resort to

"8th-bit prefixing".

Definitions                                                              Page 6

2.3. Character Set

All  characters  are  in ASCII (American national Standard Code for Information

Interchange) representation, ANSI standard X3.4-1968.  All  implementations  of

KERMIT  transmit and receive characters only in ASCII.  The ASCII character set

is listed in Appendix V.

ASCII character mnemonics:

NUL     Null, idle, ASCII character 0.

SOH     Start-of-header, ASCII character 1 (Control-A).

SP      Space, blank, ASCII 32.

CR      Carriage return, ASCII 13 (Control-M).

LF      Linefeed, ASCII 10 (Control-J).

CRLF    A carriage-return linefeed sequence.

DEL     Delete, rubout, ASCII 127.

A control character is considered to be any byte whose low order 7 bits are  in

the  range 0 through 31, or equal to 127.  In this document, control characters

are written in several ways:

Control-A

This denotes ASCII character 1, commonly referred  to  as  "Control-A".

Control-B is ASCII character 2, and so forth.

CTRL-A  This  is a common abbreviation for "Control-A".  A control character is

generally typed at a computer terminal by holding down the  key  marked

CTRL  and pressing the corresponding alphabetic character, in this case

"A".

^A      "Uparrow" notation for CTRL-A.  Many computer  systems  "echo"  control

characters in this fashion.

A  printable  ASCII character is considered to be any character in the range 32

(SP) through 126 (tilde).

2.4. Conversion Functions

Several conversion functions are useful in the description of the protocol  and

in  the  program example.  The machine that Kermit runs on need operate only on

integer data; these are functions that operate upon the numeric value of single

ASCII characters.

char(x) = x+32  Transforms the integer x, which is assumed to lie in the  range

0  to 94, into a printable ASCII character; 0 becomes SP, 1 be-

comes "!", 3 becomes "#", etc.

unchar(x) = x-32

Transforms the character x, which  is  assumed  to  be  in  the

printable  range  (SP  through  tilde),  into an integer in the

range 0 to 94.

ctl(x) = x XOR 64

Maps between control characters and their  printable  represen-

tations,  preserving  the  high-order  bit.   If x is a control

Definitions                                                              Page 7

character, then

x = ctl(ctl(x))

that  is,  the  same function is used to controllify and uncon-

trollify.  The argument is assumed to be a true control charac-

ter (0 to 31, or 127), or the result of applying CTL to a  true

control  character  (i.e.  63  to 95).  The transformation is a

mnemonic one -- ^A becomes A and vice versa.

2.5. Protocol Jargon

A Packet is a clearly delimited string of  characters,  comprised  of  "control

fields" nested around data; the control fields allow a KERMIT program to deter-

mine  whether the data has been transmitted correctly and completely.  A packet

is the unit of transmission in the KERMIT protocol.

ACK stands for "Acknowledge".  An ACK is a packet that is sent  to  acknowledge

receipt of another packet.  Not to be confused with the ASCII character ACK.

NAK  stands  for  "Negative Acknowledge".  A NAK is a packet sent to say that a

corrupted or incomplete packet was received, the wrong packet was received,  or

an expected packet was not received.  Not to be confused with the ASCII charac-

ter NAK.

A  timeout is an event that can occur if expected data does not arrive within a

specified amount of time.  The program generating the input request can  set  a

"timer  interrupt"  to  break  it out of a nonresponsive read, so that recovery

procedures may be activated.

System Requirements                                                      Page 8

3. System Requirements

The KERMIT protocol requires that:

- The  host can send and receive characters using 7- or 8-bit ASCII en-

coding over an EIA RS-232 physical connection,  either  hardwired  or

dialup.

- All  printable  ASCII  characters are acceptable as input to the host

1

and will not be transformed in any way .  Similarly, any  intervening

network  or  communications  equipment ("smart modems", TELENET, ter-

minal concentrators, port selectors, etc) must not transform or swal-

low any printable ASCII characters.

- A single ASCII control character can pass  from  one  system  to  the

other  without  transformation.    This  character is used for packet

synchronization.  The character is normally Control-A (SOH, ASCII 1),

but can be redefined.

- If a host requires a line terminator for terminal  input,  that  ter-

minator  must  be a single ASCII control character, such as CR or LF,

distinct from the packet synchronization character.

- When using a job's controlling terminal for file transfer, the system

must allow the KERMIT program to set the terminal  to  no  echo,  in-

finite  width  (no  "wraparound"  or  CRLF insertion by the operating

system), and no "formatting" of incoming or outgoing characters  (for

instance,  raising  lowercase letters to uppercase, transforming con-

trol characters to printable sequences, etc).  In short, the terminal

must be put in "binary" or "raw" mode, and, hopefully,  restored  af-

terwards to normal operation.

- The  host's terminal input processor should be capable of receiving a

single burst of 40 to 100 characters at normal  transmission  speeds.

This is the typical size of packet.

Note  that  most  of  these requirements rule out the use of KERMIT through IBM

3270 / ASCII protocol converters.

KERMIT does not require:

- That the connection run at any particular baud rate.

- That the system can do XON/XOFF or any other kind  of  flow  control.

System-  or hardware-level flow control can help, but it's not neces-

sary.  See section 5.7.

- That the system is capable of full duplex operation.  Any mixture  of

_______________

1

If  they  are  translated  to another character set, like EBCDIC, the KERMIT

program must be able to reconstruct the packet as it appeared on the communica-

tion line, before transformation.

System Requirements                                                      Page 9

half and full duplex systems is supported.

- That  the  system  can  transmit or receive 8-bit bytes.  KERMIT will

take advantage of 8-bit connections to send binary files; if an 8-bit

connection is not possible, then binary files may be  sent  using  an

optional prefix encoding.

Printable Text versus Binary Data                                       Page 10

4. Printable Text versus Binary Data

For  transmission  between  unlike systems, files must be assigned to either of

two catagories: printable text or binary.

A printable text file is one that can make sense on an unlike system -- a docu-

ment, program source, textual data, etc.  A binary file is one  that  will  not

(and probably can not) make sense on an unlike system -- an executable program,

numbers stored in internal format, etc.  On systems with 8-bit bytes, printable

2

ASCII files will have the high order bit of each byte set to zero  (since ASCII

is  a 7-bit code) whereas binary files will use the high order bit of each byte

for data, in which case its value can vary from byte to byte.

Many computers have no way to distinguish a printable file from a  binary  file

--  especially one originating from an unlike system -- so the user may have to

give an explicit command to Kermit to tell it whether to perform these  conver-

sions.

4.1. Printable Text Files

A primary goal of KERMIT is for printable text files to be useful on the target

system after transfer.  This requires a standard representation for text during

transmission.    KERMIT's  standard  is  simple:  7-bit  ASCII characters, with

"logical records" (lines) delimited by CRLFs.  It is the responsibility of sys-

tems that do not store printable files in this fashion to perform the necessary

conversions upon input and output.  For instance, IBM  mainframes  might  strip

trailing  blanks  on output and add them back on input; UNIX would prepend a CR

to its normal record terminator, LF, upon output and discard it upon input.  In

addition, IBM mainframes must do EBCDIC/ASCII translation for text files.

No other conversions (e.g. tab expansion) are performed upon text files.   This

representation  is  chosen  because  it  corresponds  to the way text files are

stored on most microcomputers and on many other systems.  In many common cases,

no transformations are necessary at all.

4.2. Binary Files

Binary files are transmitted as though they were a sequence of characters.  The

difference from printable files is that the status of the  "8th  bit"  must  be

preserved.  When binary files are transmitted to an unlike system, the main ob-

jective  is  that  they can be brought back to the original system (or one like

it) intact; no special conversions should be done during  transmission,  except

to make the data fit the transmission medium.

For  binary  files,  eight bit character transmission is permissible as long as

the two Kermit programs involved can control the value of the parity  bit,  and

_______________

2

There  are  some  exceptions,  such  as systems that store text files in so-

called "negative ASCII", or text files produced by word processors that use the

high order bit to indicate underline or boldface attributes.

Printable Text versus Binary Data                                       Page 11

no  intervening  communications equipment will change its value.  In that case,

the 8th bit of a transmitted character will match that  of  the  original  data

byte, after any control-prefixing has been done.  When one or both sides cannot

control  the  parity  bit,  a  special  prefix  character  may  be inserted, as

described below.

Systems that do not store binary data in 8-bit bytes, or whose word size is not

a multiple of 8, may make special provisions for "image mode" transfer  of  bi-

nary files.  This may be done within the basic protocol by having the two sides

implicitly  agree  upon  a  scheme  for packing the data into 7- or 8-bit ASCII

characters, or else the more flexible (but optional)  file  attributes  feature

may  be  used.    The  former method is used on PDP-10 36-bit word machines, in

which text is stored five 7-bit bytes per word; the value of the "odd  bit"  is

sent as the parity bit of every 5th word.

File Transfer                                                           Page 12

5. File Transfer

The file transfer protocol takes place over a transaction.  A transaction is an

exchange  of  packets  beginning with a Send-Init (S) packet, and ending with a

3

Break Transmission (B) or Error (E) packet , and may include  the  transfer  of

one  or more files, all in the same direction.  In order to minimize the unfor-

seen, KERMIT packets do not contain any control characters except one specially

designated to mark the beginning of a packet.  Except for  the  packet  marker,

only  printable  characters  are  transmitted.   The following sequence charac-

terizes basic Kermit operation; the sender  is  the  machine  that  is  sending

files; the receiver is the machine receiving the files.

1. The  sender  transmits  a  Send-Initiate  (S)  packet to specify its

parameters (packet length, timeout, etc; these are explained below).

2. The receiver sends an ACK (Y) packet, with its own parameters in the

data field.

3. The sender transmits a File-Header (F) packet,  which  contains  the

file's name in the data field.  The receiver ACKs the F packet, with

no data in the data field of the ACK (optionally, it may contain the

name under which the receiver will store the file).

4. The sender sends the contents of the file, in Data (D) packets.  Any

data not in the printable range is prefixed and replaced by a print-

able  equivalent.  Each D packet is acknowledged before the next one

is sent.

5. When all the file data has been sent, the sender  sends  an  End-Of-

File (Z) packet.  The receiver ACKs it.

6. If  there is another file to send, the process is repeated beginning

at step 3.

7. When no more files remain to be sent, the sender transmits  an  End-

Of-Transmission  (B)  packet.   The receiver ACKs it.  This ends the

transaction, and closes the logical connection (the physical connec-

tion remains open).

Each packet has a sequence number, starting with 0 for the Send Init.  The ack-

nowledgment (ACK or NAK) for a packet has the same packet number as the  packet

being acknowledged.  Once an acknowledgment is successfully received the packet

number is increased by one, modulo 64.

If  the  sender  is remote, it waits for a certain amount of time (somewhere in

the 5-30 second range) before transmitting the Send-Init, to give the user time

to escape back to the local KERMIT and tell it to receive files.

_______________

3

A transaction should also be considered terminated  when  one  side  or  the

other has stopped without sending an Error packet.

File Transfer                                                           Page 13

5.1. Conditioning the Terminal

KERMIT is most commonly run with the user sitting at a microcomputer, connected

through  a communications port to a remote timesharing system.  The remote KER-

MIT is using its job's own "controlling terminal" for file transfer.  While the

microcomputer's port is an ordinary device,  a  timesharing  job's  controlling

terminal  is  a special one, and often performs many services that would inter-

fere with normal operation of KERMIT.  Such services include echoing  (on  full

duplex systems), wrapping lines by inserting carriage return linefeed sequences

at  the  terminal  width,  pausing at the end of a screen or page full of text,

displaying system messages, alphabetic case conversion, control  character  in-

tepretation,  and  so  forth.   Mainframe KERMIT programs should be prepared to

disable as many of these  services  as  possible  before  packet  communication

begins,  and to restore them to their original condition at the end of a trans-

action.  Disabling these services is usually known as "putting the terminal  in

binary mode."

KERMIT's  use  of  printable  control  character  equivalents,  variable packet

lengths, redefinable markers and prefixes, and allowance for any characters  at

all  to  appear between packets with no adverse effects provide a great deal of

adaptability for those systems that do not allow certain (or any) of these fea-

tures to be disabled.

5.2. Timeouts, NAKs, and Retries

If a KERMIT program is capable of setting a timer interrupt, or setting a  time

limit on an input request, it should do so whenever attempting to read a packet

from the communication line, whether sending or receiving files.  Having read a

packet, it should turn off the timer.

If the sender times out waiting for an acknowledgement, it should send the same

packet  again,  repeating  the  process a certain number of times up to a retry

limit, or until an acknowledgement is received.   If  the  receiver  times  out

waiting  for  a packet, it can send either a NAK packet for the expected packet

or another ACK for the last packet it got.

If a packet from the sender is garbled or lost in transmission (the  latter  is

detected  when  the  sequence  number  increases by more than 1, modulo 64, the

former by a bad checksum), the receiver sends a NAK for the garbled or  missing

packet.    If  an ACK or a NAK from the receiver is garbled or lost, the sender

ignores it; in that case, one side or the other will time out and retransmit.

A retry count is maintained, and there  is  a  retry  threshold,  normally  set

around  5.   Whenever a packet is resent -- because of a timeout, or because it

was NAK'd -- the counter is incremented.  When it reaches  the  threshold,  the

transaction is terminated and the counter reset.

If  neither  side  is capable of timing out, a facility for manual intervention

must be available on the local KERMIT.  Typically, this will work  by  sampling

the  keyboard (console) periodically; if input, such as a CR, appears, then the

same action is taken as if a timeout had occurred.  The local  KERMIT  keeps  a

running  display  of the packet number or byte count on the screen to allow the

user to detect when traffic has stopped.  At this  point,  manual  intervention

should break the deadlock.

File Transfer                                                           Page 14

Shared  systems which can become sluggish when heavily used should adjust their

own timeout intervals on a per-packet basis, based on the system load, so  that

file transfers won't fail simply because the system was too slow.

Normally,  only one side should be doing timeouts, preferably the side with the

greatest knowledge of the "environment" --  system  load,  baud  rate,  and  so

forth, so as to optimally adjust the timeout interval for each packet.  If both

sides  are  timing  out,  their intervals should differ sufficiently to prevent

collisions.

5.3. Errors

During file transfer, the sender may encounter an i/o error on the disk, or the

receiver may attempt to write to a full or write-protected device.    Any  con-

dition that will prevent successful transmission of the file is called a "fatal

error".    Fatal  errors  should be detected, and the transfer shut down grace-

fully, with the pertinent information provided to  the  user.    Error  packets

provide a mechanism to do this.

If  a fatal error takes place on either the sending or receiving side, the side

which encountered the error should send an Error (E) packet.  The E packet con-

tains a brief textual error message in the data field.   Both  the  sender  and

receiver  should  be prepared to receive an Error packet at any time during the

transaction.  Both the sender and receiver of the Error packet should halt,  or

go  back  into into user command mode (a server should return to server command

wait).  The side that is local should print the error message on the screen.

There is no provision for sending nonfatal error messages, warnings, or  infor-

mation  messages during a transaction.  It would be possible to add such a fea-

ture, but this would require both sides agree to use it through  setting  of  a

bit  in the capability mask, since older KERMITs that did not know about such a

feature would encounter an unexpected packet type and would enter the fatal er-

ror state.  In any case, the utility of such a feature is  questionable,  since

there is no guarantee that the user will be present to see such messages at the

time  they  are sent; even if they are saved up for later perusal in a "message

box", their significance may be long past by the time the user reads them.  See

the section on Robustness, below.

5.4. Heuristics

During any transaction, several heuristics are useful:

1. A NAK for the current packet is equivalent to an ACK  for  the  pre-

vious  packet  (modulo  64).    This handles the common situation in

which a packet is successfully received, and then ACK'd, but the ACK

is lost.  The ACKing side then times out waiting for the next packet

and NAKs it.  The side that receives a  NAK  for  packet  n+1  while

waiting for an ACK for packet n simply sends packet n+1.

2. If  packet  n  arrives more than once, simply ACK it and discard it.

This can happen when the first ACK was lost.  Resending the  ACK  is

necessary  and  sufficient -- don't write the packet out to the file

again!

File Transfer                                                           Page 15

3. When  opening a connection, discard the contents of the line's input

buffer before reading or sending the first packet.   This  is  espe-

cially  important if the other side is in receive mode (or acting as

a server), in which case it may have been sending out periodic  NAKs

for  your  expected  SEND-INIT  or  command packet.  If you don't do

this, you may find that there are sufficient  NAKs  to  prevent  the

transfer -- you send a Send-Init, read the response, which is an old

NAK,  so  you  send another Send-Init, read the next old NAK, and so

forth, up to the retransmission limit, and give up before getting to

the ACKs that are waiting in line behind all the old NAKs.   If  the

number  of  NAKs is below the cutoff, then each packet may be trans-

mitted multiply.

4. Similarly, before sending a packet, you should clear the input buff-

er (after looking for any required handshake character).  Failure to

clear the buffer could result in propogation of the repetition of  a

packet caused by stacked-up NAKs.

5.5. File Names

The  syntax  for  file  names  can vary widely from system to system.  To avoid

problems, it is suggested that filenames be represented in the File Header  (F)

packet  in  a  "normal form", by default (that is, there should be an option to

override such conversions).

1. Delete all pathnames and attributes  from  the  file  specification.

The file header packet should not contain directory or device names;

if  it  does, it may cause the recipient to try to store the file in

an inaccessible or nonexistent area, or it  may  result  in  a  very

strange filename.

2. After  stripping  any  pathname,  convert  the remainder of the file

specification to the form "name.type", with no restriction on length

(except that it fit in the data field of the F packet), and:

a. Include no more than one dot.

b. Use digits, uppercase letters only in name and type.

Special characters like "$", "_", "-", "&", and so forth should be  disallowed,

since they're sure to cause problems on one system or another.

The  recipient, of course, cannot depend upon the sender to follow this conven-

tion, and should still take precautions.  However, since most file systems  em-

body  the  notion  of  a  file name and a file type, this convention will allow

these items to be expressed in a way that an unlike system can understand.  The

particular notation is chosen simply because it is the most common.

The recipient must worry about the length of the name and type  fields  of  the

file  name.    If  either  is  too long, they must be truncated.  If the result

(whether truncated or not) is the same as the name of a file that  already  ex-

ists  in the same area, the recipient should have the ability to take some spe-

cial action to avoid writing over the original file.

KERMIT implementations that convert  file  specifications  to  normal  form  by

default  should  have  an  option to override this feature.  This would be most

File Transfer                                                           Page 16

useful  when  transferring files between like systems, perhaps used in conjunc-

tion with "image mode" file transfer.  This could allow, for instance, one UNIX

system to send an entire directory tree to another UNIX system.

5.6. Robustness

A major feature of the KERMIT protocol is  the  ability  to  transfer  multiple

files.    Whether  a particular KERMIT program can actually send multiple files

depends on the capabilities of the program and the host operating  system  (any

KERMIT program can receive multiple files).

If  a  KERMIT  program can send multiple files, it should make every attempt to

send the entire group specified.  If it fails to send  a  particular  file,  it

should  not  terminate the entire batch, but should go on the the next one, and

proceed until an attempt has been made to send each file in the group.

Operating in this robust manner, however, gives rise to  a  problem:  the  user

must  be  notified of a failure to send any particular file.  Unfortunately, it

is not sufficient to print a message to the screen since the user  may  not  be

physically  present.   A better solution would be to have the sender optionally

keep a log of the transaction, giving the name of each file for  which  an  at-

tempt was made, and stating whether the attempt was successful, and if not, the

reason.    Additional aids to robustness are described in the Optional Features

section, below.

5.7. Flow Control

On full duplex connections, XON/XOFF flow control can generally be used in con-

junction with KERMIT file transfer with no ill effects.  This is because  XOFFs

are  sent  in the opposite direction of packet flow, so they will not interfere

with the packets themselves.  XON/XOFF, therefore, need not be  implemented  by

the  KERMIT  program,  but  can  done  by  the host system.  If the host system

provides this capability, it should be  used  --  if  both  sides  can  respond

XON/XOFF  signals,  then  buffer  overruns  and  the  resulting  costly  packet

retransmissions can be avoided.

Beware, however, of the following situation: remote Kermit is sending  periodic

NAKs, local system is buffering them on the operating system level (because the

user has not started the local end of the file transfer yet); local line buffer

becomes  full, local systems sends XOFF, remote starts buffering them up on its

end, user finally starts file transfer  on  local  end,  clears  buffer,  local

operating  system  sends  XON, and then all the remotely buffered NAKs show up,

causing the packet echoing problem described above, despite the  buffer  clear-

ing.

Flow control via modem signals can also be used when available.

Note  that  flow  control  should  not  be  confused  with "handshake" or "line

turnaround" techniques that are used on simplex  or  half-duplex  communication

lines.

File Transfer                                                           Page 17

5.8. Basic KERMIT Protocol State Table

The  KERMIT  protocol  can be described as a set of states and transitions, and

rules for what to do when changing from one state to another.    State  changes

occur  based  on  the type of packets that are sent or received, or errors that

may occur.  Packets always go back and forth; the sender of a file always sends

data packets of some kind (init, header, data) and the receiver always  returns

ACK or NAK packets.

Upon  entering  a given state, a certain kind of packet is either being sent or

is expected to arrive -- this is shown on top of the description of that state.

As a result of the action, various responses may occur; these are shown in  the

EVENT column.  For each event, an appropriate ACTION is taken, and the protocol

enters a NEW STATE.

The  following table specifies basic KERMIT operation.  Timeouts and error con-

ditions have been omitted from the following table for simplicity, but the  ac-

tion is as described above.  Server operation and some of the advanced features

are also omitted.  A full-blown state table is given subsequently.

File Transfer                                                           Page 18

STATE   EVENT           ACTION                      NEW STATE

-- SEND STATES --

Send Send-Init Packet

S       Get NAK,bad ACK (None)                          S

Get good ACK    Set remote's params, open file  SF

(Other)         (None)                          A

Send File-Header Packet

SF      Get NAK,bad ACK (None)                          SF

Get good ACK    Get bufferful of file data      SD

(Other)         (None)                          A

Send File-Data Packet

SD      Get NAK,bad ACK (None)                          SD

Get good ACK    Get bufferful of file data      SD

(End of file)   (None)                          SZ

(Other)         (None)                          A

Send EOF Packet

SZ      Get NAK,bad ACK (None)                          SZ

Get good ACK    Get next file to send           SF

(No more files) (None)                          SB

(Other)         (None)                          A

Send Break (EOT) Packet

SB      Get NAK,bad ACK (None)                          SB

Get good ACK    (None)                          C

(Other)         (None)                          A

-- RECEIVE STATES --

Wait for Send-Init Packet

R       Get Send-Init   ACK w/local params              RF

(Other)         (None)                          A

Wait for File-Header Packet

RF      Get Send-Init   ACK w/local params

(previous ACK was lost)        RF

Get Send-EOF    ACK (prev ACK lost)             RF

Get Break       ACK                             C

Get File-Header Open file, ACK                  RD

(Other)         (None)                          A

Wait for File-Data Packet

RD      Get previous

packet(D,F)    ACK it again                    RD

Get EOF         ACK it, close the file          RF

Get good data   Write to file, ACK              RD

(Other)         (None)                          A

File Transfer                                                           Page 19

-- STATES COMMON TO SENDING AND RECEIVING --

C       (Send Complete)                                 start

A       ("Abort")                                       start

Packet Format                                                           Page 20

6. Packet Format

6.1. Fields

The  KERMIT  protocol is built around exchange of packets of the following for-

mat:

+------+-----------+-----------+------+------------+-------+

| MARK | char(LEN) | char(SEQ) | TYPE |    DATA    | CHECK |

+------+-----------+-----------+------+------------+-------+

where all fields consist of ASCII characters.  The fields are:

MARK    The synchronization character that marks the beginning of  the  packet.

This should normally be CTRL-A, but may be redefined.

LEN     The  number  of  ASCII  characters  within  the packet that follow this

field, in other words the packet length minus two.  Since  this  number

is  transformed  to  a single character via the char() function, packet

character counts of 0 to 94 (decimal) are permitted, and  96  (decimal)

is  the  maximum total packet length.  The length does not include end-

of-line or padding characters, which are outside  the  packet  and  are

strictly  for  the  benefit  of  the operating system or communications

equipment, but it does include the block check characters.

SEQ     The packet sequence number, modulo 64, ranging from 0 to 63.   Sequence

numbers "wrap around" to 0 after each group of 64 packets.

TYPE    The  packet type, a single ASCII character.  The following packet types

are required:

D   Data packet

Y   Acknowledge (ACK)

N   Negative acknowledge (NAK)

S   Send initiate (exchange parameters)

B   Break transmission (EOT)

F   File header

Z   End of file (EOF)

E   Error

T   Reserved for internal use

The NAK packet is used only to indicate that the  expected  packet  was

not  received  correctly,  never  to supply other kinds of information,

such as refusal to perform a requested service.  The NAK packet  always

has  an  empty  data  field.  The T "packet" is used internally by many

KERMIT programs to indicate that a timeout occurred.

DATA    The "contents" of the packet, if any contents are required in the given

type of packet, interpreted according to  the  packet  type.    Control

characters (bytes whose low order 7 bits are in the ASCII control range

0-31, or 127) are preceded by a special prefix character, normally "#",

and "uncontrollified" via ctl().  A prefixed sequence may not be broken

across  packets.  Logical records in printable files are delimited with

CRLFs, suitably prefixed (e.g. "#M#J").  Logical records need not  cor-

respond  to  packets.  Any prefix characters are included in the count.

Packet Format                                                           Page 21

Optional  encoding  for 8-bit data and repeated characters is described

later.

CHECK   A block check on the characters in the packet between, but not  includ-

ing, the mark and the block check itself.  The check for each packet is

computed  by  both hosts, and must agree if a packet is to be accepted.

A single-character arithmetic checksum is the normal and required block

check.  Only six bits of the arithmetic sum are  included.    In  order

that  all  the bits of each data character contribute to this quantity,

bits 6 and 7 of the final value are added to  the  quantity  formed  by

bits  0-5.    Thus  if s is the arithmetic sum of the ASCII characters,

then

check = char((s + ((s AND 192)/64)) AND 63)

This is the default block check, and all Kermits  must  be  capable  of

performing it.  Other optional block check types are described later.

The  block  check is based on the ASCII values of all the characters in

the packet, including control fields and prefix characters.   Non-ASCII

systems  must translate to ASCII before performing the block check cal-

culation.

6.2. Terminator

Any line terminator that is required by the  system  may  be  appended  to  the

packet;  this  is  carriage return (ASCII 15) by default.  Line terminators are

not considered part of the packet, and are included for in the count or  check-

sum.    Terminators are not necessary to the protocol, and are invisible to it,

as are any characters that may appear between packets.  If  a  host  cannot  do

single character input from a TTY line, then a terminator will be required when

sending  to  that host.  The terminator can be specified in the initial connec-

tion exchange.

Some  KERMIT  implementations  also  use  the  terminator  for  another  reason

--  speed.   Some systems are not fast enough to take in a packet and decode it

character by character at high baud rates; by blindly reading and  storing  all

characters  between  the MARK and the EOL, they are able to absorb the incoming

characters at full speed and then process them at their own rate.

6.3. Other Interpacket Data

The space between packets may be used for any  desired  purpose.    Handshaking

characters  may  be necessary on certain connections, others may require screen

control or other sequences to keep the packets flowing.

Packet Format                                                           Page 22

6.4. Encoding, Prefixing, Block Check

MARK,  LEN, SEQ, TYPE, and CHECK are control fields.  Control fields are always

literal single-character fields, except that the CHECK field may be extended by

one or two additional check characters.   Each  control  field  is  encoded  by

char()  or  taken literally, but never prefixed.  The control fields never con-

tain 8-bit data.

The DATA field contains a string  of  data  characters  in  which  any  control

characters  are  encoded  printably  and preceded with the control prefix.  The

decision to prefix a character in this way depends upon whether its low order 7

bits are in the ASCII control range, i.e. 0-31 or 127.  Prefix characters  that

appear  in  the data must themselves be prefixed by the control prefix, but un-

like control characters, these retain their literal value in the packet.

The treatment of the high order ("8th") bit of a data byte is as follows:

- If the communication channel allows 8 data bits per  character,  then

the original value of the 8th bit is retained in the prefixed charac-

ter.  For instance, a data byte corresponding to a Control-A with the

8th  bit set would be send as a control prefix, normally "#", without

the 8th bit set, followed by ctl(^A) with the 8th bit set.  In binary

notation, this would be

00100011 10000001

In this case, the 8th bit is figured into all  block  check  calcula-

tions.

- If  the  communication channel or one of the hosts required parity on

each character, and both sides were  capable  of  8th-bit  prefixing,

then the 8th bit will be used for parity, and must not be included in

the block check.  8th bit prefixing is an option feature described in

greater detail in Section 8, below.

- If parity is being used but 8th-bit prefixing is not being done, then

the  value  of  the 8th bit of each data byte will be lost and binary

files will not be transmitted correctly.  Again, the 8th bit does not

figure into the block check.

The data fields of all packets are subject to prefix encoding, except S, I, and

A packets, and their ACKs (see below).

Initial Connection                                                      Page 23

7. Initial Connection

Initial connection occurs when the user has started up a Kermit program on both

ends  of  the physical connection.  One Kermit has been directed (in one way or

another) to send a file, and the other to receive it.

The receiving Kermit waits for a "Send-Init" packet from  the  sending  Kermit.

It  doesn't  matter  whether  the sending Kermit is started before or after the

receiving Kermit (if before,  the  Send-Init  packet  should  be  retransmitted

periodically  until  the  receiving Kermit acknowledges it).  The data field of

the Send-Init packet is optional; trailing  fields  can  be  omitted  (or  left

blank, i.e. contain a space) to accept or specify default values.

The Send-Init packet contains a string of configuration information in its data

field.   The receiver sends an ACK for the Send-Init, whose data field contains

its own configuration parameters.  The data field of the Send-Init and the  ACK

to  the  Send-Init  are literal, that is, there is no prefix encoding.  This is

because the two parties will not know how to do prefix encoding until after the

configuration data is exchanged.

It is important to note that newly invented fields are added at the  right,  so

that  old  KERMIT  programs that do not have code to handle the new fields will

act as if they were not there.  For this reason,  the  default  value  for  any

field,  indicated  by blank, should result in the behavior that occurred before

the new field was defined or added.

1      2      3      4      5      6      7      8      9      10...

+------+------+------+------+------+------+------+------+------+-------

| MAXL | TIME | NPAD | PADC | EOL  | QCTL | QBIN | CHKT | REPT | CAPAS

+------+------+------+------+------+------+------+------+------+-------

The fields are as follows (the first and second person "I" and "you"  are  used

to  distinguish  the two sides).  Fields are encoded printably using the char()

function unless indicated otherwise.

1. MAXL   The maximum length packet I want  to  receive,  a  number  up  to  94

(decimal).    You respond with the maximum you want me to send.  This

allows systems to adjust to each other's buffer sizes, or to the con-

dition of the transmission medium.

2. TIME   The number of seconds after which I want you to  time  me  out  while

waiting  for a packet from me.  You respond with the amount of time I

should wait for packets from you.  This allows the two sides  to  ac-

commodate  to different line speeds or other factors that could cause

timing problems.  Only one side needs to time out.    If  both  sides

time out, then the timeout intervals should not be close together.

3. NPAD   The  number  of  padding  characters  I want to precede each incoming

packet; you respond in kind.  Padding may be necessary  when  sending

to  a half duplex system that requires some time to change the direc-

tion of transmission, although in practice  this  situation  is  more

commonly handled by a "handshake" mechanism.

4. PADC   The  control  character  I  need  for padding, if any, transformed by

ctl() (not char()) to make it printable.  You respond in kind.   Nor-

mally NUL (ASCII 0), some systems use DEL (ASCII 127).  This field is

Initial Connection                                                      Page 24

to be ignored if the value NPAD is zero.

5. EOL    The  character  I  need to terminate an incoming packet, if any.  You

respond in kind.  Most systems that require  a  line  terminator  for

terminal input accept carriage return for this purpose (note, because

there  is no way to specify that no EOL should be sent, it would have

been better to use ctl() for this field rather than char(), but  it's

too late now).

6. QCTL   (verbatim)  The printable ASCII character I will use to quote control

characters, normally and by default "#".  You respond  with  the  one

you will use.

The  following  fields  relate  to  the  use of OPTIONAL features of the KERMIT

protocol, described in section 8.

7. QBIN   (verbatim) The printable ASCII character  I  want  to  use  to  quote

characters  which have the 8th bit set, for transmitting binary files

when the parity bit cannot be used for data.    Since  this  kind  of

quoting  increases  both  processor  and transmission overhead, it is

normally to be avoided.  If used, the quote character must be in  the

range  ASCII 33-62 ("!" through ">") or 96-126 ("`" through "~"), but

different from the control-quoting character.  This field  is  inter-

preted as follows:

Y   I agree to 8-bit quoting if you request it.

N   I will not do 8-bit quoting.

&   (or  any  other character in the range 33-62 or 96-126) I want to

do 8-bit quoting using this character (it will  be  done  if  the

other  Kermit  puts  a Y in this field, or responds with the same

prefix character, such as &).  The  recommended  8th-bit  quoting

prefix character is "&".

Anything Else : 8-bit quoting will not be done.

Note that this scheme allows either side to initiate the request, and

the  order  does  not matter.  For instance, a micro capable of 8-bit

communication will normally  put  a  "Y"  in  this  field  whereas  a

mainframe  that  uses  parity  will always put an "&".  No matter who

sends first, this combination will  result  in  election  of  8th-bit

quoting.

8. CHKT   Check  Type, the method for detecting errors.  "1" for single-charac-

ter checksum (the normal and required method), "2" for  two-character

checksum  (optional),  "3"  for three-character CRC-CCITT (optional).

If your response agrees, the designated method will be  used;  other-

wise the single-character checksum will be used.

9. REPT   The  prefix  character  I  will use to indicate a repeated character.

This can be any printable character  in  the  range  ASCII  33-62  or

96-126, but different from the control and 8th-bit prefixes.  SP (32)

denotes no repeat count processing is to be done.  Tilde ("~") is the

recommended  and  normal  repeat  prefix.  If you don't respond iden-

tically, repeat counts will not be done.  Groups of at least 3  or  4

identical  characters  may  be  transmitted  more efficiently using a

repeat count, though an individual implementation may wish to  set  a

different threshhold.

Initial Connection                                                      Page 25

10-?. CAPAS

A bit mask, in which each bit position corresponds to a capability of

KERMIT,  and is set to 1 if that capability is present, or 0 if it is

not.  Each character contains a 6-bit field (transformed by  CHAR()),

whose  low  order bit is set to 1 if another capability byte follows,

and to 0 in the last capability byte.  The  capabilities  defined  so

far are:

#1  Reserved

#2  Reserved

#3  Ability to accept "A" packets (file attributes)

The capability byte as defined so far would then look like:

bit5 bit4 bit3 bit2 bit1 bit0

+----+----+----+----+----+----+

| #1 | #2 | #3 | -- | -- |  0 |

+----+----+----+----+----+----+

If  all  these capabilities were "on", the value of the byte would be

70 (octal).  When capabilities 4, 5 and 6 are added,  the  capability

mask will look like this:

bit5 bit4 bit3 bit2 bit1 bit0    bit5 bit4 bit3 bit2 bit1 bit0

+----+----+----+----+----+----+   +----+----+----+----+----+----+

| #1 | #2 | #3 | #4 | #5 |  1 |   | #6 | -- | -- | -- | -- |  0 |

+----+----+----+----+----+----+   +----+----+----+----+----+----+

Next 4: Reserved Fields

Sites that wish to add their own parameters to the initial connection

negotiation  must  start  at  the 5th field after the last capability

byte.  Any intervening fields may be left blank (that  is,  they  may

contain  the  space character).  These fields are reserved for future

use by the standard KERMIT protocol.

The control, 8th-bit, and repeat prefixes must be distinct.

The receiving Kermit responds with an ACK ("Y") packet in the  same  format  to

indicate  its  own preferences, options, and parameters.  The ACK need not con-

tain the same number of fields as the the Send-Init.  From that point, the  two

KERMIT  programs  are  "configured"  to  communicate  with  each  other for the

remainder of the transaction.  In the case of 8th-bit quoting,  one  side  must

specify  the  character  to be used, and the other must agree with a "Y" in the

same field, but the order in which this occurs does not matter.  Similarly  for

checksums  --  if  one  side  requests 2 character checksums and the other side

responds with a "1" or with nothing at  all,  then  single-character  checksums

will  be  done, since not all implementations can be expected to do 2-character

checksums or CRCs.  And for repeat counts; if the repeat field of the send-init

and the ACK do not agree, repeat processing will not be done.

All Send-Init fields are optional.  The data field may be left  totally  empty.

Similarly,  intervening fields may be defaulted by setting them to blank.  Ker-

mit implementations should know what to do in these  cases,  namely  apply  ap-

propriate defaults.  The defaults should be:

MAXL:   80

Initial Connection                                                      Page 26

NPAD:   0, no padding

PADC:   0 (NUL)

EOL:    CR (carriage return)

QCTL:   the character "#"

QBIN:   none, don't do 8-bit quoting

CHKT:   "1", single-character checksum

REPT:   No repeat count processing

MASK:   All zeros (no special capabilities)

There are no prolonged negotiations in the initial connection sequence -- there

is  one Send-Init and one ACK in reply.  Everything must be settled in this ex-

change.

The very first Send-Init may not get through if the sending Kermit makes  wrong

assumptions about the receiving host.  For instance, the receiving host may re-

quire  certain  parity,  some  padding,  handshaking,  or a special end of line

character in order to read the Send-Init packet.  For this reason, there should

be a way for the user the user to specify whatever may be necessary to get  the

first packet through.

A  parity field is not provided in the Send-Init packet because it could not be

of use.  If the sender requires a certain kind of parity, it will also be send-

ing it.  If the receiver does not know this in advance, i.e. before getting the

Send-Init, it will not be able to read the Send-Init packet.

Optional Features                                                       Page 27

8. Optional Features

The foregoing sections have discussed basic, required operations for any KERMIT

implementation.  The following sections discuss optional and advanced features.

8.1. 8th-Bit and Repeat Count Prefixing

Prefix  quoting of control characters is mandatory.  In addition, prefixing may

also be used for 8-bit quantities or repeat counts, when both  KERMIT  programs

agree  to  do  so.   8th-bit prefixing can allow 8-bit binary data pass through

7-bit physical links.  Repeat count prefixing can  improve  the  throughput  of

certain  kinds  of  files  dramatically;  binary files (particularly executable

programs) and structured text (highly indented or columnar text) tend to be the

major beneficiaries.

When more than one type of prefixing is in effect, a single data character  can

be  preceded  by  more  than one prefix character.  Repeat count processing can

only be requested by the sender, and will only be used by  the  sender  if  the

receiver  agrees.   8th-bit prefixing is a special case because its use is nor-

mally not desirable, since it increases both processing and transmission  over-

head.   However, since it is the only straightforward mechanism for binary file

transfer available to those systems that usurp the parity bit, a receiver  must

be  able  to  request the sender to do 8th-bit quoting, since most senders will

not normally do it by default.

The repeat prefix is followed immediately by a single-character  repeat  count,

encoded  printably  via  char(),  followed  by  the  character  itself (perhaps

prefixed by control or 8th bit quotes, as explained below).  The  repeat  count

may  express values from 0 to 94.  If a character appears more than 94 times in

a row, it must be "cut off" at 94, emitted with all appropriate  prefixes,  and

"restarted".    The  following  table should clarify Kermit's quoting mechanism

(the final line shows how a sequence of 120 consecutive NULs would be encoded):

Quoted             With

Character    Representation     Repeat Count for 6

A            A                ~(A   ["(" is ASCII 40 - 32 = 6]

^A           #A               ~(#A

'A           &A               ~(&A

'^A          &#A              ~(&#A

#            ##               ~(##

'#           &##              ~(&##

&            #&               ~(#&

'&           &#&              ~(&#&

~            #~               ~(#~

'~           &#~              ~(&#~

NUL          #@               ~~#@~:#@ [120 NULs]

A represents any printable character, ^A represents any control  character,  'x

represents  any  character  with  the 8th bit set.  The # character is used for

control-character quoting, and the & character for 8-bit quoting.   The  repeat

count  must  always  precede  any  other prefix character.  The repeat count is

taken literally (after transformation by unchar(); for instance "#" and "&" im-

mediately following a "~" denote repeat counts, not control characters or 8-bit

characters.  The control quote character "#" is most closely bound to the  data

character,  then  the  8-bit prefix, then the repeat count; in other words, the

Optional Features                                                       Page 28

order  is:    repeat prefix and count, 8-bit quote, control quote, and the data

character itself.  To illustrate, note that &#A is not equivalent to #&A.

When the parity bit is available for data, then 8th-bit quoting should  not  be

done, and the 8th bit of the prefixed character will have the same value as the

8th bit of the original data byte.  In that case, the table looks like this:

Quoted             With

Character    Representation     Repeat Count for 6

'A           'A               ~('A

'^A          #'A              ~(#'A

'#           #'#              ~(#'#

'&           '&               ~('&

'~           #'~              ~(#'~

Note  that since 8th bit quoting is not being done, "&" is not being used as an

8th bit prefix character, so it does not need to be quoted  with  "#".    Also,

note  that  the 8th bit is set on the final argument of the repeat sequence, no

matter how long, and not on any of the prefix characters.

Finally, remember the following rules:

- Prefixed sequences must not be broken across packets.

- Control, 8th-bit, and repeat count prefixes must be distinct.

- Data fields of all packets must  pass  through  the  prefix  encoding

mechanism, except for S, I, and A packets, and ACKs to those packets.

In the first rule above, note that a prefixed sequence means a single character

and  all  its  prefixes,  like  ~%&#X, not a sequence like #M#J, which is two

prefixed sequences.

8.2. Server Operation

A KERMIT server is a KERMIT program running remotely with no "user  interface".

All  commands  to  the  server arrive in packets from the local KERMIT.  SERVER

operation is much more convenient than basic operation,  since  the  user  need

never  again interact directly with the remote KERMIT program after once start-

ing it up in server mode, and therefore need not issue complementary  SEND  and

RECEIVE  commands  on  the  two sides to get a file transfer started; rather, a

single command (such as SEND or GET) to the local KERMIT suffices.  KERMIT ser-

vers can also provide services beyond file transfer.

Between transactions, a Kermit server waits for packets containing server  com-

mands.  The packet sequence number is always set back to 0 after a transaction.

A  Kermit  server  in  command wait should be looking for packet 0, and command

packets sent to servers should also be packet 0.  Certain server commands  will

result  in  the exchange of multiple packets.  Those operations proceed exactly

like file transfer.

A KERMIT server program waiting for a command packet is said to be  in  "server

command  wait".    Once  put  into server command wait, the server should never

leave it until it gets a command packet telling it to do so.  This  means  that

after  any  transaction is terminated, either normally or by any kind of error,

Optional Features                                                       Page 29

the server must go back into command wait.  While in command wait, a server may

elect  to  send  out  periodic  NAKs for packet 0, the expected command packet.

Since the user may be disconnected from the server for  long  periods  of  time

(hours),  the  interval  between these NAKs should be significantly longer than

the normal timeout interval (say, 30-60 seconds, rather than 5-10).  The  peri-

odic  NAKs  are  useful  for  breaking the deadlock that would occur if a local

program was unable to time out, and sent a command that was lost.  On the other

hand, they can cause problems for local KERMIT programs that cannot clear their

input buffers, or for systems that do XON/XOFF blindly,  causing  the  NAKs  to

buffered  in the server's host system output buffer, to be suddenly released en

masse when an XON appears.  For this reason, servers should have an  option  to

set the command-wait wakeup interval, or to disable it altogher.

Server  operation  must be implemented in two places: in the server itself, and

in any KERMIT program that will be communicating with a  server.    The  server

must  have  code  to read the server commands from packets and respond to them.

The user KERMIT must have code to parse the user's server-related commands,  to

form  the  server  command packets, and to handle the responses to those server

commands.

8.2.1. Server Commands

Server commands are listed below.  Not all of them have been  implemented,  and

some  may  never  be,  but  their use should be reserved.  Although server-mode

operation is optional, certain commands should be implemented in every  server.

These  include  Send-Init  (S),  Receive-Init  (R), and the Generic Logout (GL)

and/or Finish (GF) commands.  If the server receives a command it does not  un-

derstand,  or  cannot  execute, it should respond with an Error (E) packet con-

taining a message like "Unimplemented Server Command" and both sides should set

the packet sequence number back to 0, and the server should  remain  in  server

command wait.  Only a GL or GF command should terminate server operation.

Server commands are as follows:

S   Send Initiate (exchange parameters, server waits for a file).

R   Receive Initiate (ask the server to send the specified files).

I   Initialize (exchange parameters).

X   Text header.  Allows transfer of text to the user's screen in response to a

generic  or  host  command.  This works just like file transfer except that

the destination "device" is the screen rather than a file.  Data field  may

contain a filename, title, or other heading.

C   Host Command.  The data field contains a string to be executed as a command

by the host system command processor.

K   KERMIT  Command.   The data field contains a string in the interactive com-

mand language of the KERMIT server (normally a SET command) to be  executed

as if it were typed in at command level.

G   Generic  Kermit Command.  Single character in data field (possibly followed

by operands, shown in {braces}, optional fields  in  [brackets])  specifies

the command:

I   Login [{*user[*password[*account]]}]

C   CWD, Change Working Directory [{*directory[*password]}]

L   Logout, Bye

F   Finish (Shut down the server, but don't logout).

D   Directory [{*filespec}]

Optional Features                                                       Page 30

U   Disk Usage Query [{*area}]

E   Erase (delete) {*filespec}

T   Type {*filespec}

R   Rename {*oldname*newname}

K   Copy {*source*destination}

W   Who's logged in? (Finger) [{*user ID or network host[*options]}]

M   Send a short Message {*destination*text}

H   Help [{*topic}]

Q   Server Status Query

P   Program {*[program-filespec][*program-commands]}

J   Journal {*command[*argument]}

V   Variable {*command[*argument[*argument]]}

Note  that  field  length  encoding  is  used  within the data field of all

Generic command packets, but not within the data fields of the other  pack-

ets, such as S, I, R, X, K, and C.

Asterisk  as  used  above ("*") represents a single-character length field, en-

coded using char(), for the operand that follows it; thus lengths from 0 to  94

may  be  specified.    This  allows  multiple  operands to be clearly delimited

regardless of their contents.

All server commands that send  arguments  in  their  data  fields  should  pass

through  the  prefix  encoding  mechanism.   Thus if a data character or length

field happens to correspond to an active prefix character, it  must  itself  be

prefixed.    The field length denotes the length of the field before prefix en-

coding and (hopefully) after prefix decoding.  For example, to send  a  generic

command  with  two  fields,  "ABC"  and  "ZZZZZZZZ",  first each field would be

prefixed by char() of its length, in this  case  char(3)  and  char(8),  giving

"#ABC(ZZZZZZZZ".   But "#" is the normal control prefix character so it must be

prefixed itself, and the eight Z's can be condensed to  3  characters  using  a

repeat  prefix  (if  repeat counts are in effect), so the result after encoding

would be "##ABC(~(Z" (assuming the repeat prefix is tilde ("~").  The recipient

would decode this back into the original "#ABC(ZZZZZZZZ" before  attempting  to

extract the two fields.

Since  a generic command must fit into a single packet, the program sending the

command should ensure that the command actually fits, and  should  not  include

length  fields  that  point  beyond  the  end of the packet.  Servers, however,

should be defensive and not attempt to process any characters beyond the end of

the data field, even if the argument length field would lead them to do so.

8.2.2. Timing

KERMIT does not provide a mechanism for  suspending  and  continuing  a  trans-

action.    This  means that text sent to the user's screen should not be frozen

for long periods (i.e. not longer than  the  timeout  period  times  the  retry

threshold).

Between  transactions,  when  the  server has no tasks pending, it may send out

periodic NAKs (always with type 1 checksums) to prevent a deadlock  in  case  a

command  was  sent  to  it  but  was lost.  These NAKs can pile up in the local

"user" Kermit's input buffer (if it has one), so  the  user  Kermit  should  be

prepared  to  clear  its  input  buffer  before  sending a command to a server.

Meanwhile, servers should recognize that some systems provide no function to do

Optional Features                                                       Page 31

this  (or  even  when they do, the process can be foiled by system flow control

firmware) and should therefore provide a way turn off or slow down the command-

wait NAKs.

8.2.3. The R Command

The R packet, generally sent by a local Kermit program whose user typed  a  GET

command,  tells  the server to send the files specified by the name in the data

field of the R packet.  Since we can't assume that the two Kermits are  running

on like systems, the local (user) Kermit must parse the file specification as a

character  string  and  let the server to check it.  If the server can open and

read the specified file, it sends a Send-Init (S) packet -- not an acknowledge-

ment! -- to the user, and  then  completes  the  file-sending  transaction,  as

described above.

If  the server cannot send the file, it should respond with an error (E) packet

containing a reason, like "File not found" or "Read access required".

8.2.4. The K Command

The K packet can contain a character string which the server  interprets  as  a

command  in  its own interactive command language.  This facility is useful for

achieving the same effect as a direct command without having to shut  down  the

server,  connect  back  to the remote system, continue it (or start a new one),

and issue the desired commands.  The server responds with an ACK if the command

was executed successfully, or an error packet otherwise.  The most  likely  use

for the K packet might be for transmitting SET commands, e.g. for switching be-

tween text and binary file modes.

8.2.5. Short and Long Replies

Any  request  made  of  a server may be answered in either of two ways, and any

User Kermit that makes such a request should be prepared  for  either  kind  of

reply:

- A  short reply.  This consists of a single ACK packet, which may con-

tain text in its data field.  For instance, the  user  might  send  a

disk  space query to the server, and the server might ACK the request

with a short character string in the data field, such as  "12K  bytes

free".  The user KERMIT should display this text on the screen.

- A  long  reply.    This proceeds exactly like a file transfer (and in

some cases it may be a file transfer).  It begins  with  one  of  the

following:

* A File-Header (F) packet (optionally followed by one or more At-

tributes packets; these are discussed later);

* A Text-Header (X) packet.

* A Send-Init (S) Packet, followed by an X or F packet.

After  the  X or F packet comes an arbitrary number of Data (D) pack-

Optional Features                                                       Page 32

ets, then an End-Of-File (Z) packet, and finally a Break-Transmission

(B) packet, as for ordinary file transfer.

A  long reply should begin with an S packet unless an I-packet exchange has al-

ready taken place, and the type 1 (single-character) block check is being used.

8.2.6. Additional Server Commands

The following server commands request the server to perform  tasks  other  than

sending  or receiving files.  Almost any of these can have either short or long

replies.  For instance, the Generic Erase (GE) command may elicit a simple ACK,

or a stream of packets containing the names of all  the  files  it  erased  (or

didn't  erase).  These commands are now described in more detail; arguments are

as provided in commands typed to the user KERMIT (subject to prefix  encoding);

no  transformations to any kind of normal or canonic form are done -- filenames

and other operands are in the syntax of the server's host system.

I   Login.  For use when a KERMIT server is kept perpetually running on a dedi-

cated line.  This lets a new user obtain an identity on the  server's  host

system.    If the data field is empty, this removes the user's identity, so

that the next user does not get access to it.

L   Logout, Bye.  This shuts down the server entirely, causing the  server  it-

self  to  log  out  its  own job.  This is for use when the server has been

started up manually by the user, who then wishes to shut it down  remotely.

For a perpetual, dedicated server, this command simply removes the server's

access  rights  to  the current user's files, and leaves the server waiting

for a new login command.

F   Finish.  This is to allow the user to shut down  the  server,  putting  its

terminal  back  into normal (as opposed to binary or raw) mode, and putting

the server's job back at system command level, still logged in, so that the

user can connect back to the job.  For a perpetual, dedicated server,  this

command behaves as the L (BYE) command.

C   CWD.    Change  Working Directory.  This sets the default directory or area

for file transfer on the server's host.  With  no  operands,  this  command

sets the default area to be the user's own default area.

D   Directory.    Send  a  directory listing to the user.  The user program can

display it on the terminal or store it in a  file,  as  it  chooses.    The

directory  listing  should contain file sizes and creation dates as well as

file names, if possible.  A wildcard or other file-group designator may  be

specified  to  ask  the  server  list  only  those files that match.  If no

operand is given, all files in the current area should be shown.

U   Disk Usage Query.  The server responds with the amount of  space  used  and

the  amount  left  free to use, in K bytes (or other units, which should be

specified).

E   Erase (delete).  Delete the specified file or file group.

T   Type.  Send the specified file or file group, indicating (by starting  with

an  X  packet rather than an F packet, or else by using the Type attribute)

that the file is to be displayed on the screen, rather than stored.

Optional Features                                                       Page 33

R   Rename.  Change the name of the file or files as indicated.  The string in-

dicating  the  new  name  may  contain other attributes, such as protection

code, permitted in file specifications by the host.

K   Copy.  Produce a new copy of the file or file group, as indicated,  leaving

the source file(s) unmodified.

W   Who's  logged  in? (Finger).  With no arguments, list all the users who are

logged in on the server's host  system.    If  an  argument  is  specified,

provide more detailed information on the specified user or network host.

M   Short  Message.    Send  the given short (single-packet) message to the in-

dicated user's screen.

P   Program.  This command has two  arguments,  program  name  (filespec),  and

command(s)  for  the program.  The first field is required, but may be left

null (i.e. zero length).  If it is null, the currently  loaded  program  is

"fed"  the specified command.  If not null, the specified program is loaded

and started; if a program command is given it is fed to the program  as  an

initial  command  (for instance, as a command line argument on systems that

support that concept).  In any case, the output of the program is sent back

in packets as either a long or short reply, as described above.

J   Journal.  This command controls server transaction logging.  The data field

contains one of the following:

+   Begin/resume logging transactions.  If a filename is given,  close  any

currently  open transaction and then open the specified file as the new

transaction log.  If no name given, but a log file  was  already  open,

resume  logging  to that file.  If no filename was given and no log was

open,  the  server  should  open  a  log  with  a  default  name,  like

TRANSACTION.LOG.

-   Stop  logging transactions, but don't close the current transaction log

file.

C   Stop logging and close the current log.

S   Send the transaction log as a file.  If it was open, close it first.

Transaction logging is the recording of the progress of file transfers.  It

should contain entries showing the name of each file transferred, when  the

transfer  began  and  ended, whether it completed successfully, and if not,

why.

V   Set or Query a variable.  The command can be S or Q. The first argument  is

the variable name.  The second argument, if any, is the value.

S   Set  the  specified  variable  to the specified value.  If the value is

null, then undefine the variable.  If the  variable  is  null  then  do

nothing.   If the variable did not exist before, create it.  The server

should respond with an ACK if successful, and Error packet otherwise.

Q   Query the value of the named variable.  If  no  variable  is  supplied,

display  the  value  of all active variables.  The server responds with

either a short or long reply, as described above.  If a  queried  vari-

Optional Features                                                       Page 34

able does not exist, a null value is returned.

Variables are named by character strings, and have character string values,

which may be static or dynamic.  For instance, a server might have built-in

variables  like  "system  name"  which  never changes, or others like "mail

status" which, when queried, cause the server to check to see if  the  user

has any new mail.

8.2.7. Host Commands

Host  commands  are  conceptually  simple, but may be hard to implement on some

systems.  The C packet contains a text string in its data field which is simply

fed to the server's host system command processor; any output from the  proces-

sor  is  sent  back  to  the  user in KERMIT packets, as either a short or long

reply.

Implementation of this facility under UNIX, with its forking process  structure

and i/o redirection via pipes, is quite natural.  On other systems, it could be

virtually impossible.

8.2.8. Exchanging Parameters Before Server Commands

In  basic  KERMIT, the Send-Init exchange is always sufficient to configure the

two sides to each other.  During server operation,  on  the  other  hand,  some

transactions  may  not  begin  with a Send-Init packet.  For instance, when the

user sends an R packet to ask the server to send a  file,  the  server  chooses

what  block  check option to use.  Or if the user requests a directory listing,

the server does not know what packet length to use.

The solution to this problem is the "I" (Init-Info) packet.  It is exactly like

a Send-Init packet, and the ACK works the same way too.  However, receipt of an

I packet does not cause transition to file-send state.  The  I-packet  exchange

simply  allows  the  two  sides to set their parameters, in preparation for the

next transaction.

Servers should be able to receive and ACK "I" packets when  in  server  command

wait.  User KERMITs need not send "I" packets, however; in that case, the serv-

er  will  assume  all  the defaults for the user listed on page 25, or whatever

parameters have been set by other means (e.g. SET commands typed to the  server

before it was put in server mode).

User  Kermits  which send I packets should be prepared to receive and ignore an

Error packet in response.  This could happen if the server has not  implemented

I packets.

8.3. Alternate Block Check Types

There are two optional kinds of block checks:

Type 2

A  two-character  checksum based on the low order 12 bits of the arithmetic

sum of the characters in the packet (from the LEN field  through  the  last

data character, inclusive) as follows:

Optional Features                                                       Page 35

1              2

--------+--------------+-------------+

...data | char(b6-b11) | char(b0-b5) |

--------+--------------+-------------+

For  instance, if the 16-bit result is 154321 (octal), then the 2 character

block check would be "C1".

Type 3

Three-character 16-bit CRC-CCITT. The CRC calculation treats  the  data  it

operates  upon  as  a  string  of  bits with the low order bit of the first

character first and the high order bit of the last character last.  The in-

itial value of the CRC is taken as 0; the 16-bit CRC is the remainder after

16  12  5

dividing the data bit string by the polynomial X  +X  +X +1 (this  calcula-

tion  can  actually  be  done  a  character at a time, using a simple table

lookup algorithm).  The result is represented as three printable characters

at the end of the packet, as follows:

1               2              3

--------+---------------+--------------+-------------+

...data | char(b12-b15) | char(b6-b11) | char(b0-b5) |

--------+---------------+--------------+-------------+

For instance, if the 16-bit result is 154321 (octal), then the 3  character

block check would be "-C1".  The CRC technique chosen here agrees with many

hardware  implementations  (e.g. the VAX CRC instruction).  A useful refer-

ence on table-driven CRC  calculations  can  be  found  in  "Byte-wise  CRC

Calculations" by Aram Perez in IEEE MICRO, June 1983, p.40.

The single-character checksum has proven quite adequate in practice.  The other

options  can be used only if both sides agree to do so via Init packet (S or I)

exchange.  The 2 and 3 character block checks should only be  used  under  con-

ditions of severe line noise and packet corruption.

Since  type  2 and 3 block checks are optional, not all KERMITs can be expected

to understand them.  Therefore, during initial connection,  communication  must

begin  using the type 1 block check.  If type 2 or 3 block checks are agreed to

during the "I" or "S" packet exchange, the switch will  occur  only  after  the

Send-Init  has  been sent and ACK'd with a type 1 block check.  This means that

the first packet with a type 2 or 3 block check must always be an  "F"  or  "X"

packet.   Upon completion of a transaction, both sides must switch back to type

1 (to allow for the fact that neither side has any  way  of  knowing  when  the

other  side  has  been stopped and restarted).  The transaction is over after a

"B" or "E" packet has been sent and ACK'd, or after any error  that  terminates

the transaction prematurely or abnormally.

A  consequence of the foregoing rule is that if a type 2 or 3 block check is to

be used, a long reply sent by the  server  must  begin  with  a  Send-Init  (S)

packet,  even  if  an  I packet exchange had already occurred.  If type 1 block

checks are being used, the S packet can be skipped and the transfer  can  start

with an X or F packet.

A  server  that  has  completed a transaction and is awaiting a new command may

send out periodic NAKs for that command (packet 0).  Those NAKs must have  type

1 block checks.

Optional Features                                                       Page 36

The  use  of  alternate block check types can cause certain complications.  For

instance, if the server gets a horrible error (so bad that it doesn't even send

an error packet) and reverts to command wait, sending NAKs for packet 0 using a

type 1 block check, while a transfer using type 2 or  3  block  checks  was  in

progress, neither side will be able to read the other's packets.  Communication

can  also  grind to a halt if A sends a Send-Init requesting, say, type 3 block

checks, B ACKs the request, switches to type 3 and waits for the X or F  packet

with a type 3 block check, but the ACK was lost, so A resends the S packet with

a  type 1 block check.  Situations like this will ultimately resolve themselves

after the two sides retransmit up to their retry threshhold, but  can  be  rec-

tified earlier by the use of two heuristics:

- The  packet  reader  can  assume  that if the packet type is "S", the

block check type is 1.

- A NAK packet never has anything in its data field.    Therefore,  the

block  check type can always be deduced by the packet reader from the

length field of a NAK.  In fact, it is the value of the length  field

minus  2.   A NAK can therefore be thought of as a kind of "universal

synchronizer".

These heuristics tend violate the layered nature of  the  protocol,  since  the

packet  reader  should  normally  be  totally  unconcerned with the packet type

(which is of interest  to  the  application  level  which  invokes  the  packet

reader).    A  better design would have had each packet include an indicator of

the type of its own block check; this would have allowed the block  check  type

to be changed dynamically during a transaction to adapt to changing conditions.

But it's too late for that now...

8.4. Interrupting a File Transfer

This  section  describes  an  optional  feature of the KERMIT protocol to allow

graceful interruption of file transfer.  This feature is  unrelated  to  server

operation.

To interrupt sending a file, send an EOF ("Z") packet in place of the next data

packet,  including  a  "D" (for Discard) in the data field.  The recipient ACKs

the Z packet normally, but does not retain the file.  This does  not  interfere

with  older  Kermits on the receiving end; they will not inspect the data field

and will close the file normally.  The mechanism can be triggered by typing  an

interrupt  character  at  the  console  of  the  sending  KERMIT program.  If a

(wildcard) file group is being sent, it is possible to skip to the next file or

to terminate the entire batch; the protocol is the same in either case, but the

desired action could be selected by different interrupt characters, e.g. CTRL-X

to skip the current file, CTRL-Z to skip the rest of the batch.

To interrupt receiving a file, put an "X" in the data field of  an  ACK  for  a

data packet.  To interrupt receiving an entire file group, use a "Z".  The user

could  trigger  this mechanism by typing an interrupt character by typing, say,

CTRL-X and CTRL-Z, respectively, at the receiving KERMIT's console.   A  sender

that  was  aware of the new feature, upon finding one of these codes, would act

as described above, i.e. send a "Z" packet with a "D" code; a sender  that  did

not  implement this feature would simply ignore the codes and continue sending.

In this case, and if the user wanted the whole batch to be cancelled  (or  only

one  file was being sent), the receiving KERMIT program, after determining that

Optional Features                                                       Page 37

the  sender  had ignored the "X" or "Z" code, could send an Error (E) packet to

stop the transfer.

The sender may also choose to send a Z packet containing the  D  code  when  it

detects  that  the  file  it is sending cannot be sent correctly and completely

-- for instance, after sending some packets correctly, it  gets  an  i/o  error

reading the file.  Or, it notices that the "8th bit" of a file byte is set when

the file is being sent as a text file and no provision has been made for trans-

mitting the 8th bit.

8.5. Transmitting File Attributes

The  optional  Attributes  (A)  packet provides a mechanism for the sender of a

file to provide additional information about it.  This packet can  be  sent  if

the  receiver has indicated its ability to process it by setting the Attributes

bit in the capability mask.    If  both  sides  set  this  bit  in  the  Kermit

capability  mask, then the sender, after sending the filename in the "F" packet

and receiving an acknowledgement, may (but does not have to) send an "A" packet

to provide file attribute information.

Setting the Attributes bit in the capability mask does not indicate support for

any particular attributes, only that the receiver is prepared to accept the "A"

packet.

The attributes are given in the data field of the "A" packet.  The  data  field

consists  of  0 or more subfields, which may occur in any order.  Each subfield

is of the following form:

+-----------+--------------+------+

| ATTRIBUTE | char(LENGTH) | DATA |

+-----------+--------------+------+

where

ATTRIBUTE is a single printable character other than space,

LENGTH    is the length of the data characters (0 to  94),  with  32  added  to

produce a single printable character, and

DATA      is length characters worth of data, all printable characters.

No quoting or prefixing is done on any of this data.

More  than  one attribute packet may be sent.  The only requirement is that all

the A packets for a file must immediately follow its File header (or X) packet,

and precede the first Data packet.

There may be 93 different attributes, one for each of the  93  printable  ASCII

characters other than space.  These are assigned in ASCII order.

! (ASCII 33)

Length.    The  data  field  gives the length in K (1024) bytes, as a

printable decimal number, e.g. "!#109".  This will allow the receiver

to determine in advance whether there  is  sufficient  room  for  the

file, and/or how long the transfer will take.

Optional Features                                                       Page 38

" (ASCII 34)

Type.  The data field can contain some indicator of the nature of the

file.     Operands  are  enclosed  in  {braces},  optional  items  in

[brackets].

A[{xx}] ASCII text, containing no 8-bit quantities,  logical  records

(lines)  delimited by the (quoted) control character sequence

{xx}, represented here by its  printable  counterpart  (MJ  =

CRLF,  J  =  LF,  etc).   For instance AMJ means that the ap-

pearance of #M#J (the normal prefixed  CRLF  sequence)  in  a

file  data packet indicates the end of a record, assuming the

current control prefix is "#".  If {xx} is omitted,  MJ  will

be assumed.

B[{xx}] Binary.  {xx} indicates in what manner the file is binary:

8   (default)  The  file  is a sequence of 8-bit bytes, which

must be saved as is.  The 8th bit may be sent "bare",  or

prefixed  according  to  the  Send-Init negotiation about

8th-bit prefixing.

36  The file is a PDP-10 format binary file,  in  which  five

7-bit  bytes are fit into one 36-bit word, with the final

bit of each word being represented as the "parity bit" of

every 5th character (perhaps prefixed).

D{x}    Moved from here to FORMAT attribute

F{x}    Moved from here to FORMAT attribute

I[{x}]  Image.  The file is being sent exactly as it  is  represented

on  the  system  of  origin.    For use between like systems.

There are {x} usable bits per  character,  before  prefixing.

For  instance,  to  send binary data from a system with 9-bit

bytes, it might be convenient to send three 6-bit  characters

for every two 9-bit bytes.  Default {x} is 8.

# (ASCII 35)

Creation  Date,  expressed as "[yy]yymmdd[ hh:mm[:ss]]" (ISO standard

julian format), e.g. 831009 23:59.  The time is optional;  if  given,

it should be in 24-hour format, and the seconds may be omitted, and a

single space should separate the time from the date.

$ (ASCII 36)

Creator's ID, expressed as a character string of the given length.

% (ASCII 37)

Account to charge the file to, character string.

& (ASCII 38)

Area in which to store the file, character string.

' (ASCII 39)

Password for above, character string.

( (ASCII 40)

Optional Features                                                       Page 39

Block  Size.   The file has, or is to be stored with, the given block

size.

) (ASCII 41)

Access:

N   New, the normal case -- create a new file of the given name.

S   Supersede (overwrite) any file of the same name.

A   Append to file of the given name.

* (ASCII 42)

Encoding:

A   ASCII, normal ASCII encoding with any necessary prefixing, etc.

H   Hexidecimal "nibble" encoding.

E   EBCDIC (sent as if it were a binary file).

X   Encrypted.

Q{x}

Huffman Encoded for compression.  First x bytes of the  file  are

the key.

# (ASCII 43)

Disposition  (operands  are specified in the syntax of the receiver's

host system):

M{user(s)}      Send the file as Mail to the specified user(s).

O{destination}  Send the file as  a  lOng  terminal  message  to  the

specified destination (terminal, job, or user).

S[{options}]    Submit  the  file  as a batch job, with any specified

options.

P[{options}]    Print  the  file  on  a  system  printer,  with   any

specified  options,  which  may  specify a particular

printer, forms, etc.

T               Type the file on the screen.

L[{aaa}]        Load the file into memory at the  given  address,  if

any.

X[{aaa}]        Load  the  file  into memory at the given address and

eXecute it.

A               Archive the file; save the file together with the at-

tribute packets that preceded it, so that it  can  be

sent  back  to  the system of origin with all its at-

tributes intact.  A file stored in this way should be

specially marked so that the  KERMIT  that  sends  it

back will recognize the attribute information as dis-

tinct from the file data.

, (ASCII 44)

Protection.    Protection  code  for  the  file, in the syntax of the

receiver's host file system.  With no operand, store according to the

Optional Features                                                       Page 40

system's default protection for the destination area.

- (ASCII 45)

Protection.    Protection  code  for  the  file  with  respect to the

"public" or "world", expressed generically in a 6-bit quantity  (made

printable by char()), in which the bits have the following meaning:

b0: Read Access

b1: Write Access

b2: Execute Access

b3: Append Access

b4: Delete Access

b5: Directory Listing

A  one  in the bit position means allow the corresponding type of ac-

cess, a zero means prohibit it.  For example, the letter "E" in  this

field  would  allow  read,  execute,  and  directory  listing  access

(unchar("E") = 69-32 = 37 = 100101 binary).

. (ASCII 46)

Machine and operating system of origin.  This is useful  in  conjunc-

tion  with the archive disposition attribute.  It allows a file, once

archived, to be transferred among different types of systems, retain-

ing its archive status, until it finds its way to a machine with  the

right  characteristics  to de-archive it.  The systems are denoted by

codes; the first character is the major system designator, the second

designates the specific model or operating system.  A third character

may be added to make further  distinctions,  for  instance  operating

system version.  The systems below do not form a complete collection;

many more can and probably will be added.

A   Apple microcomputers

1   Apple II, DOS

2   Apple III

3   Macintosh

4   Lisa

B   Sperry (Univac) mainframes

1   1100 series, EXEC

C   CDC mainframes

1   Cyber series, NOS

D   DEC Systems

1   DECsystem-10/20, TOPS-10

2   DECsystem-10/20, TOPS-20

3   DECsystem-10/20, TENEX

4   DECsystem-10/20, ITS

5   DECsystem-10/20, WAITS

6   DECsystem-10/20, MAXC

7   VAX-11, VMS

8   PDP-11, RSX-11

Optional Features                                                       Page 41

9   PDP-11, IAS

A   PDP-11, RSTS/E

B   PDP-11, RT-11

C   Professional-300, P/OS

D   Word Processor (WPS or DECmate), WPS

D   Honeywell mainframes

1   MULTICS systems

2   DPS series, running CP-6

F   Data General machines

1   RDOS

2   AOS

G   PR1ME machines, PRIMOS

H   Hewlett-Packard machines

1   HP-1000, RTE

2   HP-3000, MPE

I   IBM 370-series and compatible mainframes

1   VM/CMS

2   MVS/TSO

3   DOS

4   MUSIC

5   GUTS

6   MTS

J   Tandy microcomputers, TRSDOS

K   Atari micros, DOS

L-T Reserved

U   Portable Operating or File Systems

1   UNIX

2   Software Tools

3   CP/M-80

4   CP/M-86

5   CP/M-68K

6   MP/M

7   Concurrent CP/M

8   MS-DOS

9   UCSD p-System

A   MUMPS

/ (ASCII 47)

Format of the data within the packets.

A{xx}           Variable  length delimited records, terminated by the

character sequence {xx}, where xx is a string of  one

Optional Features                                                       Page 42

or more control characters, represented here by their

unprefixed  printable  equivalents,  e.g. MJ for ^M^J

(CRLF).

D{x}            Variable length undelimited records.    Each  logical

record  begins  with  an  {x}-character ASCII decimal

length field (similar to ANSI tape format "D").   For

example,  "D$"  would indicate 4-digit length fields,

like "0132".

F{xxxx}         Fixed-length  undelimited  records.    Each   logical

record is {xxxx} bytes long.

R{x}            For record-oriented transfers, to be used in combina-

tion  with  one  of  the  formats  given above.  Each

record begins (in the case of  D  format,  after  the

length field) with an x-character long position field

indicating the byte position within the file at which

this record is to be stored.

M{x}            For record-oriented transfers, to be used in combina-

tion  with  one  of the formats given above.  Maximum

record length for a variable-length record.

0 (ASCII 48)

Special system-dependent parameters for storing the file on the  sys-

tem of origin, for specification of exotic attributes not covered ex-

plicitly by any of the KERMIT attribute descriptors.  These are given

as  a  character  string  in the system's own language, for example a

list of DCB parameters in IBM Job Control Language.

1-@ (ASCII 49-64)

Reserved

Other attributes can be imagined, and can be added later if needed.    However,

two important points should be noted:

- The  receiver may have absolutely no way of honoring, or even record-

ing, a given attribute.  For instance, CP/M-80 has no slot for  crea-

tion  date  or  creator's ID in its FCB; the DEC-20 has no concept of

block size, etc.

- The sender may have no way of determining the correct values  of  any

of  the  attributes.  This is particularly true when sending files of

foreign origin.

The "A" packet mechanism only provides a way to send certain information  about

a  file to the receiver, with no provision or guarantee about what the receiver

may do with it.  That information may be  obtained  directly  from  the  file's

directory entry (FCB, FDB, ...), or specified via user command.

The  ACK  to  the  "A"  packet  may in turn have information in its data field.

However, no complicated negotiations about file attributes may take  place,  so

the  net  result  is that the receiver may either refuse the file or accept it.

The receiver may reply to the "A" packet with any of the following codes in the

data field of the ACK packet:

Optional Features                                                       Page 43

<null>  (empty data field) I accept the file, go ahead and send it.

N[{xxx}]

I  refuse  the  file  as specified, don't send it; {xxx} is a string of

zero or more of the attribute characters listed above, to specify  what

attributes I object to (e.g. "!" means it's too long, "&" means I don't

have write access to the specified area, etc).

Y[{xxx}]

I  agree to receive the file, but I cannot honor attributes {xxx}, so I

will store the file according to my own defaults.

Y       (degenerate case of Y{xxx}, equivalent to <null>, above)

How the receiver actually replies is an implementation  decision.    A  NAK  in

response  to the "A" packet means, of course, that the receiver did not receive

the "A" correctly, not that it refuses to receive the file.

8.6. Advanced KERMIT Protocol State Table

The simple table presented previously is sufficient  for  a  basic  KERMIT  im-

plementation.  The following is a state table for the full Kermit protocol, in-

cluding  both server mode and sending commands to a server Kermit.  It does not

include handling of the file attributes packet (A).   Note  that  states  whose

names  start  with "Send" always send a packet each time they are entered (even

when the previous state was the same).  States whose name  starts  with  "Rec",

always  wait for a packet to be received (up to the timeout value), and process

the received packet.  States whose names do not include either send or  receive

do  not  process  packets  directly.  These are states which perform some local

operation and then change to another state.

The initial state is determined by the user's  command.    A  "server"  command

enters  at Rec_Server_Idle.  A "send" command enters at Send_Init.  A "receive"

command (the old non-server version, not a "get" command) enters  at  Rec_Init.

Any  generic command, the "get" command, and the "host" command enter at either

Send_Server_Init or Send_Gen_Cmd, depending upon the expected response.

Under "Rec'd Msg", the packet type of the incoming message is  shown,  followed

by the packet number in parentheses; (n) means the current packet number, (n-1)

and  (n+1)  mean  the  previous  and next packet numbers (modulo 64), (0) means

packet number zero. Following the packet number may be slash and a letter,  in-

dicating  some special signal in the data field.  For instance Z(n)/D indicates

a Z (EOF) packet, sequence number n, with a "D" in the data field.

Under "Action", "r+" means that the retry count  is  incremented  and  compared

with  a  threshhold; if the threshhold is exceeded, an Error packet is sent and

the state changes to "Abort".  "n+" means that  the  packet  number  is  incre-

mented, modulo 64, and the retry count, r, is set back to zero.

Optional Features                                                       Page 44

State   Rec'd Msg       Action                  Next state

Rec_Server_Idle -- Server idle, waiting for a message

Set n and r to 0

I(0)            Send ACK                Rec_Server_Idle

S(0)            Process params,

ACK with params, n+    Rec_File

R(0)            Save file name          Send_Init

K, C or G(0)    Short reply:

ACK(0)/reply           Rec_Server_Idle

Long reply:

init needed            Send_Init

init not needed, n+    Open_File

Timeout         Send NAK(0)             Rec_Server_Idle

Other           Error                   Rec_Server_Idle

Rec_Init -- Entry point for non-server RECEIVE command

Set n and r to 0

S(0)            Process params, send

ACK with params, n+    Rec_File

Timeout         Send NAK(0), r+         Rec_Init

Other           NAK                     Abort

Rec_File -- Look for a file header or EOT message

F(n)            Open file, ACK, n+      Rec_Data

X(n)            Prepare to type on

screen, ACK, n+        Rec_Data

B(n)            ACK                     Complete

S(n-1)          ACK with params, r+     Rec_File

Z(n-1)          ACK, r+                 Rec_File

Timeout         NAK, r+                 Rec_File

Other           NAK                     Abort

Rec_Data -- Receive data up to end of file

D(n)            Store data, ACK, n+;

If interruption wanted

include X or Z in ACK  Rec_Data

D(n-1)          Send ACK, r+            Rec-Data

Z(n)            Close file, ACK, n+     Rec_File

Z(n)/D          Discard file, ACK, n+   Rec_File

F(n-1)          Send ACK, r+            Rec_Data

X(n-1)          Send ACK, r+            Rec_Data

Timeout         Send NAK, r+            Rec_Data

Other           Send E                  Abort

Optional Features                                                       Page 45

Send_Init -- Also entry for SEND command

Set n and r to 0, send S(0) with parameters

Y(0)            Process params, n+      Open_File

N, Timeout      r+                      Send_Init

Other           r+                      Send_Init

Open_File -- Open file or set up text to send

Send_File

Send_File -- Send file or text header

Send F or X(n)

Y(n), N(n+1)    Get first buffer of     Send_Data or Send_Eof if

data, n+                empty file or text

N, Timeout      r+                      Send_File

Other                                   Abort

Send_Data -- Send contents of file or textual information

Send D(n) with current buffer

Y(n), N(n+1)    n+, Get next buffer     Send_Data or Send_Eof if

at end of file or text

Y(n)/X or Z     n+                      Send_Eof

N, Timeout      r+                      Send_Data

Other                                   Abort

Send_Eof -- Send end of file indicator

Send Z(n); if interrupting send Z(n)/D

Y(n), N(n+1)    Open next file, n+      Send_File if more, or

Send_Break if no more

or if interrupt "Z".

N, Timeout      r+                      Send_Eof

Other                                   Abort

Send_Break -- End of Transaction

Send B(n)

Y(n), N(0)                              Complete

N(n), Timeout                           Send_Break

Other                                   Abort

Optional Features                                                       Page 46

Send_Server_Init - Entry for Server commands which expect large response.

Send I(0) with parameters

Y(0)            Process params          Send_Gen_Cmd

N, Timeout      r+                      Send_Server_Init

E               Use default params      Send_Gen_Cmd

Other                                   Abort

Send_Gen_Cmd - Entry for Server commands which expect short response (ACK)

Send G, R or C(0)

S(0)            Process params,

ACK with params, n+    Rec_File

X(1)            Setup to type on

terminal, n+           Rec_Data

Y(0)            Type data on TTY        Complete

N, Timeout      r+                      Send_Gen_Cmd

Other                                   Abort

Complete -- Successful Completion of Transaction

Set n and r to 0;

If server, reset params, enter Rec_Server_Idle

otherwise exit

Abort -- Premature Termination of Transaction

Reset any open file, set n and r to 0

If server, reset params, enter Rec_Server_Idle

otherwise exit

Exit, Logout states

Exit or Logout

Note that the generic commands determine the next state as follows:

1. If  the  command  is  not supported, an error packet is sent and the

next state is "Abort".

2. If the command generates a response which can be fit into  the  data

portion  of  an  ACK,  an  ACK  is  sent  with  the  text (quoted as

necessary) in the data portion.

3. If the command generates a large response or must send a file, noth-

ing is sent from the Rec_Server_Idle state, and the  next  state  is

either  Send_Init  (if either no I message was received or if alter-

Optional Features                                                       Page 47

nate  block  check types are to be used), or Open_File (if an I mes-

sage was received and the single character  block  check  is  to  be

used).

4. If  the  command  is  Logout,  an  ACK  is sent and the new state is

Logout.

5. If the command is Exit, an ACK is sent and the new state is Exit.

KERMIT Commands                                                         Page 48

9. KERMIT Commands

The  following  list  of KERMIT commands and terms is suggested.  It is not in-

tended to recommend a particular style of command parsing, only  to  promote  a

consistent vocabulary, both in documentation and in choosing the names for com-

mands.

9.1. Basic Commands

SEND    This verb tells a Kermit program to send one or more files from its own

file structure.

RECEIVE This  verb  should tell a Kermit program to expect one or more files to

arrive.

GET     This verb should tell a user Kermit to send one or more  files.    Some

Kermit  implementations  have separate RECEIVE and GET commands; others

use RECEIVE for both purposes, which creates confusion.

Since it can be useful, even necessary, to specify different names  for  source

and destination files, these commands should take operands as follows (optional

operands in [brackets]):

SEND local-source-filespec [remote-destination-filespec]

If  the destination file specification is included, this will go in the

file header packet, instead of the file's local name.

RECEIVE [local-destination-filespec]

If the destination filespec is given, the incoming file will be  stored

under that name, rather than the one in the file header pakcet.

GET remote-source-filespec [local-destination-filespec]

If  the destination filespec is given, the incoming file will be stored

under that name, rather than the one in the file header packet.

If a file group is being sent or received, alternate names should not be used.

9.2. Program Management Commands

EXIT    Leave the KERMIT program, doing  whatever  cleaning  up  must  be  done

-- deassigning of devices, closing of files, etc.

QUIT    Leave  the  KERMIT  program without cleaning up, in such a manner as to

allow further manipulation of the files and devices.

PUSH    Preserve the current KERMIT environment and enter  the  system  command

processor.

TAKE    Read and execute KERMIT program commands from a local file.

LOG     Specify  a  log for file transfer transactions, or for terminal session

loggin.

KERMIT Commands                                                         Page 49

9.3. Terminal Emulation Commands

CONNECT This  verb,  valid  only  for a local Kermit, means to go into terminal

emulation mode; present the illusion of being directly connected  as  a

terminal  to the remote system.  Provide an "escape character" to allow

the user to "get back" to the local system.  The escape character, when

typed, should take a single-character argument; the following are  sug-

gested:

0   (zero) Transmit a NUL

B   Transmit a BREAK

C   Close the connection, return to local KERMIT command level

P   Push to system command processor

Q   Quit logging (if logging is being done)

R   Resume logging

S   Show status of connection

?   Show the available arguments to the escape character

(a  second  copy  of  the  escape  character):  Transmit the escape

character itself

Lower case equivalents should be accepted.  If any invalid argument  is

typed, issue a beep.

Also see the SET command.

9.4. Special User-Mode Commands

These commands are used only by Users of Servers.

BYE     This  command  sends  a message to the remote server to log itself out,

and upon successful completion, terminate the local Kermit program.

FINISH  This command causes the remote server to shut  itself  down  gracefully

without logging out its job, leaving the local KERMIT at KERMIT command

level, allowing the user to re-CONNECT to the remote job.

9.5. Commands Whose Object Should Be Specified

Some  Kermit implementations include various local file management services and

commands to invoke them.  For instance, an implementation might  have  commands

to  let  you  get  directory  listings, delete files, switch disks, and inquire

about free disk space without having to exit and restart the program.   In  ad-

dition,  remote  servers may also provide such services.  A user Kermit must be

able to distinguish between commands aimed at its own system and those aimed at

the remote one.  When any confusion is possible, such a command may be prefixed

by one of the following "object prefixes":

REMOTE  Ask the remote Kermit server to provide this service.

LOCAL   Perform the service locally.

If the "object prefix" is omitted, the command should be executed locally.  The

services include:

KERMIT Commands                                                         Page 50

LOGIN   This  should  be  used  in its timesharing sense, to create an identity

("job", "session", "access", "account") on the system.

LOGOUT  To terminate a session that was initiated by LOGIN.

COPY    Make a new copy of the specified file with the specified name.

CWD     Change Working Directory.  This is ugly, but more  natural  verbs  like

CONNECT and ATTACH are too imprecise.  CWD is the ARPAnet file transfer

standard command to invoke this function.

DIRECTORY

Provide  a  list  of  the  names, and possibly other attributes, of the

files in the current working directory (or the specified directory).

DELETE  Delete the specified files.

ERASE   This could be a synomym for DELETE, since its meaning is clear.

(It doesn't seem wise to include UNDELETE  or  UNERASE  in  the

standard  list; most systems don't support such a function, and

users' expectations should not be toyed with...)

KERMIT  Send a command to the remote KERMIT server in its own interactive  com-

mand syntax.

RENAME  Change the name of the specified file.

TYPE    Display the contents of the specified file(s) at the terminal.

SPACE   Tell how much space is used and available for storing files in the cur-

rent working directory (or the specified directory).

SUBMIT  Submit the specified file(s) for background (batch) processing.

PRINT   Print the specified file(s) on a printer.

MOUNT   Request a mount of the specified tape, disk, or other removable storage

medium.

WHO     Show  who  is  logged in (e.g. to a timesharing system), or give infor-

mation about a specified user or network host.

MAIL    Send electronic mail to the specified user(s).

MESSAGE Send a terminal message (on a network or timesharing system).

HELP    Give brief information about how to use KERMIT.

SET     Set various parameters relating to debugging, transmission, file  mode,

and so forth.

SHOW    Display settings of SET parameters, capabilities in force, etc.

STATISTICS

Give information about the performance of the most recent file transfer

KERMIT Commands                                                         Page 51

-- elapsed time, effective baud rate, various counts, etc.

HOST    Pass  the  given command string to the specified (i.e. remote or local)

host for execution in its own command language.

LOGGING Open or close a remote transaction or debugging log.

9.6. The SET Command

A SET command should be provided to allow the user to tailor  a  connection  to

the  peculiarities  of the communication path, the local or remote file system,

etc.  Here are some parameters that should be SET-able:

BLOCK-CHECK

Specify the type of block check to be used: single character  checksum,

two-character checksum, 3-character CRC.

DEBUGGING

Display  or  log  the  packet  traffic,  packet numbers, and/or program

states.  Useful for debugging new versions of  KERMIT,  novel  combina-

tions of KERMIT programs, etc.

DELAY   How  many seconds a remote (non-server) KERMIT should wait before send-

ing the Send-Init packet, to give the user time to escape back  to  the

local KERMIT and type a RECEIVE command.

DUPLEX  For terminal emulation, specify FULL or HALF duplex echoing.

EIGHT-BIT-PREFIXING

Specify  that "8th bit" prefixing must be done; normally it will not be

done.

END-OF-LINE

Specify any line terminator that must be used after a packet.

ESCAPE  Specify the escape character for terminal emulation.

FILE attributes

Almost any of the attributes listed above  in  the  Attributes  section

(8.5).    The most common need is to tell the KERMIT program whether an

incoming or outbound file is text or binary.

FLOW-CONTROL

Specify the flow control mechanism for  the  line,  such  as  XON/XOFF,

DTR/CTS, etc.  Allow flow control to be turned off as well as on.  Flow

control is done only on full-duplex connections.

HANDSHAKE

Specify  any  line-access  negotiation  that  must be used or simulated

during file transfer.  For instance, a half duplex  system  will  often

need to "turn the line around" after sending a packet, in order to give

you  permission  to reply.  A common handshake is XON (^Q); the current

user of the line transmits an XON when done transmitting data.

LINE    Specify the line or device designator for the connection.  This is  for

KERMIT Commands                                                         Page 52

use  in  a  KERMIT program that can run in either remote or local mode;

the default line is the controlling terminal  (for  remote  operation).

If an external device is used, local operation is presumed.

LOG     Specify  a local file in which to keep a log of the transaction.  There

may be logs for debugging purposes (packet traffic, state  transitions,

etc)  and  for auditing purposes (to record the name and disposition of

each file transferred).

MARKER  Change the start-of-packet marker from the default of SOH  (CTRL-A)  to

some  other control character, in case one or both systems has problems

using CTRL-A for this purpose.

PACKET-LENGTH

The maximum length for a packet.  This should normally be no less  than

30  or  40,  and can never be greater than 96.  Short packets can be an

advantage on noisy lines; they reduce the  probabily  of  a  particular

packet  being  corrupted,  as  well as the retransmission overhead when

corruption does occur.

PADDING The number of padding  characters  that  should  be  sent  before  each

packet, and what the padding character should be.  Rarely necessary.

PARITY  Specify  the parity (ODD, EVEN, MARK, SPACE, NONE) of the physical con-

nection.  If other than none, the "8th bit" cannot be used to  transmit

data and must not be used by either side in block check computation.

PAUSE   How  many  seconds to pause after receiving a packet before sending the

next packet.  Normally 0, but when a system communication processor  or

front  end  has  trouble keeping up with the traffic, a short pause be-

tween packets may allow it to recover its  wits;  hopefully,  something

under a second will suffice.

PREFIX  Change  the default prefix for control characters, 8-bit characters, or

repeated quantities.

PROMPT  Change the program's prompt.  This is useful when  running  KERMIT  be-

tween  two  systems  whose  prompt  is the same, to eliminate confusion

about which KERMIT you are talking to.

REPEAT-COUNT-PROCESSING

Change the default for repeat count processing.  Normally, it  will  be

done if both KERMIT programs agree to do it.

RETRY   The  maximum number of times to attempt to send or receive a packet be-

fore giving up.  The normal number is about 5, but the user  should  be

able  to  adjust it according to the condition of the line, the load on

the systems, etc.

TIMEOUT Specify the length of the timer to set when waiting for a packet to ar-

rive.

KERMIT Commands                                                         Page 53

9.7. Macros, the DEFINE Command

In  addition  to the individual set commands, a "macro" facility is recommended

to allow users to combine the characteristics of specific systems into a single

SET option.  For example:

DEFINE IBM = PARITY ODD, DUPLEX HALF, HANDSHAKE XON

DEFINE UNIX = PARITY NONE, DUPLEX FULL

DEFINE TELENET = PARITY MARK

This could be done by providing a fancy runtime parser for commands  like  this

(which  could be automatically TAKEn from the user's KERMIT initialization file

upon program startup), or simply hardwired into the SET command table.

With these definitions in place, the user would simply  type  "SET  IBM",  "SET

UNIX",  and so forth, to set up the program to communication to the remote sys-

tem.

Terminal Emulation                                                      Page 54

10. Terminal Emulation

The local system must be able to act as a terminal so that the user can connect

to the remote system, log in, and start up the remote KERMIT.

Terminal  emulation should be provided by any KERMIT program that runs locally,

so that the user need not exit and restart the local KERMIT program in order to

switch between terminal and protocol operation.  On smaller  systems,  this  is

particularly important for various reasons -- restarting the program and typing

in  all  the  necessary SET commands is too inconvenient and time-consuming; in

some micros, switching in and out of terminal emulation may  cause  carrier  to

drop, etc.

Only bare-bones terminal emulation need be supplied by KERMIT; there is no need

to  emulate  any  particular  kind of "smart" terminal.  Simple "dumb" terminal

emulation is sufficient to do the job.  Emulation of fancier terminals is  nice

to  have, however, to take advantage of the remote system's editing and display

capabilities.  In some cases, microcomputer firmware will take  care  of  this.

To build emulation for a particular type of terminal into the program, you must

interpret and act upon escape sequences as they arrive at the port.

No  error  checking  is  done  during  terminal  emulation.  It is "outside the

protocol"; characters go back and forth "bare".  In this sense, terminal emula-

tion through KERMIT is no better than actually using a real terminal.

Some KERMIT implementations may allow logging of the terminal emulation session

to a local file.  Such a facility allows "capture" of  remote  typescripts  and

files,  again  with  no  error  checking  or correction.  When this facility is

provided, it is also desirable to have a convenient way of "toggling" the  log-

ging on and off.

If  the  local  system does not provide system- or firmware-level flow control,

like XON/XOFF, the terminal emulation program should attempt  to  simulate  it,

especially if logging is being done.

The terminal emulation facility should be able to handle either remote or local

echoing (full or half duplex), any required handshake, and it should be able to

transmit any parity required by the remote side or the communication medium.

A  terminal emulator works by continuously sampling both console input from the

local terminal and input from the communication line.  Simple input and  output

functions  will not suffice, however, since if you ask for input from a certain

device and there is none available, you will generally block until  input  does

become  available,  during  which time you will be missing input from the other

device.  Thus you must have a way  to  bounce  back  and  forth  regardless  of

whether input is available.  Several mechanisms are commonly used:

- Continuously jump back and forth between the port status register and

the  console  status  register,  checking  the  status bits for input

available.  This is only  practical  on  single-user,  single-process

systems, where the CPU has nothing else to do.

- Issue an ordinary blocking input request for the port, but enable in-

terrupts on console input, or vice versa.

- Handle port input in one process and console input in another, paral-

Terminal Emulation                                                      Page 55

lel process.  The UNIX KERMIT program listed in this manual uses this

method.

Any input at the port should be displayed immediately on the screen.  Any input

from the console should be output immediately to the port.  In addition, if the

connection is half duplex, console input should also be sent immediately to the

screen.

The  terminal  emulation  code must examine each console character to determine

whether it is the "escape character".  If so, it should take the next character

as a special command, which it executes.  These commands are  described  above,

in section 9.3.

The terminal emulator should be able to send every ASCII character, NUL through

DEL,  and  it  should  also  be able to transmit a BREAK signal (BREAK is not a

character, but an "escape" from ASCII transmission in which a 0 is put  on  the

line  for  about  a  quarter  of a second, regardless of the baud rate, with no

framing bits).  BREAK is important when  communicating  with  various  systems,

such as IBM mainframes.

Finally, it is sometimes necessary to perform certain transformations on the CR

character  that is normally typed to end a line of input.  Some systems use LF,

EOT, or other characters for this function.  To complicate matters, intervening

communications equipment (particularly the public packet-switched networks) may

have their own independent requirements.  Thus if using KERMIT  to  communicate

over,  say,  TRANSPAC  with  a  system  that uses LF for end-of-line, it may be

necessary to transform CR into LFCR (linefeed first -- the CR tells the network

to send the packet, which will contain the LF, and the host  uses  the  LF  for

termination).  The user should be provided with a mechanism for specifying this

transformation, a command like "SET CR sequence".

Writing a KERMIT Program                                                Page 56

11. Writing a KERMIT Program

Before writing a new implementation of KERMIT or modifying an old one, first be

sure  to  contact the KERMIT Distribution center at Columbia University to make

sure that you're not duplicating someone else's effort, and that you  have  all

the  latest material to work from.  If you do write or significantly modify (or

document) a KERMIT program, please send it back to Columbia so that it  can  be

included  in  the  standard KERMIT distribution and others can benifit from it.

It is only through this kind of sharing that KERMIT has grown from  its  modest

beginnings to its present scale.

The following sections provide some hints on KERMIT programming.

11.1. Program Organization

A  basic  KERMIT  implementation  can  usually be written as a relatively small

program, self-contained in a single source file.  However, it is often the case

that a program written to run on one system will be adapted  to  run  on  other

systems  as  well.   In that case, it is best to avoid having totally divergent

sources, because when new features are added to (or bugs fixed in) the  system-

independent parts of the program -- i.e. to the protocol itself -- only one im-

plementation will reap the benefits initially, and the other will require pain-

ful, error-prone "retrofitting" to bring it up to the same level.

Thus,  if  there  is any chance that a KERMIT program will run on more than one

machine, or under more than one operating system, or support more than one kind

of port or modem, etc, it is desirable to isolate the system-dependent parts in

a way that makes the common parts usable by the various implementations.  There

are several approaches:

1. Runtime support.  If possible, the program can inspect the  hardware

or  inquire  of  the system about relevant parameters, and configure

itself dynamically at startup time.  This is hardly ever possible.

2. Conditional compilation (or assembly).  If the number of systems  or

options  to  be supported is small, the system dependent code can be

enclosed in conditional compilation brackets  (like  IF  IBMPC  ....

ENDIF).  An example is provided in UNIX KERMIT listing included with

this  manual.    However, as the number of system dependencies to be

supported  grows,  this  method  becomes  unwieldy  and  error-prone

--  installing  support for system X tends to break the pre-existing

support for system Y.

3. Modular composition.  When there is a potentially  large  number  of

options  a  program  should  support,  it  should  be broken up into

separate modules (source  files),  with  clearly  specified,  simple

calling conventions.  This allows people with new options to provide

their  own  support for them in an easy way, without endangering any

existing support.  Suggested modules for a KERMIT program are:

- System-Indendent protocol  handling:  state  switching,  packet

formation, encoding (prefixing) and decoding, etc.

- User Interface: the command parser.  Putting this in a separate

module  allows  plug-in  of  command parsers to suit the user's

Writing a KERMIT Program                                                Page 57

taste,  to mimic the style of the host system command parser or

some popular application, etc.

- Screen i/o: This module would contain the screen control codes,

cursor positioning routines, etc.

- Port i/o: Allows support of various port hardware.  This module

can define the port status register location, the status  bits,

and so forth, and can implement the functions to read and write

characters at the port.

- Modem   control:   This   module  would  support  any  kind  of

"intelligent" modem, which is not simply a  transparent  exten-

sion  of  the communications port.  Such modems may accept spe-

cial commands to perform functions like dialing out,  redialing

a  recent  number,  hanging  up, etc., and may need special in-

itialization (for instance, setting modem signals like DTR).

- Console input: This module would supply  the  function  to  get

characters  from  the  console;  it would know about the status

register  locations  and  bits,  interrupt  structure,  key-to-

character   mappings,   etc.,  and  could  also  implement  key

redefinitions, keystroke macros,  programmable  function  keys,

expanded control and meta functions, etc.

- Terminal  Emulation:  This  module  would  interpret escape se-

quences in the incoming character  stream  (obtained  from  the

port i/o module) for the particular type of terminal being emu-

lated  and  interpret  them by making appropriate calls the the

screen i/o module, and it would send user typein (obtained from

the console input module) out the serial port (again using  the

port  i/o module).  Ideally, this module could be replacable by

other modules to emulate different  kinds  of  terminals  (e.g.

ANSI, VT52, ADM3A, etc).

- File i/o: This module contains all the knowledge about the host

system's  file  structure; how to open and close files, perform

"get next file" operations, read and write files, determine and

set their attributes, detect the end of a file, and  so  forth,

and  provides  the  functions,  including  buffering,  to get a

character from a file and put a character  to  a  file.    This

module  may  also  provide  file  management services for local

files -- directory listings, deleting, renaming,  copying,  and

so forth.

- Definitions  and  Data: Separate modules might also be kept for

compile-time parameter definitions and for global runtime data.

Writing a KERMIT Program                                                Page 58

11.2. Programming Language

The  language  to  be used in writing a KERMIT program is more than a matter of

taste.  The primary consideration is that the language  provide  the  necessary

functionality and speed.  For instance, a microcomputer implementation of BASIC

may  not  allow  the  kind of low-level access to device registers needed to do

terminal emulation, or to detect console input during file transfer, or even if

it can do these things, it might not be able to run fast enough  do  drive  the

communication line at the desired baud rate.

The  second  consideration in choosing a language is portability.  This is used

in two senses: (1) whether the language is in the  public  domain  (or,  equiv-

alently,  provided  "free"  as part of the basic system), and (2) whether it is

well standardized and in wide use on a variety of systems.  A language that  is

portable in both senses is to be preferred.

Whatever  programming  language  is selected, it is important that all lines in

the program source be kept to 80 characters or less (after expansion of  tabs).

This  is because KERMIT material must often be shipped over RJE and other card-

format communication links.

In addition, it is important that the names of all files used in  creating  and

supporting  a  particular  KERMIT  implementation be (possibly a subset) of the

form NAME.TYPE, where NAME is limited to six characters, and TYPE is limited to

three, and where the NAME of each file begin with a common  2  or  3  character

prefix.    This is so that all related files will be grouped together in an al-

phabetic directory listing, and so when all of the hundreds of  KERMIT  related

files  are  placed together on a tape, all names will be both legal and unique,

especially on systems (like PDP-11 operating  systems)  with  restrictive  file

naming conventions.

11.3. Documentation

A  new  KERMIT program should be thoroughly documented; one of the hallmarks of

KERMIT is its documentation.  The documentation should  be  at  both  the  user

level  (how  to  use  the  program,  what the commands are, etc, similar to the

documentation presently found in the Kermit Users  Guide),  and  the  implemen-

tation  level  (describe system dependencies, give pointers for adapting to new

systems, and so forth).    In  addition,  programs  themselves  should  contain

copious  commentary.   Like program source, documentation should be kept within

80-character lines.

If possible, a section for the implementation should be written for the  KERMIT

User  Guide using the UNILOGIC Scribe formatting language (subsets of which are

also to be found in some microcomputer text processing software such as Perfect

Writer or Final Word), using  the  same  general  conventions  as  the  existic

Scribe-format implementation sections.

KERMIT programs should also contain a revision history, in which each change is

briefly  explained,  assigned an "edit number", and the programmer and site are

identified.  The lines or sections comprising the edit should  be  marked  with

the corresponding edit number, and the KERMIT program, upon startup, should an-

nounce its version and edit numbers, so that when users complain of problems we

will know what version of the program is in question.

Writing a KERMIT Program                                                Page 59

The version number changes when the functionality has been changed sufficiently

to  require  major revisions of user documentation.  The edit number should in-

crease (monotonically, irrespective of version number)  whenever  a  change  is

made  to  the program.  The edit numbers are very important for program manage-

ment; after shipping out a version of, say, CP/M KERMIT-80,  we  often  receive

many copies of it, each containing its own set of changes, which we must recon-

cile in some manner.  Edit numbers help a great deal here.

11.4. Bootstrapping

Finally,  a  bootstrap  procedure should be provided.  KERMIT is generally dis-

tributed on magnetic tape to large central sites; the users at those sites need

ways of "downloading" the various implementations to  their  micros  and  other

local  systems.  A simple bootstrap procedure would consist of precise instruc-

tions on how to accomplish an "unguarded" capture of the program.    Perhaps  a

simple,  short  program  can be written for each each end that will do the job;

listings and instructions can be provided for the user to type in and run these

programs.

Packet Format and Types                                                 Page 60

I. Packet Format and Types

KERMIT Packet Layout:

+------+-----+-----+------+------------+-------+

| MARK | LEN | SEQ | TYPE |    DATA    | CHECK |

+------+-----+-----+------+------------+-------+

Send-Init Data Field Layout:

1      2      3      4      5      6      7      8      9      10...

+------+------+------+------+------+------+------+------+------+-------

| MAXL | TIME | NPAD | PADC | EOL  | QCTL | QBIN | CHKT | REPT | CAPAS

+------+------+------+------+------+------+------+------+------+-------

KERMIT  packet  types  and subtypes; required types are marked with an asterisk

(*):

Y*  Acknowledge (ACK)

N*  Negative acknowledge (NAK)

S*  Send initiate (exchange parameters)

I   Initialize (exchange parameters)

F*  File header

A   File attributes

D*  Data packet

Z*  End of file (EOF)

B*  Break transmission (EOT)

E*  Error

R   Receive Initiate (ask the server to send the specified files)

C   Host Command

K   KERMIT Command

T   Reserved for internal use

G   Generic Kermit Command; Subcommands:

I   Login (or logout)

C   CWD, Change Working Directory

L   Bye

F   Finish

D   Directory

U   Disk Usage Query

E   Erase, Delete

T   Type

R   Rename

K   Copy

W   Who's logged in?

M   Short Message

H   Help

Q   Server Status Query

P   Program Invocation

J   Journal Control

V   Variable Set/Query

List of Features                                                        Page 61

II. List of Features

There's  no  true  linear  scale along which to rate Kermit implementations.  A

basic, minimal implementation provides file transfer in both  directions,  and,

for  microcomputers  (PC's,  workstations, other single user systems), terminal

emulation.  Even within this framework, there can be variations.  For instance,

can the program send a file group in a single command, or  must  a  command  be

issued for each file?  Can it time out?  Here is a list of features that may be

present;  for  any KERMIT implementation, the documentation should show whether

these features exist, and how to invoke them.

- File groups.  Can it send a group of files  with  a  single  command,

using  "wildcard",  pattern,  or  list notation?  Can it successfully

send or receive a group of files of mixed types?  Can it recover from

an error on a particular file and go on to the next one?  Can it keep

a log of the files involved showing the disposition of each?

- Filenames.  Can it take action to avoid overwriting a local file when

a new file of the same name arrives?  Can it convert filenames to and

from legal or "normal form"?

- File types.  Can binary as well as text files be transmitted?

- 8th-Bit prefixing.  Can it send and  receive  8-bit  data  through  a

7-bit channel using the prefixing mechanism?

- Repeat-Count  processing.  Can it send and receive data with repeated

characters replaced by a prefix sequence?

- Terminal Emulation.  Does it  have  a  terminal  emulation  facility?

Does  it  provide  various  communication  options,  such  as duplex,

parity, and handshake selection?  Can it transmit all  ASCII  charac-

ters?  Can it transmit BREAK?  Can it log the remote session locally,

to provide unguarded file capture?

- Communications Options.  Can duplex, parity, handshake, and line ter-

minator be specified for file transfer?

- Block  Check  Options.    In  addition  to the basic single-character

checksum, can the two-character checksum and the three-character  CRC

be selected?

- Basic  Server.  Can it run in server mode, accepting commands to send

or receive files, and to shut itself down?

- Advanced Server.  Can it accept  server  commands  to  delete  files,

provide directory listings, send messages, and forth?

- Issue  Commands  to  Server.    Can it send commands to a server, and

handle all possible responses?

- Host Commands.  Can it parse and send remote "host commands"?  If  it

is  a  server,  can it pass these commands to the host system command

processor and return the results to the local user Kermit?

- Interrupt File Transfers.  Can it interrupt sending  or  receiving  a

List of Features                                                        Page 62

file?  Can it respond to interrupt requests from the other side?

- Local  File  Management  Services.    Are there commands to get local

directory directory listings, delete local files, and so forth?

- File Attributes.  Can it send file attribute information about  local

files,  and  can  deal with incoming file attribute information?  Can

alternate dispositions be specified.  Can files be archived?

- Debugging Capability.    Can  packet  traffic  be  logged,  examined,

single-stepped?

Unresolved Issues                                                       Page 63

III. Unresolved Issues

KERMIT doesn't do everything.  Here is a short list of things it doesn't do, or

could do better.

- KERMIT cannot be used through IBM 3270 protocol emulators.  These are

devices that allow asynchronous ASCII terminals or PCs to communicate

with  IBM mainframes as though they were synchronous full-screen dis-

plays.  The problems include: (a) the protocol  converter  translates

from  EBCDIC  to  ASCII -- that is, it assumes it is receiving EBCDIC

when KERMIT is supposed to be sending ASCII -- according to  its  own

translate  table,  which  may  vary  from  site to site, and is not a

1-to-1 mapping in any case; (b) non-printing control characters (like

SOH) cannot be sent at all; (c) the protocol converter looks for 3270

formatting commands and translates them to escape sequences  for  the

ASCII  terminal,  possibly  modifying packet data; (d) the IBM system

automatically pauses at the end of each screenful; (e)  the  protocol

converter  thinks it knows what is "on the screen" and may attempt to

optimize.  In general, one  never  knows  exactly  how  a  particular

protocol  converter will work, or how it may differ from another one.

Still, it may be possible to work around these problems if  the  Ker-

mits  on  either  end  are put into a special "3270 mode", "clear the

screen" between each packet, and agree on some special convention for

packet delimitation and data translation.

- Control character prefixing.  This can get pretty expensive  for  bi-

nary  files  or  other files that contain lots of control characters.

Since most hosts won't choke on every single  control  character,  it

might  be  a good idea for each host to inform the other of what con-

trol characters it can receive "bare".  On the other hand, this could

backfire when the two hosts are connected by a network or device that

chokes on control characters that neither of the hosts choke on.  For

hardwired connections with no unknown factors, however, many  control

characters could be sent "bare".

- When  long  sequences  of  characters  in the control range are being

sent, individually prefixing each character  is  costly.    Shift-in/

shift-out  codes  might be used more effectively here.  Same for long

strings of characters with the parity bit on, when 8th-bit  prefixing

is  being  done.  The cost would be yet another set of prefix charac-

ters, and the associated complexity of packet formation and decoding.

- In some situations, certain printable characters can't get through  a

communication  link.    For instance, a network terminal server might

reserve "@" as its attention character.  Should  the  protocol  allow

some  way  to encode such characters for translation?  A set of user-

definable escape sequences could be useful here.

- Ironically, some systems do not fare well with KERMIT's small  packet

sizes.  These are typically big mainframes that expect to communicate

with  block  mode  terminals,  receiving thousands of characters at a

time.  Such systems find that KERMIT is simply too expensive to  run.

Meanwhile,  other systems that can run KERMIT with no difficulty find

the performance disappointing (the  efficiency  generally  works  out

somewhere  between 50 and 80 percent, i.e. data characters divided by

the speed of the the transmission line, in  characters  per  second).

Unresolved Issues                                                       Page 64

These  two  problems  could  be  solved  in either of two ways: allow

longer packets, or allow multiple data packets to be sent end to end.

* Without changing the format or encoding of  the  packet  control

fields, it might be possible to have longer packets by reserving

packet lengths of 0, 1, 2, 3 to be codes for bigger numbers like

200,  500,  1000,  2000.    However,  the longer the packet, the

greater  the  probability  of  corruption,  and  the  longer  to

retransmit  once  corrupted.    In addition, the adequacy of the

single-character block check would  be  much  reduced  for  long

packets;  long packets should therefore be sent with type 2 or 3

block checks.

* It would be possible to extend the protocol to  allow  transmis-

sion  of data packets while ACKs were still outstanding, and any

ACK would be taken as an ACK for the specified  packet  and  all

previous  ones.  A limit on the number of outstanding ACKs could

be agreed upon; the maximum size of this "floating window" would

have to be less than 64, which is where  KERMIT  packet  numbers

wrap  around,  and the window could not extend over a wraparound

or else KERMIT would have no way of knowing whether n  times  64

packets  had  been skipped.  A floating window of 8 and a packet

size of 95 would give about the  same  effect  as  700-character

packets.   A NAK would require the sender to back up all the way

to the NAK'd packet.  A future edition of  the  Protocol  Manual

might include a specification for floating windows.

A KERMIT Program                                                        Page 65

IV. A KERMIT Program

What  follows  is  a listing of a real production version of KERMIT, written in

4

the C language .  It is designed to run under  various  versions  of  the  UNIX

operating  system.   The basic KERMIT functionality is present, but very little

in the way of optional features.  Only the most rudimentary  (UNIX-style)  com-

mand  parser  is  provided.  The program illustrates many of the considerations

described in this manual, with respect to the protocol, the program design, and

so forth.

It must be emphasized that this is a bare  minimum  implementation  of  Kermit.

Anyone  writing  a  new Kermit from scratch is encouraged to look at the source

for one of the more advanced implementations as a model (check the Kermit Users

Guide for a list of KERMIT implementations and their features).   Although  you

may  not  understand the language it's written in, there should be profuse com-

ments that can be useful.

The following program uses decimal notation for numbers,  and  tochar()  rather

than char() for integer-to-character conversion.

/*

*  K e r m i t  File Transfer Utility

*

*  UNIX Kermit, Columbia University, 1981, 1982, 1983

*      Bill Catchings, Bob Cattani, Chris Maio, Frank da Cruz, Alan Crosswell

*

*  Also:   Jim Guyton, Rand Corporation

*          Walter Underwood, Ford Aerospace

*

*  usage:  kermit c [lbe line baud escapechar]         to connect

*          kermit s [d..iflb line baud] file ...       to send files

*          kermit r [d..iflb line baud]                to receive files

*

*  where   c=connect, s=send, r=receive,

*          d=debug, i=image mode, f=no filename conversion, l=tty line,

*          b=baud rate, e=escape char.

*

*  For remote Kermit, format is either:

*          kermit r                                    to receive files

*  or      kermit s file ...                           to send files

*

*/

_______________

4

Kernighan & Ritchie, The C Programming Language:  Prentice-Hall (1978)

A KERMIT Program                                                        Page 66

/*

*  Modification History:

*

*  Oct. 17 Included fixes from Alan Crosswell (CUCCA) for IBM_UTS:

*          - Changed MYEOL character from \n to \r.

*          - Change char to int in bufill so getc would return -1 on

*            EOF instead of 255 (-1 truncated to 8 bits)

*          - Added read() in rpack to eat the EOL character

*          - Added fflush() call in printmsg to force the output

*          NOTE: The last three changes are not conditionally compiled

*                since they should work equally well on any system.

*

*          Changed Berkeley 4.x conditional compilation flag from

*              UNIX4X to UCB4X.

*          Added support for error packets and cleaned up the printing

*              routines.

*/

#include <stdio.h>          /* Standard UNIX definitions */

/* Conditional compilation for different machines/operating systems */

/* One and only one of the following lines should be 1 */

#define UCB4X       1       /* Berkeley 4.x UNIX */

#define TOPS_20     0       /* TOPS-20 */

#define IBM_UTS     0       /* Amdahl UTS on IBM systems */

#define VAX_VMS     0       /* VAX/VMS (not yet implemented) */

/* Conditional compilation for the different Unix variants */

/* 0 means don't compile it, nonzero means do */

#if UCB4X

#define V6_LIBS     0       /* Dont't use retrofit libraries */

#define NO_FIONREAD 0       /* We have ioctl(FIONREAD,...) for flushinput() */

#define NO_TANDEM   0       /* We have TANDEM line discipline (xon/xoff) */

#endif

#if IBM_UTS

#define V6_LIBS     0       /* Don't use retrofit libraries */

#define NO_FIONREAD 1       /* No ioctl(FIONREAD,...) for flushinput() */

#define NO_TANDEM   1       /* No TANDEM line discipline (xon/xoff) */

#endif

#if V6_LIBS

#include <retrofit/sgtty.h>

#include <retrofit/signal.h>

#include <retrofit/setjmp.h>

#else

#include <sgtty.h>

#include <signal.h>

#include <setjmp.h>

#endif

A KERMIT Program                                                        Page 67

#if NO_TANDEM

#define TANDEM      0       /* define it to be nothing if it's unsupported */

#endif

/* Symbol Definitions */

#define MAXPACKSIZ  94      /* Maximum packet size */

#define SOH         1       /* Start of header */

#define CR          13      /* ASCII Carriage Return */

#define SP          32      /* ASCII space */

#define DEL         127     /* Delete (rubout) */

#define ESCCHR      '^'     /* Default escape character for CONNECT */

#define MAXTRY      10      /* Times to retry a packet */

#define MYQUOTE     '#'     /* Quote character I will use */

#define MYPAD       0       /* Number of padding characters I will need */

#define MYPCHAR     0       /* Padding character I need (NULL) */

#if IBM_UTS

#define MYEOL       '\r'    /* End-Of-Line character for UTS systems */

#else

#define MYEOL       '\n'    /* End-Of-Line character I need */

#endif

#define MYTIME      10      /* Seconds after which I should be timed out */

#define MAXTIM      60      /* Maximum timeout interval */

#define MINTIM      2       /* Minumum timeout interval */

#define TRUE        -1      /* Boolean constants */

#define FALSE       0

/* Macro Definitions */

/*

* tochar: converts a control character to a printable one by adding a space.

*

* unchar: undoes tochar.

*

* ctl:    converts between control characters and printable characters by

*         toggling the control bit (ie. ^A becomes A and A becomes ^A).

*/

#define tochar(ch)  ((ch) + ' ')

#define unchar(ch)  ((ch) - ' ')

#define ctl(ch)     ((ch) ^ 64 )

/* Global Variables */

A KERMIT Program                                                        Page 68

int     size,               /* Size of present data */

rpsiz,              /* Maximum receive packet size */

spsiz,              /* Maximum send packet size */

pad,                /* How much padding to send */

timint,             /* Timeout for foreign host on sends */

n,                  /* Packet number */

numtry,             /* Times this packet retried */

oldtry,             /* Times previous packet retried */

ttyfd,              /* File descriptor of tty for I/O, 0 if remote */

remote,             /* -1 means we're a remote kermit */

image,              /* -1 means 8-bit mode */

debug,              /* indicates level of debugging output (0=none) */

filnamcnv,          /* -1 means do file name case conversions */

filecount;          /* Number of files left to send */

char    state,              /* Present state of the automaton */

padchar,            /* Padding character to send */

eol,                /* End-Of-Line character to send */

escchr,             /* Connect command escape character */

quote,              /* Quote character in incoming data */

**filelist,         /* List of files to be sent */

*filnam,            /* Current file name */

recpkt[MAXPACKSIZ], /* Receive packet buffer */

packet[MAXPACKSIZ]; /* Packet buffer */

FILE    *fp,                /* File pointer for current disk file */

*log;               /* File pointer for Logfile */

jmp_buf env;                /* Environment ptr for timeout longjump */

/*

*  m a i n

*

*  Main routine - parse command and options, set up the

*  tty lines, and dispatch to the appropriate routine.

*/

main(argc,argv)

int argc;                           /* Character pointers to and count of */

char **argv;                            /* command line arguments */

{

char *ttyname,                      /* tty name for LINE argument */

*cp;                            /* char pointer */

int speed,                          /* speed of assigned tty, */

cflg, rflg, sflg;               /* flags for CONNECT, RECEIVE, SEND */

struct sgttyb

rawmode,                        /* Controlling tty raw mode */

cookedmode,                     /* Controlling tty cooked mode */

ttymode;                        /* mode of tty line in LINE option */

if (argc < 2) usage();              /* Make sure there's a command line */

cp = *++argv; argv++; argc -= 2;    /* Set up pointers to args */

A KERMIT Program                                                        Page 69

/*  Initialize these values and hope the first packet will get across OK */

eol = CR;                           /* EOL for outgoing packets */

quote = '#';                        /* Standard control-quote char "#" */

pad = 0;                            /* No padding */

padchar = NULL;                     /* Use null if any padding wanted */

speed = cflg = sflg = rflg = 0;     /* Turn off all parse flags */

ttyname = 0;                        /* Default is remote mode */

#if UCB4X                               /* Default to 7-bit masking, CRLF */

image = FALSE;                      /* translation and filename case */

filnamcnv = TRUE;                   /* conversion for UNIX systems */

#else

image = TRUE;                       /* Default to no processing for */

filnamcnv = FALSE;                  /* non-UNIX systems */

#endif

escchr = ESCCHR;                    /* Default escape character */

while ((*cp) != NULL)               /* Parse characters in first arg. */

switch (*cp++)

{

case 'c': cflg++; break;    /* C = Connect command */

case 's': sflg++; break;    /* S = Send command */

case 'r': rflg++; break;    /* R = Receive command */

case 'd':                   /* D = Increment debug mode count */

debug++; break;

case 'f':

filnamcnv = FALSE;      /* F = don't do case conversion */

break;                  /*     on filenames */

case 'i':                   /* I = Image (8-bit) mode */

image = TRUE; break;    /* (this is default for non-UNIX) */

case 'l':                   /* L = specify tty line to use */

if (argc--) ttyname = *argv++;

else usage();

if (debug) printf("Line to remote host is %s\n",ttyname);

break;

case 'e':                   /* E = specify escape char */

if (argc--) escchr = **argv++;

else usage();

if (debug) printf("Escape char is \"%c\"\n",escchr);

break;

A KERMIT Program                                                        Page 70

case 'b':                   /* B = specify baud rate */

#if UCB4X

if (argc--) speed = atoi(*argv++);

else usage();

if (debug) printf("Line speed to remote host is %d\n",speed);

break;

#else

printmsg("Speed setting implemented for Unix only.");

exit(1);

#endif

}

/* Done parsing */

if ((cflg+sflg+rflg) != 1)          /* Only one command allowed */

usage();

if (ttyname)                        /* If LINE was specified, we */

{                                   /* operate in local mode */

ttyfd = open(ttyname,2);        /* Open the tty line */

if (ttyfd < 0)

{

printmsg("Cannot open %s",ttyname);

exit(1);

}

remote = FALSE;                 /* Indicate we're in local mode */

}

else                                /* No LINE specified so we operate */

{                                   /* in remote mode (ie. controlling */

ttyfd = 0;                      /* tty is the communications line) */

remote = TRUE;

}

/* Put the proper tty into the correct mode */

if (remote)                         /* If remote, use controlling tty */

{

gtty(0,&cookedmode);            /* Save current mode so we can */

gtty(0,&rawmode);               /* restore it later */

rawmode.sg_flags |= (RAW|TANDEM);

rawmode.sg_flags &= ~(ECHO|CRMOD);

stty(0,&rawmode);               /* Put tty in raw mode */

}

else                                /* Local, use assigned line */

{

gtty(ttyfd,&ttymode);

ttymode.sg_flags |= (RAW|TANDEM);

ttymode.sg_flags &= ~(ECHO|CRMOD);

A KERMIT Program                                                        Page 71

#if UCB4X                               /* Speed changing for UNIX only */

if (speed)                      /* User specified a speed? */

{

switch(speed)               /* Get internal system code */

{

case 110: speed = B110; break;

case 150: speed = B150; break;

case 300: speed = B300; break;

case 1200: speed = B1200; break;

case 2400: speed = B2400; break;

case 4800: speed = B4800; break;

case 9600: speed = B9600; break;

default:

printmsg("Bad line speed.");

exit(1);

}

ttymode.sg_ispeed = speed;

ttymode.sg_ospeed = speed;

}

#endif /* UCB4X */

stty(ttyfd,&ttymode);           /* Put asg'd tty in raw mode */

}

/* All set up, now execute the command that was given. */

if (debug)

{

printf("Debugging level = %d\n\n",debug);

if (cflg) printf("Connect command\n\n");

if (sflg) printf("Send command\n\n");

if (rflg) printf("Receive command\n\n");

}

if (cflg) connect();                /* Connect command */

if (sflg)                           /* Send command */

{

if (argc--) filnam = *argv++;   /* Get file to send */

else

{   if (remote)

stty(0,&cookedmode);    /* Restore controlling tty's modes */

usage();                    /* and give error */

}

fp = NULL;                      /* Indicate no file open yet */

filelist = argv;                /* Set up the rest of the file list */

filecount = argc;               /* Number of files left to send */

if (sendsw() == FALSE)          /* Send the file(s) */

printmsg("Send failed.");   /* Report failure */

else                            /*  or */

printmsg("done.");          /* success */

}

A KERMIT Program                                                        Page 72

if (rflg)                           /* Receive command */

{

if (recsw() == FALSE)           /* Receive the file(s) */

printmsg("Receive failed.");

else                            /* Report failure */

printmsg("done.");          /* or success */

}

if (remote) stty(0,&cookedmode);    /* Restore controlling tty's modes */

}

/*

*  s e n d s w

*

*  Sendsw is the state table switcher for sending files.  It loops until

*  either it finishes, or an error is encountered.  The routines called

*  by sendsw are responsible for changing the state.

*

*/

sendsw()

{

char sinit(), sfile(), sdata(), seof(), sbreak();

state = 'S';                        /* Send initiate is the start state */

n = 0;                              /* Initialize message number */

numtry = 0;                         /* Say no tries yet */

while(TRUE)                         /* Do this as long as necessary */

{

if (debug) printf("sendsw state: %c\n",state);

switch(state)

{

case 'S':   state = sinit();  break; /* Send-Init */

case 'F':   state = sfile();  break; /* Send-File */

case 'D':   state = sdata();  break; /* Send-Data */

case 'Z':   state = seof();   break; /* Send-End-of-File */

case 'B':   state = sbreak(); break; /* Send-Break */

case 'C':   return (TRUE);           /* Complete */

case 'A':   return (FALSE);          /* "Abort" */

default:    return (FALSE);          /* Unknown, fail */

}

}

}

/*

*  s i n i t

*

*  Send Initiate: send this host's parameters and get other side's back.

*/

char sinit()

{

int num, len;                       /* Packet number, length */

A KERMIT Program                                                        Page 73

if (numtry++ > MAXTRY) return('A'); /* If too many tries, give up */

spar(packet);                       /* Fill up init info packet */

flushinput();                       /* Flush pending input */

spack('S',n,6,packet);              /* Send an S packet */

switch(rpack(&len,&num,recpkt))     /* What was the reply? */

{

case 'N':  return(state);       /* NAK, try it again */

case 'Y':                       /* ACK */

if (n != num)               /* If wrong ACK, stay in S state */

return(state);          /* and try again */

rpar(recpkt);               /* Get other side's init info */

if (eol == 0) eol = '\n';   /* Check and set defaults */

if (quote == 0) quote = '#';

numtry = 0;                 /* Reset try counter */

n = (n+1)%64;               /* Bump packet count */

return('F');                /* OK, switch state to F */

case 'E':                       /* Error packet received */

prerrpkt(recpkt);           /* Print it out and */

return('A');                /* abort */

case FALSE: return(state);      /* Receive failure, try again */

default: return('A');           /* Anything else, just "abort" */

}

}

/*

*  s f i l e

*

*  Send File Header.

*/

char sfile()

{

int num, len;                       /* Packet number, length */

char filnam1[50],                   /* Converted file name */

*newfilnam,                     /* Pointer to file name to send */

*cp;                            /* char pointer */

if (numtry++ > MAXTRY) return('A'); /* If too many tries, give up */

A KERMIT Program                                                        Page 74

if (fp == NULL)                     /* If not already open, */

{   if (debug) printf("   Opening %s for sending.\n",filnam);

fp = fopen(filnam,"r");         /* open the file to be sent */

if (fp == NULL)                 /* If bad file pointer, give up */

{

error("Cannot open file %s",filnam);

return('A');

}

}

strcpy(filnam1, filnam);            /* Copy file name */

newfilnam = cp = filnam1;

while (*cp != '\0')                 /* Strip off all leading directory */

if (*cp++ == '/')               /* names (ie. up to the last /). */

newfilnam = cp;

if (filnamcnv)                      /* Convert lower case to upper  */

for (cp = newfilnam; *cp != '\0'; cp++)

if (*cp >= 'a' && *cp <= 'z')

*cp ^= 040;

len = cp - newfilnam;               /* Compute length of new filename */

printmsg("Sending %s as %s",filnam,newfilnam);

spack('F',n,len,newfilnam);         /* Send an F packet */

switch(rpack(&len,&num,recpkt))     /* What was the reply? */

{

case 'N':                       /* NAK, just stay in this state, */

num = (--num<0 ? 63:num);   /* unless it's NAK for next packet */

if (n != num)               /* which is just like an ACK for */

return(state);          /* this packet so fall thru to... */

case 'Y':                       /* ACK */

if (n != num) return(state); /* If wrong ACK, stay in F state */

numtry = 0;                 /* Reset try counter */

n = (n+1)%64;               /* Bump packet count */

size = bufill(packet);      /* Get first data from file */

return('D');                /* Switch state to D */

case 'E':                       /* Error packet received */

prerrpkt(recpkt);           /* Print it out and */

return('A');                /* abort */

case FALSE: return(state);      /* Receive failure, stay in F state */

default:    return('A');        /* Something else, just "abort" */

}

}

A KERMIT Program                                                        Page 75

/*

*  s d a t a

*

*  Send File Data

*/

char sdata()

{

int num, len;                       /* Packet number, length */

if (numtry++ > MAXTRY) return('A'); /* If too many tries, give up */

spack('D',n,size,packet);           /* Send a D packet */

switch(rpack(&len,&num,recpkt))     /* What was the reply? */

{

case 'N':                       /* NAK, just stay in this state, */

num = (--num<0 ? 63:num);   /* unless it's NAK for next packet */

if (n != num)               /* which is just like an ACK for */

return(state);          /* this packet so fall thru to... */

case 'Y':                       /* ACK */

if (n != num) return(state); /* If wrong ACK, fail */

numtry = 0;                 /* Reset try counter */

n = (n+1)%64;               /* Bump packet count */

if ((size = bufill(packet)) == EOF) /* Get data from file */

return('Z');            /* If EOF set state to that */

return('D');                /* Got data, stay in state D */

case 'E':                       /* Error packet received */

prerrpkt(recpkt);           /* Print it out and */

return('A');                /* abort */

case FALSE: return(state);      /* Receive failure, stay in D */

default:    return('A');        /* Anything else, "abort" */

}

}

/*

*  s e o f

*

*  Send End-Of-File.

*/

char seof()

{

int num, len;                       /* Packet number, length */

if (numtry++ > MAXTRY) return('A'); /* If too many tries, "abort" */

A KERMIT Program                                                        Page 76

spack('Z',n,0,packet);              /* Send a 'Z' packet */

switch(rpack(&len,&num,recpkt))     /* What was the reply? */

{

case 'N':                       /* NAK, just stay in this state, */

num = (--num<0 ? 63:num);   /* unless it's NAK for next packet, */

if (n != num)               /* which is just like an ACK for */

return(state);          /* this packet so fall thru to... */

case 'Y':                       /* ACK */

if (n != num) return(state); /* If wrong ACK, hold out */

numtry = 0;                 /* Reset try counter */

n = (n+1)%64;               /* and bump packet count */

if (debug) printf("   Closing input file %s, ",filnam);

fclose(fp);                 /* Close the input file */

fp = NULL;                  /* Set flag indicating no file open */

if (debug) printf("looking for next file...\n");

if (gnxtfl() == FALSE)      /* No more files go? */

return('B');            /* if not, break, EOT, all done */

if (debug) printf("   New file is %s\n",filnam);

return('F');                /* More files, switch state to F */

case 'E':                       /* Error packet received */

prerrpkt(recpkt);           /* Print it out and */

return('A');                /* abort */

case FALSE: return(state);      /* Receive failure, stay in Z */

default:    return('A');        /* Something else, "abort" */

}

}

/*

*  s b r e a k

*

*  Send Break (EOT)

*/

char sbreak()

{

int num, len;                       /* Packet number, length */

if (numtry++ > MAXTRY) return('A'); /* If too many tries "abort" */

spack('B',n,0,packet);              /* Send a B packet */

switch (rpack(&len,&num,recpkt))    /* What was the reply? */

{

case 'N':                       /* NAK, just stay in this state, */

num = (--num<0 ? 63:num);   /* unless NAK for previous packet, */

if (n != num)               /* which is just like an ACK for */

return(state);          /* this packet so fall thru to... */

A KERMIT Program                                                        Page 77

case 'Y':                       /* ACK */

if (n != num) return(state); /* If wrong ACK, fail */

numtry = 0;                 /* Reset try counter */

n = (n+1)%64;               /* and bump packet count */

return('C');                /* Switch state to Complete */

case 'E':                       /* Error packet received */

prerrpkt(recpkt);           /* Print it out and */

return('A');                /* abort */

case FALSE: return(state);      /* Receive failure, stay in B */

default:    return ('A');       /* Other, "abort" */

}

}

/*

*  r e c s w

*

*  This is the state table switcher for receiving files.

*/

recsw()

{

char rinit(), rfile(), rdata();     /* Use these procedures */

state = 'R';                        /* Receive-Init is the start state */

n = 0;                              /* Initialize message number */

numtry = 0;                         /* Say no tries yet */

while(TRUE)

{

if (debug) printf(" recsw state: %c\n",state);

switch(state)                   /* Do until done */

{

case 'R':   state = rinit(); break; /* Receive-Init */

case 'F':   state = rfile(); break; /* Receive-File */

case 'D':   state = rdata(); break; /* Receive-Data */

case 'C':   return(TRUE);           /* Complete state */

case 'A':   return(FALSE);          /* "Abort" state */

}

}

}

/*

*  r i n i t

*

*  Receive Initialization

*/

char rinit()

{

int len, num;                       /* Packet length, number */

A KERMIT Program                                                        Page 78

if (numtry++ > MAXTRY) return('A'); /* If too many tries, "abort" */

switch(rpack(&len,&num,packet))     /* Get a packet */

{

case 'S':                       /* Send-Init */

rpar(packet);               /* Get the other side's init data */

spar(packet);               /* Fill up packet with my init info */

spack('Y',n,6,packet);      /* ACK with my parameters */

oldtry = numtry;            /* Save old try count */

numtry = 0;                 /* Start a new counter */

n = (n+1)%64;               /* Bump packet number, mod 64 */

return('F');                /* Enter File-Receive state */

case 'E':                       /* Error packet received */

prerrpkt(recpkt);           /* Print it out and */

return('A');                /* abort */

case FALSE:                     /* Didn't get packet */

spack('N',n,0,0);           /* Return a NAK */

return(state);              /* Keep trying */

default:     return('A');       /* Some other packet type, "abort" */

}

}

/*

*  r f i l e

*

*  Receive File Header

*/

char rfile()

{

int num, len;                       /* Packet number, length */

char filnam1[50];                   /* Holds the converted file name */

if (numtry++ > MAXTRY) return('A'); /* "abort" if too many tries */

switch(rpack(&len,&num,packet))     /* Get a packet */

{

case 'S':                       /* Send-Init, maybe our ACK lost */

if (oldtry++ > MAXTRY) return('A'); /* If too many tries "abort" */

if (num == ((n==0) ? 63:n-1)) /* Previous packet, mod 64? */

{                           /* Yes, ACK it again with  */

spar(packet);           /* our Send-Init parameters */

spack('Y',num,6,packet);

numtry = 0;             /* Reset try counter */

return(state);          /* Stay in this state */

}

else return('A');           /* Not previous packet, "abort" */

A KERMIT Program                                                        Page 79

case 'Z':                       /* End-Of-File */

if (oldtry++ > MAXTRY) return('A');

if (num == ((n==0) ? 63:n-1)) /* Previous packet, mod 64? */

{                           /* Yes, ACK it again. */

spack('Y',num,0,0);

numtry = 0;

return(state);          /* Stay in this state */

}

else return('A');           /* Not previous packet, "abort" */

case 'F':                       /* File Header (just what we want) */

if (num != n) return('A');  /* The packet number must be right */

strcpy(filnam1, packet);    /* Copy the file name */

if (filnamcnv)              /* Convert upper case to lower */

for (filnam=filnam1; *filnam != '\0'; filnam++)

if (*filnam >= 'A' && *filnam <= 'Z')

*filnam |= 040;

if ((fp=fopen(filnam1,"w"))==NULL) /* Try to open a new file */

{

error("Cannot create %s",filnam1); /* Give up if can't */

return('A');

}

else                        /* OK, give message */

printmsg("Receiving %s as %s",packet,filnam1);

spack('Y',n,0,0);           /* Acknowledge the file header */

oldtry = numtry;            /* Reset try counters */

numtry = 0;                 /* ... */

n = (n+1)%64;               /* Bump packet number, mod 64 */

return('D');                /* Switch to Data state */

case 'B':                       /* Break transmission (EOT) */

if (num != n) return ('A'); /* Need right packet number here */

spack('Y',n,0,0);           /* Say OK */

return('C');                /* Go to complete state */

case 'E':                       /* Error packet received */

prerrpkt(recpkt);           /* Print it out and */

return('A');                /* abort */

case FALSE:                     /* Didn't get packet */

spack('N',n,0,0);           /* Return a NAK */

return(state);              /* Keep trying */

default:    return ('A');       /* Some other packet, "abort" */

}

}

A KERMIT Program                                                        Page 80

/*

*  r d a t a

*

*  Receive Data

*/

char rdata()

{

int num, len;                       /* Packet number, length */

if (numtry++ > MAXTRY) return('A'); /* "abort" if too many tries */

switch(rpack(&len,&num,packet))     /* Get packet */

{

case 'D':                       /* Got Data packet */

if (num != n)               /* Right packet? */

{                           /* No */

if (oldtry++ > MAXTRY)

return('A');        /* If too many tries, abort */

if (num == ((n==0) ? 63:n-1)) /* Else check packet number */

{                       /* Previous packet again? */

spack('Y',num,6,packet); /* Yes, re-ACK it */

numtry = 0;         /* Reset try counter */

return(state);      /* Don't write out data! */

}

else return('A');       /* sorry, wrong number */

}

/* Got data with right packet number */

bufemp(packet,len);         /* Write the data to the file */

spack('Y',n,0,0);           /* Acknowledge the packet */

oldtry = numtry;            /* Reset the try counters */

numtry = 0;                 /* ... */

n = (n+1)%64;               /* Bump packet number, mod 64 */

return('D');                /* Remain in data state */

case 'F':                       /* Got a File Header */

if (oldtry++ > MAXTRY)

return('A');            /* If too many tries, "abort" */

if (num == ((n==0) ? 63:n-1)) /* Else check packet number */

{                           /* It was the previous one */

spack('Y',num,0,0);     /* ACK it again */

numtry = 0;             /* Reset try counter */

return(state);          /* Stay in Data state */

}

else return('A');           /* Not previous packet, "abort" */

case 'Z':                       /* End-Of-File */

if (num != n) return('A');  /* Must have right packet number */

spack('Y',n,0,0);           /* OK, ACK it. */

fclose(fp);                 /* Close the file */

n = (n+1)%64;               /* Bump packet number */

return('F');                /* Go back to Receive File state */

case 'E':                       /* Error packet received */

prerrpkt(recpkt);           /* Print it out and */

return('A');                /* abort */

A KERMIT Program                                                        Page 81

case FALSE:                     /* Didn't get packet */

spack('N',n,0,0);           /* Return a NAK */

return(state);              /* Keep trying */

default:     return('A');       /* Some other packet, "abort" */

}

}

/*

*  c o n n e c t

*

*  Establish a virtual terminal connection with the remote host, over an

*  assigned tty line.

*/

connect()

{

int pid,                            /* Holds process id of child */

connected;                      /* Boolean connect flag */

char bel = '\07',

c;

struct sgttyb

rawmode,                        /* Controlling tty raw mode */

cookedmode;                     /* Controlling tty cooked mode */

if (remote)                         /* Nothing to connect to in remote */

{                                   /* mode, so just return */

printmsg("No line specified for connection.");

return;

}

gtty(0,&cookedmode);                /* Save current mode so we can */

gtty(0,&rawmode);                   /* restore it later */

rawmode.sg_flags |= (RAW|TANDEM);

rawmode.sg_flags &= ~(ECHO|CRMOD);

stty(0,&rawmode);                   /* Put tty in raw mode */

pid = fork();           /* Start fork to get typeout from remote host */

A KERMIT Program                                                        Page 82

if (pid)                        /* Parent: send type-in to remote host */

{

printmsg("connected...\r");

connected = TRUE;               /* Put us in "connect mode" */

while (connected)

{

read(0,&c,1);               /* Get a character */

if ((c&0177) == escchr)     /* Check for escape character */

{

read(0,&c,1);

if ((c&0177) == escchr)

write(ttyfd,&c,1);

else

switch (c&0177)

{

case 'c':

case 'C':

connected = FALSE;

write(0,"\r\n",2);

break;

case 'h':

case 'H':

write(0,"\r\nYes, I'm still here...\r\n",26);

break;

default:

write(0,&bel,1);

break;

}

}

else

{                           /* If not escape charater, */

write(ttyfd,&c,1);      /* write it out */

c = NULL;               /* Nullify it (why?) */

}

}

kill(pid,9);                    /* Done, kill the child */

wait(0);                        /* and bury him */

stty(0,&cookedmode);            /* Restore tty mode */

printmsg("disconnected.");

return;                         /* Done */

}

else                  /* Child does the reading from the remote host */

{

while(1)                        /* Do this forever */

{

read(ttyfd,&c,1);

write(1,&c,1);

}

}

}

A KERMIT Program                                                        Page 83

/*

*      KERMIT utilities.

*/

clkint()                                /* Timer interrupt handler */

{

longjmp(env,TRUE);                  /* Tell rpack to give up */

}

/*

*  s p a c k

*

*  Send a Packet

*/

spack(type,num,len,data)

char type, *data;

int num, len;

{

int i;                              /* Character loop counter */

char chksum, buffer[100];           /* Checksum, packet buffer */

register char *bufp;                /* Buffer pointer */

if (debug>1)                        /* Display outgoing packet */

{

if (data != NULL)

data[len] = '\0';           /* Null-terminate data to print it */

printf("  spack type: %c\n",type);

printf("         num:  %d\n",num);

printf("         len:  %d\n",len);

if (data != NULL)

printf("        data: \"%s\"\n",data);

}

bufp = buffer;                      /* Set up buffer pointer */

for (i=1; i<=pad; i++) write(ttyfd,&padchar,1); /* Issue any padding */

*bufp++ = SOH;                      /* Packet marker, ASCII 1 (SOH) */

*bufp++ = tochar(len+3);            /* Send the character count */

chksum  = tochar(len+3);            /* Initialize the checksum */

*bufp++ = tochar(num);              /* Packet number */

chksum += tochar(num);              /* Update checksum */

*bufp++ = type;                     /* Packet type */

chksum += type;                     /* Update checksum */

A KERMIT Program                                                        Page 84

for (i=0; i<len; i++)               /* Loop for all data characters */

{

*bufp++ = data[i];              /* Get a character */

chksum += data[i];              /* Update checksum */

}

chksum = (((chksum&0300) >> 6)+chksum)&077; /* Compute final checksum */

*bufp++ = tochar(chksum);           /* Put it in the packet */

*bufp = eol;                        /* Extra-packet line terminator */

write(ttyfd, buffer,bufp-buffer+1); /* Send the packet */

}

/*

*  r p a c k

*

*  Read a Packet

*/

rpack(len,num,data)

int *len, *num;                         /* Packet length, number */

char *data;                             /* Packet data */

{

int i, done;                        /* Data character number, loop exit */

char t,                             /* Current input character */

type,                           /* Packet type */

cchksum,                        /* Our (computed) checksum */

rchksum;                        /* Checksum received from other host */

#if UCB4X                               /* TOPS-20 can't handle timeouts... */

if (setjmp(env)) return FALSE;      /* Timed out, fail */

signal(SIGALRM,clkint);             /* Setup the timeout */

if ((timint > MAXTIM) || (timint < MINTIM)) timint = MYTIME;

alarm(timint);

#endif /* UCB4X */

while (t != SOH)                    /* Wait for packet header */

{

read(ttyfd,&t,1);

t &= 0177;                      /* Handle parity */

}

done = FALSE;                       /* Got SOH, init loop */

while (!done)                       /* Loop to get a packet */

{

read(ttyfd,&t,1);               /* Get character */

if (!image) t &= 0177;          /* Handle parity */

if (t == SOH) continue;         /* Resynchronize if SOH */

cchksum = t;                    /* Start the checksum */

*len = unchar(t)-3;             /* Character count */

read(ttyfd,&t,1);               /* Get character */

if (!image) t &= 0177;          /* Handle parity */

if (t == SOH) continue;         /* Resynchronize if SOH */

cchksum = cchksum + t;          /* Update checksum */

*num = unchar(t);               /* Packet number */

A KERMIT Program                                                        Page 85

read(ttyfd,&t,1);               /* Get character */

if (!image) t &= 0177;          /* Handle parity */

if (t == SOH) continue;         /* Resynchronize if SOH */

cchksum = cchksum + t;          /* Update checksum */

type = t;                       /* Packet type */

for (i=0; i<*len; i++)          /* The data itself, if any */

{                               /* Loop for character count */

read(ttyfd,&t,1);           /* Get character */

if (!image) t &= 0177;      /* Handle parity */

if (t == SOH) continue;     /* Resynch if SOH */

cchksum = cchksum + t;      /* Update checksum */

data[i] = t;                /* Put it in the data buffer */

}

data[*len] = 0;                 /* Mark the end of the data */

read(ttyfd,&t,1);               /* Get last character (checksum) */

rchksum = unchar(t);            /* Convert to numeric */

read(ttyfd,&t,1);               /* get EOL character and toss it */

if (!image) t &= 0177;          /* Handle parity */

if (t == SOH) continue;         /* Resynchronize if SOH */

done = TRUE;                    /* Got checksum, done */

}

#if UCB4X

alarm(0);                           /* Disable the timer interrupt */

#endif

if (debug>1)                        /* Display incoming packet */

{

if (data != NULL)

data[*len] = '\0';          /* Null-terminate data to print it */

printf("  rpack type: %c\n",type);

printf("         num:  %d\n",*num);

printf("         len:  %d\n",*len);

if (data != NULL)

printf("        data: \"%s\"\n",data);

}

/* Fold in bits 7,8 to compute */

cchksum = (((cchksum&0300) >> 6)+cchksum)&077; /* final checksum */

if (cchksum != rchksum) return(FALSE);

return(type);                       /* All OK, return packet type */

}

/*

*  b u f i l l

*

*  Get a bufferful of data from the file that's being sent.

*  Only control-quoting is done; 8-bit & repeat count prefixes are

*  not handled.

*/

A KERMIT Program                                                        Page 86

bufill(buffer)

char buffer[];                          /* Buffer */

{

int i,                              /* Loop index */

t;                              /* Char read from file */

char t7;                            /* 7-bit version of above */

i = 0;                              /* Init data buffer pointer */

while((t = getc(fp)) != EOF)        /* Get the next character */

{

t7 = t & 0177;                  /* Get low order 7 bits */

if (t7 < SP || t7==DEL || t7==quote) /* Does this char require */

{                                   /* special handling? */

if (t=='\n' && !image)

{                           /* Do LF->CRLF mapping if !image */

buffer[i++] = quote;

buffer[i++] = ctl('\r');

}

buffer[i++] = quote;        /* Quote the character */

if (t7 != quote)

{

t = ctl(t);             /* and uncontrolify */

t7 = ctl(t7);

}

}

if (image)

buffer[i++] = t;            /* Deposit the character itself */

else

buffer[i++] = t7;

if (i >= spsiz-8) return(i);    /* Check length */

}

if (i==0) return(EOF);              /* Wind up here only on EOF */

return(i);                          /* Handle partial buffer */

}

/*

*      b u f e m p

*

*  Put data from an incoming packet into a file.

*/

bufemp(buffer,len)

char  buffer[];                         /* Buffer */

int   len;                              /* Length */

{

int i;                              /* Counter */

char t;                             /* Character holder */

A KERMIT Program                                                        Page 87

for (i=0; i<len; i++)               /* Loop thru the data field */

{

t = buffer[i];                  /* Get character */

if (t == MYQUOTE)               /* Control quote? */

{                               /* Yes */

t = buffer[++i];            /* Get the quoted character */

if ((t & 0177) != MYQUOTE)  /* Low order bits match quote char? */

t = ctl(t);             /* No, uncontrollify it */

}

if (t==CR && !image)            /* Don't pass CR if in image mode */

continue;

putc(t,fp);

}

}

/*

*  g n x t f l

*

*  Get next file in a file group

*/

gnxtfl()

{

if (debug) printf("   gnxtfl: filelist = \"%s\"\n",*filelist);

filnam = *(filelist++);

if (filecount-- == 0) return FALSE; /* If no more, fail */

else return TRUE;                   /* else succeed */

}

/*

*  s p a r

*

*  Fill the data array with my send-init parameters

*

*/

spar(data)

char data[];

{

data[0] = tochar(MAXPACKSIZ);          /* Biggest packet I can receive */

data[1] = tochar(MYTIME);           /* When I want to be timed out */

data[2] = tochar(MYPAD);            /* How much padding I need */

data[3] = ctl(MYPCHAR);             /* Padding character I want */

data[4] = tochar(MYEOL);            /* End-Of-Line character I want */

data[5] = MYQUOTE;                  /* Control-Quote character I send */

}

A KERMIT Program                                                        Page 88

/*  r p a r

*

*  Get the other host's send-init parameters

*

*/

rpar(data)

char data[];

{

spsiz = unchar(data[0]);            /* Maximum send packet size */

timint = unchar(data[1]);           /* When I should time out */

pad = unchar(data[2]);              /* Number of pads to send */

padchar = ctl(data[3]);             /* Padding character to send */

eol = unchar(data[4]);              /* EOL character I must send */

quote = data[5];                    /* Incoming data quote character */

}

/*

*  f l u s h i n p u t

*

*  Dump all pending input to clear stacked up NACK's.

*  (Implemented only for Berkeley Unix at this time).

*/

#if UCB4X&(~NO_FIONREAD)

flushinput()

{

long int count;                     /* Number of bytes ready to read */

long int i;                         /* Number of bytes to read in loop */

ioctl(ttyfd, FIONREAD, &count);     /* See how many bytes pending read */

if (!count) return;                 /* If zero, then no input to flush */

while (count)                       /* Loop till all are flushed */

{

i = (count<sizeof(recpkt)) ?    /* Read min of count and size of */

count : sizeof(recpkt);     /*  the read buffer */

read(ttyfd, recpkt, i);         /* Read a bunch */

count -= i;                     /* Subtract from amount to read */

}

}

#else

flushinput()            /* Null version for non-Berkeley Unix */

{}

#endif /* UCB4X&(~FIONREAD) */

A KERMIT Program                                                        Page 89

/*

*  Kermit printing routines:

*

*  usage - print command line options showing proper syntax

*  printmsg -  like printf with "Kermit: " prepended

*  error - like printmsg if local kermit; sends a error packet if remote

*  prerrpkt - print contents of error packet received from remote host

*/

/*

*  u s a g e

*

*  Print summary of usage info and quit

*/

usage()

{

#if UCB4X

printf("Usage: kermit c[lbe line baud esc.char]      (connect mode)\n");

printf("or:    kermit s[diflb line baud] file ...    (send mode)\n");

printf("or:    kermit r[diflb line baud]             (receive mode)\n");

#else

printf("Usage: kermit c[le line esc.char]            (connect mode)\n");

printf("or:    kermit s[difl line] file ...          (send mode)\n");

printf("or:    kermit r[difl line]                   (receive mode)\n");

#endif

exit(1);

}

/*

*  p r i n t m s g

*

*  Print message on standard output if not remote.

*/

/*VARARGS1*/

printmsg(fmt, a1, a2, a3, a4, a5)

char *fmt;

{

if (!remote)

{

printf("Kermit: ");

printf(fmt,a1,a2,a3,a4,a5);

printf("\n");

fflush(stdout);                 /* force output (UTS needs it) */

}

}

A KERMIT Program                                                        Page 90

/*

*  e r r o r

*

*  Print error message.

*

*  If local, print error message with printmsg.

*  If remote, send an error packet with the message.

*/

/*VARARGS1*/

error(fmt, a1, a2, a3, a4, a5)

char *fmt;

{

char msg[80];

int len;

if (remote)

{

sprintf(msg,fmt,a1,a2,a3,a4,a5); /* Make it a string */

len = strlen(msg);

spack('E',n,len,msg);           /* Send the error packet */

}

else

printmsg(fmt, a1, a2, a3, a4, a5);

return;

}

/*

*  p r e r r p k t

*

*  Print contents of error packet received from remote host.

*/

prerrpkt(msg)

char *msg;

{

printf("Kermit aborting with following error from remote host:\n%s\n",msg);

return;

}

The ASCII Character Set                                                 Page 91

V. The ASCII Character Set

ASCII Code (ANSI X3.4-1968)

There  are 128 characters in the ASCII (American national Standard Code for In-

formation Interchange) "alphabet".  The characters are listed in order of ASCII

value; the columns are labeled as follows:

Bit             Even parity bit for ASCII character.

ASCII Dec       Decimal (base 10) representation.

ASCII Oct       Octal (base 8) representation.

ASCII Hex       Hexadecimal (base 16) representation.

EBCDIC Hex      EBCDIC hexadecimal equivalent for Kermit translate tables.

Char            Name or graphical representation of character.

Remark          Description of character.

The first group consists of nonprintable 'control' characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   000   000   00   00    NUL   ^@, Null, Idle

1   001   001   01   01    SOH   ^A, Start of heading

1   002   002   02   02    STX   ^B, Start of text

0   003   003   03   03    ETX   ^C, End of text

1   004   004   04   37    EOT   ^D, End of transmission

0   005   005   05   2D    ENQ   ^E, Enquiry

0   006   006   06   2E    ACK   ^F, Acknowledge

1   007   007   07   2F    BEL   ^G, Bell, beep, or fleep

1   008   010   08   16    BS    ^H, Backspace

0   009   011   09   05    HT    ^I, Horizontal tab

0   010   012   0A   25    LF    ^J, Line feed

1   011   013   0B   0B    VT    ^K, Vertical tab

0   012   014   0C   0C    FF    ^L, Form feed (top of page)

1   013   015   0D   0D    CR    ^M, Carriage return

1   014   016   0E   0E    SO    ^N, Shift out

0   015   017   0F   0F    SI    ^O, Shift in

1   016   020   10   10    DLE   ^P, Data link escape

0   017   021   11   11    DC1   ^Q, Device control 1, XON

0   018   022   12   12    DC2   ^R, Device control 2

1   019   023   13   13    DC3   ^S, Device control 3, XOFF

0   020   024   14   3C    DC4   ^T, Device control 4

1   021   025   15   3D    NAK   ^U, Negative acknowledge

1   022   026   16   32    SYN   ^V, Synchronous idle

0   023   027   17   26    ETB   ^W, End of transmission block

0   024   030   18   18    CAN   ^X, Cancel

1   025   031   19   19    EM    ^Y, End of medium

1   026   032   1A   3F    SUB   ^Z, Substitute

0   027   033   1B   27    ESC   ^[, Escape, prefix, altmode

1   028   034   1C   1C    FS    ^\, File separator

0   029   035   1D   1D    GS    ^], Group separator

0   030   036   1E   1E    RS    ^^, Record separator

1   031   037   1F   1F    US    ^_, Unit separator

The last four are usually associated with the  control  version  of  backslash,

right  square  bracket,  uparrow (or circumflex), and underscore, respectively,

The ASCII Character Set                                                 Page 92

but some terminals do not transmit these control characters.

The following characters are printable:

First, some punctuation characters.

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

1   032   040   20   40    SP    Space, blank

0   033   041   21   5A    !     Exclamation mark

0   034   042   22   7F    "     Doublequote

1   035   043   23   7B    #     Number sign, pound sign

0   036   044   24   5B    $     Dollar sign

1   037   045   25   6C    %     Percent sign

1   038   046   26   50    &     Ampersand

0   039   047   27   7D    '     Apostrophe, accent acute

0   040   050   28   4D    (     Left parenthesis

1   041   051   29   5D    )     Right parenthesis

1   042   052   2A   5C    *     Asterisk, star

0   043   053   2B   4E    +     Plus sign

1   044   054   2C   6B    ,     Comma

0   045   055   2D   60    -     Dash, hyphen, minus sign

0   046   056   2E   4B    .     Period, dot

1   047   057   2F   61    /     Slash

Numeric characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   048   060   30   F0    0     Zero

1   049   061   31   F1    1     One

1   050   062   32   F2    2     Two

0   051   063   33   F3    3     Three

1   052   064   34   F4    4     Four

0   053   065   35   F5    5     Five

0   054   066   36   F6    6     Six

1   055   067   37   F7    7     Seven

1   056   070   38   F8    8     Eight

0   057   071   39   F9    9     Nine

More punctuation characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   058   072   3A   7A    :     Colon

1   059   073   3B   5E    ;     Semicolon

0   060   074   3C   4C    <     Left angle bracket

1   061   075   3D   7E    =     Equal sign

1   062   076   3E   6E    >     Right angle bracket

0   063   077   3F   6F    ?     Question mark

1   064   100   40   7C    @     "At" sign

The ASCII Character Set                                                 Page 93

Upper-case alphabetic characters (letters):

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   065   101   41   C1    A

0   066   102   42   C2    B

1   067   103   43   C3    C

0   068   104   44   C4    D

1   069   105   45   C5    E

1   070   106   46   C6    F

0   071   107   47   C7    G

0   072   110   48   C8    H

1   073   111   49   C9    I

1   074   112   4A   D1    J

0   075   113   4B   D2    K

1   076   114   4C   D3    L

0   077   115   4D   D4    M

0   078   116   4E   D5    N

1   079   117   4F   D6    O

0   080   120   50   D7    P

1   081   121   51   D8    Q

1   082   122   52   D9    R

0   083   123   53   E2    S

1   084   124   54   E3    T

0   085   125   55   E4    U

0   086   126   56   E5    V

1   087   127   57   E6    W

1   088   130   58   E7    X

0   089   131   59   E8    Y

0   090   132   5A   E9    Z

More punctuation characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

1   091   133   5B   AD    [     Left square bracket

0   092   134   5C   E0    \     Backslash

1   093   135   5D   BD    ]     Right square bracket

1   094   136   5E   5F    ^     Circumflex, up arrow

0   095   137   5F   6D    _     Underscore, left arrow

0   096   140   60   79    `     Accent grave

The ASCII Character Set                                                 Page 94

Lower-case alphabetic characters (letters):

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

1   097   141   61   81    a

1   098   142   62   82    b

0   099   143   63   83    c

1   100   144   64   84    d

0   101   145   65   85    e

0   102   146   66   86    f

1   103   147   67   87    g

1   104   150   68   88    h

0   105   151   69   89    i

0   106   152   6A   91    j

1   107   153   6B   92    k

0   108   154   6C   93    l

1   109   155   6D   94    m

1   110   156   6E   95    n

0   111   157   6F   96    o

1   112   160   70   97    p

0   113   161   71   98    q

0   114   162   72   99    r

1   115   163   73   A2    s

0   116   164   74   A3    t

1   117   165   75   A4    u

1   118   166   76   A5    v

0   119   167   77   A6    w

0   120   170   78   A7    x

1   121   171   79   A8    y

1   122   172   7A   A9    z

More punctuation characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   123   173   7B   C0     {    Left brace (curly bracket)

1   124   174   7C   4F     |    Vertical bar

0   125   175   7D   D0     }    Right brace (curly bracket)

0   126   176   7E   7E     ~    Tilde

Finally, one more nonprintable character:

0    127   177  7F   07    DEL   Delete, rubout

Index                                                                   Page 95

Index

8th Bit   5, 27                           NAK   7, 36

Normal Form for File Names   15

ACK   7

ASCII   6, 10, 91                         Packet   7, 20

Parity   22, 26, 91

Baud   8                                  Prefix   27, 30

Binary Files   9, 10                      Prefixed Sequence   28

Binary Mode   8                           Printable Files   10

Bit Positions   5                         Program, Kermit   56, 65

Block Check   21, 22                      Protocol   3

Bootstrap   59

BREAK   55                                Raw Mode   8

Records   10

Capabilies   25                           Remote   5, 8

CAPAS   25                                Repeat Prefix   27

Char(x)   6                               Reserved Fields   25

Checksum   21

Control Character   6                     Send-Init   23

Control Characters   20, 91               Sequence Number   12

Control Fields   22                       Sequential Files   3

Ctl(x)   6                                Server   5

Server Command Wait   28

Data Encoding   22                        Server Commands   32

DEFINE   53                               Server Operation   28

Duplex   8                                Short Reply   31

SOH   8

EBCDIC   8, 10, 91

Edit Number   58                          Tab Expansion   10

Encoding   27, 30                         Text Files   10

End-Of-Line (EOL)   8, 21                 Timeout   7

Errors   14                               Transaction   12

Transaction Log   16

Fatal Errors   14                         TTY   5

File Names   15

Flow Control   8, 16                      Unchar(x)   6

Full Duplex   8                           User   5

GET Command   31                          XON/XOFF   8, 16, 91

Half Duplex   8

Host   5

Initial Connection   23

KERMIT   3

Language, Programming   58

Line Terminator   21

Line Terminator (see End-Of-Line)

Local   5

Logical Record   10

Logical Records   10

Long Reply   31

Table of Contents                                                        Page i

Table of Contents

1. Introduction                                                               3

1.1. Background                                                               3

1.2. Overview                                                                 3

2. Definitions                                                                5

2.1. General Terminology                                                      5

2.2. Numbers                                                                  5

2.3. Character Set                                                            6

2.4. Conversion Functions                                                     6

2.5. Protocol Jargon                                                          7

3. System Requirements                                                        8

4. Printable Text versus Binary Data                                         10

4.1. Printable Text Files                                                    10

4.2. Binary Files                                                            10

5. File Transfer                                                             12

5.1. Conditioning the Terminal                                               13

5.2. Timeouts, NAKs, and Retries                                             13

5.3. Errors                                                                  14

5.4. Heuristics                                                              14

5.5. File Names                                                              15

5.6. Robustness                                                              16

5.7. Flow Control                                                            16

5.8. Basic KERMIT Protocol State Table                                       17

6. Packet Format                                                             20

6.1. Fields                                                                  20

6.2. Terminator                                                              21

6.3. Other Interpacket Data                                                  21

6.4. Encoding, Prefixing, Block Check                                        22

7. Initial Connection                                                        23

8. Optional Features                                                         27

8.1. 8th-Bit and Repeat Count Prefixing                                      27

8.2. Server Operation                                                        28

8.2.1. Server Commands                                                  29

8.2.2. Timing                                                           30

8.2.3. The R Command                                                    31

8.2.4. The K Command                                                    31

8.2.5. Short and Long Replies                                           31

8.2.6. Additional Server Commands                                       32

8.2.7. Host Commands                                                    34

8.2.8. Exchanging Parameters Before Server Commands                     34

Table of Contents                                                       Page ii

8.3. Alternate Block Check Types                                             34

8.4. Interrupting a File Transfer                                            36

8.5. Transmitting File Attributes                                            37

8.6. Advanced KERMIT Protocol State Table                                    43

9. KERMIT Commands                                                           48

9.1. Basic Commands                                                          48

9.2. Program Management Commands                                             48

9.3. Terminal Emulation Commands                                             49

9.4. Special User-Mode Commands                                              49

9.5. Commands Whose Object Should Be Specified                               49

9.6. The SET Command                                                         51

9.7. Macros, the DEFINE Command                                              53

10. Terminal Emulation                                                       54

11. Writing a KERMIT Program                                                 56

11.1. Program Organization                                                   56

11.2. Programming Language                                                   58

11.3. Documentation                                                          58

11.4. Bootstrapping                                                          59

I. Packet Format and Types                                                   60

II. List of Features                                                         61

III. Unresolved Issues                                                       63

IV. A KERMIT Program                                                         65

V. The ASCII Character Set                                                   91

Index                                                                        95