CHAPTER 1

BUILD COMMAND IMPLEMENTATION NOTES

1.1  ABSTRACT

This  distribution  contains a Fortran program which is

implemented as a command to simplify the  use  of  the  task

builder  options.   Most  of the task builder options can be

supplied on a single line.  The program parses  the  command

line  to determine options, libraries, etc., and generates a

task builder command file, then spawns the task builder.  It

may  optionally  compile the program first too, and will use

the fast task builder if it thinks it is possible.  Commonly

used  libraries and resident libraries are automatically in-

cluded.  This program is considerably faster and more flexi-

ble than some of the command file driven parsers.

It  should work on any recent RSX-11M or M-Plus system,

with either Fortran-IV or Fortran-77.

1.2  AUTHOR

If  you  have  any  (polite)  comments  or suggestions,

please feel free to contact me.

Dr. David Villeneuve

Division of Physics M23A

National Research Council

Ottawa Ont.   K1A 0R6

Canada

613-993-9975

BUILD COMMAND IMPLEMENTATION NOTES                  Page 1-2

This  program  is the property of the Government of Ca-

nada, and as such may not be sold  or  copied  commercially,

etc., etc.

1.3  IMPLEMENTATION

The  syntax  of  the  command  is  given  in the second

chapter of this writeup.  This writeup  is  taken  from  the

users  guide  for  our system, so a lot of it is specific to

our site, in particular where we keep our libraries.

The  program retrieves its command line, parses it into

either positional or keyword parameters, and creates a  task

builder command file.  The syntax comes from an little known

IBM operating system, TSS, but it seems to be  quite  effec-

tive here where you use the first two parameters most of the

time, but use the keyword defined ones less often.

The  program then spawns the task builder (choosing FTB

if possible, i.e.  no CLSTR), and then STOPFRs, which  means

that  it  does not compete for memory while the task builder

is active, and only comes back when the task builder is fin-

ished.  It then deletes the command file, and optionally the

object file if it compiled first.

The  program  will  be generated by the BUI.CMD command

file, which will ask for which Fortran compiler you are  us-

ing,  where  the  Fortran  OTS  library  is, and if you have

FCSRES.  If you also have a FORRES (as we do), you will have

to  modify BUI.CMD accordingly.  On our system, with Fortran

IV, FCSRES and FORRES, this task is only  3456  words  long.

The BUI.CMD file works in any UIC, and does not require pri-

vilege unless it needs to install a utility.

It is recommended that you install BUI, rather than use

CCL, since it is one of the most commonly used tasks on  our

system.

You  will  almost certainly have to modify the code, in

BUI.FTE, before you compile it, since parts of it are imple-

mentation  dependent,  particularly where you put libraries.

This should not be too difficult since the code is  modular.

Note that it is coded in an extended Fortran, the preproces-

sor of which is included.  If you know any structured  lang-

uages,  it  should  be  clear  to you, otherwise look at the

writeup in FTE.FTN.

BUILD COMMAND IMPLEMENTATION NOTES                  Page 1-3

Subroutine  SECT contains the keywords which are recog-

nized.  You can add or modify these as you wish.  Subroutine

BUILD1  generates  the part of the task builder command file

up to the slash;  you should modify this  part  to  link  to

your most common libraries automatically (as our FTNLIB is),

and to translate the leading $ to your system account.   Su-

broutine  BUILD2  generates  the options part of the command

file, and you will probably want to change the resident  li-

brary choices.

Note  that the command added after the BUI command name

must by recognized by your system, and must accept a  single

parameter consisting of the file name without extension.  In

our system we use  CCL  to  understand  commands  like  FTN,

FTNLIST,  FTNCREF.   If  you don't have this, you can modify

the part where the compiler is spawned, in subroutine TKB.

CHAPTER 2

BUILD COMMAND SYNTAX

This chapter comes from the users guide for our system,

so a lot of stuff will make no sense on yours,  particularly

where  we keep libraries and what resident libraries we sup-

port.  However it describes how the command works,  and  may

give  you some ideas on how it should be customized for your

system.

2.1  BUILD COMMAND

The task builder is a complicated program which is des-

cribed fully in the DEC manuals.  It allows a number of  ob-

ject  modules  to  be  bound  together  from object files or

libraries.  It has options which set the number of I/O units

required by the program, etc.

The  BUILD  command  is designed to simplify the use of

the task builder.  It produces a command file which is given

to  the  task  builder, which then follows the contained in-

structions.  If the BUILD command doesn't provide  the  fea-

tures  you  require,  you  may  have to use the task builder

directly.  The full BUILD syntax is:

BUILD FILE=(...), LIB=(...), MAP=Y, PRI=n, ACTFIL=n,

MAXBUF=n, FMTBUF=n, COMMON=(...), LIBR=(...),

CLSTR=(...), ASG=(...), UNITS=n, TASK=name,

SIZE=n, DEBUG=Y

or  BUILD/compiler ...

The  syntax allows arguments to be specified either po-

sitionally, or by specifying the keyword  of  the  argument.

If more than one item is specified for an argument, they are

enclosed in parentheses and separated by  commas.   If  more

than  one  line  is required, a hyphen is placed as the last

BUILD COMMAND SYNTAX                                Page 2-2

character of the first  line,  and  a  second  line  may  be

entered.  Examples of BUILD commands are shown at the end of

this section, and serve to illustrate the syntax.

FILE =    list of input files.  The first one named is to be

the name of the task file, and must  exist  as  an

object  file.   The  others may be object files or

object modules extracted from libraries,  such  as

HPALIB/LB:SWCHAR:SPACE1.   Options may be appended

to the first name, such as /DA for  debugging  aid

(ODT), or /PR for privileged.

LIB =     list  of  object  libraries to be searched.  These

libraries will be  searched  for  required  object

modules  in  the  order  they are listed.  If your

current account has a USERLIB.OLB, it is automati-

cally  included  at the top of the list.  The gen-

eral Fortran library SF:[5,3]FTNLIB is also always

included between USERLIB and the listed libraries.

If a library name is preceeded by $, the specified

library  will be found in system account SF:[5,3],

such as NPLIB, HPLIB, INENLIB, and CAMACLIB.

MAP = Y   If  this  is  specified, a map file is produced by

the task builder to show how the task was put  to-

gether.  This may be printed.

SIZE =0-2 Determines  the  maximum  size of the task through

the number of resident libraries  which  it  links

to.   SIZE=0 is the default, and uses two resident

libraries to produce the most compact  task  size;

the maximum task size is 24K (not counting all the

code in the resident libraries).  SIZE=1 uses  the

same two libraries but "clustered" so they use the

same address space;  the maximum task size is  now

28K;   the  larger available task size is slightly

offset by somewhat  slower  execution  and  longer

task build time.  SIZE=2 does not use any resident

libraries, so that all the code  is  contained  in

the task image;  the maximum task size is 32K, but

since all the code that was in  the  resident  li-

braries  is now in the task, this never results in

more space for the program;  it also takes  longer

to task build.

PRI =     decimal  number defining the priority given to the

task when it is run.  The default is 50.  If  your

task requires a lot of CPU time, be kind to others

and set its priority down, say to PRI=8.

BUILD COMMAND SYNTAX                                Page 2-3

ACTFIL =  the  number  of  files  which will be open concur-

rently in a Fortran program.  Default  is  4,  but

reducing this number will save space.

MAXBUF =  maximum  buffer  length  allowed in a Fortran pro-

gram.  If you use files with long records, you may

have to increase this.  Default is 132 bytes.

FMTBUF =  length  of  the  buffer  used  by  run time format

statements.  Default is 64 bytes.

COMMON =  list  of  external  common blocks to be linked to.

This permits several independent  tasks  to  share

data,  such as COCO-II tasks through COCOCM.  They

must each be followed by the access attribute, ei-

ther RO or RW.  For example COMMON=COCOCM:RO.

LIBR =    list  of  core resident libraries to be linked to.

We have FORRES, which contains much  of  the  code

used  by Fortran tasks, and FCSRES, which contains

the file control services.   These  libraries  are

used  automatically, and reduce each task by about

4K words.  If  this  option  is  explicitly  used,

FORRES and FCSRES will not be referenced automati-

cally.

CLSTR =   list  of  core resident libraries to be linked to.

Unlike LIBR however, the libraries are "clustered"

together  so they use the same address space.  See

SIZE=1 above and the task builder manual.

UNITS =   number  of  logical  units  available for use by a

Fortran program.  Default is 6.

TASK =    name  of task when it is installed, specified as 6

characters.

ASG =     list  of  device  assignments  at task build time.

These may be  changed  at  run  time,  using  CALL

ASNLUN.  Default is ASG=(SY:1:4,TI:5,CL:6).

DEBUG = Y is  specified  if you wish to see the command file

produced by BUILD.  Useful if you  wish  to  learn

about the task builder.

This  full description appears complicated, but most of

the options are rarely used.  Some examples are shown below:

BUILD COMMAND SYNTAX                                Page 2-4

BUILD MYPROG

Uses  the  object  file MYPROG.OBJ to build task MYPROG.TSK.

Automatically searches USERLIB.OLB if the user has one,  and

SF:[5,3]FTNLIB,  and  automatically  saves  4K  of memory by

linking to FORRES and FCSRES resident libraries.

BUILD MYPROG,$PFLIB

As  above,  but also searches SF:[5,3]PFLIB for the plotting

routines.

BUILD MYPROG,($INENLIB,$NPLIB)

As  above,  but searches SF:[5,3]INENLIB before NPLIB.  Note

the use of parentheses to include more than one  library  in

the LIB section.

BUILD (MYMAIN,MYSUB)

Uses  two object files MYMAIN.OBJ and MYSUB.OBJ to build the

task.  Presumably MYMAIN contains the main program, and  MY-

SUB the subroutines.

BUILD MYPROG,$HPLIB,PRI=8,ACTFIL=2,UNITS=5,DEBUG=Y

Shows  a  mixture  of  positional location of arguments (the

first two) and keyword location (the rest).

The  error message "Task has illegal memory limits" me-

ans that the program is too large.   Try  SIZE=1  or  reduce

some arrays.  As a last resort, the subroutines may be over-

laid, but this is complicated.  The command SIZE myprog will

show the task size.

The  build  command can also compile your program first

if you wish;  this save entering an extra line.  To  do  so,

at  the  end  of the BUILD command name, add a slash and the

command name you would normally use to compile your program,

e.g.,  FTN  or FTCLIST, etc.  For example, BUILD/FTN myprog.

The object file will be deleted at the end.  If the compiler

returns  severe error messages, the task build phase will be

skipped.

Fortran 77 users have a slightly different command name

which must include the numbers "77" somewhere in  the  name,

for  example BUILD77, BUI77, or BUILD/FTN77.  This will then

task  build  using  the  Fortran  77  runtime  library,  and

FTNLIB77  instead of FTNLIB.  Any other libraries you choose

should also be the Fortran  77  version,  such  as  PFLIB77.

Also,  since we don't have a Fortran 77 resident library, it

will only link to FCSRES, so that maximum task size  is  28K

words,  and  there  is  no  difference between SIZE=0 and 1.

Generally you will not be able to build as large  a  program

BUILD COMMAND SYNTAX                                Page 2-5

with Fortran 77 as with Fortran IV.

If  you  require a complicated BUILD command for a par-

ticular program, you may imbed the command in  the  comments

at the beginning of the source program.  The line must start

with (exactly) C--BUILD with the rest of the line containing

the  usual options;  a compiler name is not allowed.  To use

this feature, issue a BUILD command (with a compiler name if

desired)  and  the name of the source file with a preceeding

at-sign.  If the extension is not given, it will try FTN and

then FTE.  For example, BUILD @MYPROG, BUI/FTC @HISPROG.FTE.