Instructions for TWOPAGE

The following proceedure can be used to produce manuals from ru-

noff  text.   The resultant manual will be printed on both sides

of the 9&1/2 X 11 paper with  perforated  edges.   The  sprocket

feed  edges  can  be  easily  removed to produce a standard size

8&1/2 X 11 manual.

1.  Runoff source should specify the following:

1.  .LEFT MARGIN 10

2.  .RIGHT MARGIN 74

3.  .PAPER SIZE 58,74

4.  The output file should  have  the  standard  ".DOC"

file type specification

2.  After using RNO to produce the "XXXXXX.DOC"  file,  run

the basic program "TWOPAGE" via the command line:

BA3 DP:[1,1]TWOPAGE/RN

"TWOPAGE will do the following:

1.  Seperate  the  "XXXXXX.DOC"  file  into  the  files

"XXXXXX.LFT" and "XXXXXX.RGT"

2.  Append another "FF" character  to  "XXXXXX.LFT"  if

necessary to make the total number of pages of each

file equal.

3.  Change the printer you wish to use to FORM TYPE  2  (to

ensure  that  no other output is queued to it,) via the

command:

OPR LP1:/FO:2

4.  Load the 9&1/2 X 11 paper in  the  printer  and  verify

printer is at TOP-OF-FORM with paper properly aligned

5.  Mark the starting page so one can return to that  point

after printing the "right-hand" pages.

6.  QUE LP1:/FO:2=XXXXXX.RGT(/CO:N)   (number of copies  is

optional)

7.  When  printing  of  right  pages  is  finished,  Remove

printer output with 1 or 2 extra blank pages

Instructions for TWOPAGE                                  PAGE 2

8.  Align paper to same fan-fold as in step #4 above

9.  QUE LP1:/FO:2=XXXXXX.LFT(/CO:N)   (same number  of  co-

pies as above)

10. After "left-hand" pages have finished, re-load  printer

with standard paper.

11. Return printer to standard FORMS TYPE via the command:

OPR LP1:/FO:2

Frank R. Borger

Michael Reese Medical Center

Department of Medical Physics

March 20, 1978