Author: Martin Heller, New England Nuclear Corporation

Modified by: Glenn Everhart, RCA ,F. Borger, MRHMC

Date: 24-Oct-79

Date: 23-March-1983

Date: 15-June-1984

Last source edit: 15-June-83

Source version: 1.20

Documentation edit date: 15-June-83

Purpose:  to provide the capability of having two computers  talk

to one another over serial lines, while having access to file fa-

cilities.

Operating System:  IAS

Source Language:  Macro-11

Units:

1.       terminal at which user is typing

2.       line to another computer

3.       file to be transmitted

4.       RECEIVED.DAT

DUPLEX Documentation                                       PAGE 2

Usage:

1.  Define logical units

2.  Hook up cables and/or dial other computer

3.  Set characteristics of terminal ports as needed

4.  Run DUPLEX

5.  Type away -- you will effectively be talking to the oth-

er computer

6.  Use control characters as needed to control DUPLEX

Control characters used:

1.  Control-B:  exit from DUPLEX.  Terminal talks to MCR.

2.  Control-T:  transmit a file.  Program will prompt for an

input          file          specification,         i.e.

DUPLEX>=SY0:[100,200]FileName.Typ;4.  The equals sign is

necessary for the file to be recognized as input.

3.  Control-P:  toggle between line-by-line  and  whole-file

transmission.

4.  Control-N:  transmit one line in line-by-line  transmis-

sion mode.

5.  Control-A:  abort transmission of a file.

6.  Control-F:  open/close file RECEIVED.DAT  of  characters

received  from unit 2.  This file is also closed by Con-

trol-B.

7.  Control-]:  Sets up line delay value by reading 1  char-

acter  and subtracting 48.  from it (ASCII 0).  If nega-

tive sets line delay to 4 ticks.

8.  Control-\:  Toggles line delay off/on.  In line mode, if

line  delay  is  on, the file is transmitted 1 line at a

time but with automatic proceeding after  "delay"  ticks

between  lines.  This defaults to 4 ticks but may be set

by control-] to anything reasonable.

9.  Control-Y:  Toggles character delay.  This will insert a

1  tick  delay  between sending characters if set.  Note

this forces single character QIO$s and is  expensive  of

CPU time, however is provided for remote computers where

the delay is needed.  When turned off,  transmission  is

by  qio$  of  a  line at a time.  The line delay is more

likely to be useful for general purposes.

DUPLEX Documentation                                       PAGE 3

Internal mechanics of program:  The program initializes  its

buffers,  attaches  units  1 and 2, and declares AST handlers for

both units.  It then clears event flag 3 and waits for event flag

3 to be set.  A character at either port will be delivered to the

AST handler.  Normal characters will be queued for output to  the

other  port;   special  control  characters  will  cause  the AST

handler to set event flag 3, mark the appropriate words to vector

the  main  program, and exit.  When the main program regains con-

trol it will perform the required control function,  clear  event

flag  3,  and return to a wait state.  Control-B, of course, will

cause the main program to clean up  outstanding  I/O  operations,

close  open files, and exit, returning control of the terminal to

MCR.

Notes:  Custom tailoring of the program can be done most ef-

fectively  by  adding to the list of special characters in one or

the other of the AST handlers.  Special functions can be  indexed

through  TABLE,  with indexing passed from the AST handler to the

main program through SENTIN.  CIRSIZ and BUFSIZ could be enlarged

if  necessary,  although  on my PDP-11/34 they are sufficient for

9600 baud operation under normal system loads.

Once DUPLEX is running, the user's terminal should remain at

its  normal  speed and parity, but should be set for full or half

duplex (no echo or echo) appropriate to the computer on  the  re-

mote  port.   The  program's  default settings are appropiate for

talking to DEC computers;  when addressing IBM systems, a  single

control-P  at run time will set file transmission to line-by-line

mode.  When talking to a VAX, set the VAX port to have  HOSTSYNC,

and  the  control-s  and  control-q mechanism will keep both com-

puters from overflowing their buffers.  It is  also  possible  to

activate  the  RECEIVED.DAT  file  and then to control-O your own

terminal, if you don't want to waste paper.

DUPLEX has been used successfully with computers  hard-wired

and  connected  via  modems and ordinary phone lines.  It has had

both ports at the same speed, and  the  ports  set  to  different

speeds.   DEC, IBM, CDC, and even Motorola systems have been suc-

cessfully interfaced to PDP-11's.  File transfer  from  micropro-

cessors  has  turned  out to be highly useful:  the micro is then

given the advantage of hard disks, high speed printers, and other

mini-computer  peripherals,  albeit  via a 9600 baud serial line.

The incoming Bell 103 on a DZ-11 port has turned out to  function

perfectly well as an outgoing modem port.  There is no reason au-

to-dialing and such features should not work without any  changes

to the software.

For further documentation, please refer to the source  list-

ing.

Note:  DUPLEXX should normally be run with a fast  terminal.

If  your  console  is slower than (or even the same speed as) the

remote line, it can behave strangely.   Speed  combinations  have

not  been  well  investigated but running a 1200 baud line from a

9600 baud terminal works well.  Be careful of buffered I/O.   You

DUPLEX Documentation                                       PAGE 4

may want to run Duplex noncheckpointable and/or at higher priori-

ty than usual.  It's I/O bound so high priority is normally not a

bad  idea.  Making it non- checkpointable improves AST throughput

and (on older RSXes) allows multiple outstanding QIOs.