DL Device CSRs

Author:	Richard Neitzel

312 Laveta Pass

Golden, CO 80401

As you know, DL devices, especially the DLV-11J, now the

DLVJ-1, have a problem in losing interrupts when the interrupt enable

bit in the receive control status register (rcsr) is cleared. This

hardware flaw means that your DL line suddenly goes dead for input. If

like us you are using it for serial communications this is a serious

problem, since normally the only recourse is to reboot. Fortunately,

you can directly access the rcsr and test the interrupt enable bit

(ieb). If it is zero, the ieb can be set to restart the line.

Unfortunately, it must be remembered that the operating system

has no knowledge of the DL locking up, which in turn means that your

program gets no indication of the problem either. As result, you must

be very careful. Issuing a Fortran read or a qiow directive means that

the program will sit forever waiting on the DL. The qiow directive can

specifiy a time out, but you won't know why it occured. Similarly, a

qio will continue to show an I/O status of zero forever, so testing

the status as a means of determining completion is risky. Use time outs

again. When a time out occurs, in order to determine what happened the

easiest thing to do is to spawn a program similar to CSET.MAC, which

returns a status that tells if the interrupt bit was set. If it was

not then you know why the time out occurred and can act accordingly.

The routines supplied to 'diddle' the ieb are as follows:

set.mac - Set the ieb.

clr.mac - Clear the ieb.

cset.mac - Check ieb, set if clear and return status.

conset.mac - Continuously check and set ieb if clear.

They all are coded for the system console, but it's trival to change

them to work for any other terminal. Have fun!