RTR - RSTS to RSX Transfer

==========================

Users' Guide

============

1. Introduction

===============

RTR  is  a small program which provides two facilities.  The first is

to  allow  a  limited  form  of  copying files from a RSTS disk to an

RSX-11M disk, without having to bother with shutting down the RSX-11M

system and transferring the files to tape first.  The second facility

allows  the  user  to  examine  a (possibly corrupted) RSTS directory

structure - in particular allowing him to bypass the MFD in accessing

the UFD.

2. Command-Line Format

======================

For  all  purposes the RSTS/E disk must be loaded onto a drive on the

RSX-11M system, but not mounted.  The command then takes the standard

form of:-

RTR <output-filespec=><input filespec><switches>

The format of files as held on a RSTS/E disk are completely different

to  those  on  an  RSX-11M disk (unless they are RMS files) and, even

worse,  they  are not as well documented.  Thus it helps to know what

sort  of  file  you  are  copying  before asking RTR to copy it.  The

format of a copy command is:-

RTR <output-filespec>=<input-filespec><switches>

where the input-filespec may contain wildcards in UIC and/or filename

the output-filespec  may  contain just a disk/UIC in which case

the  input  filename(s)  are  used  for the new files, or a

filename  as  well in which case (all) the input file(s) is

copied into that filename

switches are discussed below

RSTS/E  disks  are  very  fussy  about their directory structure, and

should one link break are often thereafter unusable from RSTS itself.

RTR  attempts  both  to  list out RSTS directories, and to circumvent

them where possible.

The standard command-line for listing a directory is:-

RTR	<output-filespec=><input-filespec><switches>

where the output-filespec,  if  given,  is where the listing is to be

put

the input-filespec  may  contain  just  a  directory  (possibly

wildcarded) or a filename (possibly wildcarded) as well.

switches are discussed below

3. Switches

===========

By  default  files  are transferred as variable-length records, where

any   linefeed  or  carriage-return  character  (or  any  combination

thereof)  is  regarded  as a record terminator. This is OK except for

BASIC-PLUS files which use linefeed as a continuation marker, and the

/BA  switch  simply  transfers  them  in  a format that can be passed

through  the  BASIC-PLUS-2  Translator  Utility  (after going through

FORMAT  first).  It  simply  regards each record of the input file as

terminating  at a carriage-return/linefeed pair, and passes all other

carriage-returns and linefeeds through unchanged.

/IM is used for copying files in Image Mode - i.e. copying the file a

block  at  a  time, and creating (as output) a fixed-length 512. byte

file.  This  switch is particularly useful for copying RMS-11 Indexed

Files  -  where  most of the control information resides in the first

few  blocks - but in that case it is vital to update the RSX-11M File

Header  with  the  special RMS-11K values (RTR is not yet intelligent

enough to do that).

/DI  produces  a  listing of the MFD - that is, a listing of each UFD

followed  by  the  password  for  the UFD. Note that as RSX-11M is an

octal  system  the  UFD  numbers  are output in octal rather than the

original  decimal.  Naturally this will only work if the directory is

not corrupt.

/LI  is  used  to produce a listing of the contents of a given UFD or

UFDs as specified in the commandline. The listing will be output into

a  file,  if  one was specified, or on the terminal otherwise. If the

UFD  links  are corrupt, RTR will list as far as it can and then give

up.

All  UFD  Headers in the MFD are linked together in a list.  Normally

the  way  to find a UFD is to start at the first such link and follow

the  list through.  However a common accident that occurs to a RSTS/E

disk  is  losing one of the MFD blocks, and as the linked list twists

and turns from block to block that virtually guarantees that the link

will  be  broken  very early on.  However, in such cases, most of the

UFDs (and their files) are usually intact. If /NM:xxxxxx is specified

the  RTR  will simply check each blockette in the MFD in turn looking

for  one  which  matches  the  UFD  given  in  the UFD field, and the

password  given  (xxxxxx)  in  the  password  field.  The password is

required to guard against corruption of the MFD producing a blockette

which looks like the desired UFD header.

/NM  may  be  used either in a directory listing, or a file transfer,

but obviously only when a single UFD is specified.

/SP  requests  that  the  output file be spooled when finished. It is

usually  used  with  a directory listing, and is ignored if no output

file was specified.

4. Error Messages

=================

RTR may produce the following error messages:-

1.  RTR - CMD READ ERROR

The input command-line could not be read succesfully.

2.  RTR - CMD SYNTAX ERROR

An  error  was  detected  in  the  syntax  of  the input command.

Correct it and respecify the line.

3.  RTR - ERROR IN INPUT SPEC

The  input  filespec  was  invalid  (possibly  a  decimal UFD was

specified).  Correct the mistake and respecify the line.

4.  RTR - ERROR IN OUTPUT SPEC

The  output  filespec  was  invalid.    Correct  the  mistake and

respecify the line.

5.  RTR - PARSE ERROR

The output filespec could not be parsed successfully (possibly an

invalid UIC).  Correct the mistake and respecify the line.

6.  RTR - OPEN ERROR ON OUTPUT

An error occurred while trying to open the output file.

7.  RTR - NO SUCH FILE(S)

No files could be found to match the specification the user gave.

8.  RTR - ERROR = nnnnn WHILE READING DISK

RTR  detected an error of some kind while reading the input disk.

Determine  from the error code given what the problem is and take

any possible action.

9.  RTR - Pack is not a RSTS-E Pack!

RTR  could not find the block signalling the start of the MFD and

so has concluded that the pack is not a valid RSTS/E pack.

RTR - RSTS to RSX Transfer

==========================

System Guide

============

RTR is written as one program, split into several logical 'modules' as follows:-

1.  RTR

=======

This  is  the  main  routine.    It  calls GETCMD to get the next

command-line.  If /NM was specified it sets up default values for

searching  the  disk, and if not it calls GETMFD to find the MFD.

If  a copy was requested it then calls FILCPY and returns for the

next command.  If an MFD list was requested it then calls LSTMFD.

If  a  UFD  list was requested it then calls LSTUFD.  If anything

was listed, and /SP was specified, the output file is spooled and

then the routine returns to get the next command.

2.  GETCMD

==========

GETCMD  merely requests and parses a commandline via GCML and CSI

in  a  virtually  standard  format.  The only major difference is

that  the  input  filespec  is  parsed  as  three  separate parts

(device,  UIC  & filename) as the disk will be accessed by direct

QIOs and the UIC may contain wildcards.

3.  GETMFD

==========

The  first  block of the MFD starts in Cluster 1 of the disk (the

bootstrap  block  starts in cluster 0) but it is not easy to tell

what the cluster-size of a disk is, so this routine simply checks

the  first  few  (10) blocks until it finds one whose second word

(the MFD ID) is -1 and assumes that to be the start of the MFD.

4.  GETUFD

==========

This routine calls NXTUFD to get the next UFD entry and checks to

see  if it is one the user requested.  If not it branches back to

get  the  next  one  (exiting  with  an indicator if there are no

more).    Else it reads in the first UFD block and returns to the

caller.

5.  LSTMFD

==========

This  routine  simply  repetitively calls NXTUFD to find the next

UFD in the MFD, and outputs a line for each.

6.  LSTUFD

==========

This  routine  simply  repetitively calls GETUFD to find the next

UFD  that  the  user wanted, and for each such repetitively calls

NXTFIL  to  find  the next file in the UFD, and if it matches the

user's  specifications  outputs a line describing it.  At the end

of  each  UFD  it outputs a line totalling up the number of files

and blocks allocated in the UFD.

7.  FILCPY

==========

FILCPY is similar to LSTUFD in finding each file to copy.  Having

done  so it then copies all the data from that file to the output

device via OUTREC.

8.  OUTREC

==========

This  routine  simply outputs the next record to the output file,

depending on which (if any) of the switches /BA and /IM have been given.

9. NXTUFD

=========

If  /NM  was  specified,  this  routine  merely  returns the next

blockette  marked  as  a  UFD  header.    Else it calls CNVLNK to

convert the link word and gets the blockette that points to.

10. NXTFIL

==========

This routine simply reads the next block in the UFD via GETBLK.

11. GETBLK

==========

This  routine  converts a link word via CNVLNK, and then converts

the  resultant  cluster  number to a block number via CNVCLS.  If

the block is not already in memory it is read in.

12. CNVLNK

==========

This  routine simply decodes a standard RSTS/E link word into its

constituent  block  number,  cluster  number,  blockette  number,

blockette offset and use flag.

13. CNVCLS

==========

This routine simply converts a cluster number into a block number.

14. CNVDAT

==========

This  routine  simply  converts  a  date in RSTS/E format into an

ASCII string.

15. CNVUIC

==========

This routine simply converts a binary UIC into ASCII.

RTR  is a non-privileged task with a taskname of ...RTR, which uses 4

units (of which 2 and 4 should be assigned to TI:).