Terminal Security System

Technical Manual

Prepared for

Cargill Grain Lab

Minneapolis, Minnesota

by

Cargill Grain Lab

Computer Systems Group

Jim Bostwick

June 20, 1985

Sy42:[1,100]SCLIMAN.DOC

CHAPTER 1

Introduction

This is the technical support manual for  the  security

CLI  -  SCLI.   It  contains module design information along

with overall design guidelines.  Additional  information  is

provided  for the use of maintainers of the program, includ-

ing instructions for modifying  passwords,  response  lines,

protected terminals, and so on.  A chapter describing modif-

ications to the RSX system required by the  security  system

is  included,  along  with  instructions  for installing the

changes.

1.1  Overview

SCLI is a user-written command line interpreter,  which

provides  additional access protection to one or more serial

ports, beyond that provided by the RSX login procedure.  The

secure system consists of three 'components':  the SCLI task

itself, a modified RSX BYE program, and a patch to  the  RSX

TT driver.

The BYE modification causes BYE to determine if a  pro-

tected  terminal  is being logged off.  If this is the case,

BYE resets the CLI for that terminal to SCLI.

The terminal driver patch prevents the driver from  ar-

bitrarily  and  unconditionally setting a remote port to MCR

when the modem is answered.  This is a two-line  patch,  and

may be implemented using ZAP or OPEN/SAVE.

These two modifications set up a  situation  where  any

protected  port  which  is  not currently logged on is under

control of SCLI.  SCLI then obtains all input  to  the  pro-

tected  port.   SCLI responds to the proper input by setting

the port to MCR CLI, allowing the user to log on and use the

system normally.

This system has been  designed  to  operate  fully  un-

attended.   It will be run at system startup, and never shut

down.  In fact, the system is very difficult to eliminate.

CHAPTER 2

SCLI Narrative

SCLI is a non-priveleged task.  However, when installed

as  a  CLI, SCLI takes on several normally priveleged attri-

butes.  SCLI implements access protection by simply  sitting

on the terminal(s), absorbing all input.  When SCLI receives

a valid password on a protected terminal, it  gives  control

of the terminal to MCR, and remains 'quiet' until the termi-

nal is logged off.  At that point, BYE gives control back to

SCLI, re-establishing protection.

When SCLI is first  started,  normally  just  following

system  boot,  it  takes control of the protected terminals.

As long as SCLI remains  active  (normally  'forever'),  the

protected  terminals are in one of three states:  protected,

logged on, or disabled.  A terminal is protected  when  SCLI

is  that  terminal's  CLI.  A logged on terminal has already

been 'admitted' to the system, and is  in  use  by  a  valid

user.   A terminal can become disabled in the unlikely event

that SCLI is shut down.  In this state, no CLI is  connected

to  the  terminal,  and it is effectively slaved.  Note that

this state occurs briefly during  system  shutdown,  between

the  time  that  SHUTUP  disables the CLI, and the time that

shutdown completes.

SCLI places terminals which it is protecting in one  of

two  internal  states:   normal, and mumble mode.  In normal

mode, SCLI will accept a valid password, and give  the  user

access  to  the  system.   In mumble mode, SCLI has detected

unauthorized attempts to access the  system;  all  input  is

treated as invalid, and the user will 'never' gain access to

the system.

Mumble mode is triggered by a preset number of  sequen-

tial  invalid login attempts.  Once in mumble mode, the (po-

tential) user must wait a preset time interval before  again

attempting  to  log  in.  If the timer expires, and no input

has been received, SCLI assumes that the villian  has  left,

and  a  new user is attempting to gain access.  On the other

hand, if the user keeps  trying,  SCLI  simply  re-sets  the

timer, perpetuating mumble mode.

SCLI maintains a log of all activity on both  the  Con-

sole  (CO:  or COT), and the system error log file.  The log

to CO:  is more detailed, since it is assumed that the  con-

SCLI Narrative                                      Page 2-2

sole  log  will be purged more frequently than the error log

file.

SCLI is also sensitive to messages issued via  the  MCR

CLI  commands.  There are currently three messages:  two en-

able/disable dumping of all invalid  input  to  the  console

file.  These are used for observing activity during attempt-

ed break-ins to the system.  The third message provides  the

only  known  means  of  shutting SCLI down while keeping the

system running.

NOTE

This third message is known to a  few  programmers

at  the Grain Lab only.  It is not to be made ava-

ilable to Grain Lab staff in general, nor to plant

staff  under  any circumstances.  Should it be re-

quired to provide plant staff with a shutdown  me-

chanism,  another  message  should  be defined for

thier use.

2.1  SCLI Walkthru

This section comprises a brief walkthrough of the  SCLI

task itself.  Details of individual module action are conta-

inied in subsequent chapters.

SCLI is normally in a stopped state.  Initialization is

only  performed  at  system  startup, or following a program

crash.  Note that the RSX executive will  automatically  re-

start  SCLI as needed, so that the only truly fatal error is

one which continually crashes SCLI.

Initialization is brief, consiting of assigning  a  LUN

(2)  to  the  CO:  device, and clearing out the user-id, and

invalid counts for each protected terminal.

Following initialization, SCLI lives in  a  loop  which

obtains  a  'command'  from  the system (RSX), processes it,

then loops.  The executive will STOP SCLI until a command is

available.

Commands take two forms:   user  input,  and  messages.

User  input  comes  from  one  of  the  protected terminals.

Messages come from the  system,  either  automatically  when

events  affecting SCLI occur, or in response to the MCR "CLI

/MES" command.

SCLI Narrative                                      Page 2-3

The main code immediately determines if  the  'command'

is  user input or a message.  All message processing is done

in subroutine DOMSG.  User input is processed  in  the  main

SCLI module.

All input is encrypted.  The  encrypted  text  is  then

compared  against a command table, containing pre-encrypted,

known commands.

2.1.1  Command Input

Input commands are matched against the  command  table.

The  issuing  tt  is  also  determined and the corresponding

entry in the tt-table is located.  If no entry  exists,  the

command is ignored (a log entry is made, however).  Assuming

the command comes from a known tt, the action taken  depends

upon  two  things:   1)  the action specified in the command

table (pass/fail); and 2) the current mumble status  of  the

terminal.

If the terminal is in mumble-mode, all  input  commands

are treated alike:  a 'random' reply is issued to the termi-

nal, and the foo-count is incremented.  If the  terminal  is

not in mumble mode, commands with the ac.pas attribute cause

SCLI to admit the user.  Commands with  the  fail  attribute

(ac.fai), or those not found in the command table, cause the

foo-count to be incremented, and the reply message from  the

table  issued  to the terminal.  If, after incrementing, the

foo count exceeds the pre-defined MAXFOO, MBLON is called to

place the terminal in mumble mode.

SCLI admits a user to the system by first clearing  the

foo-count,  and  total-foo,  saving  the  user-id in the tt-

table, marking the terminal as 'logged on',  and  issuing  a

SCLI$  directive to set the terminal to MCR.  This gives the

user access to the standard RSX login procedures.

2.1.2  Mumble Processing

Mumble mode simply means that no input from the  termi-

nal  will  be  accepted as a valid password.  Mumble mode is

triggered by a number of invalid commands which exceeds  the

MAXFOO limit.  It is terminated by a timer.  Once in mumble-

mode, any command input causes mumble mode to be  perpetuat-

ed.   Only  if  the  terminal  is 'quiet' (e.g., no commands

received by SCLI from that  terminal)  for  a  predetermined

time  will  mumble mode be reset.  This is handled in module

SCLI Narrative                                      Page 2-4

MUMBLE, which contains  the  MBLON  initiator  routine,  and

MBLAST, the timer service routine.

When mumble mode is entered, a timer is  started,  with

MBLAST specified as the entry point.  Since several terminal

may be in mumble mode at one time, the terminal  is  identi-

fied by a unique event flag.  When the timer expires, MBLAST

is entered, and retrieves the EFN  number  form  the  stack.

The  EFN  is  located in tt-table, which gives access to the

foo-count.  If the foo-count is zero, MBLAST resets the mum-

ble  bit  for  that tt, and exits.  This clears mumble mode,

allowing the next valid password to be accepted.

On the other hand, if the foo-count is non-zero, MBLAST

adds the count to the total-foo (for logging purposes), cle-

ars the foo-count, and re-issues  the  mark-time  directive.

This perpetuates mumble mode for another interval.

2.1.3  Message Processing

Messages are key to SCLI - as they inform  the  CLI  of

system   actions   which  might  affect  terminal  security.

Several operator-initiated messgaes are also accepted, which

control SCLI functions.

SCLI makes great attempts to maintain it's control over

protcted terminals.  Basic protection is provided by the op-

erating system.  All input to a terminal goes to that termi-

nal's   CLI,   without   exception   (the  standard  control

characters like CTRL-U, -S, etc.  continue to  be  processed

by  the  tt driver).  Also, a CLI is non-abortable - it must

be  first  eliminated  before  the  task  can  be   aborted.

Finally,  if  the  CLI  task  exits for some reason (such as

fatal error), the executive will re-activate it when a  mes-

sage or command is received for that CLI.  This more or less

makes SCLI a crash-proof program.  SCLI is allowed to die on

rare errors, such as low-pool conditions, without compromiz-

ing security.

A cli may only be eliminated if no terminals are linked

to  it.  SCLI goes beyond this protection by reacting to the

'you have been eliminated' message with a RPOI of MCR to re-

establish  the  cli.   This  occurrs very rapidly, providing

good security (not perfect) against a fast typist's  'elimi-

nate, abort' sequence.

The MCR SET command may be  used  (from  a  priveleged,

logged-on terminal), to alter the CLI associated with a ter-

minal.  SCLI reacts to this as well.  If a protected  termi-

SCLI Narrative                                      Page 2-5

nal  which  is not known by SCLI to be 'logged on', is reset

to a different CLI, SCLI simply sets the  terminal  back  to

SCLI with the SCLI$ directive.

.hl 2 User Messages

There are currently three messages known to SCLI,  bey-

ond the system-initiated ones.  THese are all issued via the

MCR CLI /MESSAGE directive.   Two  messages  set  and  reset

'verbose' logging.  Verbose logging is normally OFF (reset).

When set, all invalid input to scli is passed on  in  a  log

entry.  When reset, only the invalid command causing mumble-

mode to be initiated is so logged.

The third user message provides the only known means to

effect the orderly shutdown of SCLI.  When issued, this mes-

sage sets a flag, telling SCLI to accept normally unfriendly

behavior such as terminal CLI reassignment, and CLI elimina-

tion.  Note that, even though  this  'maintenance  shutdown'

mode  is  set by the message, security protection remains in

effect until the user disconnects SCLI  from  the  protected

terminals, and elimintates SCLI.

2.1.4  Logging

Module LOG handles all logging functions.  A macro, de-

fined  in SCLIDAT.MAC, eases the setup and call of LOG.  All

logging is via pre-defined messages, with  variable  parame-

ters.   The  DOLOG  macro  accepts  the message number and a

message-specific parameter list.

Messages may be routed to either  the  console  (device

CO:),  or  to  the  system error log file, or both.  Message

routing is controlled by flag bits in the message definition

(module  LOGMSG.MAC).   The  RSX $EDMSG routines are used to

format the messages, handling all conversion  and  parameter

substitution.

Since messages are routed through CO:, logging  is  de-

pendent  upon the settings of the console output task (COT).

Using the MCR SET /COLOG commands, it is possible  to  route

console  messages to a terminal, a file, or both.  If COT is

turned off entirely, messages will appear on the system con-

sole device.

A later chapter covers the format and  content  of  all

log messages in full detail.

CHAPTER 3

Standards

3.1  Coding standards

This section presents coding and design standards to be

followed for this project.

3.1.1  Modularity

This is a fully modular program.  Each source file must

contain code for a single function.  This may include sever-

al related subroutines, or a single routine.  An example  of

the former might be the MUMBLE.MAC module, containing MBLON,

MBLOFF, and MBLAST -- all routines  relating  to  initiating

and  terminating  mumble  mode.  It would also be acceptable

for these three routines to exist in separate source files.

3.1.2  Source Format

Each source module will be maintained in a  SourceTools

archive  (xxx.SCN  file).  File TEMPLATE.MAC will be used as

the basis for all source files.  The  fields  surrounded  by

tilde  characters (~) are supplied by GETSRC, and should not

be altered.

There will be a module header for  each  routine.   The

skeleton  header is provided by the CTRL-P command, assuming

the system edt include file "MACRO"  is  used.   The  header

shall  contain  the module title, a description of its func-

tion, call site, input and output parameters,  and  register

usage.   A  list  of  routines called will be included.  The

description will include the module's  function,  error  re-

turns, and algorithm if not self-evident.

3.1.3  Source Control

This project is being implemented under the SourceTools

system.   Every  source file will be under "SrcCon" control.

New modules are to be entered into the system following ini-

Standards                                           Page 3-2

tial  test.  Subsequent modifications are to be under source

control.  Test programs will also be  subject  to  the  same

controls.

3.1.4  Test Programs

Most modules will  have  an  associated  test  program,

which demonstrates the module functions.  Test programs need

not exhaustively test the module, and should be as simple as

is reasonable.  Extensive testing will be done during integ-

ration.

Individual test programs may exercise multiple modules,

so  long  as  it  remains  easy  to  determine the source of

faults.  Test programs which evolve to test several  related

modules are ideal.

3.1.5  PSECT Usage

This section defines the PSECT (program section)  usage

for the project.

1.  BLANK - all program code will reside in  the  blank

(default) psect.

2.  GDATA - global common data (rw,  gbl,  ovr).   This

psect  defined by module SCLIDAT, which must be in-

cluded in the assembly of using modules.

3.  DATA - global private data (rw, gbl).

4.  LDAT - local data.  This psect shall be used by mo-

dules  for scratch storage - assume that it will be

destroyed across module invokations.

5.  DPBS - static directive DPBs - (rw, lcl)

Except for GDATA, each module may contribute  to  these

psects.   Data which is shared among modules must be defined

in SCLIDAT.MAC.

Psect DATA is global, concatenated, and may be used  by

any  module  which  needs data to remain across calls to the

module.  Psect LDAT is  overlaid  and  is  to  be  used  for

scratch  storage.   The  contents of LDAT are undefined upon

entry to a module.

Standards                                           Page 3-3

NOTE

Any program contributing to one of the data psects

must  include an .EVEN directive following defini-

tions for the psect.  This  forces  each  module's

contribution  to be of even size, and prevents er-

rors due to TKB's byte alignment of data psects.

3.1.6  Exceptions

The only exceptions to the above standards are the mod-

ifications  to DEC system software.  These modifications, to

module TTMOD.MAC of the RSX terminal driver, and to BYE.MAC,

shall  adhere to the format and standards employed by DEC in

constructing and maintaining RSX system software.  Note that

these  modules  stand alone from SCLI, even though the modi-

fied programs are required for proper function of the  secu-

rity  system.   Modificatons to TTDRV and BYE are documented

in later chapters.  The related  files  (including  separate

runoff  source files for each modification) are found in uic

[1,101] (11MPLUS and common) and [1,102] (11M  specific)  on

the SCLI sources disk.

CHAPTER 4

Database

This chapter describes the database used by SCLI.   The

database  consists of global data defined in module SCLIDAT,

internal error messages defined in ERNUM.TXT (these are  im-

plicit  in  the  SCLI  code),  logging  messages  defined in

LOGMSG.DAT, and the command table, defined in KEYTAB.  These

are all internal databases - SCLI does no file operations of

it's own.

4.1  Global Data

Module SCLIDAT defines the global data for SCLI.   This

module  must be included ( .include directive) in all source

modules which must access the global database.

4.1.1  Constants

A number of constants are defined.  Most of  these  de-

fine  the  offsets and flag bit values for the tt-table, and

the command table.   Three  additional  constants  tune  the

mumble-mode functions.

*  MAXFOO sets the trigger point for mumble mode.  When

more than maxfoo invalid commands are received, mum-

ble mode is entered.  This should  remain  a  fairly

small number (3-5).

*  MBLUNI defines the time units for the mumble  inter-

val timer.  This must have a value of 2 (seconds) or

3 (minutes).

*  MBLTIM defines the count  for  the  mumble  interval

timer.  A time interval of 2-5 minutes is suggested.

4.1.2  TT-TABLE

This table contains one entry per  protected  terminal.

It  must  be terminated with a zero word.  Macro DEFTT, also

defined in SCLIDAT, is used to construct the table.  A table

entry contains the following fields:

Database                                            Page 4-2

*  tt.num - integer number of the terminal

*  tt.foo - integer count of invalid commands

This value is incremented for each invalid com-

mand received from the terminal.  It is reset by the

mumble  timer  completion  routine  (MBLAST).   This

value must remain zero for an entire mumble interval

before MBLAST will reset mumble mode.

*  tt.efn - event flag used  in  mumble-mode  for  this

terminal

*  tt.flg - flags word

tt.flg bits are:

*  fl.mbl - 1-> terminal is in mumble mode

*  fl.pro - 1-> this  terminal's  cli  is  currently

SCLI

*  tt.uid - pointer to user-id string, or nil

This pointer is initialized when a user is  ad-

mitted  to the system.  It is then used when the use

loggs out (SCLI resumes control over the  terminal),

to write a log entry.

*  tt.tot - integer total foo count since last login

This value is maintained entirely  for  logging

purposes.   Since tt.foo is reset at the end of each

mumble interval, tt.tot is used  to  accumulate  the

total  number of invalid login attempts.  This value

is cleared when a user is admitted to the system.

4.1.3  Command Table

There  are  two  command  tables  in  SCLI.   The  pre-

encrypted  command strings are in a table defined locally to

the LOOKUP subroutine.  Lookup returns an  index  into  this

table, or zero if no entry is found.

The command table defined in SCLIDAT  contains  corres-

ponding  entries.   The  index returned by LOOKUP is used to

access CMDTBL.  Cmdtbl is also zero terminated, and constant

MAXCMD   also  gives  the  address  of  the  end  of  table.

Currently, only MAXCMD is used.  The fields of cmdtbl are as

follows:

*  cm.rpl - pointer to reply string

The reply string is issued to the user's termi-

nal  when this command is input, and the terminal is

NOT in mumble mode.

Database                                            Page 4-3

*  cm.id - pointer to the plain-text  user  id  string.

This  string  is only defined for commands which are

valid passwords.  When the user is  logged  on,  the

user-id string is used to greet the user, and is en-

tered in a log entry.  The user-id is also saved  in

the  tt-table,  and appears in a logout message when

SCLI resumes control of the terminal.

*  cm.act  -  the  action  to  be   taken   when   this

command/message  is received.  There are two actions

only for command input:  ac.fai  and  ac.pas.   They

are  fail/pass respectively.  The presence of a fail

action allows dummy table entries.  Most often these

are  used  for  common  login sequences such as HEL,

LOGIN, etc.  This allows non-standard replys  to  be

issued for these commands.

AC.MES  defines  the   first   message   entry.

Actions  greater  than or equal to ac.mes are by de-

finition messages received via the CLI /MESSAGE  fa-

cility.    The  first  such  message  has  the  same

commmand number as ac.mes.  Three messages  are  de-

fined  at  this  time:   ac.mai  enables maintenance

shutdown mode; ac.vrb enables verbose logging of in-

valid input; ac.qui disables verbose logging.

The first  entry  in  cmdtbl  is  required  to  be  the

unknown-command  action.   Unknown  commands,  commands from

non-protected terminals, and mumble-mode all use  the  reply

string from this entry.

Other entries in the table may be  in  any  order,  al-

though  by convention the message entries are grouped at the

end of the table.

4.1.4  Reply Strings

These are simply ASCIZ strings which are pointed to  by

the command table entries.  There need not be a one-one cor-

respondence between the  number  of  such  strings  and  the

number  of  command  table  entries.  The reply string for a

valid login command should be some sort of greeting  to  the

user.  Other reply strings are up to the programmer.

Database                                            Page 4-4

4.1.5  User-id Strings

These are plain-text identifiers for each valid user of

the system.  They are placed in log entries at login and lo-

gout (to and from SCLI) time.

4.1.6  Global Variables

Several variables are defined  in  SCLIDAT,  which  are

used  globally.   Their  use  is fairly obvious.  One is the

Flags word, consisting of  two  flag  bits  which  configure

SCLI.   GF.KIL  is  the maintenance shutdown bit.  It is set

when the ac.mai message is received by  SCLI,  and  disables

several  protection  features.  When set, it it possible for

the knowlegable user to shut down SCLI in an orderly manner.

When  clear,  it is essentially impossible to shut SCLI down

other than by pathological means.

The other flag bit is GF.VRB, which controls  the  ver-

bose logging of invalid input commands.

4.2  Internal Errors

Internal errors are logged through the same logging me-

chanism (DOLOG) used for other activity.  Log message number

9.  signifies an internal error log entry.  Because  parame-

ters  must  be passed by reference, some care is required in

setting up an internal error log entry.  A typical  internal

error call is as follows:

mov     $dsw, p1

mov      1, nparm

mov      5, err

DOLOG   9.,<err,nparm,p1>

where p1, nparm, and err are all variable labels.  Note that

the  number  of  parameters beyond nparm is variable.  Nparm

tells the log routine how many parameters to include in  the

log  entry,  and how many to pull off the R5 stack.  Err and

nparm themselves are always required, and are NOT counted in

the value of nparm.

Due to limitations on log entry size,  only  the  error

number and parameter values are logged.  File ERNUM.TXT, re-

produced in an appendix, gives the error numbers, initiating

routine, number of parameters, and their meaning.  Using the

above example, error 5 is issued by SCLI upon failure to in-

Database                                            Page 4-5

itialize  (ALUN$)  the  console  LUN.   P1  is the directive

status word.

4.3  Log Messages

Log messages are defined  in  module  LOGMSG.MAC.   For

each  message,  an ASCIZ string is defined, which is used as

the 'control string' in an EDMSG call.  See the  RSX  System

Library Reference Manual for the format and functions of the

EDMSG routines.

Two other structures are associated with the  log  mes-

sages.   Table msgtbl contains the pointer to the ASCIZ con-

trol string for each message.  The strings are  accessed  by

using the message number as an index into msgtbl, from which

the string address is retrieved.

The other table, MSGFLG, is  a  byte  table  containing

control  bits  for  each message.  It is also indexed by the

message number.  The control bits specify  the  desitination

of the message - the console, error logger, or both.

A  prefix  (PREMSG)  is  added  to  each   log   entry.

Currently , the prefix consists of the word "SECURE:" only.

4.4  Key Table

For security reasons, the text of commands and messages

is  stored  internally  in encrypted form.  Input to SCLI is

then encrypted using the same algorithm, and  the  scrambled

text compared against the internal tables.

Module LOOKUP performs the encryption and table  lookup

functions.   It  'includes' module KEYTAB.MAC, which defines

the table.  KEYTAB.MAC is in turn produced by  program  KEY-

GEN,  which takes input file KEYTAB.TXT (the plain text mes-

sages), encrypts the text, and  writes  KEYTAB.MAC  as  it's

output.

KEYTAB.MAC  defines  a  table  of  pointers  to   ASCII

strings.   The  table entries must correspond to the entries

in CMDTBL (defined in SCLIDAT).  It is critical  that  their

be a one-one correspondence between KEYTAB and CMDTBL.  That

is, each entry in CMDTBL must have a corresponding entry  in

KEYTAB, and vice versa.  If these two tables are not confor-

mant, SCLI will fail in various and often obscure ways.

CHAPTER 5

Maintenance Instructions

This chapter contains sections  devoted  to  rebuilding

and  modifying  SCLI.   It  will make reference to the USRSB

routine and the modified BYE program, both of which are more

fully  described  in a later chapter.  Also found in a later

chapter are details of required  modifications  to  the  RSX

terminal driver.

Maintenance functions for SCLI primarily  resolve  into

three  areas:   protected  terminals,  passwords, and users.

These areas can be expected to change for each installation.

5.1  Protected Terminals

The number and identity of terminals protected by  SCLI

is   coded   in   two  areas:   first  in  the  tt-table  in

SCLIDAT.MAC; second, in the USRSB  module  included  in  the

modified  BYE  program.  It is necessary that both these mo-

dules define the same terminals.

5.1.1  USRSB

USRSB contains a table (JMBTBL) with an entry for  each

protected  terminal.   The  entry  consists  of the terminal

number (an integer), and the default CLI  name  (in  RAD50).

Macro  ENT  defined  in  USRSB.MAC  will construct a correct

table entry.  It is normal to leave several empty entries in

the table.  Having empty entries allows the table to be mod-

ified using ZAP, without having to rebuild the BYE  program.

BYE ignores any null entries in the table.

While it is necessary that each terminal  protected  by

SCLI have an entry in JMBTBL, it is possible to use the same

mechanism to change the default CLI for  other  (non-secure)

terminals.  Thus, JMBTBL may have additional entries, refer-

encing CLIs other than SCLI.  Such entries will  not  effect

security  operations.   Whenever  a terminal logs off (using

the BYE program), BYE checks the table for a terminal number

match.   If  a  match  is  found, BYE forces the CLI to that

given in the table.  The effect is to have the CLI for  that

Maintenance Instructions                            Page 5-2

terminal  as  other than MCR whenever the terminal is logged

off.

5.1.2  SCLIDAT

As previously described, SCLIDAT defines  TTTBL,  which

has  an  entry for each protected terminal.  It is necessary

that any terminal with the default CLI of SCLI have an entry

in  TTTBL.   If  a mismatch between TTTBL and JMBTBL occurs,

either the CLI will not be re-established on logout, or SCLI

will never allow access through the terminal.  In the latter

case, SCLI will issue an error message "Message from unknown

terminal".

It is possible to alter TTTBL using ZAP, but the normal

mode will be to rebuild SCLI after editing SCLIDAT.

5.2  Passwords

Passwords are defined by KEYTAB.TXT.  These  are  ASCII

strings,  up  to 72.  characters in length.  The key strings

may contain any printable  character  (including  tab),  and

some  control characters (those which are not intercepted by

the terminal driver).  CASE IS SIGNIFICANT!  Unlike the  MCR

interface, which automatically converts input to upper case,

the SCLI interface accepts upper or lower case.   This  fact

should be used in designing passwords.

Passwords should be phrases, and should be at least 10-

12 characters in length.

To alter a password, edit KEYTAB.TXT to define the  ap-

propriate  plain-text  string.   Recall  that the entries in

KEYTAB correspond to the command table entries  in  SCLIDAT.

Once KEYTAB.TXT is as desired, run program KEYGEN.  The nor-

mal command to KEYGEN is:

KGN>KEYTAB.MAC=KEYTAB.TXT

KEYGEN will write the output file containing  the  encrypted

text  strings.   For  further information on KEYGEN, see the

source file KEYGEN.MAC.

Once KEYGEN.MAC has  been  created,  reassemble  module

LOOKUP,  which  is the only module to include KEYTAB.  Then,

rebuild SCLI.

Maintenance Instructions                            Page 5-3

NOTE

All  that  is  really  required  is  the  edit  of

KEYTAB.TXT.  Then, use the MAKE program to rebuild

SCLI.  MAKE will detect the new KEYTAB, run KEYGEN

(rebuilding IT if necessary), and rebuild SCLI.

5.3  Command Table

The command table in SCLIDAT  defines  the  known  com-

mands,  and  the action to be taken.  Probably the most fre-

quent alterations will be to the uiser-id strings.

Again, it is necessary  to  maintain  a  correspondence

between the entries in CMDTBL and those in KEYTAB.

5.4  Summary

This section summarizes the required  edits  to  effect

various configuration changes to SCLI.

5.4.1  Change Terminals

To alter the protected terminals, JMBTBL in  USRSB  and

TTTBL in SCLIDAT must be modified.  Rebuild BYE and SCLI.

5.4.2  Change Passwords

To alter password text, for either  input  commands  or

messages,  edit module KEYTAB.TXT.  Then, run KEYGEN to cre-

ate KEYTAB.MAC.  Reassemble LOOKUP, then rebuild SCLI.

5.4.3  Change Users

To change a user's id string, edit the string in  SCLI-

DAT,  and/or  the string pointer in CMDTBL, also in SCLIDAT.

Rebuild SCLI.

To add or remove a user, edit CMDTBL in SCLIDAT, possi-

bly  adding  a  user-id string.  Edit KEYTAB.TXT to maintain

correspondence between password strings  and  command  table

Maintenance Instructions                            Page 5-4

entries.  To rebuild, follow instructions above for changing

passwords.

5.4.4  Change Messages

The procedure for adding or modifying messages  is  es-

sentially the same as that for user-ids, as described above.

The message text is in KEYTAB, while the corresponding  com-

mand table entry is in SCLIDAT.

5.5  Build Instructions

SCLI is composed of three components:   the  SCLI  task

itself,  the  modified  BYE  task,  and  the  modified TTDRV

driver.  Each is built in a separate  operation.   For  most

maintenance  operatons,  it will not be necessary to rebuild

all three components.

SCLI will normally be installed by copying  the  source

kit  to  the SYSGEN work disk for the target system.  If ne-

cessary, copy all files in uic [1,100], [1,101], and [1,102]

from the SCLI kit to the target disk.  The following discus-

sion assumes that these accounts do in fact exist on a  sys-

gen  work  disk.   The  process  is similar for other source

locations.

It is also assumed that required modifications  to  the

sources  have been made, and the archive (.scn) files updat-

ed.

5.5.1  SCLI

SCLI rebuild is controlled by the sourcetools MAKE pro-

gram.   Use  of MAKE greatly simplifies the rebuild process.

To rebuild SCLI, enter the following MCR commands:

1.  SET /UIC=[1,100]

2.  @MAKE SCLI

When the command file completes, copy SCLI.TSK  to  the

library account on the target system disk.

Maintenance Instructions                            Page 5-5

5.5.2  Documentation

SCLI documentation can be rebuild manually from the ar-

chive  files.  Simply GET the required modules and pass them

through RNO.  A full documentation set may be built with the

following MCR commands:

1.  SET /UIC=[1,100]

2.  MAK SCLIDOC.CMD=SCLI/target:"Dockit"

3.  @SCLIDOC.CMD

Be warned that some of the runoff source files  include

(.require) additional runoff sources.  Users unfamiliar with

the sources should either rebuild the entire  kit,  as  des-

cribed above, or examine the source files to ensure that the

required include files are present.

Individual documents may  be  rebuilt  by  substituting

their   name   for   the   'target'   above.    For  example

/target:"Scliman.doc" would produce this manual.

5.5.3  BYE

BYE is built in [1,101] (M+) or [1,102] (M), and copied

to  the library account of the target disk.  BYEASM.CMD will

assemble BYE.

USRSB is reassembled as follows:

GET USRSB

MAC USRSB,USRSB=USRSB

TKB @BYE2BLD will rebuild  BYE2.TSK.   BYE2BLD  assumes

that  device  TK:  points to the sysgen work disk (to obtain

RSX11M.STB, MCR.OLB, etc), and that  BYE.OBJ  and  USRSB.OBJ

are in the current account.  It leaves BYE2.TSK and BYE2.MAP

in the current account.

5.5.4  TTDRV

The terminal  driver  modification  is  not  automated.

Normally,  the  modified  source  file  will  be copied from

[1,101] (or [1,102]) into [11,10] of the sysgen  disk  prior

to  SYSGEN.   SYSGEN  then builds the modified driver by de-

Maintenance Instructions                            Page 5-6

fault.  If the modification must be done after  SYSGEN,  the

following  instructions  should  be followed for MPLUS.  The

procedure for M is similar.

1.  Copy TTMOD.MAC and  TTMODASM.CMD  from  [1,101]  to

[11,10].

2.  Execute MAC @TTMODASM, from [11,10].

3.  From [1,24]:  LBR TTDRV/RP=[11,24]TTMOD.

4.  From [1,24]:  TKB @[200,200]TTDRVBLD

5.  Use VMR to UNLoad the old driver and LOAd the modi-

fied one.

5.6  Initialization

SCLI should be installed with the CLI=YES option either

in  VMR  or  STARTUP.CMD.   Very  early in STARTUP.CMD, file

STARTSCLI.CMD should be executed.  Logons should be disabled

until after STARTSCLI is executed.  Alternately, SCLI may be

initialized directly from STARTUP.CMD, using STARTSCLI as an

example of the commands.

It is important that the proper switches be included in

the  CLI  command  initializing SCLI.  If "/QUI/MES/LGO" are

not present, the security system will not function properly.

In  most cases, the protected terminals will become unusable

(usually slaved).  The prompts may be changed as desired.

CHAPTER 6

Module Descriptions

This chapter contains a section for each module in  the

system,  excepting  test programs.  The information in these

sections is essentially a duplicate of the  header  informa-

tion in the source module.

6.1  Global Data

Module SCLIDAT defines the global database for the SCLI

program.   This  file is "included" (.INCLUDE or in assembly

command line) in any source program which requires access to

the  global data.  Note that many modules do not require ac-

cess to the global data.

6.2  CRYPT

Crypt performs a simple, in-place encryption  of  the  input

string.   The algorithm is the familiar and relatively inse-

cure XOR of data bytes with a  key  text.   This  works  for

crypt  because  the  key is guaranteed to be longer than the

input text, and the messages  are  relatively  short.   Both

combine  to  frustrate normal breaking techniques.  If r0 is

odd, the first character of the key is skipped, resulting in

word alignment for most of the routine.

CALL

jsr pc,CRYPT

INPUT

r0 -> input string

r1 =  length of input string

OUTPUT

input string encrypted in place

REGS

all preserved

Program TSCRYPT exercises this routine.

Module Descriptions                                 Page 6-2

6.3  DOMSG

DOMSG processes all messages (as opposed  to  commands)  re-

ceived by SCLI.  Messages come in two forms:  those produced

automatically by RSX, and those initiated by  someone  using

the MCR "CLI /MESSAGE" command.  Both types are important to

SCLI.

CALL

jsr pc, domsg

INPUT

r0 -> GCCI command buffer

OUTPUT

various (including immediate exit)

depending upon message received.

REGS

preserved

Module Descriptions                                 Page 6-3

6.4  LOG

LOG logs messages to the  error  logging  file  (via  SMSG$)

and/or  to  the console (CO).  For testing, if logtt3 is de-

fined (just before entry point), logs all  messages  in  raw

form  to  LUN   3,  which is hardcoded to TT3.  Messages are

identified by number.  A variable number of  parameters  are

required  depending  upon the message.  For message numbers,

required parameters, see module LOGMSG.MAC.  Message  number

is  a pass-by-value.  All of the variable params are pass by

reference.

CALL

call r5,log,<msg,params>

msg = message number (messages defined

above)

params = parameters dependant upon message

number

see individual message definition

above

INPUT

params on r5 stack

OUTPUT

message formatted and logged as indicated by

flags word

in message database (defined above)

REGS

preserved

CALLS

uses $EDMSG from SYSLIB to format messages

includes LOGMSG.MAC to define messages

6.4.1  Notes

LOG does a QIO no-wait to CO:, followed by a mark-time.

Both  specify  AST  routines.  Whichever one completes first

cancells the other.  This provides output timeout  services,

since a hung QIOW to CO:  would hand the entire SCLI system.

LOG also contains code to fake out a call to itself  in

the  event  of an internal error.  This code is a bit wierd,

in that it must protect against infinite looping on multiple

errors.  However, it is known to work.

Module Descriptions                                 Page 6-4

6.5  LOOKUP

LOOKUP is called with a    plain-text command line.  It

first  encrypts the command line, then searches the password

table for a match.  LOOKUP returns the offset of the  match-

ing  entry  in the command table, or 0 (zero) if no matching

entry exists.  Also, a no-match condition returns carry set.

NOTE

The returned index is the  byte  offset  into  the

command table (NOT the password table).  The pass-

word table contains one less entry (the entry with

index=0) than the command table.  In simple terms,

add the base of the command table to r2 on  return

and look at the command.

CALL

jsr pc, lookup

INPUT

r0 -> command line

r1 = length of command line

OUTPUT

r0 -> same command line, but encrypted now

r2 = command index (0 -> no match)

carry set -> no match

carry clear -> match found

REGS

r2 modified, string referenced by r0 modified

others preserved

CALLS

CRYPT

NOTES:

1.  In order to work properly, trailing whitespace (tab

or  space)  should  be  stripped from command line.

Leading whitespace is ok, allowing tricky passwords

like <tab><tab>hi there!

2.  Source module KEYTAB.MAC is .INCLUDED in  this  mo-

dule.  It is generated by KEYGEN.

Program TSTLOOKUP exercises this module.

Module Descriptions                                 Page 6-5

6.6  MUMBLE

Module MUMBLE.MAC contains  two  routines:   MBLON  and

MBLAST.   Together these routines implement mumble mode ser-

vices.

MBLON initiates mumble mode for the  current  tt.   The

current tt is pointed to by ttptr (defined by sclidat).

CALL

jsr,pc mblon

INPUT

ttptr -> tt record to be put in mumble mode

OUTPUT

tt placed in mumble mode,

foo-count added to total for tt

foo-count for tt cleared

mumble-timer started for tt's efn

REGS

preserved

MBLAST is entered upon expiration of  the  mumble-timer

for  a  given tt.  The associated tt is determined, and it's

foo-count examined.  Either the mumble-timer is perpetuated,

or the tt is kicked out of mumble-mode.

CALL

AST service routine

INPUT

on stack: efn (remove before exit)

OUTPUT

if tt.foo > 0

mumble-timer restarted

if tt.foo = 0

mumble-mode cleared

in either case

tt.foo added to tt.tot, then cleared

REGS

preserved

Module Descriptions                                 Page 6-6

6.7  SLEN

SLEN determines the length of an asciz  string.   ASCIZ

strings  are  terminated  by a null (zero) byte.  Length re-

turned does not include the null byte.

CALL

jsr pc, SLEN

INPUT

r0 -> input string

OUTPUT

r0 -> input string

length on stack

REGS

all preserved

NOTE: 	1. Caller must pop length off of stack on return

2. Input string had better have a null terminator!

Program TSTSLEN exercises this routine.

Module Descriptions                                 Page 6-7

6.8  TYPE

TYPE outputs a message to one of  two  terminals:   the

CLI  command input terminal, or the task ti:.  The latter is

assumed pointed to by LUN 5.  For the former,  the  terminal

is  in  the  GCCI  buffer pointed to by r0, and LUN 1 is as-

signed to that terminal.

TYPE uses QIO (no-wait), with MRKT, in order to provide

positive  timeout  action.   In  the event the qio times-out

(user hit no-scroll for example, or hung  up),  the  QIO  is

killed.  No other error reporting is done, to prevent an in-

finite error loop!

CALL

CALL TYPE,r5,buf,flag

INPUT

r0 -> GCCI return buffer

(reqd if cli command term is output)

on r5 parameter list -

buf -> ASCIZ buffer to output

flag = 0 --> use cli command term

= 1 --> output to TI:

OUTPUT

message sent, errors ignored (!)

if flag = 0, LUN 1 set to comand input terminal

REGS

preserved

Program TSTTYPE exercises this module.

CHAPTER 7

Test Programs

This chapter contains brief descriptions of  the  test  pro-

grams used to validate the various modules.

7.1  TSTCRYPT

This program validates the CRYPT module.  It accepts  a

line  of input from the terminal, encrypts it, then decrypts

it.  The input and decrypted text are printed on the  termi-

nal,  along  with  the  result of a check that the encrypted

text does NOT match the input.  Note that TSTCRYPT does  not

validate  the  encryption algorithm for security - only that

it will decrypt a duplicate of the original plain text.

7.2  TSTLOG

TSTLOG exercises the logging  facility.   It  basically

calls  LOG  once per message type.  Several 'internal error'

messages are logged, in order to exercise the variable  par-

ameter  features.  Output from tstlog will be on the console

log and error log.  This is a useful program  for  producing

sample log reports as well.

7.3  TSTLOOKUP

This program validates LOOKUP, and indirectly CRYPT and

KEYGEN.  It accepts a string from the operator, passes it to

lookup, and prints the returned index value.  CTRL-Z  termi-

nates.   The  returned  value is zero if no record found, or

the byte offset into a table of responses.  It is just  pos-

sible, especially if the STARTUP.CMD file is faulty, or fol-

lowing maintenance shutdowns, that a user may be  logged  on

to the terminal at the time SCLI takes control.  This is the

only time that this situation can arrise.  In the event this

happens,  the  user must satisfy SCLI, in order to return to

MCR.

Test Programs                                       Page 7-2

7.4  TSTTYPE

This program validates TYPE.   It  dummies  up  a  GCCI

buffer  with  device  info (HARDWIRED to tt23:!), then calls

TYPE twice.  First, a message to the 'command  tt'  (tt23:),

next a call to the 'task ti:'.  We don't try to exhaustively

exercise the various error conditions, since most are  fatal

anyway.   You  can  check  the timeout by hitting S on tt23:

before running the program.

7.5  TSTSLEN

This program validates SLEN.   It  accepts  a  line  of

input from TI:, and prints its length as returned by SLEN.

CHAPTER 8

Terminal Driver Modifications

This chapter discusses the  modifications  to  the  RSX

terminal  driver  required  to  support  the security system

(SCLI) for remote dial-up lines.

8.1  Overview

The RSX-11 (M and M+) terminal  driver  unconditionally

forces  MCR  to  a terminal being answered on a remote line.

While intended to provide a consistent interface  to  remote

terminals,  this  functionality  breaks the security system,

which depends upon  SCLI  owning  any  protected  terminals.

Equally  unfortunate is the fact that TTDRV does not use the

normal SCLI$ mechanism to establish MCR, but instead  simply

hammers the executive data structures directly.  The exec is

thus unaware that a port's CLI has changed, and does not in-

form  SCLI of the action.  This prevents SCLI from re-taking

the port.

Fortunately, a very simple patch to the terminal driver

will  'correct'  this  situation.   The  remainder  of  this

chapter describes the modification,  and  includes  instruc-

tions  for making the patch either in the sources or via ZAP

to an existing system.

8.2  Modification Details

The relevant code is in subroutine MANS (modem answer),

in  module TTMOD.MAC.  This file can be found on the distri-

bution kits in [11,10].  MANS is called when a  remote  line

is  answered  (e.g.,  carrier  detected).  Its purpose is to

place the terminal in a known (and benign) state.  MANS mod-

ifies the terminal characteristics words, sets the buffer to

72., and forces the CLI to MCR.  It then initiates  autobaud

(if the terminal is set ABAUD), and returns.

The switch to MCR is accomplished by  one  of  two  in-

structions, depending upon certain SYSGEN conditionals.  For

MPLUS, this is a BIC instruction, while for M it  is  a  MOV

instruction.   Both  are  commented  as  "SET  MCR  AS CLI".

Terminal Driver Modifications                       Page 8-2

Comment out these instructions.

The driver also resets  the  lower-case  bit,  enabling

lower-to-upper   case   conversion.   Since  SCLI  is  case-

sensitive, we wish the terminal  to  be  left  in  lowercase

mode.   This is accomplished by locating the BIC instruction

containing "U2.LWC" in the first operand.  Remove the refer-

ence to "U2.LWC", leaving the other bits intact.

NOTE

Several other characteristics are set in this code

section.   Most  may  be  eliminated, if it is de-

sired.   Examples  include  U2.CRT,  U2.ESC,   and

S3.8BC.   It is also possible to alter the default

buffer size.  The buffer size must be at least 72.

for SCLI to operate properly.

8.3  Source Modification

The desired method of modifying the driver  is  at  the

source level.  UIC [1,101] contains the MPLUS files; [1,102]

those for M.  These uics contains the modified TTMOD.MAC, an

unmodified TTMOD.VGN, and TTMOD.COR.

Although M and M+ share the same source module, the up-

date  versions  are  different, requiring patch sets in both

[1,101] and [1,102].  The patches may have  to  be  manually

applied,  and  new correction files built, for later updates

or releases.   Both  patch  sets  contain  the  audit  trail

"JMB227".

The simplest way to apply these patches is to copy  the

modified  TTMOD  to [11,10] of the sysgen work disk prior to

SYSGEN.  Since TTDRV is always  re-assembled  and  built  by

SYSGEN,  this will incorporate the patches.  It is also pos-

sible to manually assemble TTMOD, and  rebuild  TTDRV  post-

SYSGEN.   Follow  the  instructions in the release notes for

rebuilding a driver.  The database doesn't change, so it  is

possible  to  use  VMR to UNLoad and LOAd the patched driver

without creating a new system image.

The secure source accounts contain TTMODASM.CMD,  which

will reassemble TTMOD.  This file assumes that TTMOD.MAC ex-

ists in [11,10] on the sysgen work disk, which is  also  the

current  SY:.  Replace module TTMOD in [1,24]TTDRV.OLB, then

rebuild.  To rebuild TTDRV, use [200,200]TTDRVBLD.CMD.

Terminal Driver Modifications                       Page 8-3

8.4  Patching TTDRV.TSK

It is also possible to apply these patches to TTDRV.TSK

using  ZAP,  or to a running system using OPEN.  In order to

do this, TTDRV.MAP, plus an assembly listing  of  TTMOD  are

required.   The  patches add no code, and are thus simple to

make.  Simply comment out the  "SET  MCR"  instruction  (all

three  words!),  and  alter the bitmask for the BIC instruc-

tion.

The unmodified  bitmask  is  currently  3047,  and  the

patched value 3046.

CONTENTS

1.  Introduction . . . . . . . . . . . . . . . . . . . . 1-1

1.1  Overview  . . . . . . . . . . . . . . . . . . . 1-1

2.  SCLI Narrative . . . . . . . . . . . . . . . . . . . 2-1

2.1  SCLI Walkthru . . . . . . . . . . . . . . . . . 2-2

2.1.1  Command Input  . . . . . . . . . . . . . 2-3

2.1.2  Mumble Processing  . . . . . . . . . . . 2-3

2.1.3  Message Processing . . . . . . . . . . . 2-4

2.1.4  Logging  . . . . . . . . . . . . . . . . 2-5

3.  Standards  . . . . . . . . . . . . . . . . . . . . . 3-1

3.1  Coding standards  . . . . . . . . . . . . . . . 3-1

3.1.1  Modularity . . . . . . . . . . . . . . . 3-1

3.1.2  Source Format  . . . . . . . . . . . . . 3-1

3.1.3  Source Control . . . . . . . . . . . . . 3-1

3.1.4  Test Programs  . . . . . . . . . . . . . 3-2

3.1.5  PSECT Usage  . . . . . . . . . . . . . . 3-2

3.1.6  Exceptions . . . . . . . . . . . . . . . 3-3

4.  Database . . . . . . . . . . . . . . . . . . . . . . 4-1

4.1  Global Data . . . . . . . . . . . . . . . . . . 4-1

4.1.1  Constants  . . . . . . . . . . . . . . . 4-1

4.1.2  TT-TABLE . . . . . . . . . . . . . . . . 4-1

4.1.3  Command Table  . . . . . . . . . . . . . 4-2

4.1.4  Reply Strings  . . . . . . . . . . . . . 4-3

4.1.5  User-id Strings  . . . . . . . . . . . . 4-4

4.1.6  Global Variables . . . . . . . . . . . . 4-4

4.2  Internal Errors . . . . . . . . . . . . . . . . 4-4

4.3  Log Messages  . . . . . . . . . . . . . . . . . 4-5

4.4  Key Table . . . . . . . . . . . . . . . . . . . 4-5

5.  Maintenance Instructions . . . . . . . . . . . . . . 5-1

5.1  Protected Terminals . . . . . . . . . . . . . . 5-1

5.1.1  USRSB  . . . . . . . . . . . . . . . . . 5-1

5.1.2  SCLIDAT  . . . . . . . . . . . . . . . . 5-2

5.2  Passwords . . . . . . . . . . . . . . . . . . . 5-2

5.3  Command Table . . . . . . . . . . . . . . . . . 5-3

5.4  Summary . . . . . . . . . . . . . . . . . . . . 5-3

5.4.1  Change Terminals . . . . . . . . . . . . 5-3

5.4.2  Change Passwords . . . . . . . . . . . . 5-3

5.4.3  Change Users . . . . . . . . . . . . . . 5-3

5.4.4  Change Messages  . . . . . . . . . . . . 5-4

5.5  Build Instructions  . . . . . . . . . . . . . . 5-4

5.5.1  SCLI . . . . . . . . . . . . . . . . . . 5-4

5.5.2  Documentation  . . . . . . . . . . . . . 5-5

5.5.3  BYE  . . . . . . . . . . . . . . . . . . 5-5

5.5.4  TTDRV  . . . . . . . . . . . . . . . . . 5-5

5.6  Initialization  . . . . . . . . . . . . . . . . 5-6

6.  Module Descriptions  . . . . . . . . . . . . . . . . 6-1

6.1  Global Data . . . . . . . . . . . . . . . . . . 6-1

6.2  CRYPT	  . . . . . . . . . . . . . . . . . . . . 6-1

6.3  DOMSG . . . . . . . . . . . . . . . . . . . . . 6-2

6.4  LOG . . . . . . . . . . . . . . . . . . . . . . 6-3

6.4.1  Notes  . . . . . . . . . . . . . . . . . 6-3

6.5  LOOKUP  . . . . . . . . . . . . . . . . . . . . 6-4

6.6  MUMBLE  . . . . . . . . . . . . . . . . . . . . 6-5

6.7  SLEN  . . . . . . . . . . . . . . . . . . . . . 6-6

6.8  TYPE  . . . . . . . . . . . . . . . . . . . . . 6-7

7.  Test Programs  . . . . . . . . . . . . . . . . . . . 7-1

7.1  TSTCRYPT  . . . . . . . . . . . . . . . . . . . 7-1

7.2  TSTLOG  . . . . . . . . . . . . . . . . . . . . 7-1

7.3  TSTLOOKUP . . . . . . . . . . . . . . . . . . . 7-1

7.4  TSTTYPE . . . . . . . . . . . . . . . . . . . . 7-2

7.5  TSTSLEN . . . . . . . . . . . . . . . . . . . . 7-2

8.  Terminal Driver Modifications  . . . . . . . . . . . 8-1

8.1  Overview  . . . . . . . . . . . . . . . . . . . 8-1

8.2  Modification Details  . . . . . . . . . . . . . 8-1

8.3  Source Modification . . . . . . . . . . . . . . 8-2

8.4  Patching TTDRV.TSK  . . . . . . . . . . . . . . 8-3